3GPP TSG-RAN5 Meeting #102	R5-241819

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0806 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SDR RMC for more FDD channel BW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current reference measurement channel for FDD SDR is limited to 20MHz BW |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 30MHz and 40MHz BW cases for FDD SDR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be incomplete for FDD SDR tests that need to test higher BW |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-241137r1 R5-241819 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## A.3.2_1	Reference measurement channels for Sustained downlink data rate performance requirements

### A.3.2_1.1	FDD

#### A.3.2_1.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2_1.1.1-1: Sustained Downlink Data Rate Reference Channel for FDD 15kHz SCS FR1 (64QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 10 | 15 | 52 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 20496 | 24 | 3 | 44928 | 17.422 |
|  | 20 | 15 | 106 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 42016 | 24 | 5 | 91584 | 35.714 |
|  | 30 | 15 | 160 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 62504 | 24 | 8 | 138240 | 53.128 |
|  | 40 | 15 | 216 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 83976 | 24 | 10 | 186624 | 71.38 |
|  | 10 | 15 | 52 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 29192 | 24 | 4 | 44928 | 24.813 |
|  | 20 | 15 | 106 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 59432 | 24 | 8 | 91584 | 50.517 |
|  | 30 | 15 | 160 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 90176 | 24 | 11 | 138240 | 76.65 |
|  | 40 | 15 | 216 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 120936 | 24 | 15 | 186624 | 102.796 |
|  | 10 | 15 | 52 | 13 | 23 | 64QAM | 0.7 | 1 | 1 | 31752 | 24 | 4 | 44928 | 26.989 |
|  | 20 | 15 | 106 | 13 | 23 | 64QAM | 0.7 | 1 | 1 | 64552 | 24 | 8 | 91584 | 54.869 |
|  | 30 | 15 | 160 | 13 | 23 | 64QAM | 0.7 | 1 | 1 | 96264 | 24 | 12 | 138240 | 81.824 |
|  | 40 | 15 | 216 | 13 | 23 | 64QAM | 0.7 | 1 | 1 | 131176 | 24 | 16 | 186624 | 111.5 |
|  | 10 | 15 | 52 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 39936 | 24 | 5 | 44928 | 33.946 |
|  | 20 | 15 | 106 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 81976 | 24 | 10 | 91584 | 69.68 |
|  | 30 | 15 | 160 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 122976 | 24 | 15 | 138240 | 104.53 |
|  | 40 | 15 | 216 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 163976 | 24 | 20 | 186624 | 139.38 |
|  | 10 | 15 | 52 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 40976 | 24 | 5 | 89856 | 34.83 |
|  | 20 | 15 | 106 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 83976 | 24 | 10 | 183168 | 71.38 |
|  | 30 | 15 | 160 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 125016 | 24 | 15 | 276480 | 106.264 |
|  | 40 | 15 | 216 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 167976 | 24 | 20 | 373248 | 142.78 |
|  | 10 | 15 | 52 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 58384 | 24 | 7 | 89856 | 49.626 |
|  | 20 | 15 | 106 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 118896 | 24 | 15 | 183168 | 101.062 |
|  | 30 | 15 | 160 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 180376 | 24 | 22 | 276480 | 153.32 |
|  | 40 | 15 | 216 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 241720 | 24 | 29 | 373248 | 205.462 |
|  | 10 | 15 | 52 | 13 | 23 | 64QAM | 0.7 | 2 | 1 | 63528 | 24 | 8 | 89856 | 53.999 |
|  | 20 | 15 | 106 | 13 | 23 | 64QAM | 0.7 | 2 | 1 | 129128 | 24 | 16 | 183168 | 109.759 |
|  | 30 | 15 | 160 | 13 | 23 | 64QAM | 0.7 | 2 | 1 | 192624 | 24 | 23 | 276480 | 163.73 |
|  | 40 | 15 | 216 | 13 | 23 | 64QAM | 0.7 | 2 | 1 | 262376 | 24 | 32 | 373248 | 223.02 |
|  | 10 | 15 | 52 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 79896 | 24 | 10 | 89856 | 67.912 |
|  | 20 | 15 | 106 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 163976 | 24 | 20 | 183168 | 139.38 |
|  | 30 | 15 | 160 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 245976 | 24 | 30 | 276480 | 209.08 |
|  | 40 | 15 | 216 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 327888 | 24 | 39 | 373248 | 278.705 |
|  | 10 | 15 | 52 | 13 | 19 | 64QAM | 0.5 | 4 | 1 | 83976 | 24 | 10 | 164736 | 71.38 |
|  | 20 | 15 | 106 | 13 | 19 | 64QAM | 0.5 | 4 | 1 | 167976 | 24 | 20 | 335808 | 142.78 |
|  | 30 | 15 | 160 | 13 | 19 | 64QAM | 0.5 | 4 | 1 | 254176 | 24 | 31 | 506880 | 216.05 |
|  | 40 | 15 | 216 | 13 | 19 | 64QAM | 0.5 | 4 | 1 | 344376 | 24 | 41 | 684288 | 292.72 |
|  | 10 | 15 | 52 | 13 | 23 | 64QAM | 0.7 | 4 | 1 | 114776 | 24 | 14 | 164736 | 97.56 |
|  | 20 | 15 | 106 | 13 | 23 | 64QAM | 0.7 | 4 | 1 | 237776 | 24 | 29 | 335808 | 202.11 |
|  | 30 | 15 | 160 | 13 | 23 | 64QAM | 0.7 | 4 | 1 | 352440 | 24 | 42 | 506880 | 299.574 |
|  | 40 | 15 | 216 | 13 | 23 | 64QAM | 0.7 | 4 | 1 | 483464 | 24 | 58 | 684288 | 410.944 |
|  | 10 | 15 | 52 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 125016 | 24 | 15 | 164736 | 106.264 |
|  | 20 | 15 | 106 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 254176 | 24 | 31 | 335808 | 216.05 |
|  | 30 | 15 | 160 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 385272 | 24 | 46 | 506880 | 327.481 |
|  | 40 | 15 | 216 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 516312 | 24 | 62 | 684288 | 438.865 |
|  | 10 | 15 | 52 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 147576 | 24 | 18 | 164736 | 125.44 |
|  | 20 | 15 | 106 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 295176 | 24 | 36 | 335808 | 250.9 |
|  | 30 | 15 | 160 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 450984 | 24 | 54 | 506880 | 383.336 |
|  | 40 | 15 | 216 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 606504 | 24 | 72 | 684288 | 515.528 |
| Note 1: Allocated full DL slots are with slot index i, if i is not in {0,10,11} for i = 0,1,...,19. So total number of allocated slots per 2 frames is 17. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is not enabled. MCS 18 and 19 are equivalent to MCS 11 and 12 in 256QAM table, respectively. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.1.1-2: Sustained Downlink Data Rate Reference Channel for FDD 15kHz SCS FR1 (256QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 10 | 15 | 52 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 39936 | 24 | 5 | 59904 | 33.946 |
|  | 20 | 15 | 106 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 81976 | 24 | 10 | 122112 | 69.68 |
|  | 30 | 15 | 160 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 122976 | 24 | 15 | 184320 | 104.53 |
|  | 40 | 15 | 216 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 163976 | 24 | 20 | 248832 | 139.38 |
|  | 10 | 15 | 52 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 42016 | 24 | 5 | 59904 | 35.714 |
|  | 20 | 15 | 106 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 83976 | 24 | 10 | 122112 | 71.38 |
|  | 30 | 15 | 160 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 127080 | 24 | 16 | 184320 | 108.018 |
|  | 40 | 15 | 216 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 172176 | 24 | 21 | 248832 | 146.35 |
|  | 10 | 15 | 52 | 13 | 26 | 256QAM | 0.9 | 1 | 1 | 53288 | 24 | 7 | 59904 | 45.295 |
|  | 20 | 15 | 106 | 13 | 26 | 256QAM | 0.9 | 1 | 1 | 108552 | 24 | 13 | 122112 | 92.269 |
|  | 30 | 15 | 160 | 13 | 26 | 256QAM | 0.9 | 1 | 1 | 163976 | 24 | 20 | 184320 | 139.38 |
|  | 40 | 15 | 216 | 13 | 26 | 256QAM | 0.9 | 1 | 1 | 221376 | 24 | 27 | 248832 | 188.17 |
|  | 10 | 15 | 52 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 79896 | 24 | 10 | 119808 | 67.912 |
|  | 20 | 15 | 106 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 163976 | 24 | 20 | 244224 | 139.38 |
|  | 30 | 15 | 160 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 245976 | 24 | 30 | 368640 | 209.08 |
|  | 40 | 15 | 216 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 327888 | 24 | 39 | 497664 | 278.705 |
|  | 10 | 15 | 52 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 83976 | 24 | 10 | 119808 | 71.38 |
|  | 20 | 15 | 106 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 167976 | 24 | 20 | 244224 | 142.78 |
|  | 25 | 15 | 133 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 213176 | 24 | 26 | 306432 | 181.2 |
|  | 30 | 15 | 160 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 254176 | 24 | 31 | 368640 | 216.05 |
|  | 40 | 15 | 216 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 344376 | 24 | 41 | 497664 | 292.72 |
|  | 10 | 15 | 52 | 13 | 26 | 256QAM | 0.9 | 2 | 1 | 106576 | 24 | 13 | 119808 | 90.59 |
|  | 20 | 15 | 106 | 13 | 26 | 256QAM | 0.9 | 2 | 1 | 217128 | 24 | 26 | 244224 | 184.559 |
|  | 30 | 15 | 160 | 13 | 26 | 256QAM | 0.9 | 2 | 1 | 327888 | 24 | 39 | 368640 | 278.705 |
|  | 40 | 15 | 216 | 13 | 26 | 256QAM | 0.9 | 2 | 1 | 442632 | 24 | 53 | 497664 | 376.237 |
|  | 10 | 15 | 52 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 159880 | 24 | 19 | 219648 | 135.898 |
|  | 20 | 15 | 106 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 327888 | 24 | 39 | 447744 | 278.705 |
|  | 30 | 15 | 160 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 500136 | 24 | 60 | 675840 | 425.116 |
|  | 40 | 15 | 216 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 671976 | 24 | 80 | 912384 | 571.18 |
|  | 10 | 15 | 52 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 172176 | 24 | 21 | 219648 | 146.35 |
|  | 20 | 15 | 106 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 352440 | 24 | 42 | 447744 | 299.574 |
|  | 25 | 15 | 133 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 434280 | 24 | 52 | 561792 | 369.138 |
|  | 30 | 15 | 160 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 524640 | 24 | 63 | 675840 | 445.944 |
|  | 40 | 15 | 216 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 704904 | 24 | 84 | 912384 | 599.168 |
|  | 10 | 15 | 52 | 13 | 26 | 256QAM | 0.9 | 4 | 1 | 196776 | 24 | 24 | 219648 | 167.26 |
|  | 20 | 15 | 106 | 13 | 26 | 256QAM | 0.9 | 4 | 1 | 401640 | 24 | 48 | 447744 | 341.394 |
|  | 30 | 15 | 160 | 13 | 26 | 256QAM | 0.9 | 4 | 1 | 606504 | 24 | 72 | 675840 | 515.528 |
|  | 40 | 15 | 216 | 13 | 26 | 256QAM | 0.9 | 4 | 1 | 819256 | 24 | 98 | 912384 | 696.368 |
| Note 1: Allocated full DL slots are with slot index i, if i is not in {0,10,11} for i = 0,1,...,19. So total number of allocated slots per 2 frames is 17. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is enabled. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.1.1-3: Sustained Downlink Data Rate Reference Channel for FDD 15kHz SCS FR1 (1024QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 10 | 15 | 52 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 58384 | 24 | 7 | 74880 | 49.626 |
|  | 20 | 15 | 106 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 120936 | 24 | 15 | 152640 | 102.796 |
|  | 30 | 15 | 160 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 180376 | 24 | 22 | 230400 | 153.32 |
|  | 40 | 15 | 216 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 245976 | 24 | 30 | 311040 | 209.08 |
|  | 10 | 15 | 52 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 62504 | 24 | 8 | 74880 | 53.128 |
|  | 20 | 15 | 106 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 127080 | 24 | 16 | 152640 | 108.018 |
|  | 30 | 15 | 160 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 192624 | 24 | 23 | 230400 | 163.73 |
|  | 40 | 15 | 216 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 258144 | 24 | 31 | 311040 | 219.422 |
|  | 10 | 15 | 52 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 118896 | 24 | 15 | 149760 | 101.062 |
|  | 20 | 15 | 106 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 241720 | 24 | 29 | 305280 | 205.462 |
|  | 30 | 15 | 160 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 360488 | 24 | 43 | 460800 | 306.415 |
|  | 40 | 15 | 216 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 491800 | 24 | 59 | 622080 | 418.03 |
| Note 1: Allocated full DL slots are with slot index i, if i is not in {0,10,11} for i = 0,1,...,19. So total number of allocated slots per 2 frames is 17. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 1024QAM is enabled. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.3.2_1.2	TDD

<< End of Changes >>