3GPP TSG-RAN5 Meeting #102	R5-241853

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5490 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to CBW for multiples CA combos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT, Sporton International |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some CA configuration are missing in clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added CA combos: - CA_30A-48A - CA_2A-2A-29A-66A - CA_2A-29A-66A-66A - CA_2A-2A-29A-66A-66A - CA_2A-2A-29A-30A-66A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA band combinations keep missing in test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised fro R5-240940 to correct typos “6” instead of “66” and “8-“ instead of “80” in Table 5.4.2A.1-2b. |  |  |  |  |  |  |  |  |

## {Start of changes}

### 5.4.2A	Channel bandwidth for CA

<Unchanged Sections Skipped>

#### 5.4.2A.1	Channel bandwidths per operating band for CA

<Unchanged Sections Skipped>

Table 5.4.2A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 3 |  | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_1A-1A-3A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.4.2A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |  |  |
| CA_29A-70A | - | 29 |  |  | Yes | Yes |  |  | 25 | 0 |
|  |  | 70 |  |  | Yes | Yes | Yes |  |  |  |
| CA_29A-70C | - | 29 |  |  | Yes | Yes |  |  | 35 | 0 |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |
| CA_30A-48A | CA_30A-48A | 30 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_30A-66A | CA_30A-66A | 30 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_30A-66A-66A | - | 30 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |  |  |
| CA_32A-43A | - | 32 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.4.2A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 5: For TDD inter-band Carrier Aggregation only non-simultaneous Rx/Tx uplink CA configurations can be supported by UE supporting corresponding DL CA configuration without simultaneous Rx/Tx. NOTE 6: Void NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 8: For the corresponding CA configuration, UE may not support PCell transmissions in this E-UTRA band. NOTE 9: 8Rx Requirements are applicable for this band configuration if UE supports 8Rx. |  |  |  |  |  |  |  |  |  |  |

Table 5.4.2A.1-2a: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (three bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A |  | CA_1A-3A CA_1A-5A6 CA_3A-5A | 1 |  |  |  |  | Yes |  | Yes | Yes | Yes | 50 | 0 |
|  |  |  | 3 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
|  |  |  | 5 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 1 |  |  |  |  | Yes |  | Yes |  |  | 40 | 1 |
|  |  |  | 3 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
|  |  |  | 5 |  |  |  |  | Yes |  | Yes |  |  |  |  |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66B |  | CA_2A-5A CA_5A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  | 5 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66C |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  |  | 5 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A-12A |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  | 7 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
|  |  |  | 12 |  |  |  |  | Yes |  | Yes |  |  |  |  |
| CA_2A-2A-7A-66A |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  |  | 7 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| CA_2A-2A-12B-66A |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| CA_2A-2A-29A-66A |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  | 29 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| CA_2A-2A-29A-66A-66A |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  |  | 29 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
|  |  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A-66A |  | CA_2A-13A CA_13A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  | 13 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| CA_2A-5A-12B |  | - | 2 |  |  |  |  | Yes |  | Yes | Yes | Yes | 45 | 0 |
|  |  |  | 5 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-29A-30A |  | - | 2 | See CA_2C Bandwidth Combination set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  |  | 29 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  |  |  |
| CA_2A-29A-66A |  | - | 2 |  |  |  |  | Yes |  | Yes | Yes | Yes | 50 | 0 |
|  |  |  | 29 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| CA_2A-29A-66A-66A |  | - | 2 |  |  |  |  | Yes |  | Yes | Yes | Yes | 70 | 0 |
|  |  |  | 29 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-30A-66A |  | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| CA_2A-30A-66A-66A |  | - | 2 |  |  |  |  | Yes |  | Yes | Yes | Yes | 70 | 0 |
|  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  |  |  |  |  |  |
| CA_2A-30A-66A |  | - | 2 |  |  |  |  | Yes |  | Yes | Yes | Yes | 50 | 0 |
|  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 66 |  |  |  |  | Yes |  | Yes | Yes | Yes |  |  |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66C-70C-71A |  | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 71 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.4.2A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 6: If the UE supports any uplink CA configuration for corresponding downlink CA configuration it shall support this uplink CA configuration. NOTE 7: UL carrier shall be supported in Band 3 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 8: UL carrier shall be supported in Band 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 9: UL carrier is only supported on Band 1 or Band 3 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 10: UL carrier is only supported on Band 1 or Band 42 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 11: UL carrier is only supported on Band 1 or Band 5 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 12: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 13: UL carrier shall be supported in Band 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 14: UL carrier shall be supported in Band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 15: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 16: UL carrier shall be supported in Band 1 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.4.2A.1-2b: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (four bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A-7A | CA_1A-3A CA_1A-5A6 CA_1A-7A CA_3A-5A CA_3A-7A CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-30A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-14A-30A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-29A-30A-66A |  | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.4.2A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-4A-5A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-5A-29A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |

## {End of changes}