3GPP TSG-RAN5 Meeting #102	R5-241858

# Athens, Greece, 26th Feb – 1st Mar 2024

Title:							MU discussion on FR2c

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

There is an action point to define MTSU of FR2 RF test cases for FR2c (n259). In the previous RAN5 meetings, we have agreed on MTSU/TT values for Frequency error [1][2], MOP (EIRP, TRP), REFSENS, EIS spherical coverage [3], and Tx/Rx spurious emissions up to 80 GHz [4]. This paper basically repeats the proposals of [3] for remaining test cases and provides further analysis for Transmit OFF power to proceed MU discussion on FR2c. Unless otherwise commented, observations and proposals in this paper are for PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Table 1 Summary of the current agreement on FR2c MU (NTC)

| Test case | MTSU | Relaxation |
| --- | --- | --- |
| MOP (EIRP) | 6.64 dB | - |
| MOP (TRP) | 6.16 dB | - |
| MOP (Spherical coverage) | No agreement | No agreement |
| MPR | No agreement | No agreement |
| Minimum output power | No agreement | No agreement |
| Transmit OFF power | No agreement | No agreement |
| Frequency error | 0.01 ppm | - |
| OBW | No agreement | No agreement |
| SEM | No agreement | No agreement |
| ACLR | No agreement | No agreement |
| REFSENS | 6.34 dB | - |
| EIS spherical coverage | 6.04 dB | - |
| ACS | No agreement | No agreement |
| In-band blocking | No agreement | No agreement |
| Tx Spurious |  |  |
| Up to 80 GHz | Same as FR2b | Same as FR2b |
| 80 GHz to 87 GHz | No agreement | No agreement |
| Rx Spurious |  |  |
| Up to 80 GHz | Same as FR2b | Same as FR2b |
| 80 GHz to 87 GHz | No agreement | No agreement |

# 2.	Discussion

## 2.1.	Testability

Except OBW, our analysis on testability of each test case which was not agreed in the last meeting has not changed from [3] and summarized in the tables below. Please refer to [5] for the detailed analysis.

Table 2 Summary of testability analysis for FR2c

| Test case | Testability |
| --- | --- |
| MOP (Spherical coverage) | No testability issues SNR = 5.9 dB |
| MPR | No testability issues SNR = 8.7 dB |
| Minimum output power | Relaxation of 1 dB larger than FR2b is required due to low UL power. SNR is the same as FR2b, when the required relaxation is applied. |
| SEM | No testability issues SNR is the same as FR2b. |
| ACLR | The red marked test points in Table 3 are not testable due to low UL power. SNR is the same as FR2b at the green marked test points in Table 2. |
| ACS | For case 1, the interferer level needs to be relaxed by 3.8 dB for CBW = 50 MHz and 6.8 dB for CBW = 100 MHz due to high DL power. Case 1 with CBW > 100 MHz and case 2 are not testable. |
| In-band blocking | Same as ACS (case 1) |
| Spurious emissions | The same testability as 80 GHz is applicable to the frequency range 80 GHz to 87 GHz. |

Table 3 Testability of ACLR test points for FR2c

| Test ID | Adjacent channel level [dBm] |  | SNR [dB] |  |  |  | Influence of noise [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz – 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 2 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 3 | -1.3 | -2.8 | 12.2 | 9.2 | 6.2 | 1.7 | 0.25 | 0.49 | 0.93 | 2.24 |
| 4 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 5 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 6 | -1.3 | -2.8 | 12.2 | 9.2 | 6.2 | 1.7 | 0.25 | 0.49 | 0.93 | 2.24 |
| 7 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 8 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 9 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 10 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 11 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 12 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 13 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| 14 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| 15 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |

For transmit OFF power, there is testability issue due to low UL power as same as FR2a. Considering that the core requirements of FR2a and FR2c are the same and the TE noise floor is degraded by 6.1 dB due to the higher frequency, the required relaxation for FR2c is 6.1 dB larger than that for FR2a. The concrete required relaxation values for FR2c are 27.5 dB for CBW = 50 MHz, 30.5 dB for CBW = 100 MHz, 33.5 dB for CBW = 200 MHz, and 36.5 dB for CBW = 400 MHz.

For OBW, we assume that the same MTSU as FR2b is achievable in FR2c with CBW up to 200 MHz, but CBW = 400 MHz is not testable due to low UL power. To imporve the testability of CBW = 400 MHz, we propose to reduce the measured frequency range to less than 1.5-times the OBW requirement.

According to Table 2 the above analysis on transmit OFF power and OBW, and the outcome of MU discussion, we propose the following relaxations/test points for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 1: Do not relax the test requirement of MOP (Spherical coverage) for FR2c.

Proposal 2: Do not relax the test requirement of MPR for FR2c.

Proposal 3: Relax the test requirement of Minimum output power for FR2c by [5.5] dB for CBW = 50 MHz, [8.5] dB for CBW = 100 MHz, [11.5] dB for CBW = 200 MHz, and [14.5] dB for CBW = 400 MHz.

Proposal 4: Relax the test requirement of Transmit OFF power for FR2c by 27.5 dB for CBW = 50 MHz, 30.5 dB for CBW = 100 MHz, 33.5 dB for CBW = 200 MHz, and 36.5 dB for CBW = 400 MHz.

Proposal 5: For FR2c OBW, do not relax the test requirement for CBW  200 MHz.

Proposal 6: For FR2c with CBW = 400 MHz, consider reducing the measured frequency range to less than 1.5-times the OBW requirement.

Proposal 7: Do not relax the test requirement of SEM for FR2c.

Proposal 8: For FR2c ACLR, do not relax the test requirement of the green marked test points in Table 3 and do not test others.

Proposal 9: For FR2c ACS (case 1) and In-band blocking, relax the test requirement by [3.8] dB for CBW = 50 MHz and [6.8] dB for CBW = 100 MHz, and do not test CBW > 100 MHz and ACS (case 2).

Proposal 10: Do not relax the test requirement of General Tx Spurious for FR2c.

Proposal 11: For Rx Spurious, apply the same relaxation as f = 80 GHz to the frequency range 80 GHz – 87 GHz.

## 2.2.	MU element

MU elements for spurious test cases

Following MU elements have specific values for spurious test cases up to f = 80 GHz. We assume that the same value as f = 80 GHz is applicable to the frequency range 80 GHz – 87 GHz.

- Quality of Quiet Zone

- Mismatch

- Uncertainty of the RF power measurement equipment

- Amplifier Uncertainties

- Uncertainty of the Network Analyzer

- Quality of the Quiet Zone for Calibration Process

Influence of noise

Based on the testability analysis in clause 2.1, influence of noise for FR2c is calculated as below.

Table 4 Influence of noise for FR2c

| Test case | Influence of noise |
| --- | --- |
| MOP (Spherical coverage) | 1.0 dB |
| MPR | 0.55 dB |
| Minimum output power | 1.0 dB (with relaxation) |
| Transmit OFF power | 1.0 dB (with relaxation) |
| SEM | 1.0 dB |
| ACLR | 1.0 dB (at testable test points) |
| Tx Spurious (80 GHz – 87 GHz) | 0.41 dB (Same as 80 GHz) |
| Rx Spurious (80 GHz – 87 GHz) | 0.64 dB (Same as 80 GHz) |

According to the above analysis and the outcome of MU discussion, we propose the following uncertainty values for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 12: For Quality of Quiet Zone, Mismatch, Uncertainty of the RF power measurement equipment, Amplifier Uncertainties, Uncertainty of the Network Analyzer, Quality of the Quiet Zone for Calibration Process, adopt the same uncertainty values as f = 80 GHz to the frequency range 80 GHz – 87 GHz in spurious test cases.

Proposal 13: For FR2c Influence of noise, adopt values in Table 4 without [ ] for MOP (Spherical coverage), SEM, and with [ ] for MPR, Minimum output power.

## 2.3.	MTSU and TT

Based on the above analysis in clause 2.1 and 2.2, total MU for each test case is calculated as below.

Table 5 Summary of total MU in each test case

| Test case | Total MU |
| --- | --- |
| MOP (Spherical coverage) | 6.84 dB |
| MPR | [6.89] dB |
| Minimum output power | [7.34] dB |
| Transmit OFF power | 6.86 dB |
| OBW |  |
| BW = 50 MHz | 0.4 % |
| BW = 100 MHz | 0.4 % |
| BW = 200 MHz | 1.3 % |
| BW = 400 MHz | TBD |
| SEM | 6.86 dB |
| ACLR | 7.61 dB |
| ACS (case 1) | 9.19 dB |
| In-band blocking | 9.19 dB |
| Tx Spurious (80 GHz to 87 GHz) | 7.72 dB |
| Rx Spurious (80 GHz to 87 GHz) | 7.95 dB |

Proposal 14: Adopt Table 5 for MTSU of MOP (Spherical coverage), MPR, Minimum output power, SEM for FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Using the same approach as FR2a and FR2b, test tolerance for FR2c is calculated as below.

Table 6 Summary of TT in each test case

| Test case | TT | Formula |
| --- | --- | --- |
| MOP (Spherical coverage) | 3.50 dB | TT = 0.60 x (MTSUIFF – 1.0) |
| MPR | [4.12] dB | TT = 0.65 x (MTSUIFF - 0.55) |
| Minimum output power |  | TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) -R |
| BW = 50 MHz | [1.39] dB | ΔSNRmr = 2.77 dB |
| BW = 100 MHz | [0.06] dB | ΔSNRmr = 4.44 dB |
| BW = 200 MHz | [0] dB | ΔSNRmr = 6.58 dB |
| BW = 400 MHz | [0] dB | ΔSNRmr = 9.07 dB |
| Transmit OFF power | 0 dB | - |
| OBW | 0 kHz | - |
| SEM | 4.46 dB | TT = 0.65 x MTSUIFF |
| ACLR |  | TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-1.0)) -R + TT due to metric change - R = 0 dB - TT due to metric change = 1.0 dB |
| BW = 50 MHz (Test ID 1-2, 4-5) | 5.61 dB | ΔSNRmr = 0.32 dB |
| BW = 50 MHz (Test ID 3, 6) | 5.55 dB | ΔSNRmr = 0.25 dB |
| BW = 50 MHz (Test ID 7-9) | 5.79 dB | ΔSNRmr = 0.49 dB |
| BW = 50 MHz (Test ID 13-15) | 5.85 dB | ΔSNRmr = 0.55 dB |
| BW = 100 MHz (Test ID 1-2, 4-5) | 5.91 dB | ΔSNRmr = 0.61 dB |
| BW = 100 MHz (Test ID 3, 6) | 5.79 dB | ΔSNRmr = 0.49 dB |
| BW = 100 MHz (Test ID 7-9) | 6.23 dB | ΔSNRmr = 0.93 dB |
| BW = 200 MHz (Test ID 3, 6) | 6.23 dB | ΔSNRmr = 0.93 dB |
| ACS (case 1) | 0 dB | - |
| In-band blocking | 0 dB | - |
| Tx Spurious | 0 dB | - |
| Rx Spurious | 0 dB | - |

Proposal 15: Adopt Table 6 for TT of MOP (Spherical coverage), MPR, Minimum output power, SEM for FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

# 3.	Conclusion

RAN5 is asked to endorse following proposals for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 1: Do not relax the test requirement of MOP (Spherical coverage) for FR2c.Proposal 2: Do not relax the test requirement of MPR for FR2c.

Proposal 3: Relax the test requirement of Minimum output power for FR2c by [5.5] dB for CBW = 50 MHz, [8.5] dB for CBW = 100 MHz, [11.5] dB for CBW = 200 MHz, and [14.5] dB for CBW = 400 MHz.

Proposal 4: Relax the test requirement of Transmit OFF power for FR2c by 27.5 dB for CBW = 50 MHz, 30.5 dB for CBW = 100 MHz, 33.5 dB for CBW = 200 MHz, and 36.5 dB for CBW = 400 MHz.

Proposal 5: For FR2c OBW, do not relax the test requirement for CBW  200 MHz.

Proposal 6: For FR2c with CBW = 400 MHz, consider reducing the measured frequency range to less than 1.5-times the OBW requirement.

Proposal 7: Do not relax the test requirement of SEM for FR2c.

Proposal 8: For FR2c ACLR, do not relax the test requirement of the green marked test points in Table 3 and do not test others.

Proposal 9: For FR2c ACS (case 1) and In-band blocking, relax the test requirement by [3.8] dB for CBW = 50 MHz and [6.8] dB for CBW = 100 MHz, and do not test CBW > 100 MHz and ACS (case 2).

Proposal 10: Do not relax the test requirement of General Tx Spurious for FR2c.

Proposal 11: For Rx Spurious, apply the same relaxation as f = 80 GHz to the frequency range 80 GHz – 87 GHz.

Proposal 12: For Quality of Quiet Zone, Mismatch, Uncertainty of the RF power measurement equipment, Amplifier Uncertainties, Uncertainty of the Network Analyzer, Quality of the Quiet Zone for Calibration Process, adopt the same uncertainty values as f = 80 GHz to the frequency range 80 GHz – 87 GHz in spurious test cases.

Proposal 13: For FR2c Influence of noise, adopt values in Table 4 without [ ] for MOP (Spherical coverage), SEM, and with [ ] for MPR, Minimum output power.

Proposal 14: Adopt Table 5 for MTSU of MOP (Spherical coverage), MPR, Minimum output power, SEM for FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Proposal 15: Adopt Table 6 for TT of MOP (Spherical coverage), MPR, Minimum output power, SEM for FR2c, PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

# 4.	References

[1] R5-233971, “Testability and MU analysis for FR2c”, Keysight Technologies UK Ltd, RAN5#100

[2] R5-235753, “MU discussion on FR2c”, Anritsu, RAN5#100

[3] R5-237724, “MU discussion on FR2c”, Anritsu, RAN5#101

[4] R5-237723, “Discussion on MU and TT analysis for FR2c”, Keysight Technologies UK Ltd, RAN5#101

[5] R5-232983, “MU discussion on FR2c”, Anritsu, RAN5#99