3GPP TSG-RAN5 Meeting #102	R5-241878

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2952 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex F for RedCap RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To provide TE uncertainties and test tolerance derivations for following RedCap RRM test cases: - SDT tests 16.2.1.1, 16.2.1.2; - RACH tests 16.3.2.2.2, 16.3.2.2.4, 16.3.2.2.6, 16.3.2.2.8; - Inter-RAT CGI w/ autonomous gap tests 16.6.5.3, 16.6.5.4; - SS-RSRP accuracy tests 16.7.1.1.1, 16.7.1.1.2, 16.7.1.2.1, 16.7.1.2.2 - SS-RSRQ accuracy tests 16.7.2.1, 16.7.2.2, 16.7.2.3.1, 16.7.2.3.2, 16.7.2.4.1, 16.7.2.4.2; - SS-SINR accuracy tests: 16.7.3.1, 16.7.3.2, 16.7.3.3.1, 16.7.3.3.2, 16.7.3.4.1, 16.7.3.4.2 - L1-RSRP accuracy tests: 16.7.4.1.1, 16.7.4.1.2, 16.7.4.2.1, 16.7.4.2.2, 16.7.4.3.1, 16.7.4.3.2, 16.7.4.4.1, 16.7.4.4.2. - Inter-RAT measurement accuracy tests 16.7.5.1, 16.7.5.2, 16.7.6.1, 16.7.6.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-240732r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table F.1.1.2-9: Maximum test system uncertainty for RRM requirements for SA FR1 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 16.1.1.1 NR SA FR1 Cell reselection for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.1.1.2 NR SA FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.1.1.3 NR SA FR1-FR1 Cell reselection for 1 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.1.1.4 NR SA FR1-FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.1.1.5 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 6.1.1.4 | Same as 6.1.1.4 |
| 16.1.1.6 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.4 | Same as 6.1.1.4 |
| 16.1.1.7 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 6.1.1.5 | Same as 6.1.1.5 |
| 16.1.1.8 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.5 | Same as 6.1.1.5 |
| 16.1.2.1 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.1.2.2 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.1.2.3 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.4 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.5 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.6 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.2.1.1 NR SA FR1 CG-SDT for 1Rx UE | Same as 6.2.1 | Same as 6.2.1 |
| 16.2.1.2 NR SA FR1 CG-SDT for 2Rx UE | Same as 6.2.1 | Same as 6.2.1 |
| 16.3.1.1 NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.2 NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.3 NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.4 NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.5 NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.6 NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.7 NR - E-UTRA handover for 1Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.1.8 NR - E-UTRA handover for 2Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.1.9 NR - E-UTRA handover with unknown target cell for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.1.10 NR - E-UTRA handover with unknown target cell for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.2.1.1 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.1.2 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.1.3 NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.3.2.1.4 NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.3.2.1.5 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.1.6 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.2.1 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.2 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.3 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.4 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.5 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.6 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.7 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.8 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.3.1 NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.3.2 NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.3.3 NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.2.3.4 NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.4.1.1 NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.1.2 NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.3.1 NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.4.3.2 NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.5.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.2 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.3 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.4 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.6 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.7 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.8 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.9 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.10 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.11 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.12 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.13 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.14 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.15 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.3 NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.4 NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.5 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.2.6 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.2.7 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.2.8 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.3.1.1 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.3.2.2 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.4.1 NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.5.4.2 NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.6.1.1 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.2 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.3 NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.4 NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.5 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.6 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.7 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.8 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.9 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.10 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.11 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.12 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.2.1 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.2 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.3 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.7 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.8 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.9 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.10 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.11 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.12 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.3 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.4 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.5 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.4.6 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.4.7 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.4.8 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.5.1 NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.5.2 NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.5.3 NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on Cell 1 frequency. Noc1 is the AWGN on Cell 2 frequency. Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN. Ês2 / Noc2 is the ratio of Cell 2 signal / AWGN. |
| 16.6.5.4 NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE | Same as 16.6.5.3 | Same as 16.6.5.3 |
| 16.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 |
| 16.7.1.2.1 NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.1.2.2 NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 |
| 16.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 16.7.2.2 NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 16.7.2.3.1 NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.2.3.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.2.4.1 NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.2.4.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.1 NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.3.2 NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.3.3.1 NR SA FR1-FR1 Inter-frequency absloute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.3.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.4.1 NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.4.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.2.1 NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.2.2 NR SA FR1 SSB based L1-RSRP relative measurement for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.3.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.3.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.4.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.4.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.5.1 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.5.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.6.1 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.6.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |

<End of modified section 1>

<Start of modified section 2>

Table F.1.3.2-7: Derivation of test requirements for NR SA FR1 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 16.1.1.1 NR SA FR1 Cell reselection for 1 Rx UE | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -3.11dB Ês2 / Noc: +2.79dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +2.79dB Ês2 / Noc: -3.11dB Qoffsets, n = 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB In signalling -2dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -3.11dB Ês2 / Noc: +2.79dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +2.79dB Ês2 / Noc: -3.11dB Qoffsets, n = -2dB |
| 16.1.1.2 NR SA FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.1.1.3 NR SA FR1-FR1 Cell reselection for 1 Rx UE | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -4dB 1.6dB 0.3dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -102dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 16.1.1.4 NR SA FR1-FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.1.1.5 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB -0.55dB During T2: 0dB -0.55dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +15.45dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +15.45dB Ês2 / Noc: +13dB |
| 16.1.1.6 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.4 | Same as 6.1.1.4 | Same as 6.1.1.4 |
| 16.1.1.7 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 16.1.1.8 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.5 | Same as 6.1.1.5 | Same as 6.1.1.5 |
| 16.1.2.1 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB Qrxlevmin of Cell 2: -141 | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.6dB During T2: 0dB 1.6dB -2dB 0.4dB In signalling: +1dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.6dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +15.6dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.6dB Qrxlevmin of Cell 2: -140 |
| 16.1.2.2 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX | Same as 6.1.2.1 | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.1.2.3 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14Db Qrxlevmin of Cell 1: 140 (137) | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB In signalling: +1dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB Qrxlevmin of Cell 1: 139 (136) |
| 16.1.2.4 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.5 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 16.1.2.5 | Same as 16.1.2.5 | Same as 16.1.2.5 |
| 16.1.2.6 R SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.2.1.1 NR SA FR1 CG-SDT for 1Rx UE | Test configuration 1, 2, 4: During T1 Noc: -100dBm/15kHz Es/Noc: 0dB During T2 Noc: -100dBm/15kHz Es/Noc: 8dB During T3 Noc: -100dBm/15kHz Es/Noc: 0dB During T4 Noc: -100dBm/15kHz Es/Noc: 18dB During T5 Noc: -100dBm/15kHz Es/Noc: 18dB During T6 Noc: -100dBm/15kHz Es/Noc: 0dB In Signaling cg-SDT-RSRP-ThresholdSSB: -110 dBm/SCS | During T1 0dB -0.55dB During T2 0dB 0.55dB During T3 0dB -0.55dB During T4 0dB 0dB During T5 0dB 0dB During T6 0dB -0.55dB In Signaling -3dB | During T1 Noc: -100dBm/15kHz Es/Noc: -0.55dB During T2 Noc: -100dBm/15kHz Es/Noc: 8.55dB During T3 Noc: -100dBm/15kHz Es/Noc: -0.55dB During T4 Noc: -100dBm/15kHz Es/Noc: 18dB During T5 Noc: -100dBm/15kHz Es/Noc: 18dB During T6 Noc: -100dBm/15kHz Es/Noc: -0.55dB In Signaling cg-SDT-RSRP-ThresholdSSB: -113 dBm/SCS |
|  | Test configuration 3: During T1 Noc: -100dBm/15kHz Es/Noc: 0dB During T2 Noc: -100dBm/15kHz Es/Noc: 8dB During T3 Noc: -100dBm/15kHz Es/Noc: 0dB During T4 Noc: -100dBm/15kHz Es/Noc: 18dB During T5 Noc: -100dBm/15kHz Es/Noc: 18dB During T6 Noc: -100dBm/15kHz Es/Noc: 0dB | During T1 -0.55dB -0.55dB During T2 -0.55dB 0.55dB During T3 -0.55dB -0.55dB During T4 -0.55dB 0dB During T5 -0.55dB 0dB During T6 -0.55dB -0.55dB | During T1 Noc: -100.55dBm/15kHz Es/Noc: -0.55dB During T2 Noc: -100.55dBm/15kHz Es/Noc: 8.55dB During T3 Noc: -100.55dBm/15kHz Es/Noc: -0.55dB During T4 Noc: -100.55dBm/15kHz Es/Noc: 18dB During T5 Noc: -100.55dBm/15kHz Es/Noc: 18dB During T6 Noc: -100.55dBm/15kHz Es/Noc: -0.55dB |
| 16.2.1.2 NR SA FR1 CG-SDT for 2Rx UE | Test configuration 1, 2, 4: During T1 Noc: -100dBm/15kHz Es/Noc: 0dB During T2 Noc: -100dBm/15kHz Es/Noc: 8dB During T3 Noc: -100dBm/15kHz Es/Noc: 0dB During T4 Noc: -100dBm/15kHz Es/Noc: 18dB During T5 Noc: -100dBm/15kHz Es/Noc: 18dB During T6 Noc: -100dBm/15kHz Es/Noc: 0dB | During T1 0dB 0dB During T2 0dB 0dB During T3 0dB 0dB During T4 0dB 0dB During T5 0dB 0dB During T6 0dB 0dB | During T1 Noc: -100dBm/15kHz Es/Noc: 0dB During T2 Noc: -100dBm/15kHz Es/Noc: 8dB During T3 Noc: -100dBm/15kHz Es/Noc: 0dB During T4 Noc: -100dBm/15kHz Es/Noc: 18dB During T5 Noc: -100dBm/15kHz Es/Noc: 18dB During T6 Noc: -100dBm/15kHz Es/Noc: 0dB |
|  | Test configuration 3: During T1 Noc: -100dBm/15kHz Es/Noc: 0dB During T2 Noc: -100dBm/15kHz Es/Noc: 8dB During T3 Noc: -100dBm/15kHz Es/Noc: 0dB During T4 Noc: -100dBm/15kHz Es/Noc: 18dB During T5 Noc: -100dBm/15kHz Es/Noc: 18dB During T6 Noc: -100dBm/15kHz Es/Noc: 0dB | During T1 -0.5dB 0dB During T2 -0.5dB 0dB During T3 -0.5dB 0dB During T4 -0.5dB 0dB During T5 -0.5dB 0dB During T6 -0.5dB 0dB | During T1 Noc: -100.5dBm/15kHz Es/Noc: 0dB During T2 Noc: -100.5dBm/15kHz Es/Noc: 8dB During T3 Noc: -100.5dBm/15kHz Es/Noc: 0dB During T4 Noc: -100.5dBm/15kHz Es/Noc: 18dB During T5 Noc: -100.5dBm/15kHz Es/Noc: 18dB During T6 Noc: -100.5dBm/15kHz Es/Noc: 0dB |
| 16.3.1.1 NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE | During T1 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 12 dB During T3 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 12 dB In signalling A3-Offset 0 dB | 0 dB 0 dB 0 dB 0 dB 0 dB -1.85dB 0 dB 0 dB -1.85dB -1.5dB | During T1 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 10.15 dB During T3 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 10.15 dB In signalling A3-Offset -1.5 dB |
| 16.3.1.2 NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.3 NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE | Same as 6.3.1.2 | Same as 6.3.1.2 | Same as 6.3.1.2 |
| 16.3.1.4 NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE | Same as 6.3.1.2 | Same as 6.3.1.2 | Same as 6.3.1.2 |
| 16.3.1.5 NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.6 NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.8 NR - E-UTRA handover for 2Rx UE | Same as 6.3.1.4 | Same as 6.3.1.4 | Same as 6.3.1.4 |
| 16.3.1.10 NR - E-UTRA handover with unknown target cell for 2 Rx UE | Same as 6.3.1.5 | Same as 6.3.1.5 | Same as 6.3.1.5 |
| 16.3.2.1.2 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 16.3.2.1.4 NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 16.3.2.1.6 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.2.2 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.4 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.6 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.8 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.3.2 NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 16.3.2.3.4 NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 |
| 16.4.1.1 NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.1.2 NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.3.1 NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.4.3.2 NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.5.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.2 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.3 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.4 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.6 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.7 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.8 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.9 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.10 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.11 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.12 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.13 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.14 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.15 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered |
| 16.5.2.3 NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB at T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB at T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB at T3-5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB at T1: 0dB -3.25dB q1 SSB at T2: 0dB -3.25dB q1 SSB at T3-5: 0dB +3.25dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB q1 SSB at T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -13.25dB q1 SSB at T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -13.25dB q1 SSB at T3-5: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.25dB |
| 16.5.2.4 NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered |
| 16.5.2.6 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered |
| 16.5.2.8 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered |
| 16.5.3.1.1 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.3.2.2 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.4.1 NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.5.4.2 NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.6.1.1 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.2 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.3 NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.4 NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.5 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.6 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.7 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.8 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.9 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.10 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.11 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.12 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.2.1 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 16.6.2.2 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.3 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.7 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.8 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.9 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.10 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.11 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.12 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE | During T1: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 3 dB Test Configuration 1,2,4 Reported SSB#1 absolute L1-RSRP values: -88.15 ± 13.03 dBm/SCS Reported SSB#0 differential L1-RSRP values: -6.5 ± 6.42 dB Test Configuration 3 Reported SSB#1 absolute L1-RSRP values: -85.15 ± 13.03 dBm/SCS Reported SSB#0 differential L1-RSRP values: -6.5 ± 6.42 dB | 0 dB 0 dB 0 dB 0 dB 0 dB 3.5 dB Via mapping | During T1: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 6.5 dB Test Configuration 1,2,4 Reported SSB#1 absolute L1-RSRP Range: RSRP_55 to RSRP_81 Reported SSB#0 differential L1-RSRP range: DIFFRSRP_0 to DIFFRSRP_6 Test Configuration 3 Reported SSB#1 absolute L1-RSRP Range: RSRP_58 to RSRP_84 Reported SSB#0 differential L1-RSRP range: DIFFRSRP_0 to DIFFRSRP_6 |
| 16.6.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.3 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 16.6.4.1 | Same as 16.6.4.1 | Same as 16.6.4.1 |
| 16.6.4.4 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.5 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.6 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.7 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.8 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.5.3 NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB | During T1 -0.55dB 0dB 0dB 0dB During T2 -0.55dB 0dB 0dB 0.55dB During T3 -0.55dB 0dB 0dB 0.55dB | During T1 Noc1: -106.55dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106.55dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7.55dB During T3 Noc1: -106.55dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7.55dB |
| 16.6.5.4 NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB | During T1 0dB 0dB 0dB 0dB During T2 0dB 0dB 0dB 0dB During T3 0dB 0dB 0dB 0dB | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB |
| 16.7.1.1.1 NR SA FR1 SS- | TEST CONFIGURATION 1, 2, 4 |  |  |
| RSRP absolute measurement accuracy for 1 Rx UE | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±5.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB Reported RSRP values: ±5.5dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_43 to RSRP_57 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_59 to RSRP_80 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0dB RSRP_35 to RSRP_50 RSRP_36 to RSRP_50 RSRP_36 to RSRP_51 RSRP_37 to RSRP_51 RSRP_37 to RSRP_52 RSRP_38 to RSRP_52 RSRP_38 to RSRP_53 RSRP_39 to RSRP_53 depending on operating band |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: N/A Test 2: Noc: -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB Reported RSRP values: ±5.5dB | Test 1: N/A Test 2: 0dB 0dB +0.4dB Via mapping Test 3: -1.2dB 0dB 0dB Via mapping | Test 1: N/A Test 2: Noc: -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_56 to RSRP_77 Test 3: Noc: -115.2dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0dB RSRP_37 to RSRP_51 RSRP_38 to RSRP_52 RSRP_38 to RSRP_52 RSRP_39 to RSRP_53 RSRP_39 to RSRP_53 RSRP_40 to RSRP_54 RSRP_40 to RSRP_54 RSRP_41 to RSRP_55 depending on operating band |
| 16.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | Test 1: Noc: Test configuration 1, 2, 4: -106dBm/15kHz Test configuration 3: N/A Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: Test configuration 1, 2, 4: -88dBm/15kHz Test configuration 3 -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB Reported re. SS-RSRP values: ±3dB | Test 1: 0 dB N/A 0 dB +1.0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -106 dBm/15kHz N/A Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| 16.7.1.2.1 NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.1.2.2 NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.2.1 NR SA FR1 SS- | TEST CONFIGURATION 1, 2, 4 |  |  |
| RSRQ measurement accuracy for 1 Rx UE | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_50 to RSRQ_64 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_44 to RSRQ_62 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB RSRQ_42 to RSRQ_61 |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_50 to RSRQ_64 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB RSRQ_42 to RSRQ_61 |
| 16.7.2.2 NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE | Same as 6.7.2.1 | Same as 6.7.2.1 | Same as 6.7.2.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.2.3.1 NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Test configuration 1,2,4: Test 1 Noc1: -80.18dBm/15kHz Noc2: -80.18dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 -1.5dB -1.5dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 |
| 16.7.2.3.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Test configuration 1,2,4: Test 1 Noc1: -80.18dBm/15kHz Noc2: -80.18dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±4.25dB Extreme condition, Reported RSRQ values: ±5.25dB Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±4.25dB Extreme condition, Reported RSRQ values: ±5.25dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±4.21dB Extreme condition, Reported RSRQ values: ±5.21dB | Test 1 -1.5dB -1.5dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-13 to RSRQ_x+4 Extreme condition, RSRQ_x-15 to RSRQ_x+6 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-13 to RSRQ_x+4 Extreme condition, RSRQ_x-15 to RSRQ_x+6 |
| 16.7.2.4.1 NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Test configuration 1,2,4: Same as 6.7.2.2.2 test configuration 1,2 | Same as 6.7.2.2.2 test configuration 1,2 | Same as 6.7.2.2.2 test configuration 1,2 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 |
| 16.7.2.4.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Test configuration 1,2,4: Same as 6.7.2.2.2 test configuration 1, 2 | Same as 6.7.2.2.2 test configuration 1, 2 | Same as 6.7.2.2.2 test configuration 1, 2 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-7 to RSRQ_x+7 Extreme condition, RSRQ_x-9 to RSRQ_x+9 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-7 to RSRQ_x+7 Extreme condition, RSRQ_x-9 to RSRQ_x+9 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-11 to RSRQ_x+2 Extreme condition, RSRQ_x-13 to RSRQ_x+4 |
| 16.7.3.1 NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Test configuration 1, 2, 4: Same as 6.7.3.1 test configuration 1, 2 | Same as 6.7.3.1 test configuration 1, 2 | Same as 6.7.3.1 test configuration 1, 2 |
|  | Test configuration 3: Test 1 Noc: -93dBm/15kHz Es1/Noc: 4.54dB Es2/Noc: 2.66dB Normal condition, Reported SINR values: ±4.40dB Extreme condition, Reported SINR values: ±4.90dB Test 2 Noc: -116dBm/15kHz + ΔBG_offset Es1/Noc: -4dB Es2/Noc: -4dB Normal condition, Reported SINR values: ±4.17dB Extreme condition, Reported SINR values: ±4.67dB | 0dB 0dB 0dB Via mapping Via mapping 0dB 0.5dB 0.5dB Via mapping Via mapping | Test 1 Noc: -93dBm/15kHz Es1/Noc: 4.54dB Es2/Noc: 2.66dB Normal condition, SINR_31 to SINR_49 Extreme condition, SINR_30 to SINR_50 Test 2 Noc: -116dBm/15kHz + ΔBG_offset Es1/Noc: -3.5dB Es2/Noc: -3.5dB Normal condition, SINR_28 to SINR_45 Extreme condition, SINR_27 to SINR_44 |
| 16.7.3.2 NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 16.7.3.1 | Same as 16.7.3.1 | Same as 16.7.3.1 |
| 16.7.3.3.1 NR SA FR1-FR1 Inter-frequency absloute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported SINR values: ±4.30dB Extreme condition, Reported SINR values: ±5.30dB Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, Reported SINR values: ±4.30dB Extreme condition, Reported SINR values: ±5.30dB Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -4dB Es2/Noc2: -4dB Normal condition, Reported SINR values: ±4.80dB Extreme condition, Reported SINR values: ±5.30dB | 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0.35dB 0.35dB Via mapping Via mapping | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, SINR_34 to SINR_52 Extreme condition, SINR_32 to SINR_54 Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, SINR_78 to SINR_95 Extreme condition, SINR_76 to SINR_97 Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -3.65dB Es2/Noc2: -3.65dB Normal condition, SINR_30 to SINR_49 Extreme condition, SINR_29 to SINR_50 |
| 16.7.3.3.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported SINR values: ±4.92dB Extreme condition, Reported SINR values: ±5.42dB Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, Reported SINR values: ±4.92dB Extreme condition, Reported SINR values: ±5.42dB Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -4dB Es2/Noc2: -4dB Normal condition, Reported SINR values: ±5.42dB Extreme condition, Reported SINR values: ±5.42dB | 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0.35dB 0.35dB Via mapping Via mapping | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, SINR_x-10 to SINR_x+10 Extreme condition, SINR_x-11 to SINR_x+11 Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, SINR_x-10 to SINR_x+10 Extreme condition, SINR_x-11 to SINR_x+11 Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -3.65dB Es2/Noc2: -3.65dB Normal condition, SINR_x-11 to SINR_x+11 Extreme condition, SINR_x-11 to SINR_x+11 |
| 16.7.3.4.1 NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.3.2.1 | Same as 6.7.3.2.1 | Same as 6.7.3.2.1 |
| 16.7.3.4.2 NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE | Same as 6.7.3.2.2 | Same as 6.7.3.2.2 | Same as 6.7.3.2.2 |
| 16.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement for 1 Rx UE | Test configuration 1, 2, 4: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, RSRP_59 to RSRP_85 Extreme condition, RSRP_56 to RSRP_88 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition: RSRP_27 to RSRP_46 RSRP_28 to RSRP_47 RSRP_28 to RSRP_47 RSRP_29 to RSRP_48 RSRP_29 to RSRP_48 RSRP_30 to RSRP_49 RSRP_30 to RSRP_49 RSRP_31 to RSRP_50 Depending on band groups Extreme condition: RSRP_23 to RSRP_51 RSRP_23 to RSRP_51 RSRP_24 to RSRP_52 RSRP_24 to RSRP_52 RSRP_25 to RSRP_53 RSRP_25 to RSRP_53 RSRP_26 to RSRP_54 RSRP_26 to RSRP_54 Depending on band groups |
|  | Test configuration 3: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, RSRP_62 to RSRP_88 Extreme condition, RSRP_59 to RSRP_91 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition: RSRP_30 to RSRP_49 RSRP_31 to RSRP_50 RSRP_31 to RSRP_50 RSRP_32 to RSRP_51 RSRP_32 to RSRP_51 RSRP_33 to RSRP_52 RSRP_33 to RSRP_52 RSRP_34 to RSRP_53 Depending on band groups Extreme condition: RSRP_26 to RSRP_54 RSRP_26 to RSRP_54 RSRP_27 to RSRP_55 RSRP_27 to RSRP_55 RSRP_28 to RSRP_56 RSRP_28 to RSRP_56 RSRP_29 to RSRP_57 RSRP_29 to RSRP_57 Depending on band groups |
| 16.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement for 1 Rx UE | Test configuration 1, 2, 4: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 |
|  | Test configuration 3: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 |
| 16.7.4.2.1 NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.2.2 NR SA FR1 SSB based L1-RSRP relative measurement for 2 Rx UE | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 16.7.4.3.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.3.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 1 Rx UE | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 16.7.4.4.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.4.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 16.7.5.1 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE | Test 1 Noc2: -91.65dBm/15kHz Es2/Noc2: 10dB Normal condition, Reported LTE RSRP values: ±10.53dB Extreme condition, Reported LTE RSRP values: ±13.53dB Test 2 Noc2: -117dBm/15kHz + ΔBG_offset Es2/Noc2: -4dB Normal condition, Reported LTE RSRP values: ±7.03dB Extreme condition, Reported LTE RSRP values: ±11.53dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Noc2: -91.65dBm/15kHz Es2/Noc2: 10dB Normal condition, RSRP_48 to RSRP_69 Extreme condition, RSRP_45 to RSRP_72 Noc2: -117dBm/15kHz + ΔBG_offset Es2/Noc2: -3.65dB Normal condition, RSRP_13 to RSRP_27 RSRP_13 to RSRP_27 RSRP_14 to RSRP_28 RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_16 to RSRP_30 Depending on band groups Extreme condition, RSRP_8 to RSRP_31 RSRP_9 to RSRP_32 RSRP_9 to RSRP_32 RSRP_10 to RSRP_33 RSRP_10 to RSRP_33 RSRP_11 to RSRP_34 RSRP_12 to RSRP_35 Depending on band groups |
| 16.7.5.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.6.1 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE | Test 1 Noc2: -83dBm/15kHz Es2/Noc2: -1.75dB Normal condition, Reported LTE RSRQ values: ±3.68dB Extreme condition, Reported LTE RSRQ values: ±5.18dB Test 2 Noc2: -104.7dBm/15kHz Es2/Noc2: -4dB Normal condition, Reported LTE RSRQ values: ±4.72dB Extreme condition, Reported LTE RSRQ values: ±5.22dB Test 3 Noc2: -119.5dBm/15kHz + ΔBG_offset Es2/Noc2: -4dB Normal condition, Reported LTE RSRQ values: ±4.72dB Extreme condition, Reported LTE RSRQ values: ±5.22dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Noc2: -83dBm/15kHz Es2/Noc2: -1.75dB Normal condition, RSRQ_3 to RSRQ_11 Extreme condition, RSRP_0 to RSRP_20 Noc2: -104.7dBm/15kHz Es2/Noc2: -3.65dB Normal condition, RSRQ_0 to RSRQ_17 Extreme condition, RSRP_0 to RSRP_18 Noc2: -119.5dBm/15kHz + ΔBG_offset Es2/Noc2: -3.65dB Normal condition, RSRQ_0 to RSRQ_17 Extreme condition, RSRP_0 to RSRP_18 |
| 16.7.6.2 NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE | Same as 6.7.6.1 | Same as 6.7.6.1 | Same as 6.7.6.1 |

<End of modified section 2>