3GPP TSG-RAN5 Meeting #102 	R5-241924

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1752 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to p-Max value in out of band test cases for FR1 inter-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In OOB test cases for FR1 inter-band EN-DC: - p-MaxUE-FR1-r15 is used for E-UTRA CC at RRC. - DPS does not affect the test points of OOB test cases, and the the value for not DPS UE can meet all these requirements. Therefore, the difference of p-Max value between DPS UE and not DPS UE is unnecessary. - The smallest possible difference between PCMAX_L,c for E-UTRA CC and PCMAX_L,c for NR CC is expected. For EN-DC PC2 UE, power classes other than PC2 are allowed for the power class of E-UTRA CC or NR CC. If one of CC is PC3 while the other is PC2, it causes 3 dB difference between PCMAX_L,c for E-UTRA CC and PCMAX_L,c for NR CC. To avoid it, it is appropriate to set both of p-NR-FR1 and p-MaxEUTRA-r15 as 23 for EN-DC PC2 UE and 20 for EN-DC PC3 UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In OOB test cases for FR1 inter-band EN-DC: - IE at E-UTRA CC was corrected from p-MaxUE-FR1-r15 to p-MaxEUTRA-r15. - p-Max value was defined independently of DPS. - p-Max value for PC2 UE was corrected to 23, and p-Max value for PC3 UE was corrected to 20. - Editorial corrections in 7.6B.3.3_1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL difference between E-UTRA CC and NR CC will depend on the power class of each CC, and also the absolute power of each CC will be different according to the DPS support status. It will deviate from the OOB test purpose and make the OOB to be more complicated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6B.3.3, 7.6B.3.3_1, 7.7B.3, 7.7B.3_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-241112 and the outcome of 3 revisions to this document. r1: Typo correction r2: Spaces were added before p-MaxEUTRA-r15. r3: “nonCriticalExtension SEQUENCE {“ was removed. rev1: Removed spaces before p-MaxEUTRA-r15. Invisible lines were corrected to solid lines. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.6B.3.3	Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs)

<<Unchanged sections skipped>>

7.6B.3.3.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 with the following exceptions.

Table 7.6B.3.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

Table 7.6B.3.3.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

<<Unchanged sections skipped>>

7.6B.3.3_1	Out-of-band blocking for EN-DC within FR1 (>2 CCs)

7.6B.3.3_1.1	Out-of-band blocking for EN-DC within FR1 (3 CCs)

<<Unchanged sections skipped>>

7.6B.3.3_1.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.6B.3.3_1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

Table 7.6B.3.3_1.1.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

7.6B.3.3_1.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.3.3_1.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.6B.3.3_1.1.5-1: Out-of-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [5]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

7.6B.3.3_1.2	Out-of-band blocking for EN-DC within FR1 (4 CCs)

<<Unchanged sections skipped>>

7.6B.3.3_1.2.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.6B.3.3_1.2.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

Table 7.6B.3.3_1.2.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

7.6B.3.3_1.2.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.6B.3.3_1.2.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.6B.3.3_1.2.5-1: Out-of-band blocking for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [5]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

<<Unchanged sections skipped>>

7.7B.3	Spurious Response for inter-band EN-DC within FR1 (2 CCs)

<<Unchanged sections skipped>>

7.7B.3.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 with the following exceptions.

Table 7.7B.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

Table 7.7B.3.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

<<Unchanged sections skipped>>

7.7B.3_1	Spurious Response for EN-DC within FR1 (>2 CCs)

7.7B.3_1.1	Spurious Response for EN-DC within FR1 (3 CCs)

<<Unchanged sections skipped>>

7.7B.3_1.1.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.7B.3_1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

Table 7.7B.3_1.1.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

7.7B.3_1.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.7B.3_1.1.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.7B.3_1.1.5-1: Spurious Response for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

7.7B.3_1.2	Spurious Response for EN-DC within FR1 (4 CCs)

<<Unchanged sections skipped>>

7.7B.3_1.2.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.7B.3_1.2.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

Table 7.7B.3_1.2.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 20 |  | Power Class 3 UE |
|  | 23 |  | Power Class 2 UE |

7.7B.3_1.2.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [10] Annex A.3 and TS 38.521-1 [8] Annex A.3 for E-UTRA CG and NR CG respectively with parameters specified in Table 7.7B.3_1.2.5-1 for the specified wanted signal mean power in the presence of interfering signals.

Table 7.7B.3_1.2.5-1: Spurious Response for intra-band contiguous EN-DC

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 in TS 38.101-1 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [5] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

<<End of change>>