3GPP TSG-RAN WG5 Meeting #102	R5-242001

Athens, Greece, 26th February – 3rd March 2024

Title:	Ephemeris definition for NR NTN RRM neighbour cells

Source:	Keysight Technologies UK Ltd, Thales

Agenda item:	5.3.10.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN4 has been working on the solutions for NR to support non-terrestrial networks (NTN) as part of the work item defined in [1] during Rel-17. One of this work item outcomes has been the definition of NR NTN RRM test cases defined in 3GPP TS 38.133[2] Annex A.14.

Some of these RRM test cases require the definition of satellite neighbour cells:

Table 1. NR NTN RRM test cases requiring a neighbour cell according to TS 38.133[2]

| TC ID# | TC Title |
| --- | --- |
| A.14.1.1 | Cell reselection to FR1 intra-frequency NR case |
| A.14.1.2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements |
| A.14.1.3 | Time-based measurement initiation to FR1 intra-frequency NR cell reselection |
| A.14.1.4 | Location-based cell measurement initiation to FR1 intra-frequency NR cell reselection |
| A.14.1.5 | Cell reselection to FR1 inter-frequency NR case |
| A.14.1.6 | Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] |
| A.14.1.7 | Time-based Cell measurement initiation to FR1 inter-frequency cell reselection |
| A.14.1.8 | Location-based measurement initiation to FR1 inter-frequency NR cell reselection |
| A.14.1.9 | Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion |
| A.14.1.10 | Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion |
| A.14.2.1.1 | Intra-frequency SAN Handover from FR1 to FR1 |
| A.14.2.1.2 | Inter-frequency SAN Handover from FR1 to FR1 |
| A.14.2.1.3 | Intra-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.4 | Inter-frequency SAN time-based conditional Handover from FR1 to FR1 |
| A.14.2.1.5 | Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.2.16 | Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 |
| A.14.2.2.1.1 | Intra-frequency RRC Re-establishment in FR1 |
| A.14.2.2.1.2 | Inter-frequency RRC Re-establishment in FR1 |
| A.14.2.2.3.1 | Redirection from NR in FR1 to NR in FR1 |
| A.14.4.2.5 | Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode |
| A.14.4.2.6 | Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode |
| A.14.5.1.1 | SA event triggered reporting tests without gap under non-DRX |
| A.14.5.1.2 | SA event triggered reporting tests without gap under DRX |
| A.14.5.1.3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.14.5.1.4 | SA event triggered reporting tests with single measurement gap under non-DRX for satellite access |
| A.14.5.1.5 | SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access |
| A.14.5.1.6 | SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access |
| A.14.5.2.1 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access |
| A.14.5.2.2 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access |
| A.14.5.2.3 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access |
| A.14.5.2.4 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access |
| A.14.5.2.6 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overlapping for satellite access |
| A.14.5.2.7 | Event triggered reporting test without gap under non-DRX |
| A.14.5.2.8 | Event triggered reporting tests without gap under DRX |
| A.14.6.1.1 | SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell (SS-RSRP) |
| A.14.6.1.2 | SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell (SS-RSRP) |
| A.14.6.2.1 | SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access (SS-RSRQ) |
| A.14.6.2.2 | SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access (SS-RSRQ) |
| A.14.6.3.1 | SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell (SS-SINR) |
| A.14.6.3.2 | SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell |

To be noticed that those test cases in bold require 2 neighbour cells (3 cells in total).

Additionally, RAN5 is working in the corresponding work item [3] to define conformance test cases for NR NTN.

As part of this work, RAN5 defined the ephemeris of the serving cell for all type of test cases in [4]. At that point, ephemeris for GSO (7º inclination) and NGSO were defined.

Additionally, the definition of the ephemeris for the neighbour cell required in RRM test cases listed in Table 1 is still pending according to the latest work plan [5].

This document analyses the NR NTN RRM scenarios where neighbour cells are required and makes proposals for the definition of their ephemeris.

# 2. 	Discussion

## 2.1 Scenarios

Analysing the list of test cases included in Table 1 and its content in 3GPP TS 38.133 [2], it can be seen that:

- For NR NTN RRM test cases requiring neighbour cells:

  - All test cases in table 1 are applicable to both GSO and NGSO satellites.

  - Either one or two neighbour cells are required depending on the test case (test cases in bold in Table 1 required 2 neighbour cells while others only one).

  - Neighbour cell is defined as both intra-frequency or inter-frequency.

Observation 1: According to 3GPP TS 38.133 [2], for NR NTN RRM test cases requiring neighbour cells, either one or two neighbour cells either intra-frequency or inter-frequency are required for both GSO and NGSO satellites for Rel-17 UEs.

As a difference compared to IoT NTN, RAN2 has defined corresponding information elements to introduce satellite assistance information of neighbour cells in system information block 19 in Rel-17 NR NTN [6]:

Proposal 1: Define ephemeris for intra-frequency/inter-frequency neighbour cell(s) for NR NTN RRM test cases requiring neighbour cells for both GSO and NGSO satellites scenarios.

## 2.2 RAN4 assumptions for neighbour cells

For NR NTN RRM test cases, RAN4 agreed in [7] as part of general aspects of this work item (Issue 1-9-1) that RRM requirements apply provided that serving cell and neighbour satellites on the same layer are of same satellite type (LEO or GEO):

Regarding the altitude for serving and neighbour cells for NGSO satellite scenarios, it was previously agreed in [8] for NR NTN RRM Intra-NTN for both RRC Connected and Idle/Inactive modes and (new) SMTC and Measurement Gap based requirements, that same altitude will be considered for both serving and neighbour cells in this case.

Keysight’s understanding is that when leveraging satellites of the same type, same altitude for NGSO satellites is also leveraged.

Proposal 2: When defining ephemeris for neighbour cell(s) for NR NTN, assume that neighbour cell(s) satellite type is same as serving cell, including its altitude.

Another important aspect that has been analysed is if RAN4 discussed whether both serving and neighbour cells are on the same satellite or in different satellites. As per [8], RAN4 considered both scenarios, i.e. serving and neighbour cells within a satellite or belonging to different satellites.

Observation 2: RAN4 requirements/test cases considered both scenarios, i.e. serving and neighbour cell(s) within a satellite or belonging to different satellites.

## 2.3 Criteria to define ephemeris for neighbour cell(s) in both GSO and NGSO satellites scenarios

### 2.3.1 Same vs different satellites and Doppler for serving and neighbour cells in NTN

First criterion that RAN5 needs to decide upon is whether serving and neighbour cells for testing will be located in same or different satellites.

Test cases will have a wider coverage if UEs are required to handle 2/3 different satellites (with different Doppler and delays) instead of only one satellite, while maximizing the Doppler difference between satellites.

As Rel-17 is only covering the testing of static satellite conditions, the simplest approach could be evaluated based on the following assumptions:

- All required cells satellite follow the same orbital plane but with a time shift (as shown in Figures 1, 2 and 3).

- The ephemeris of each satellite is chosen such that Doppler difference is maximized (as shown in figures 1, 2 and 3).

- In GSO case, equivalent selection can be done in case totally different elevation angles for serving and neighbour cells are used.

Figure 1. Doppler for a GSO satellite orbit with 55 UE latitude and 121.56 UE longitude

Figure 2. Doppler for a NGSO- LEO600km satellite orbit with 25.08 UE latitude and 121.56 UE longitude

Figure 3. Doppler for a NGSO-LEO1200km satellite with 25.08 UE latitude and 121.56 UE longitude

For the GSO case, figure 1 shows minimal difference in the elevation angle perceived by the UE for the serving and neighbour cells. This is not the case for NGSO satellites, where significative differences can be observed in the elevation angles for serving and neighbour cells even using the same satellite orbit with a time shift between satellites.

Hence, in order to have a wider test coverage, it is proposed to use different satellites orbits for serving and neighbour cells in the case of GSO satellites and maximize the Doppler difference between satellites while, for NGSO satellites, all satellites in the same orbital plane can provide good test coverage

Proposal 3: For GSO satellites, use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells.

Proposal 4: For NGSO satellites, use same satellite orbit for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells.

### 2.3.2 Elevation angle

According to 3GPP TS 38.133 [2], Annex B.5:

As it is not specified that highlighted statement applies only to serving cell, the assumption is that the minimum elevation angle relative to the UE position shall be equal or higher than 30 degrees during the entire test time for both serving and neighbour cells.

Observation 3: Minimum elevation angle shall be 30 degrees for both serving and neighbour cells in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells.

For the GSO case, figure 1 shows minimal difference in the elevation angle perceived by the UE for the serving and neighbour cells when all satellites follow exactly the same orbital plane.

Figure 4 shows the comparison in terms of delay and Doppler of 3 GSO satellites orbits with same inclination for the same UE location:

a) Satellite longitude equal to UE longitude for serving cell,

b) +20º shifted satellite longitude regarding UE longitude for neighbour cell

c) -20º shifted satellite longitude regarding UE longitude for neighbour cell.

Red crosses indicate GSO satellite configuration proposed.

| Serving cell | Serving cell |
| --- | --- |
| (a) |  |
| Neighbour cell 1 | Neighbour cell 1 |
| (b) |  |
| Neighbour cell 2 | Neighbour cell |
| (c) |  |

Figure 4. Doppler for 3 GSO satellite orbits (7º inclination) with 55 UE latitude and 121.56 UE longitude:

a) Satellite longitude equal to UE longitude for serving cell,

b) +20º shifted satellite longitude regarding UE longitude for neighbour cell

c) -20º shifted satellite longitude regarding UE longitude for neighbour cell.

Red crosses indicate GEO satellite configuration proposed.

Hence, in order to have a wider test coverage in the case of GSO satellites, it is proposed to use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between satellites.

Proposal 3: For GSO satellites, use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells (as shown in Figure 4).

Proposal 4: For NGSO satellites, use same satellite orbit for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells (as shown in Figures 2 and 3).

# 3. 	Conclusion

This document analyses the NR NTN RRM scenarios where neighbour cells are required and makes proposals for the definition of their ephemeris.

The following observations and proposals are made:

Observation 1: According to 3GPP TS 38.133 [2], for NR NTN RRM test cases requiring neighbour cells, either one or two neighbour cells either intra-frequency or inter-frequency are required for both GSO and NGSO satellites for Rel-17 UEs.

Proposal 1: Define ephemeris for intra-frequency/inter-frequency neighbour cell(s) for NR NTN RRM test cases requiring neighbour cells for both GSO and NGSO satellites scenarios.

Proposal 2: When defining ephemeris for neighbour cell(s) for NR NTN, assume that neighbour cell(s) satellite type is same as serving cell, including its altitude.

Observation 2: RAN4 requirements/test cases considered both scenarios, i.e. serving and neighbour cell(s) within a satellite or belonging to different satellites.

Proposal 3: For GSO satellites, use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells.

Proposal 4: For NGSO satellites, use same satellite orbit for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells.

These proposals are submitted for approval in [9].

# 4. 	References

- RP-221169, “WID on Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#96.

- 3GPP TS 38.133 v17.12.0, “NR; Requirements for support of radio resource management”.

- RP-233929, “WID on UE Conformance – Solutions for NR to support non-terrestrial networks (NTN) plus CT aspects”, Qualcomm Incorporated, RAN#102.

- R5-237863, “Common test environment for NR NTN RF/RRM testing”, Keysight UK Ltd, Thales, Qualcomm.

- R5-237269, “WP UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN)”, WP UE Conformance Test Aspects - Solutions for NR to support non-terrestrial networks (NTN), RAN5#101.

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”

- R4-2210610, “WF on NR NTN RRM requirements”, Qualcomm Incorporated, RAN4#103-e.

- R4-2103682, “WF on NTN RRM measurement requirements”, Qualcomm Incorporated, RAN4#98-e.

- R5-241393, “Updates to Test environment for NR NTN RRM testing”, Keysight UK Ltd, RAN5#102.