3GPP TSG-RAN5 Meeting #102 	R5-242022

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3087 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to message exceptions for Performance test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Performance test cases message exception clean-up: - Performance message exception for searchSpaceZero is used to avoid SSB and CSS conflict. Currently, this message exception is only specified for PDCCH-ConfigCommon (Table 5.4.2.0-4). However, searchSpaceZero is only configured under PDCCH-ConfigCommon of initial BWP for NSA. For SA, searchSpaceZero is configured under PDCCH-ConfigSIB1 of MIB. -> So, searchSpaceZero message exception is missing for Perf SA. - Table 5.4.2.0-7: PDCCH Search Space includes common and ue-Spacific searchSpaceType, but there are no specified conditions. -> So, the condition of searchSpaceType setting in Table 5.4.2.0-7 is required. - Table 5.4.2.0-20: ZP-CSI-RS-Resource includes setting for qcl-InfoPeriodicCSI-RS, but there is no such IE for ZP-CSI-RS-Resource as per TS 38.331. - CSI reporting is not configured for Demod TCs. However, Table 5.4.2.1-2/Table 5.4.2.2-2: CSI-MeasConfig does not specify setting for csi-ReportConfigToAddModList and related parameters. And, if we refer to its derivation path Table 4.6.3-38: CSI-MeasConfig, csi-ReportConfigToAddModList should be configured, which is unnecessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Message exception for searchSpaceZero was separated such that both PDCCH-ConfigCommon and PDCCH-ConfigSIB1 can refer to it. - Removed searchSpaceZero setting in Table 5.4.2.0-4: PDCCH-ConfigCommon - Created Table 5.4.2.0-42: SearchSpaceZero - Clarified condition of searchSpaceType setting in Table 5.4.2.0-7: PDCCH Search Space. - Removed not applicable IE qcl-InfoPeriodicCSI-RS in Table 5.4.2.0-20: ZP-CSI-RS-Resource. - Added settings for csi-ReportConfigToAddModList, reportTriggerSize, and aperiodicTriggerStateList (=Not present) in Table 5.4.2.1-2/Table 5.4.2.2-2: CSI-MeasConfig. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0, 5.4.2.1, 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-241095 and the outcome of 2 revisions to this document. r1: Condition of searchSpaceType was clarified in Table 5.4.2.0-7 instead of removing searchSpaceType. r2: Definition of condition ‘USS’ was added under Table 5.4.2.0-7, and ‘common SEQUENCE’ was removed from searchSpaceType of Table 5.4.2.0-7. rev 1: Removal of ‘}’ from Table 5.4.2.0-7 was reversed. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

5.4.2.0	Parameters common to all Demod and CSI tests

<<Unchanged sections skipped>>

PDCCH-ConfigCommon

Table 5.4.2.0-4: PDCCH-ConfigCommon

| Derivation Path: Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon::= SEQUENCE { |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry | 1 search space for both SA and NSA |  |
| SearchSpace [1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |

<<Unchanged sections skipped>>

Table 5.4.2.0-7: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Starting symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| USS | UE-Specific SearchSpace |

<<Unchanged sections skipped>>

ZP-CSI-RS-Resource

Table 5.4.2.0-20: ZP-CSI-RS-Resource

| Derivation Path: Table 4.6.3-204 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  | DEMOD_FR1 DEMOD_FR2 |
| zp-CSI-RS-ResourceId | ZP-CSI-RS-ResourceId |  |  |
| resourceMapping | ZP CSI-RS-ResourceMapping |  |  |
| periodicityAndOffset | ZP CSI-ResourcePeriodicityAndOffset |  |  |
| } |  |  |  |

<<Unchanged sections skipped>>

PUCCH-Config

Table 5.4.2.0-41: PUCCH-Config

| Derivation Path: Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 4 entries |  | FR1.15-1 FR1.30-2 FR2.120-1 FR2.120-1A |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 6 | entry 4 |  |
| } |  |  |  |
| } |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 7 entries |  | FR1.30-1 FR1.30-1A FR1.30-4 |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 5 | entry 4 |  |
| INTEGER[5] | 6 | entry 5 |  |
| INTEGER[6] | 7 | entry 6 |  |
| INTEGER[7] | 8 | entry 7 |  |
| } |  |  |  |
| } |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 4 entries |  | FR1.30-3 |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 5 | entry 4 |  |
| } |  |  |  |
| } |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 2 entries |  | FR1.30-5 FR1.30-6 |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 3 entries |  | FR2.60-1 FR2.120-2 |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 5 | entry 3 |  |
| } |  |  |  |
| } |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 2 entries |  | FR2.120-1_CSI |
| INTEGER[1] | 3 | entry 1 |  |
| INTEGER[2] | 6 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 3 entries |  | FR2.120-2_CSI |
| INTEGER[1] | 6 | entry 1 |  |
| INTEGER[2] | 7 | entry 2 |  |
| INTEGER[3] | 11 | entry 3 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1.15-1 | TDD UL-DL pattern FR1.15-1 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-1 | TDD UL-DL pattern FR1.30-1 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-2 | TDD UL-DL pattern FR1.30-2 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-3 | TDD UL-DL pattern FR1.30-3 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-4 | TDD UL-DL pattern FR1.30-4 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-5 | TDD UL-DL pattern FR1.30-5 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-6 | TDD UL-DL pattern FR1.30-6 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR2.60-1 | TDD UL-DL pattern FR2.60-1 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-1 | TDD UL-DL pattern FR2.120-1 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-2 | TDD UL-DL pattern FR2.120-2 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-1_CSI | TDD UL-DL pattern FR2.120-1 is used for CSI reporting test cases. Ref Table 8.1.2-1 of TS 38.521-4 |
| FR2.120-2_CSI | TDD UL-DL pattern FR2.120-2 is used for CSI reporting test cases. Ref Table 8.1.2-1 of TS 38.521-4 |

SearchSpaceZero

Table 5.4.2.0-42: SearchSpaceZero

| Derivation Path: Table 4.6.3-164 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| searchSpaceZero | 4 |  | FDD FR1.15-1 FR1.30-1 FR1.30-2 FR1.30-3 FR1.30-4 FR2.60-1 FR2.120-1 FR2.120-2 |
|  | 2 |  | FR1.30-5 FR1.30-6 |

5.4.2.1	Message contents for PDSCH Demodulation requirements

<<Unchanged sections skipped>>

CSI-MeasConfig

Table 5.4.2.1-2: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | dependent on test condition |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | CSI-RS-Resource for beam refinement | entry 5 CSI-RS resource 6 | DEMOD_FR2 |
| NZP-CSI-RS-Resource[7] | CSI-RS-Resource for beam refinement | entry 5 CSI-RS resource 7 | DEMOD_FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | n entries | dependent on test condition |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| NZP-CSI-RS-ResourceSet[3] | CSI-RS-ResourceSet for beam refinement | entry 3 | DEMOD_FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | n entries | dependent on test condition |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for beam refinement | entry 3 | DEMOD_FR2 |
| } |  |  |  |
| csi-ReportConfigToAddModList | Not present |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList | Not present |  |  |
| } |  |  |  |

<<Unchanged sections skipped>>

5.4.2.2	Message contents for PDCCH Demodulation requirements

<<Unchanged sections skipped>>

CSI-MeasConfig

Table 5.4.2.2-2: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | Dependent on test condition |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for beam management (5) | entry 4 CSI-RS resource 5 | PDCCH_FR2 |
| NZP-CSI-RS-Resource[6] | NZP-CSI-RS-Resource for beam management (6) | entry 4 CSI-RS resource 6 | PDCCH_FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for beam management | entry 1 | PDCCH_FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for beam management | entry 1 | PDCCH_FR2 |
| } |  |  |  |
| csi-ReportConfigToAddModList | Not present |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList | Not present |  |  |
| } |  |  |  |

<<End of change>>