3GPP TSG-RAN WG5 Meeting #102	R5-242024

Athens, Greece, 26th February – 3rd March 2024

Title:	FR2 UL Gaps measurement uncertainties and test tolerances

Source:	Keysight Technologies UK Ltd, Apple Inc

Agenda item:	5.3.19.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN5 has been working on the definition of an FR2 RF test case “6.2.5 UE Maximum Output Power - EIRP with UL Gaps” as part of the work item defined in [1] during Rel-17 to be added to 38.521-2 [2], according to the Work plan in [3] through documents in [4-8].

Test tolerances for this test case have been initially drafted in [7] as shown below while measurement uncertainties are still undefined:

This document makes a proposal to define measurement uncertainties for FR2 RF test case “6.2.5 UE Maximum Output Power - EIRP with UL Gaps” and to complete test tolerances for this test case.

# 2. 	Discussion

## 2.1 Measurement uncertainties

To define measurement uncertainties for UE Maximum Output Power – EIRP with UL Gaps, an analysis of its minimum requirements is required:

It can be seen that final verdict is composed of 2 metrics:

- A relative power measurement: PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF

- A reported value: reported P-MPRf,c when UL gap for Tx power management is configured and activated or P bit in PHR when UL gap for Tx power management is not configured and activated if PHR is configured.

For the reported values, there is no need to define any measurement uncertainty as test system will only read the value reported by the UE.

For the relative power measurement, uncertainties need to be analysed for both sides on the inequality defining such requirement:

- Relative power measurement in the left side of the inequality:

In this case,  measurement uncertainty for Relative power control tolerance for EIRP could be used as a baseline [9]:

When this uncertainty was defined initially ([10]), the only measurement uncertainty factor considered was the uncertainty of the RF relative power measurement equipment. However, in a later revision through discussion paper in [11], it was agreed to add 2 new MU contributors:

- One associated to the compression of the amplifier experienced during RB change (which could lead up to 24 dB considering the UE power tolerance for power values in the lower range).

- One associated to the frequency response of the test equipment also related to RB change.

In UE Maximum Output Power – EIRP with UL Gaps, there power step is not caused by RB change but the expected power step in the relative power measurement will have a minimum of 3 dB, we should consider the factor associated to the frequency response as negligible but the one associated to the compression of the amplifier experience during the power step (in this case caused not by the RB change but for the UL gap instead) shall be kept.

- Absolute power measurement in the right side of the inequality:

According to minimum requirement, EIRPmeas_peak is the measured UE peak EIRP with zero MPR/A-MPR/P-MPR as specified in clause 6.2.1 for the corresponding power class. Hence, measurement uncertainty for this measurement could be leveraged from the peak EIRP measurement in the maximum output power test case.

Proposal 1a: Define measurement uncertainties for UE Maximum Output Power – EIRP with UL Gaps (PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF metric) with 3 MU contributors:

- - uncertainty of the RF relative power measurement equipment, leveraging its value from Relative power control tolerance for EIRP test case.

- Amplifier uncertainties, leveraging its value from Relative power control tolerance for EIRP test case.

- Influence of noise

Proposal 1b: Define measurement uncertainties for EIRP meas_peak as per EIRP peak measurement uncertainty in the UE Maximum Output Power test.

## 2.2 Test tolerances

Test tolerances for UE Maximum Output Power – EIRP with UL Gaps have been recently introduced as in [7]:

The rationale behind the proposed test tolerances figures is the following: for the (P_gap_on) – (P_gap_Off), the Test system uplink relative power TT is specified in Table F.3.2-1 -> 6.3.4.3 can be used. Considering that MU factor affecting relative power control test case related to frequency response is not defined but applicable in the UL gaps test case, square brackets could be removed from the first test tolerance defined in table 6.2.5.5-2..

Proposal 2a: Remove square brackets from test tolerance term associated to PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF. in TS 38.521-2 Table 6.2.5.5-2 for PC3 and Nominal temperature conditions.

Regarding the second test tolerance term defined in TS 38.521-2 Table 6.2.5.5-2, as it is related to the right side of the inequality in the requirement, it should be clearly specified that it relates to EIRPmeas_peak and its value could be leveraged from the test tolerances applicable to EIRPpeak in maximum output power test.

Proposal 2b: Redefine the second test tolerance term in TS 38.521-2 as EIRPmeas_peak and leverage its value from the EIRP peak measurement in Maximum output power test cases.

As P-MPR measurement is not measured by the test system but reported by the UE instead and no accuracy requirements have been defined for P-MPR reporting in 3GPP TS 38.133 [13], there is no need to define test tolerances for P-MPR metric in UE Maximum Output Power – EIRP with UL Gaps test case.

Proposal 3: Remove the place holder to define test tolerance for P-MPR in UE Maximum Output Power – EIRP with UL Gaps test case.

# 3. 	Conclusion

This document makes a proposal to define measurement uncertainties for FR2 RF test case “6.2.5 UE Maximum Output Power - EIRP with UL Gaps” and to complete test tolerances for this test case.

The following proposals are made:

Proposal 1a: Define measurement uncertainties for UE Maximum Output Power – EIRP with UL Gaps (PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF metric) with 3 MU contributors:

- - uncertainty of the RF relative power measurement equipment, leveraging its value from Relative power control tolerance for EIRP test case.

- Amplifier uncertainties, leveraging its value from Relative power control tolerance for EIRP test case.

- Influence of noise

Proposal 1b: Define measurement uncertainties for EIRP meas_peak as per EIRP peak measurement uncertainty in the UE Maximum Output Power test.

Proposal 2a: Remove square brackets from test tolerance term associated to PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF. in TS 38.521-2 Table 6.2.5.5-2 for PC3 and Nominal temperature conditions.

Proposal 2b: Redefine the second test tolerance term in TS 38.521-2 as EIRPmeas_peak and leverage its value from the EIRP peak measurement in Maximum output power test cases.

Proposal 2b: Redefine the second test tolerance term in TS 38.521-2 as EIRPmeas_peak and leverage its value from the EIRP peak measurement in Maximum output power test cases.

Proposal 3: Remove the place holder to define test tolerance for P-MPR in UE Maximum Output Power – EIRP with UL Gaps test case.

These proposals are submitted for approval in multiple CRs ([14-18]).

# 4. 	References

- RP-221932, “UE Conformance – Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, Apple, RAN#97-e

- 3GPP TS 38.521-2, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 standalone”

- R5-236085, “WP UE Conformance – Further enhancements of NR RF requirements for FR2”, WP UE Conformance – Further enhancements of NR RF requirements for FR2”, Nokia, Nokia Shanghai Bell, Apple, RAN5#101

- R5-227774, “Introduce framework for UL-Gaps related Tx Power tests”, Apple Portugal, RAN5#97

- R5-231373, “Updates to FR2 RF test case 6.2.5 for EIRP with UL-Gaps”, Apple Inc, RAN5#98

- R5-235668, “Updates to FR2 RF test case 6.2.5 for EIRP with UL-Gaps”, Apple Benelux B.V., RAN5#100

- R5-237657, “Adding TT for FR2 RF test case 6.2.5 EIRP with UL-Gaps”, Qualcomm Korea, RAN5#101

- R5-237235, “Updates to EIRP test with UL-Gaps”, Apple Benelux B.V., RAN5#101

- 3GPP TR 38.903, “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”

- R5-213852, “Measurement uncertainties for FR2 Relative and aggregate power tolerance”, Keysight Technologies, RAN5#91-e.

- R5-215485, “On the relative power measurement MU”, Rohde & Schwarz, RAN5#92e

- R5-216105, “Correction of power control in 38.903”, Anritsu, RAN5#92-e

- 3GPP TS 38.133, “NR; Requirements for support of radio resource management”

- R5-241404, “Measurement uncertainty definition for UE Maximum Output Power – EIRP with UL Gaps test case” (TR 38.903)

- R5-241405, “Updates to Annex F for UE Maximum Output Power – EIRP with UL Gaps test case” (TS 38.521-2)

- R5-241406, “Updates to UE Maximum Output Power – EIRP with UL Gaps test case” (TS 38.521-2)

- R5-241407, “Updates to Annex F for 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case” (TS 38.521-3)

- R5-241408, “Updates to 6.2B.1.6 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP with UL Gaps test case” (TS 38.521-3)