3GPP TSG-RAN5 Meeting #102	R5-242038

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3065 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configuration of SMTC for NCD-SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#114bis, it was agreed that NW shall ensure NCD-SSB time domain location is a subset of the time domain of CD-SSB in TDD. Agreement: For RedCap UE in TDD, the NW ensures that the NCD-SSB time domain location is a subset of the time domain location of CD-SSB.. A LS R1-2310566 is also sent to RAN2. As a result, R2-2314057 is agreed in RAN2#124 to update the field description of ssb-TimeOffset as follows. ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, RedCap UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For RedCap UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell’s CD-SSB.. Correspondingly, in RAN4#109/RAN5#101 CR R4-2321485/R5-237885/ R5-237897 were also agreed to update RMC SMTC.2 RedCap in RAN4/RAN5 specs to align with RAN1/RAN2’s conclusion. However, - SMTC.4 RedCap is also used for NCD-SSB based measurements and needs to be updated too. |
| Summary of change: |  | Default configuration of SMTC.4 RedCap are updated to align with RAN1/RAN2 conclusions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2401304 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-240676r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	SSB-MTC

Table 7.3.1-3: SSB-MTC

| Derivation Path: Table 4.6.3-185 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf10 | 0 |  | SMTC.8 |
| sf20 | 0 |  | SMTC.1, SMTC.2 |
|  | 5 |  | SMTC.7 |
|  | 10 |  | SMTC.4, SMTC.5 |
|  | 17 |  | SMTC.6 |
|  | 13 |  | SMTC.9 |
| sf40 | 0 |  | SMTC.1 RedCap |
|  | 20 |  | SMTC.3 RedCap |
| sf80 | 0 |  | SMTC.2 RedCap, SMTC.4 RedCap |
| sf160 | 0 |  | SMTC.3 |
| } |  |  |  |
| duration | sf1 |  | SMTC.1, SMTC.3, SMTC.4, SMTC.8, SMTC.9, SMTC.1 RedCap SMTC.2 RedCap SMTC.3 RedCap |
|  | sf5 |  | SMTC.2, SMTC.5 SMTC.6, SMTC.7 SMTC.4 RedCap |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n as defined in 38.533 Annex A.4 |
| SMTC.n RedCap | RedCap-specific SMTC pattern n as defined in 38.533 Annex A.4A |

–	SubcarrierSpacing

<End of modified section 1>