3GPP TSG-RAN5 Meeting #102	R5-242042

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2932 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap RRM test case 16.7.2.4.1 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update test case in following aspects according to TT analysis: - Editor’s note is removed; - Test parameters; - Test requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2401304 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-240712r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.7.2.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

16.7.2.4.1.1	Test purpose

To verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.8.1.1

16.7.2.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from Release 17 onwards.

16.7.2.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.4.

16.7.2.4.1.4	Test description

16.7.2.4.1.4.1	Initial conditions

Same as the initial conditions given in 6.7.2.2.1 with followings exceptions:

-	Table 6.7.2.2.1.4.1-1 is replaced by Table 16.7.2.4.1.4.1-1;

-	Table 6.7.2.2.1.4.1-2 is replaced by Table 16.7.2.4.1.4.1-2;

Table 16.7.2.4.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.2.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.2.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.2.4.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.2.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.2.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.2.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

16.7.2.4.1.4.2	Test procedure

Same as the test procedure in clause 6.7.2.2.1.4.2 with followings exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 16.7.2.4.1.5-1;

-	Table 6.7.2.2.1.5-2 is replaced by Table 16.7.2.4.1.5-2.

16.7.2.4.1.4.3	Message contents

Same as the message contents in clause 6.7.2.2.1.4.3 with following exceptions:

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.2.4.1-3.

16.7.2.4.1.5	Test requirement

Same as the test requirements in clause 6.7.2.2.1.5 with followings exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 16.7.2.4.1.5-1;

-	Table 6.7.2.2.1.5-2 is replaced by Table 16.7.2.4.1.5-2;

Table 16.7.2.4.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Gap pattern ID |  |  | 0 |  |  |  |  |  |
| BWP BW | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |  |  |
|  | Config 3 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | dBm/ 15kHz | -81.68 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -86.27 |  | -113 |  | -116 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/ SCS | -81.68 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -83.27 |  | -110 |  | -113 + ΔBG_offset |  |
|  |  | dB | -1.75 |  | -1.75 |  | 3 | -1.75 |
|  |  | dB | -1.75 |  | -1.75 |  | 3 | -1.75 |
| SS-RSRPNote3 | Config 1,2,4 | dBm/ SCS | -83.43 |  | -107.75 |  | -113+ ΔBG_offset | -117.7+ ΔBG_offset |
|  | Config 3 |  | -85.02 |  | -111.75 |  | -110+ ΔBG_offset | -114.7+ ΔBG_offset |
| SS-RSRQNote3 |  | dB | -14.77 |  | -14.77 |  | -12.56 | -14.76 |
| IoNote3 | Config 1,2,4 | dBm/ Ch BW | -51.51 |  | -75.83 |  | -83.28+ ΔBG_offset | -85.83+ ΔBG_offset |
|  | Config 3 |  | -53.17 |  | -79.90 |  | -80.36+ ΔBG_offset | -82.90+ ΔBG_offset |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 16.7.2.4.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_52 | SS-RSRQ_52 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_62 | SS-RSRQ_62 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_49 | SS-RSRQ_49 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_65 | SS-RSRQ_65 |

<End of modified section 1>