3GPP TSG-RAN5 Meeting #102	R5-240052r1

# Athens, Greece, February 26 – March 1 , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2588 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Configured transmitted power TC for ATG UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Configured transmitted power test case for ATG UE needs to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Configured transmitted power test case for ATG UE has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2J.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0366 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related CR is R5-240334. MU and TT of ATG Configured transmitted power TC are added in R5-240334. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Add MU and TT editor notes. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2J.2	Configured transmitted power

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- Addition to applicability spec is incomplete.

- [RatedMOPATG] is pending.

- Annex F MU/TT is FFS.

6.2J.2.1	Test purpose

To verify the measured NR ATG UE configured maximum output power PUMAX,f,c for the ATG UE is within the specified bounds.

6.2J.2.2	Test applicability

This test case applies to all types of NR ATG UE release 18 and forward.

6.2J.2.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, PMaxOutputPower}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7];

PMaxOutputPower is the maximum ATG UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability [RatedMOPATG];

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2J.2.

6.2J.2.4	Test description

6.2J.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1 that are restricted to the ATG UE. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2J.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2J.2.4.1-1: Test Configuration Table for ATG UE

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n41, n78 and n79. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2J.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2J.2.4.3.

6.2J.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2J.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the PUMAX level of the test point.

3.	Measure the sum of the mean transmitted power of the UE at each transmit antenna connector in table 6.2J.2.5-1 according to the test configuration from table 6.2J.2.4.1-1. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots with transient periods are not under test.

6.2J.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2J.2.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2J.2.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2J.2.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2J.2.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2J.2.4.3-4: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2J.2.4.3-2: FrequencyInfoUL-SIB: Test point 5

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 25 |  |  |

Table 6.2J.2.4.3-3: FrequencyInfoUL-SIB: Test point 6

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 30 |  |  |

Table 6.2J.2.4.3-4: FrequencyInfoUL-SIB: Test point 7

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 35 |  |  |

Table 6.2J.2.4.3-5: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2J.2.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2J.2.5-1.

Table 6.2J.2.5-1: PCMAX configured UE output power for ATG UE

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (2+TT) | -10 dBm ± (2+TT) |
| Measured UE output power test point 2 | 10 dBm ± (2+TT) | 10 dBm ± (2+TT) |
| Measured UE output power test point 3 | 15 dBm ± (2+TT) | 15 dBm ± (2+TT) |
| Measured UE output power test point 4 | 20 dBm ± (2+TT) | 20 dBm ± (2+TT) |
| Measured UE output power test point 5 | 25 dBm ± (2+TT) | 25 dBm ± (2+TT) |
| Measured UE output power test point 6 | 30 dBm ± (2+TT) | 30 dBm ± (2+TT) |
| Measured UE output power test point 7 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2J.2.5-2. Note 2: UE capability [RatedMOPATG] is an integer value in the range 23 to 40 dBm. Note 3: The maximum output power shall be within the range in Table 6.2J.2.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2J.2.5-1b. |  |  |

Table 6.2J.2.5-1a: Measured UE output power test point 7 for Test ID 1,2 for ATG UE

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 35 dBm ±(2+TT) |
| n3 | 35 dBm ±(2+TT) |
| n34 | 35 dBm ±(2+TT) |
| n39 | 35 dBm ±(2+TT) |
| n41 | 35 dBm ±(2+TT) |
| n78 | 35 dBm ±(2+TT) |
| n79 | 35 dBm ±(2+TT) |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2J.2.5-2. |  |

Table 6.2J.2.5-1b: Measured UE output power test point 7 for Test ID 3 for ATG UE

| NR band | Tolerance (dB) |
| --- | --- |
| n41 | 38 dBm ±(2+TT) |
| n78 | 38 dBm ±(2+TT) |
| n79 | 38 dBm ±(2+TT) |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2J.2.5-2. |  |

Table 6.2J.2.5-2: Test Tolerance (Configured transmitted power) for ATG UE

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

<< END OF CHANGES >>