3GPP TSG-RAN5 Meeting #102	R5-240095r1

Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4161 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 8.1.6.1.2.14 for SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism to add one test conditions and modify the test procedure and related specific message content to resolve the problem that the UE may not be able to detect the IDC problem. The idle UE logging suspension is “at least 10s gap” is not accurate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update SON/MDT TC 8.1.6.1.2.14 Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism - Add one test condition to facilitate the subsequent test procedure - Modify the test procedure and related specific message content to resolve the problem that the UE may not be able to detect the IDC problem. - The idle UE logging suspension “at least 10s gap” is revised into “about 10s gap”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the spec version from to 17.5.0 to 17.5.1. CATT is added as co-source company. “Table 8.1.6.1.2.14.3.2-1: Main behaviour” is updated as per the comments from CATT. |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.6.1.2.14	Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism

8.1.6.1.2.14.1	Test Purpose (TP)

(1)

with { UE is performing measurement logging and T330 is running }

ensure that {

when { UE detects IDC problem }

then { UE suspend measurement logging and tag MDT report with inDeviceCoexDetected-r17 flag in logMeasInfoList-r16 in VarLogMeasReport }

}

(2)

with { UE has suspended measurement logging and T330 is running }

ensure that {

when { The IDC problems detected by the UE is resolved during the last logging interval }

then { UE resume measurement logging }

}

8.1.6.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.5a.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331 clause 5.5a.3.2]

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

...

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

...

8.1.6.1.2.14.3	Test description

8.1.6.1.2.14.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, WLAN AP 1 (Cell 27);

-	NR Cell 1 with NR operating band n40

UE:

None;

Preamble:

The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.14.3.2	Test procedure sequence

Table 8.1.6.1.2.14.3.2-1a for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.2.14.3.2-1.

Table 8.1.6.1.2.14.3.2-1a: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | WLAN AP 1 (Cell 27) is configured as being off. NR Cell 1 is configured as far from 2400MHz as possible. |
|  | BeaconRSSI | dBm |  | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Channel 1 (2401-2423MHz) is configured to WLAN AP 1 (Cell 27). NR Cell 1 is configured as close to 2400MHz as possible. |
|  | BeaconRSSI | dBm |  | -80 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | WLAN AP 1 (Cell 27) is configured as being off. NR Cell 1 is configured as far from 2400MHz as possible. |
|  | BeaconRSSI | dBm |  | Off |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Channel 1 (2401-2423MHz) is configured to WLAN AP 1 (Cell 27). NR Cell 1 is configured as close to 2400MHz as possible. |
|  | BeaconRSSI | dBm |  | -80 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | WLAN AP 1 (Cell 27) is configured as being off. NR Cell 1 is configured as far from 2400MHz as possible. |
|  | BeaconRSSI | dBm |  | Off |  |

Table 8.1.6.1.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | The SS transmits a RRCReconfiguration message including OtherConfig with idc-AssistanceConfig-r16 including CandidateServingFreqListNR-r16 including center frequency for NR cell 1. | <-- | NR RRC: RRCReconfiguration |  |  |
| 0B | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |  |  |
| 0C | UE does not transmits UEAssistanceInformation message including IDC-Assistance-r16 within 10s. Note: Test Step 0C is used to confirm whether UE can not detect IDC problems with “T0” configuration. If UEAssistanceInformation message including IDC-Assistance-r16 is not received by SS within 10s, it is confirmed that UE with “T0” configuration can not detect IDC problem in RRC_CONNECTED which will be recorded as a reference to the test result. | - | - | - | - |
| 0D | The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.2.14.3.2-1a. | - | - | - | - |
| 0E | SS starts timer = 10s. | - | - | - | - |
| - | EXCEPTION: Step 0Fa1 describes UE’s behaviour when detecting IDC problem in RRC_CONNECTED state; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. Note: Some UE may not detect IDC problem under this condition. Test Step 0Fa1 will be used to confirm whether UE can detect IDC problems with “T1” configuration. If UEAssistanceInformation message including IDC-Assistance-r16 is received by SS, it is confirmed that UE with “T1” configuration can detect IDC problem in RRC_CONNECTED which will be recorded as a reference to the test result. |  |  |  |  |
| 0Fa1 | UE transmits an UEAssistanceInformation message including IDC-Assistance-r16 with AffectedCarrierFreq set to NR carrier frequencies of NR Cell 1. | --> | NR RRC: UEAssistanceInformation |  |  |
| 0Fb1 | The SS waits for Timer expiry. | - | - | - | - |
| 0G | The SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.6.1.2.14.3.2-1a. | - | - | - | - |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging. | - | - | - | - |
| 3a | The SS re-adjusts the SS/PBCH EPRE level according to row "T3" in Table 8.1.6.1.2.14.3.2-1a. Note: UE can detect IDC problem in RRC_IDLE with configuration “T3” which is the same as “T1”. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 4a | The SS re-adjusts the SS/PBCH EPRE level according to row "T4" in Table 8.1.6.1.2.14.3.2-1a after 10s. Note: UE can not detect IDC problem in RRC_IDLE with configuration “T4” which is the same as “T0”. The IDC problems detected by the UE was resolved with configuration “T4”. | - | - | - | - |
| 5 | Wait 10s to allow UE to resume logging. | - | - | - | - |
| 6-20 | Steps 1 to 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 21 | The SS transmits an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 22 | Check: Does the UE transmit an UEInformationResponse message including logMeasReport with a LogMeasInfoList consisting of at least three LogMeasInfo entries, and one of the LogMeasInfo entries, except the first one and the last one, is tagged with inDeviceCoexDetected-r17, and also there exists about 10s gap between the relativeTimeStamp of the LogMeasInfoList entry with inDeviceCoexDetected-r17 flag and the relativeTimeStamp of the next LogMeasInfoList entry without inDeviceCoexDetected-r17 flag? Note 1: The relativeTimeStamp is increased between the subsequent LogMeasInfoList entries. Note 2: About 10s gap (Step 4a) between the relativeTimeStamp of the LogMeasInfoList entry with inDeviceCoexDetected-r17 flag and the relativeTimeStamp of the next LogMeasInfoList entry without inDeviceCoexDetected-r17 flag is used to check that the UE suspends measurement logging when IDC problem is detected. Note 3: After 10s gap (Step 5), the next LogMeasInfoList entry without inDeviceCoexDetected-r17 flag is used to check that the UE resume logging when IDC problem is resolved. | --> | NR RRC: UEInformationResponse | 1, 2 | P |

8.1.6.1.2.14.3.3		Specific message contents

Table 8.1.6.1.2.14.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.14.3.3-2: UEInformationRequest (steps 21, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.14.3.3-3: UEInformationResponse (step 22, Table 8.1.6.1.2.14.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 3 entries where at least 3 entry complies to the entries with index ‘x-1’, ‘x’, ‘x+1’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x-1] SEQUENCE { |  | entry x-1 |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x-1 |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |
| } |  |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x, and relativeTimeStamp-r16_x-1 < relativeTimeStamp-r16_x, relativeTimeStamp-r16_x + 10s ≤ relativeTimeStamp-r16_x+1 |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | true |  |  |
| } |  |  |  |
| LogMeasInfo-r16[x+1] SEQUENCE { |  | entry x+1 |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x+1 |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.14.3.3-4: RRCReconfiguration (step 0A, Table 8.1.6.1.2.14.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-r16 CHOICE { |  |  |  |
| Setup IDC-AssistanceConfig-r16 SEQUENCE { |  |  |  |
| candidateServingFreqListNR-r16 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR { | 1 entry |  |  |
| ARFCN-ValueNR[1] | ARFCN-ValueNR of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.14.3.3-5: UEAssistanceInformation (step 0C, Table 8.1.6.1.2.14.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-Assistance-r16 SEQUENCE { |  |  |  |
| affectedCarrierFreqList-r16 SEQUENCE { |  |  |  |
| AffectedCarrierFreq-r16 SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueNR of NR Cell 1 |  |  |
| interferenceDirection-r16 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>