3GPP TSG-RAN WG5 Meeting #102	R5-240126r1

Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0227 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of BDS B2a and B3I signal default test conditions in TS 37.571-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the global B2a and B3I signals in the network-assisted BDS System, as part of A-GNSS positioning methods in LTE and NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding reference of B2a and B3I ICD and symbol of these two signals - Adding BDS B2a into the Table 6.1.3.3-1 note 3 to clarify that GNSS-AuxiliaryInformation should also be provided when the UE supporting BDS B2a. - Adding description of BDS B2a and BDS B3I signal values in the GNSS IONOSPHERIC MODEL assistance data information elements - Adding description of BDS B2a and BDS B3I signal values in the GNSS NAVIGATION MODEL assistance data information elements - Adding description of BDS B2a and BDS B3I signal values in the GNSS ACQUISITION MODEL assistance data information elements - Adding description of BDS B2a and BDS B3I signal values in the GNSS ALMANAC assistance data information elements - Adding description of BDS B2a and BDS B3I signal values in the GNSS UTC MODEL assistance data information elements - Adding description of BDS B2a and BDS B3I signal values in the GNSS AUXILIARY information assistance data information elements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to BDS B2a and B3I signal testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 6.1.3.3, 6.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: Corrected the meeting date to resolve 3GU issue |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[38]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[39]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[40]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[41]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[44]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[45]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[46]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[47]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[48]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[49]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[50]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[51]	NMEA standard 0183, Version 4.11, November 2018.

[52]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[53]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

< Unchanged sections omitted >

3.2	Symbols

For the purposes of the present document, the following symbols apply:

B1I	BeiDou B1I navigation signal with carrier frequency of 1561.098 MHz.

B1C	BeiDou B1C navigation signal with carrier frequency of 1575.420 MHz.

B2a	BeiDou B2a navigation signal with carrier frequency of 1176.450 MHz.

B3I	BeiDou B3I navigation signal with carrier frequency of 1268.520 MHz.

E1	Galileo E1 navigation signal with carrier frequency of 1575.420 MHz.

E5	Galileo E5 navigation signal with carrier frequency of 1191.795 MHz.

E6	Galileo E6 navigation signal with carrier frequency of 1278.750 MHz.

G1	GLONASS navigation signal in the L1 sub-bands with carrier frequencies 1602 MHz ± k  562.5 kHz.

G2	GLONASS navigation signal in the L2 sub-bands with carrier frequencies 1246 MHz ± k  437.5 kHz.

k		GLONASS channel number, k = -7…13.

L1 C/A	GPS or QZSS L1 navigation signal carrying the Coarse/Acquisition code with carrier frequency of 1575.420 MHz.

L1C	GPS or QZSS L1 Civil navigation signal with carrier frequency of 1575.420 MHz.

L2C	GPS or QZSS L2 Civil navigation signal with carrier frequency of 1227.600 MHz.

L5	GPS or QZSS L5 navigation signal with carrier frequency of 1176.450 MHz.

< Unchanged sections omitted >

6.1.3.3	Default Assistance Data for TS 37.571-2 subclauses 7 and 9

This subclause defines the GNSS assistance data elements which shall be provided to the UE in certain tests in TS 37.571-2 [7] subclauses 7 and 9 in the LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message. The GNSS assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSS(s) supported by the UE. GNSS assistance data IEs not supported by the UE shall not be sent. GNSS assistance data IEs supported by the UE but not listed below shall not be sent.

Table 6.1.3.3-1: Default GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE supported by the UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted. GNSS-Acquisition Assistance supported by the UE | UE-assisted. GNSS-Acquisition Assistance not supported by the UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes(1) | No | Yes(1) |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes(2) | Yes(2) | Yes(2 |
| GNSS-AuxiliaryInformation | Yes(3) | Yes(3) | Yes(3) |
| Note1: Only if more than one GNSS supported by the UE Note2: Only if GLONASS and at least one other GNSS supported by the UE. Note3: Only if GLONASS supported by the UE, and/or if the UE supports multiple GPS signals and/or if BDS B1C and/or BDS B2a supported by the UE. |  |  |  |

6.1.3.4	Assistance Data values for TS 37.571-2 subclauses 7 and 9

Assistance data that is marked as “time varying” and the gnss-TimeOfDay field are created and used in 1 second increments.

The accuracy of the gnss-TimeOfDay and assistance data that is marked as “time varying” in the provided assistance data shall be within +/- 2 s relative to the GNSS time in the system simulator. In the case that assistance data is required but satellite signals are not required then this clause does not apply.

Assistance data Information Elements and fields that are not specified shall not be used.

The information elements detailed below are fully defined in TS 37.355 [8].

< Unchanged sections omitted >

# 6.1.3.4.3	GNSS IONOSPHERIC MODEL

# GNSS-IonosphericModel (Klobuchar Model): If GPS or GLONASS or BDS B1I or BDS B3I supported by the UE

Derived from data in clause 6.1.2 and the following information:

dataID: 00

neQuickModel: not present unless Galileo is also supported by the UE

klobucharModel2: not present unless BDS B1C or BDS B2a is also supported by the UE

# GNSS-IonosphericModel (NeQuick Model): If Galileo supported by the UE

Derived from data in clause 6.1.2 and the following information:

klobucharModel: not present unless GPS or GLONASS or BDS B1I or BDS B3I is also supported by the UE

klobucharModel2: not present unless BDS B1C or BDS B2a is also supported by the UE

# GNSS-IonosphericModel (Klobuchar2 Model): If BDS B1C or BDS B2a supported by the UE

Derived from data in clause 6.1.2 and the following information:

klobucharModel: not present unless GPS or GLONASS or BDS B1I or BDS B3I is also supported by the UE

neQuickModel: not present unless Galileo is also supported by the UE

< Unchanged sections omitted >

# 6.1.3.4.5	GNSS NAVIGATION MODEL:

# GNSS-NavigationModel (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-2): If GPS supported by the UE

Derived from data in clause 6.1.2 and the following information:

GNSS-ClockModel: nav-ClockModel, Model-2

GNSS-OrbitModel: nav-KeplerianSet, Model-2

Note: in the case that the UE supports Modernized GPS then the UE may also support the GNSS-NavigationModel (Model-3), however in this case the GNSS-NavigationModel (Model-2) for GPS shall still be used.

# GNSS-NavigationModel (Model-4): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-4): If GLONASS supported by the UE

Derived from data in clause 6.1.2 and the following information:

svHealth: 00000000

GNSS-ClockModel: glonass-ClockModel, Model-4

GNSS-OrbitModel: glonass-ECEF, Model-4

# GNSS-NavigationModel (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-1): If Galileo supported by the UE

Derived from data in clause 6.1.2 and the following information:

svHealth: 0

GNSS-ClockModel: standardClockModelList, Model-1.

standardClockModelList: (SIZE) 1 if the UE supports only Galileo E1, (SIZE) 2 if the UE supports multiple Galileo signals.

StandardClockModelElement (I/NAV):

- stanClockTgd: Not present if the UE supports multiple Galileo signals.

- stanModelID: 0 (I/NAV). Present only if the UE supports multiple Galileo signals

StandardClockModelElement (F/NAV): Present only if the UE supports multiple Galileo signals

- stanClockTgd: Not present

- stanModelID: 1 (F/NAV)

GNSS-OrbitModel: keplerianSet, Model-1

# GNSS-NavigationModel (Model-6): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-6): If BDS supported by the UE

Derived from data in clause 6.1.2 and the following information:

svHealth: 0

GNSS-ClockModel: BDS-ClockModel-r12, Model-6

GNSS-OrbitModel: BDS-KeplerianSet-r12, Model-6

Note: in the case that the UE supports BDS B1C or BDS B2a then the UE may also support the GNSS-NavigationModel (Model-7), however in this case the GNSS-NavigationModel (Model-6) for BDS shall still be used.

# 6.1.3.4.6	GNSS ACQUISITION ASSISTANCE:

In the case that the UE only supports GPS L1 C/A for GPS then the GNSS-AcquisitionAssistance (GPS L1 C/A) shall be used for GPS.

In the case that the UE supports Modernized GPS then the GNSS-AcquisitionAssistance to be used for GPS depends on the GNSS-AcquisitionAssistance(s) supported by the UE for GPS. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (GPS L1 C/A)

GNSS-AcquisitionAssistance (Modernized GPS L5)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_GPS_L1CA and pc_GNSS_AcquAssist_GPS_L5, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (GPS L1 C/A) shall be used.

# GNSS-AcquisitionAssistance (GPS L1 C/A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (GPS L1 C/A) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (Modernized GPS L5)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 3 (GPS L5) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Modernized GPS L5)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (GLONASS G1) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement: If GLONASS supported by the UE

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

In the case that the UE only supports Galileo E1 for Galileo then the GNSS-AcquisitionAssistance (Galileo E1) shall be used for Galileo.

In the case that the UE supports more than one Galileo signal then the GNSS-AcquisitionAssistance to be used for Galileo depends on the GNSS-AcquisitionAssistance(s) supported by the UE for Galileo. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (Galileo E1)

GNSS-AcquisitionAssistance (Galileo E5A)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_Galileo_E1 and pc_GNSS_AcquAssist_Galileo_E5A, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (Galileo E1) shall be used.

# GNSS-AcquisitionAssistance (Galileo E1)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (Galileo E1) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Galileo E1)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (Galileo E5A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 1 (Galileo E5A) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Galileo E5A)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

In the case that the UE only supports BDS B1I for BDS then the GNSS-AcquisitionAssistance (BDS B1I) shall be used for BDS.

In the case that the UE supports more than one BDS signal then the GNSS-AcquisitionAssistance to be used for BDS depends on the GNSS-AcquisitionAssistance(s) supported by the UE for BDS. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (BDS B1I)

GNSS-AcquisitionAssistance (BDS B1C)

GNSS-AcquisitionAssistance (BDS B2a)

GNSS-AcquisitionAssistance (BDS B3I)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_BDS_B1I, pc_GNSS_AcquAssist_BDS_B1C, pc_GNSS_AcquAssist_BDS_B2a and pc_GNSS_AcquAssist_BDS_B3I in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (BDS B1I) shall be used.

# GNSS-AcquisitionAssistance (BDS B1I)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (BDS B1I) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B1I)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (BDS B1C)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 9 (BDS B1C) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B1C)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (BDS B2a)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 12 (BDS B2a) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B2a)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (BDS B3I)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 3 (BDS B3I) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B3I)

These fields are time varying (see clause 6.1.3.4) and are derived from data in clause 6.1.2 and the following information:

Doppler uncertainty: 2.5 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# 6.1.3.4.7	GNSS ALMANAC:

# GNSS-Almanac (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | Derived from data in clause 6.1.2 |
| toa |  | Derived from data in clause 6.1.2 |
| ioda |  | Not present |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 31 |

# GNSS-AlmanacElement (Model-2): If GPS supported by the UE

FFS

GNSS-AlmanacElement:keplerianNAV-Almanac (Model-2)

Note: in the case that the UE supports Modernized GPS then the UE may also support the GNSS-Almanac (Model-3) and/or GNSS-Almanac (Model-4), however in this case the GNSS-Almanac (Model-2) for GPS shall still be used.

# GNSS-Almanac (Model-5): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 24 |

# GNSS-AlmanacElement (Model-5): If GLONASS supported by the UE

FFS

GNSS-AlmanacElement: keplerianGLONASS (Model-5)

# GNSS-Almanac (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | Derived from data in clause 6.1.2 |
| toa |  | Derived from data in clause 6.1.2 |
| ioda |  | 0 |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 29 |

# GNSS-AlmanacElement (Model-1): If Galileo supported by the UE

FFS

GNSS-AlmanacElement: keplerianAlmanacSet (Model-1)

kepSV-StatusFNAV: 0. Present only if the UE supports multiple Galileo signals.

# GNSS-Almanac (Model-7): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | Derived from data in clause 6.1.2 |
| toa |  | Derived from data in clause 6.1.2 |
| ioda |  | Not present |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 30 |

# GNSS-AlmanacElement (Model-7): If BDS supported by the UE

FFS

GNSS-AlmanacElement: BDS-AlmanacSet-r12 (Model-7)

Note: in the case that the UE supports BDS B1C or BDS B2a then the UE may also support the GNSS-Almanac (Model-3) and/or GNSS-Almanac (Model-4), however in this case the GNSS-Almanac (Model-7) for BDS shall still be used.

# 6.1.3.4.8	GNSS UTC MODEL:

# GNSS-UTC-Model: If both GPS and GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-UTC-Model |  |  |
| utcModel1 |  |  |

# UTC-ModelSet1: If both GPS and GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-Utc-A1 |  | 0 |
| gnss-Utc-A0 |  | 0 |
| gnss-Utc-Tot |  | Derived from data in clause 6.1.2 |
| gnss-Utc-WNt |  | Derived from data in clause 6.1.2 |
| gnss-Utc-DeltaTls |  | Derived from data in clause 6.1.2 |
| gnss-Utc-WNlsf |  | Derived from data in clause 6.1.2 |
| gnss-Utc-DN |  | Derived from data in clause 6.1.2 |
| gnss-Utc-DeltaTlsf |  | Derived from data in clause 6.1.2 |

# 6.1.3.4.9	GNSS AUXILIARY INFORMATION:

# GNSS-AuxiliaryInformation: If multiple GPS signals supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-GPS |  | (SIZE) 6 |
| svID |  | Derived from data in clause 6.1.2 |
| signalsAvailable |  | As supported by the UE |

# GNSS-AuxiliaryInformation: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-GLONASS |  | (SIZE) 6 |
| svID |  | Derived from data in clause 6.1.2 |
| signalsAvailable |  | G1 |
| channelNumber |  | Derived from data in clause 6.1.2 |

# GNSS-AuxiliaryInformation: If BDS B1C or BDS B2a supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-BDS-r16 |  | (SIZE) 6 |
| svID-r16 |  | Derived from data in clause 6.1.2 |
| satType-r16 |  | Derived from data in clause 6.1.2 |

< Unchanged sections omitted >