3GPP TSG-RAN5 Meeting #102	R5-240222r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2881 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Table H.3.1-8A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the IE RadioLinkMonitoringConfig with CoreSpec |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IE RadioLinkMonitoringConfig was updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this RRC IE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 version : to add closed WIC in coversheet |  |  |  |  |  |  |  |  |

<<Start of changes >>

Table H.3.1-8A: RadioLinkMonitoringConfig-Release 17: Release 17 configuration for TRP specific RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| beamFailure-r17 ::= SEQUENCE { |  |  |  |
| failureDetectionSet1-r17::= SEQUENCE { |  |  |  |
| bfdResourcesToAddModList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-r17 { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| BeamLinkMonitoringRS-r17[1] ::= SEQUENCE { |  | entry 1 |  |
| beamLinkMonitoringRS-Id-r17 | 0 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS-r17[2] ::= SEQUENCE { |  | entry 2 |  |
| beamLinkMonitoringRS-Id-r17 | 1 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bfdResourcesToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-Id-r17 { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| BeamLinkMonitoringRS-Id-r17[1] | 0 | entry 1 |  |
| BeamLinkMonitoringRS-Id-r17[2] | 1 | entry 2 |  |
| } |  |  |  |
| beamFailureInstanceMaxCount-r17 | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer-r17 | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |
| failureDetectionSet2-r17::= SEQUENCE { |  |  |  |
| bfdResourcesToAddModList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-r17 { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| BeamLinkMonitoringRS-r17[1] ::= SEQUENCE { |  | entry 1 |  |
| beamLinkMonitoringRS-Id-r17 | 2 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 2 | Index of SSB #2 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS-r17[2] ::= SEQUENCE { |  | entry 2 |  |
| beamLinkMonitoringRS-Id-r17 | 3 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 3 | Index of SSB #3 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bfdResourcesToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-Id-r17 { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| BeamLinkMonitoringRS-Id-r17[1] | 2 | entry 1 |  |
| BeamLinkMonitoringRS-Id-r17[2] | 3 | entry 2 |  |
| } |  |  |  |
| beamFailureInstanceMaxCount-r17 | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer-r17 | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |
| additionalPCI-r17 | not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

<<End of changes >>