3GPP TSG WG RAN5 Meeting #102	R5-240261r1

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0372 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Additional Information of 6.5.3.1 in 38.521-1 need to be revised as per the endorsed proposal in R5-237831. - LTE anchor agnostic approach testing in 6.5B.3.3.2 is not identical to the corresponding SA test case as the test requirements are not the same(not non-exception requirement) and it is only for the testing of NSA only UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Additional Information of 6.5.3.1 in 38.521-1 were revised as per the endorsed proposal in R5-237831. - Additional Information for 6.5B.3.3.2 in 38.521-3 were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional Information for 6.5.3.1 in 38.521-1 and 6.5B.3.3.2 in 38.521-3 will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1, 4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion in R5-240259. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Additional Information for 6.5.3.1 in Table 4.1.1-1 was updated to be the same wording as the example in R5-237831. - Additional Information for 6.5B.3.3.2 in Table 4.1.3-1 was updated. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001l | UEs supporting 5GS FR1 not supporting txDiversity-r16 and not supporting RedCap | D001 | PC1 PC2 PC3 |  |
| << Unchanged contents omitted >> |  |  |  |  |  |  |  |
| 6.5.2.4.2 | UTRA ACLR | Rel-15 | C001a | UEs supporting 5GS FR1 PC3 not supporting txDiversity-r16 | D001 |  | Skip TC 6.5.2.4.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.2 has been executed. |
| 6.5.3.1 | General spurious emissions | Rel-15 | C001h | UEs supporting 5GS FR1 not supporting txDiversity-r16 | D001 |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.1.1 or 6.5B.3.2.1 or 6.5B.3.3.1(non-exception requirements) has been executed. |
| 6.5.3.2 | Spurious emissions for UE co-existence | Rel-15 | C001h | UEs supporting 5GS FR1 not supporting txDiversity-r16 | D001 |  |  |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C001h | UEs supporting 5GS FR1 not supporting txDiversity-r16 | D001 |  | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.3 has been executed. |
| 6.5.4 | Transmit intermodulation | Rel-15 | C001h | UEs supporting 5GS FR1 not supporting txDiversity-r16 | D001 |  | Skip TC 6.5.4 if UE supports NSA and TS 38.521-3 TC 6.5B.5.3 has been executed. |

<< Unchanged sections omitted >>

4.1.3	NR interworking between NR FR1 and NR FR2 and between NR and LTE conformance test cases

Table 4.1.3-1: Applicability of RF EN-DC FR1 and FR2 conformance test cases, ref. TS 38.521-3 [3]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA/DC-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter characteristics |  |  |  |  |  |  |
| 6.2B | Transmitter power for DC |  |  |  |  |  |  |
| 6.2B.1.1 | UE Maximum Output Power for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  |  |
| << Unchanged contents omitted >> |  |  |  |  |  |  |  |
| 6.5B.3.2.2 | Spurious emission band UE co-existence for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.5B.3.3.1 | General spurious emissions for Inter-band EN-DC within FR1 | Rel-15 | C011 | UEs supporting Inter-band EN-DC within FR1 (2UL CCs) | E005 |  | Test only one EN-DC combination per 5G NR band. Skip LTE anchor agnostic approach testing in TC 6.5B.3.3.1 if UE supports SA and TS 38.521-1 TC 6.5.3.1 has been executed. |
| 6.5B.3.3.2 | Spurious emission band UE co-existence for Inter-band within FR1 | Rel-15 | C011 | UEs supporting Inter-band EN-DC within FR1 (2UL CCs) | E005 |  | NOTE 5 applied for LTE anchor agnostic approach testing in TC 6.5B.3.3.2. |
| 6.5B.3.4.1 | General Spurious Emissions for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.3.4.1 if UE supports SA and TS 38.521-2 TC 6.5.3.1 has been executed. |
| 6.5B.3.4.1a | General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Rel-16 | C012w | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC and mpr-PowerBoost-FR2-r16 | E010 | PC1 (NOTE 1) PC2 (NOTE 1) PC3 PC4 (NOTE 1) | NOTE 5 Skip TC 6.5B.3.4.1a if UE supports SA and TS 38.521-2 TC 6.5.3.1_1 has been executed. |

<< End of changes >>