3GPP TSG WG RAN5 Meeting #102	R5-240270r1

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0851 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test points analysis for ATG test cases in 38.521-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test points analysis for ATG test cases 6.2J.1, 6.2J.2, 6.3J.1, 6.5J.1, 6.5J.3.1, and 7.4J to 7.9J in 38.521-1 are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test points analysis for ATG test cases 6.2J.1, 6.2J.2, 6.3J.1, 6.5J.1, 6.5J.3.1, and 7.4J to 7.9J in 38.521-1 were introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_ATG-UEConTest work item cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Test cases CRs in R5-230265 to R5-240269, R5-240051, R5-240052, R5-240054, R5-240057, R5-240062, R5-240064 and R5-240065. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Addition of test cases 6.2J.1, 6.2J.2, 6.5J.1, 6.5J.3.1, 7.7J, 7.8J.2, 7.9J and addition of CMCC as co-source company. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

4.1.1.7	FR1 CA with UL MIMO test cases

This section contains information on test point selection for FR1 CA with UL MIMO test cases in TS 38.521-1[2]. The general rule in this section apply to all test cases for FR1 CA with UL MIMO test cases. Separate analysis for each FR1 CA with UL MIMO test case is not provided, unless deemed necessary.

For Test Environment, Test frequency, Test channel bandwidths, Test Subcarrier Spacing, modulation and RB allocation: adopt the same selection as the corresponding FR1 NR CA test cases.

As for waveform, CP-OFDM is mandatory in case of UL MIMO with two layer transmission and DFT-s-OFDM is not valid. Therefore CP-OFDM waveform is selected.

The test point selection for FR1 CA with UL MIMO test cases in [2] clause 6 is listed in Table 4.1.1.7-1.

Table 4.1.1.7-1: UE receiver test point selection for FR1 shared spectrum channel access

| Subclause | Number of test points | Justification | Comments |
| --- | --- | --- | --- |
| 6.2H.1.2 UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO | 320 | General rules of test point selection apply. Test points of TC 6.2A.2.1 for intra-band contiguous CA from TS 38.521-1 can be leveraged excluding DFT-s-OFDM test points. | RAN5#101 |
| 6.2H.1.4 Configured transmitted power for intra-band UL contiguous CA with UL MIMO | 10 | General rules of test point selection apply. Test points of TC 6.2A.4.1 for intra-band contiguous CA from TS 38.521-1 can be leveraged with CP-OFDM waveform replacement. | RAN5#101 |
| 6.3H.1.1 Minimum output power for intra-band UL contiguous CA with UL MIMO | 20 | General rules of test point selection apply. Test points of TC 6.3A.1.1 for intra-band contiguous CA from TS 38.521-1 can be leveraged with CP-OFDM waveform replacement. | RAN5#101 |
| 6.3H.1.3 Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO | 40 | General rules of test point selection apply. Test points of TC 6.3A.3.1 for intra-band contiguous CA from TS 38.521-1 can be leveraged with CP-OFDM waveform replacement. | RAN5#101 |
| 6.5H.1.2.1 Spectrum emission mask for intra-band UL contiguous CA for UL MIMO | 32 | General rules of test point selection apply. Test points of TC 6.5A.2.2.1 for intra-band contiguous CA from TS 38.521-1 can be leveraged excluding DFT-s-OFDM test points. | RAN5#101 |
| 6.5H.1.2.3 NR ACLR for intra-band UL contiguous CA for UL MIMO | 320 | General rules of test point selection apply. Test points of TC 6.5A.2.4.1.1 for intra-band contiguous CA from TS 38.521-1 can be leveraged excluding DFT-s-OFDM test points. | RAN5#101 |

4.1.1.8	FR1 ATG test cases

This section contains information on test point selection for ATG test cases in [2]. The general rules in this section apply to all the NR ATG test cases. Separate analysis is not provided for each single test case unless specific test requirement deviates from the general rules.

General rules of test point selection:

Considering that NR ATG operating bands are defined like FR1 bands, test environment, frequencies, SCSs, channel bandwidths, waveforms, and modulations of a Tx or Rx test for an NR ATG band should be selected based on the same principles of the corresponding single carrier test for an FR1 band.

Table 4.1.1.8-1: NR UE transmitter test point selection for ATG test cases in FR1

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 6.2J.1 UE maximum output power for ATG | 540 | General rules of test point selection apply. Test points of TC 6.2.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.2J.2 Configured transmitted power for ATG | 30 | General rules of test point selection apply. Test points of TC 6.2.4 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.3J.1 Minimum output power for ATG | 45 | General rules of test point selection apply. Test points of TC 6.3.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.5J.1 Occupied bandwidth for ATG | 10 | General rules of test point selection apply. Test points of TC 6.5.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.5J.3.1 General spurious emissions for ATG | 27 | General rules of test point selection apply. Test points of TC 6.5.3.1 from TS 38.521-1 can be leveraged. | RAN5#102 |

Table 4.1.1.8-2: NR UE receiver test point selection for ATG test cases in FR1

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 7.4J Maximum input level for ATG | 6 | General rules of test point selection apply. Test points of TC 7.4 from TS 38.521-1 can be leveraged limiting the modulation to 64QAM and 256QAM. | RAN5#102 |
| 7.5J Adjacent channel selectivity for ATG | 3 | General rules of test point selection apply. Test points of TC 7.5 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.6J.2 In-band blocking for ATG | 3 | General rules of test point selection apply. Test points of TC 7.6.2 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.6J.3 Out-of-band blocking for ATG | 3 | General rules of test point selection apply. Test points of TC 7.6.3 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.7J Spurious response for ATG | 3 | General rules of test point selection apply. Test points of TC 7.7 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.8J.2 Wide band intermodulation for ATG | 3 | General rules of test point selection apply. Test points of TC 7.8.2 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.9J Spurious emissions for ATG | 3 | General rules of test point selection apply. Test points of TC 7.9 from TS 38.521-1 can be leveraged. | RAN5#102 |

4.1.2	Test point analysis per NS value

4.1.2.1	A-MPR, A-SEM and A-SE FR1 test cases for single carrier and UL MIMO

This section contains information on test point selection for single carrier test cases 6.2.3, Additional Maximum Power Reduction (A-MPR), 6.5.2.3 Additional spectrum emission mask (A-SEM) and 6.5.3.3 Additional spurious emissions (A-SE); and for correspondent UL-MIMO test cases in 6.2D.3 and 6.5D.3.3 in [2].

Selection of test points should include some possible worst combinations based on the A-MPR characteristics specified for each NS value and these shall be selected so that they match with corresponding spectrum emission requirements test points. The number of test points should be realistic.

For pi/2 BPSK with Rel-16 DMRS, the correspondent A-MPR requirements are the same as that for Rel-15 DMRS but the PAPR is lower. Given the UE can pass the requirements using Rel-15 DMRS, it can be expected that the UE can pass the requirements using Rel-16 DMRS. Therefore there is no need to additionally test A-MPR, A-SEM and A-SE for Rel-16 DMRS.

<< End of changes >>