3GPP TSG-RAN5 Meeting #102	R5-240379r1

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4217 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.2.3.18.4 inter-SN CPC for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The new test case for inter-SN CPC for EN-DC was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case for inter-SN CPC for EN-DC was missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-SN CPC cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.18.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.508 CR 1462 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Correct a lot of typos. - Add “OCTET STRING” to the value of some IEs to align with 38.331. |  |  |  |  |  |  |  |  |

<Start of Changes>

Table 8.2.3.18.3.3.3-12: CellGroupConfig (Table 8.2.3.18.3.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| } |  |  |  |
| newUE-Identity | ‘4148’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.3.18.4	MN initiated inter-SN Conditional PSCell change / Success / EN-DC

8.2.3.18.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with EN-DC and receiving an RRCConnectionReconfiguration message for conditional PSCell change }

ensure that {

when { None of CPC execution conditions is satisfied }

then { UE maintains connection with source PSCell and continues to evaluate the CPC execution conditions }

}

(2)

with { UE in NR RRC_CONNECTED state with EN-DC and receiving an RRCConnectionReconfiguration message for conditional PSCell change }

ensure that {

when { Any CPC execution condition is satisfied }

then { UE performs conditional PSCell change to the candidate target cell }

}

(3)

with { UE in NR RRC_CONNECTED state with EN-DC and receiving an RRCConnectionReconfiguration message for conditional PSCell change }

ensure that {

when { UE performs conditional PSCell change successfully }

then { UE releases stored CPC configurations }

}

8.2.3.18.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 37.340 clause 10.5.1 and 10.6. Unless otherwise stated these are Rel-17 requirements.

[TS 37.340, clause 10.5.1]

The Secondary Node Change procedure is initiated either by MN or SN and used to transfer a UE context from a source SN to a target SN and to change the SCG configuration in UE from one SN to another. In case of inter-SN CPC, the Conditional Secondary Node Change procedure initiated either by the MN or SN is also used for inter-SN CPC configuration and inter-SN CPC execution.

…

MN initiated conditional SN Change

The MN initiated conditional inter-SN change procedure is used for inter-SN CPC configuration and inter-SN CPC execution.

Figure 10.5.1-3: Conditional SN Change – MN initiated

…

3.	The MN sends to the UE an RRCConnectionReconfiguration message including the CPC configuration, i.e. a list of RRCConnectionReconfiguration* messages and associated execution conditions, in which each RRCConnectionReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 2 and possibly an MCG configuration. Besides, the RRCConnectionReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements.

4.	The UE applies the RRCConnectionReconfiguration message received in step 3, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

4a.	Upon receiving the RRCConnectionReconfigurationComplete message from the UE, the MN triggers the Data Forwarding Address Indication procedure to the source SN to inform that the CPC has been configured, the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding.

NOTE 5b:	Separate Data Forwarding Address Indication procedures may be invoked to provide different forwarding addresses of the prepared candidate target SNs. In this case, it is up to the MN and the source SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional SN change procedures.

NOTE 5c:	For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s).

5.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration* message corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete* message, including an NR RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

6a-6b.	The MN triggers the MeNB initiated SgNB Release procedure to inform the source SN to stop providing user data to the UE, and, if applicable, the address of the SN of the selected candidate PSCell to start data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

8.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 5.

…

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for inter-SN CPC configuration and inter-SN CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing SN initiated inter-SN CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

NOTE 6a0:	To modify or release an existing intra-SN CPC configuration, the source SN triggers an SN initiated Conditional SN Modification (with or without SRB3) without MN involvement, as specified in 10.3.

Figure 10.5.1-4: Conditional SN Change – SN initiated

…

6.	The MN sends to the UE an RRCConnectionReconfiguration message including the CPC configuration, i.e. a list of RRCConnectionReconfiguration* messages and associated execution conditions, in which each RRCConnectionReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCConnectionReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required conditional measurements.

7.	The UE applies the RRCConnectionReconfiguration message received in step 6, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message, which can include an NR RRCReconfigurationComplete*** message. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

8.	If an NR RRC response message is included, the MN informs the source SN with the NR RRCReconfigurationComplete*** message via SgNB Change Confirm message. If step 4 and 5 are skipped, the MN will indicate the candidate PSCells accepted by each candidate SN to the source SN in the SgNB Change Confirm message.

The MN sends the SgNB Change Confirm message towards the source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the candidate SN(s), the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SgNB ID and list of data forwarding addresses to the source SN.

NOTE 6b:	The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some PDCP SDUs if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional PSCell change.

NOTE 6c:	For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s).

9a-9d.	The source SN may send the SgNB Modification Required message to trigger an update of CPC execution condition and/or corresponding SCG measurement configuration for CPC. In such case in step 9b, the MN reconfigures the UE and in step 9c the UE responds with RRCConnectionReconfigurationComplete, similarly as in steps 6 and 7.

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies the RRCConnectionReconfiguration* message corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete* message, including the NR RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

11a-11b.	The MN triggers the MeNB initiated SgNB Release procedure to inform source SN to stop providing user data to the UE, and if applicable, provides the address of the SN of the selected candidate PSCell to start late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 10.

…

[TS 37.340, clause 10.6]

A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change or PCell change is triggered. Intra-SN CPC without MN involvement, inter-SN CPC initiated either by MN or SN are supported.

The following principles apply to CPC:

-	The CPC configuration contains the configuration of CPC candidate PSCell(s) and execution condition(s) and may contain the MCG configuration for inter-SN CPC, to be applied when CPC execution is triggered.

-	An execution condition may consist of one or two trigger condition(s) (see CondEvent, as defined in TS 38.331 [4] or TS 36.331 [10]). Only single RS type and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be used for the evaluation of CPC execution condition of a single candidate PSCell.

-	Before any CPC execution condition is satisfied, upon reception of PSCell change command or PCell change command, the UE executes the PSCell change procedure as described in clause 10.3 and 10.5 or the PCell change procedure as described in clause 9.2.3.2 in TS 38.300[3] or clause 10.1.2.1 in TS 36.300 [2], regardless of any previously received CPC configuration. Upon the successful completion of PSCell change procedure or PCell change procedure, the UE releases all stored CPC configurations.

-	While executing CPC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s) or PCell(s).

-	Once the CPC procedure is executed successfully, the UE releases all stored conditional reconfigurations (i.e. for CPC and for CHO, as specified in TS 38.300 [3] or TS 36.300 [2]).

-	Upon the release of SCG, the UE releases the stored CPC configurations.

-	MN can inform SN of the maximum number of conditional reconfigurations the SN is allowed to configure for SN initiated CPC including both intra-SN and inter-SN CPC.

CPC configuration in HO command, in PSCell addition/change command or within any conditional reconfiguration (i.e CPA, CPC or CHO configuration) is not supported.

8.2.3.18.4.3	Test description

8.2.3.18.4.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell, NR Cell 1 is the source PSCell and NR Cell 3 and NR Cell 6 are the candidate target PSCell.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.2-1.

-	Else, 	the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC), Bearers (MCG(s) and SCG) and Test Loop Function (On) with UE test loop mode B activated according to TS 38.508-1 [4], table 4.5.4.2-1.

8.2.3.18.4.3.2	Test procedure sequence

Tables 8.2.3.18.4.3.2-1 and 8.2.3.18.4.3.2-2 illustrate the downlink power levels to be applied for NR Cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and T2, are applied at the point indicated in the Main behaviour description in Table 8.2.3.18.4.3.2-3.

Table 8.2.3.18.4.3.2-1: Power levels in FR1

|  | Parameter | Unit | E-UTRA Cell 1 | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | - | - | Power levels are such that event B1 is not satisfied for NR Cell 3 and NR Cell 6. |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -94 | -125 | -125 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  | - | - | Power levels are such that entry condition for event B1 is satisfied for NR Cell 3 |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -94 | -85 | -125 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 |  | - | - | Power levels are such that entry condition for event B1 is satisfied for NR Cell 6 |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -94 | -125 | -94 |  |

Table 8.2.3.18.4.3.2-2: Power levels in FR2

|  | Parameter | Unit | E-UTRA Cell 1 | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Power levels are such that event B1 is not satisfied for NR Cell 3 and NR Cell 6. |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -91 | -125 | -125 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Power levels are such that entry condition for event B1 is satisfied for NR Cell 3 |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -91 | -82 | -125 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Power levels are such that entry condition for event B1 is satisfied for NR Cell 6 |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -91 | -125 | -82 |  |

Table 8.2.3.18.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing NR RRCReconfiguration message to perform conditional PSCell change with ConditionalReconfiguration to set NR Cell 3 and NR Cell 6 as target candidate cells. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 2 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 3 or Cell 6 within 10s? | --> | (PRACH Preamble) | 1 | F |
| 3 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on E-UTRA Cell 1 and the ID of the selected conditional reconfiguration associated with NR Cell 3? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete, CondReconfigurationId) | 2 | P |
| 5 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 3 within 10s? | --> | (PRACH Preamble) | 2 | P |
| 6 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on SCG DRB using NR radio path? | - | - | 2 | - |
| 8 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 9 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 6 within 10s? | --> | (PRACH Preamble) | 3 | F |

8.2.3.18.4.3.3	Specific message contents

Table 8.2.3.18.4.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.3.18.4.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition Inter-SN CPC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.2.3.18.4.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.2.3.18.4.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.4.3.3-2: MeasConfig (Table 8.2.3.18.4.3.3-1)

| Derivation Path: TS 36.508 [7] Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1.. maxObjectId)) OF MeasObjectToAddMod { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| MeasObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.2.3.18.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| MeasObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR with condition CPC_NR Cell 3 | Table 8.2.3.18.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | entry 3 |  |
| MeasObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR with condition CPC_NR Cell 6 | Table 8.2.3.18.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | reportConfigInterRAT-condEventB1 | Table 8.2.3.18.4.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 3 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[3] SEQUENCE { |  | entry 3 |  |
| measId | 3 |  |  |
| measObjectId | 3 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.1.3.3-3: MeasObjectNR (Table 8.2.3.18.1.3.3-2)

| Derivation Path: TS 36.508 [7] Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| carrierFreq-r15 | ARFCN-ValueNR for SSB of NR Cell 1 | ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
|  | ARFCN-ValueNR for SSB of NR Cell 3 | ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 | CPC_NR Cell 3 |
|  | ARFCN-ValueNR for SSB of NR Cell 6 | ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 | CPC_NR Cell 6 |
| } |  |  |  |

Table 8.2.3.18.4.3.3-4: reportConfigInterRAT-condEventB1 (Table 8.2.3.18.4.3.3-2)

| Derivation Path: TS 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| condReconfigurationTriggerNR-r17 SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventB1-NR-r17 SEQUENCE { |  |  |  |
| b1-ThresholdNR-r17 CHOICE { |  |  |  |
| nr-RSRP-r15 | 39 |  |  |
| } |  |  |  |
| hysteresis-r17 | 0 |  |  |
| timeToTrigger-r17 | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.4.3.3-5: ConditionalReconfiguration (Table 8.2.3.18.4.3.3-1)

| Derivation Path: TS 36.508 [7] Table 4.6.5-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16 { | 2 entries |  |  |
| CondReconfigurationAddMod-r16 [1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16 ::= SEQUENCE { |  |  |  |
| MeasId[1] | 2 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | OCTET STRING including RRCConnectionReconfiguration with condition CPC_NR Cell 3 | Table 8.2.3.18.4.3.3-6 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16 [2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16 ::= SEQUENCE { |  |  |  |
| MeasId[1] | 3 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | OCTET STRING including RRCConnectionReconfiguration with condition CPC_NR Cell 6 | Table 8.2.3.18.4.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.4.3.3-6: RRCConnectionReconfiguration (Table 8.2.3.18.4.3.3-5)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including RRCReconfiguration with condition CPC_NR Cell 3 | Table 8.2.3.18.4.3.3-7 | CPC_NR Cell 3 |
|  | OCTET STRING including RRCReconfiguration with condition CPC_NR Cell 6 | Table 8.2.3.18.4.3.3-7 | CPC_NR Cell 6 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.4.3.3-7: RRCReconfiguration (Table 8.2.3.18.4.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup SEQUENCE { | CellGroupConfig with condition PCI_NR Cell 3 | OCTET STRING (CONTAINING Table 8.2.3.18.4.3.3-8) | CPC_NR Cell 3 |
|  | CellGroupConfig with condition PCI_NR Cell 6 | OCTET STRING (CONTAINING Table 8.2.3.18.4.3.3-8) | CPC_NR Cell 6 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.3.18.4.3.3-8: CellGroupConfig (Table 8.2.3.18.4.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  | PCI_NR Cell 3 |
|  | Physical Cell Identity of NR Cell 6 |  | PCI_NR Cell 6 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.3.18.4.3.3-9: RRCReconfigurationComplete (step 4)

| Derivation Path: TS 36.508 [7], Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| scg-ConfigResponseNR-r15 |  | OCTET STRING including the RRCReconfigurationComplete message according TS 38.508-1 [67], table 4.6.1-14. |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| selectedCondReconfigurationToApply-r17 |  | 1 |  | CPC_NR Cell 3 |
|  |  | 2 |  | CPC_NR Cell 6 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| CPC_NR Cell 3 | When NR Cell 3 is selected as target PSCell |
| CPC_NR Cell 6 | When NR Cell 6 is selected as target PSCell |

<End of Changes>