3GPP TSG-RAN5 Meeting #102	R5-240387r2

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0579 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UE capability for inter-SN conditional PSCell change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE capability for inter-SN conditional PSCell change was missing in TS 38.508-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding UE capability for inter-SN conditional PSCell change in TS 38.508-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases of inter-SN conditional PSCell change cannot be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0435 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Correcting CR number and Tdoc number to solve 3GU issue. The 2nd revision: - Replace hyphens with underscore in mnemonics of new PICS. |  |  |  |  |  |  |  |  |

<Start of Changes>

A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item | UE Mobility Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support inter-RAT Handover to EUTRA connected to EPC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_EUTRA_Handover | Yes |  |  |
| 2 | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. | 38.306, 4.2.9 | Rel-15 | pc_handoverInterF | Yes |  |  |
| 3 | Support Handover between FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_FR1toFR2_Handover | Yes |  |  |
| 4 | Support Handover between FDD and TDD | 38.306, 4.2.9 | Rel-15 | pc_FDDtoTDD_Handover | Yes |  |  |
| 5 | Support inter-RAT Handover to E-UTRA connected to 5GC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_eLTE_Handover | Yes |  |  |
| 6 | Support inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC | 36.306, 4.3.34.9 | Rel-15 | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 | Yes |  |  |
| 7 | Support inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC | 36.306, 4.3.34.8 | Rel-15 | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 | Yes |  |  |
| 8 | Support inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC | 36.306, 4.3.34.11 | Rel-15 | pc_eutra_EPC_HO_ToNR_TDD_FR2_r15 | Yes |  |  |
| 9 | Support intra-frequency DAPS handover | 38.306, 4.2.7.5 | Rel-16 | pc_intraFreqDAPS_r16 | No |  | It is mandated if the UE supports asynchronous intra-frequency DAPS handover |
| 10 | Support inter-RAT Handover from NR to EN-DC | 38.306, 4.2.10 | Rel-16 | pc_interRAT_NR_ToENDC | CY |  | It is mandated if the UE supports EN-DC. |
| 11 | Support conditional handover | 38.306, 4.2.7.2 | Rel-16 | pc_condHandover_r16 | No |  |  |
| 12 | Support conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover | 38.306, 4.2.7.2 | Rel-16 | pc_condHandoverFailure_r16 | No |  |  |
| 13 | Support 2 trigger events for same execution condition of conditional handover | 38.306, 4.2.7.2 | Rel-16 | pc_condHandoverTwoTriggerEvents_r16 | CY |  | It is mandated if the UE supports condHandover-r16. |
| 14 | Support inter-RAT Handover from NR to UTRA-FDD CELL_DCH CS | 38.306, 4.2.9 | Rel-16 | pc_handoverUTRA_FDD_r16 | No |  |  |
| 15 | Support inter-frequency DAPS handover | 38.306, 4.2.7.4 | Rel-16 | pc_interFreqDAPS_r16 | No |  | It is mandated if the UE supports asynchronous inter-frequency DAPS handover or supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |
| 16 | UE supports asynchronous intra-frequency DAPS handover | 38.306, 4.2.7.5 | Rel-16 | pc_intraFreqAsyncDAPS_r16 | No |  |  |
| 17 | UE supports asynchronous inter-frequency DAPS handover | 38.306, 4.2.7.5 | Rel-16 | pc_interFreqAsyncDAPS_r16 | No |  |  |
| 18 | UE supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover | 38.306, 4.2.7.5 | Rel-16 | pc_ inteFreqDiffSCS_DAPS_r16 | No |  |  |
| 19 | Support conditional PSCell change | 38.306, 4.2.7.2 | Rel-16 | pc_condPSCellChange_r16 | No |  |  |
| 20 | Support handover from 5GS to EPC/ePDG | 24.302, 7.2.2.1 | Rel-15 | pc_HO_from_5GS_to_EPC_ePDG | No |  |  |
| 21 | Support handover from EPC/ePDG to 5GS | 23.502, 4.11.4.1 | Rel-15 | pc_HO_from_EPC_ePDG_to_5GS | No |  |  |
| 22 | Support Handover from EPS to 5GC-N3IWF | 23.502, 4.11.3.1 | Rel-15 | pc_HO_from_EPS_to_5GC_N3IWF | No |  |  |
| 23 | Support Handover from 5GC-N3IWF to EPS | 23.502, 4.11.3.2 | Rel-15 | pc_HO_from_5GC_N3IWF_to_EPS | No |  |  |
| 24 | Support Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access | 23.502, 4.9.2.1 | Rel-15 | pc_HO_from_5GC_N3IWF_to_5GC | No |  |  |
| 25 | Support Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access | 23.502, 4.9.2.2 | Rel-15 | pc_HO_from_5GC_to_5GC_N3IWF | No |  |  |
| 26 | Supports location based conditional handover, i.e., CondEvent D1 | 38.306, 4.2.7.2 | Rel-17 | pc_locationBasedCondHandover_r17 | No |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |
| 27 | Support time based conditional handover, i.e., CondEvent T1 | 38.306, 4.2.7.2 | Rel-17 | pc_timeBasedCondHandover_r17 | No |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |
| X1 | Support of MN initiated conditional PSCell change in NR-DC | 38.306, 4.2.7.2 | Rel-17 | pc_mn_InitiatedCondPSCellChangeNRDC_r17 | No |  | A UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. |
| X2 | Support of MN initiated conditional PSCell change within all supported FR1-FDD bands in EN-DC | 38.306, 4.6.9a | Rel-17 | pc_mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17 | No |  | The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. |
| X3 | Support of MN initiated conditional PSCell change within all supported FR1-TDD bands in EN-DC | 38.306, 4.6.9a | Rel-17 | pc_mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17 | No |  | The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. |
| X4 | Support of MN initiated conditional PSCell change within all supported FR2-TDD bands in EN-DC | 38.306, 4.6.9a | Rel-17 | pc_mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17 | No |  | The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. |

<End of Changes>