3GPP TSG-RAN5 Meeting #102	R5-240616r1

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3059 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RF TRx testing - P-Max configuration extension to RX tests to enable TxD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Custom P-Max configuration for RF test cases defined in 38.508-1, section 5.4.1, only applies for Tx test cases. That means that for RX tests, the default configuration in section 4.6.3 applies, that is limited to power class 3. In such a case, for power classes other than 3, in RX test cases, the UE will never exercise Tx Diversity even if supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In message exceptions for RF in section 5.4.1, applied P-Max exception not limited to Tx test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TxD will not be correctly exercised in Rx test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -r1: corrected WI code from TEI15_Test to TEI17_Test. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## 5.4	Default NG-RAN RRC message and information elements contents

### 5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

For test cases in which Power Class 3 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-1; For test cases in which Power Class 2 UEs verifying Power Class 2 requirements, refer to Table 5.4.1-2; For test cases in which Power Class 1.5 UEs verifying Power Class 1.5 requirements, refer to Table 5.4.1-2A.

Table 5.4.1-1: P-Max-PC3

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |
|  | 23 |  | EN-DC |

Table 5.4.1-2: P-Max-PC2

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |
|  | 26 |  | EN-DC |

| Condition | Explanation |
| --- | --- |
| SA | NR standalone RF tests cases |
| EN-DC | EN-DC RF tests cases specified in 38.521-3 |

Table 5.4.1-2A: P-Max-PC1.5

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |

Table 5.4.1-3: Void

## <<< END OF CHANGES >>>