3GPP TSG-RAN5 Meeting #102	R5-240650r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4265 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer Test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 6.7.1.4 needs to be added as per NTN WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 6.7.1.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR ABCD |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact, Dependent on approval of R5-240648 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Remark for T2 has been removed, comment changed for Test procedure sequence for T0 (to provide further clarity), Exception for Rel.15 UE removed as it is not required for NTN UEs |  |  |  |  |  |  |  |  |

<Beginning of changes>

<Unchanged Sections Omitted >

#### 6.7.1.4	NR-NTN / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer

6.7.1.4.1	Test Purpose (TP)

(1)

with { UE configured with “MinimumPeriodicSearchTimer“ and camped on an VPLMN cell}
ensure that {
  when {Cells of a higher priority PLMN available, the higher priority PLMN search timer T stored in the USIM }
then { T shall be set to the MinimumPeriodicSearchTimer.

When the T expires the UE selects and camps on a cell of the highest priority PLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user }
             }

6.7.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1 and 4.4.3.3.1.

Unless otherwise stated these are Rel-17 requirements.

[TS 23.122, clause 4.4.3.1]

At switch on, following recovery from lack of coverage, or when the MS stops operating in the SNPN access mode, the MS selects the registered PLMN or equivalent PLMN (if it is available) using all access technologies that the MS is capable of and if necessary (in the case of recovery from lack of coverage, see clause 4.5.2) attempts to perform a Location Registration.

NOTE 1:	The MS in automatic network selection mode can end the PLMN search procedure once the registered PLMN or equivalent PLMN is found on an access technology.

NOTE 2:	An MS in automatic network selection mode can use location information to determine which PLMNs can be available in its present location.

…..

If there is no registered PLMN, or if registration is not possible due to the PLMN being unavailable or registration failure, the MS follows one of the following two procedures depending on its PLMN selection operating mode. At switch on, if the MS provides the optional feature of user preferred PLMN selection operating mode at switch on then this operating mode shall be used. Otherwise, the MS shall use the PLMN selection mode that was used before switching off.

….

The MS selects and attempts registration on other PLMN/access technology combinations, if available and, for bullets i, ii, iii, iv, v, allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

NOTE 1:	High quality signal is defined in the appropriate AS specification.

v)	other PLMN/access technology combinations in order of decreasing signal quality.

vi)	PLMN/NG-RAN combinations for any forbidden PLMNs broadcasting the PLMN ID of the MS determined PLMN with disaster condition or broadcasting the disaster related indication and matching the below conditions:

a)	if the indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' is set to true:

-	each PLMN in the "list of PLMN(s) to be used in disaster condition" stored in the ME which is associated with the PLMN ID of the MS determined PLMN with disaster condition, if any, ordered based on this list; otherwise

-	if the ME does not have a stored "list of PLMN(s) to be used in disaster condition" associated with the PLMN ID of the MS determined PLMN with disaster condition, each PLMN in the "list of PLMN(s) to be used in disaster condition" stored in the ME which is associated with the PLMN ID of the HPLMN, if any, ordered based on this list.

b)	if the indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' is set to false:

-	each PLMN in the "list of PLMN(s) to be used in disaster condition" stored in the ME which is associated with the HPLMN, if any, ordered based on this list.

vii)	PLMN /NG-RAN combinations for other forbidden PLMNs broadcasting the PLMN ID of the MS determined PLMN with disaster condition or broadcasting the disaster related indication, in random order.

When following the above procedure the following requirements apply:

. . .

t)	In i to vii, if the MS detects a PLMN in satellite NG-RAN access technology which fulfils the conditions related to the list of "PLMNs not allowed to operate at the present UE location" as defined in clause 3.1, it shall not consider the PLMN as PLMN selection candidate for satellite NG-RAN access technology.

. . .

[TS 23.122, clause 4.4.3.3.1.1]

If the MS is in a VPLMN and not registered for disaster roaming services, the MS shall periodically attempt to obtain service on its HPLMN (if the EHPLMN list is not present or is empty) or one of its EHPLMNs (if the EHPLMN list is present) or a higher priority PLMN/access technology combinations listed in "user controlled PLMN selector" or "operator controlled PLMN selector" by scanning in accordance with the requirements that are applicable to i), ii) and iii) as defined in the Automatic Network Selection Mode in clause 4.4.3.1.1. For this purpose, a value of timer T may be stored in the SIM. The interpretation of the stored value depends on the radio capabilities supported by the MS:

-	For an MS that does not support any of the following: EC-GSM-IoT, Category M1 or Category NB1 (as defined in 3GPP TS 36.306 [54]), if:

a)	the MS is in a VPLMN through satellite NG-RAN access or satellite E-UTRAN access with a shared MCC, T is in the range 6 multiplied by integer M minutes to 8 multiplied by integer M hours in 6 multiplied by integer M minutes steps or T indicates that no periodic attempts shall be made. If no value for M is stored in the SIM, a default value of M equal to one is used; otherwise

b)	T is either in the range 6 minutes to 8 hours in 6 minutes steps or it indicates that no periodic attempts shall be made. If no value for T is stored in the SIM, a default value of 60 minutes is used for T.

. . .

If the MS is configured with the MinimumPeriodicSearchTimer as specified in 3GPP TS 24.368 [50] or 3GPP TS 31.102 [40], the MS shall not use a value for T that is less than the MinimumPeriodicSearchTimer. If the value stored in the SIM, or the default value for T (when no value is stored in the SIM), is less than the MinimumPeriodicSearchTimer, then T shall be set to the MinimumPeriodicSearchTimer.

6.7.1.4.3	Test description

6.7.1.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 11, 12 and 13 as specified in TS 38.508-1 [4] Table 4.4.2-3.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.7.1.4.3.1.1-1.

Table 6.7.1.4.3.1.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN1 |
| NR Cell 12 | PLMN15 |
| NR Cell 13 | PLMN16 |

UE:

The UE is in Automatic PLMN selection mode.

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

-	The UE is registered to PLMN15 before it is switched off.

-	The UE is configured with a value of MinimumPeriodicSearchTimer set to 6 minutes

-	The UE is equipped with a USIM containing default values (as per TS 38.508-1) except for those listed in Table 6.7.1.4.3.1–1.

Table 6.7.1.4.3.1–1: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | Default | Default |
|  | 2 | PLMN16 | All specified |
| EFOPLMNwACT | 1 | PLMN15 | All specified |
|  |  | Remaining defined entries use default values |  |
| EFHPLMNwAcT | 1 | PLMN1 | NR-NTN |
| EFHPPLMN |  | 1 (6 minutes) |  |
| EFNASCONFIG |  | MinimumPeriodicSearchTimer set to [6 minutes] |  |

Preamble:

-	The UE performs a successful registration on PLMN15 after which it is switched OFF (State 0N-B) as per TS 38.508-1 [4] table 4.4A.2-0.

6.7.1.4.3.2	Test procedure sequence

Table 6.7.1.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" to be applied during preamble condition, while columns marked "T1" and “T2” to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.7.1.4.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 11 | NR Cell 12 | NR Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | -88 | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | -88 |  |

Table 6.7.1.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.6.3.2-1/2. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3-22a1 | Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 are performed on NR Cell 12. | - | - | - | - |
| 23 | Check: Does the UE send an RRCSetupRequest on NR Cell 13 after 2 minutes, but before 6 minutes (“MinimumPeriodicSearchTimer”) from power on? (Note 1 and Note 2) | --> | NR RRC: RRCSetupRequest | 1 | P |
| 24-25 | Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 26a1-26a16a1 | IF 5GS registration type is set as Initial Registration in step 25, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13. | - | - | - | - |
| 26b1-26b3a1 | IF 5GS registration type is set as Mobility Registration in step 25, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 13. | - | - | - | - |
| 27 | SS adjusts cell levels according to row T2 of table 6.1.1.6.3.2-1/2. | - | - | - | - |
| 28 | Check: Does the UE send an RRCSetupRequest on NR Cell 11 after 6 minutes (“MinimumPeriodicSearchTimer”) from step 27? (Note 1) | --> | NR RRC: RRCSetupRequest | 1 | P |
| 29-30 | Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 31a1-31a16a1 | IF 5GS registration type is set as Initial Registration in step 45, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 11. | - | - | - | - |
| 31b1-31b3a1 | IF 5GS registration type is set as Mobility Registration in step 45, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| Note 1: Timers in Steps 23 and 40 are derived from the value defined by the “MinimumPeriodicSearchTimer”. Note 2: Tolerance of 5min is added to allow time for the UE to find the proper PLMN. |  |  |  |  |  |

##### 6.7.1.4.3.3	Specific message contents

Table 6.7.1.4.3.3-1: SIB19 for on NR Cell 11 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [6], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| EphemerisInfo-r17 ::= CHOICE | Refer to the Second cell in TS 38.508-1 Table 6.3.4.1-1 |  | GSO |
| EphemerisInfo-r17 ::= CHOICE | FFS |  | NGSO |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.4.3.3-2: SIB19 for on NR Cell 12 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [6], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| EphemerisInfo-r17 ::= CHOICE | Refer to the First cell in TS 38.508-1 Table 6.3.4.1-1 |  | GSO |
| EphemerisInfo-r17 ::= CHOICE | FFS |  | NGSO |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.4.3.3-3: SIB19 for on NR Cell 13 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [6], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| EphemerisInfo-r17 ::= CHOICE | Refer to the Third cell in TS 38.508-1 Table 6.3.4.1-1 |  | GSO |
| EphemerisInfo-r17 ::= CHOICE | FFS |  | NGSO |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<Unchanged Sections Omitted >

<End of Changes >