3GPP TSG-RAN5 Meeting #102	R5-240703r2

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2923 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EN-DC and NR SA SCell activation test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Minimum requirements need to be updated to align with 38.133 - 4.5.3.1, 4.5.3.2, 4.5.3.3 - Editor’s note should be removed since the TT analysis for 15kHz SCS+30kHz SCS test configuration is already added in 38.903 (“38.533 4.5.3+6.5.3 TT”). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.0, 4.5.3.1, 4.5.3.2, 4.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - WIC and affected clause on coversheet are corrected to fix 3GU issues. 2nd revision: - Changes to the start timing of T2 is reversed based on comments. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.5.3.0	Minimum conformance requirements

4.5.3.0.1	SCell activation and deactivation delay

The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in 38.213 [8].

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB + 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

-	TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

If the SCell being activated belongs to FR2, and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB + 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in TS 38.133 [6] clause 3.6.3.

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell.

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE is not provided with any SMTC for the target SCell, Tactivation_time is 3ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3 ms + max(Tuncertainty_MAC +TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0, if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP)

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max ((THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)).

Where,

TSMTC_MAX:

-	In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There is no requirements if the SSB transmission periodicity is not 5ms.

TFirstSSB: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after n +  THARQ+3msNR slot length.

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in TS 38.133 [6] clause 9.5 assuming M=1.

TL1-RSRP,report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in [13].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5 measCycleSCell, 5 DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in TS 38.133 [6] section 9.2 and 9.3.

-	the SSB measured during the period equal to max(5 measCycleSCell, 5 DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in TS 38.133 [6] section 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

The requirements for FR1 unknown SCell activation specified in this clause apply when one of the following conditions is met

-	‘ssb-PositionInBurst' indicates only one SSB is being actually transmitted, or

-	‘ssb-PositionInBurst' indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell  activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in TS 38.133 [6] clause 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [13] signalling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [13] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in TS 38.133[6] clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes TFineTiming.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requirements  on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the SCell.Starting from the slot specified in clause 4.3 of TS 38.213 [8] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [8] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

Upon receiving SCell deactivation command or upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + THARQ+3msNR slot length.

The starting point of an interruption on SpCell or any activated SCell in the same cell group as SCell being deactivated for NR standalone, EN-DC, NE-DC or NR-DC mode specified in clause 8.2 shall not occur before slot  n+1+THARQNR slot length and not occur after slot  n+1+THARQ+3msNR slot length. where NR slot length is with respect to the numerology used in the SCell being deactivated.

Upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + 3msNR slot length. The starting point of an interruption window on spCell or any activated SCell, as specified in TS 38.133 [6] clause 8.2, shall not occur before slot n+1 and not occur after slot n+1+ 3msNR slot length, where NR slot length is with respect to the numerology used in the SCell being deactivated.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The normative reference for this requirement is TS 38.133 [6] clause 8.3.

<End of modified section 1>

<Start of modified section 2>

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle

4.5.3.1.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements, when the SCell in FR1 is known by the UE at the time of activation.

4.5.3.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards and supporting 2DL CA in NR.

4.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 4.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.3.1.

4.5.3.1.4	Test description

4.5.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for LTE PCell and NR PSCell are shown in Table 4.5.3.1.4.1-1. Supported test configurations for NR SCell are shown in Table 4.5.3.1.4.1-1A below. Test configuration for LTE PCell and NR PSCell and test configuration for NR SCell are chosen independently.

Table 4.5.3.1.4.1-1: supported test configurations for LTE PCell and NR PSCell

| Config | Description |
| --- | --- |
| 4.5.3.1-1 | LTE FDD, NR 15 kHz SSB SCS, ≥10MHz bandwidth, FDD duplex mode |
| 4.5.3.1-2 | LTE FDD, NR 15 kHz SSB SCS, ≥10MHz bandwidth, TDD duplex mode |
| 4.5.3.1-3 | LTE FDD, NR 30 kHz SSB SCS, ≥40MHz bandwidth, TDD duplex mode |
| 4.5.3.1-4 | LTE TDD, NR 15 kHz SSB SCS, ≥10MHz bandwidth, FDD duplex mode |
| 4.5.3.1-5 | LTE TDD, NR 15 kHz SSB SCS, ≥10MHz bandwidth, TDD duplex mode |
| 4.5.3.1-6 | LTE TDD, NR 30 kHz SSB SCS, ≥40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table 4.5.3.1.4.1-1A: supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Configure the test equipment and the DUT according to the parameters in Table 4.5.3.1.4.1-2 and Table 4.5.3.1.4.1-3.

Table 4.5.3.1.4.1-2: Initial conditions for known FR1 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.3.1.5-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 4.5.3.1.4.1-3: General test parameters for known FR1 SCell activation case,
160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in section A.3.7.2.1 of TS38.133 [6] |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell3 timing offset to cell2 | s | 0 |  |
| Time alignment error between cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1*NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13] |
| K | slot |  | As specified in section 4.3 of TS 38.213 [8] |

1.	Message contents are defined in clause 4.5.3.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells. Cell 2 is the PSCell and Cell 3 is the deactivated SCell.

4.5.3.1.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+THARQ+Tactivation_time+TCSI_ReportingNR slot length. The UE shall start reporting CSI in PSCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell or PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3ms+TXNR slot length+Ninterruption. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [23] section 7.32.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n+THARQ+3msNR slot length. The starting point of any PSCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length. The starting point of any E-UTRA PCell interruption due to the deactivation shall occur in the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length, where n1 and n2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot n.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 4.5.3.1.5-1 and A.6.1.1-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 4.5.3.1.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete.

3a.	The UE send a MeasurementReport message embedded in ULInformationTransferMRDC.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m and T2 starts in slot m. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

6.	After at least one CSI-RS transmission occasion for channel measurement, the UE shall start sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PSCell according to the following criteria:

-	If the first CSI report for SCell is received by the SS no later than slot sfirst_CSI=m+1+THARQ+3ms+TCSI_ReportingNR slot length,

-	or slot sfirst_CSIint=m+1+THARQ+3ms+TCSI_Reporting+TXNR slot length+Ninterruption+1 if the slot sfirst_CSI was subject to interruption,

-	or the next CSI report occasion if there is no CSI report occasion in slots sfirst_CSI or sfirst_CSIint,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot snonzero_CQI=m+THARQ+Tactivation_time+TCSI_ReportingNR slot length,

-	or slot snonzero_CQIint=maxsfirst_CSIint,snonzero_CQI if slot snonzero_CQI was subject to interruption,

-	or the next available CSI report occasion if there is no CSI report occasion for reporting the valid CSI in slot snonzero_CQI, or slot snonzero_CQIint if snonzero_CQI was subject to interruption,

-	and DTX is not observed by the SS outside the slot m+1+THARQNR slot length to m+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 9.

7.	When T2 expires, the SS deactivate SCC by sending the deactivation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n and T3 starts in slot n. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 8, otherwise go to step 9.

8.	The UE shall stop sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PSCell during SCell deactivation.

-	If the last CSI report is received by the SS earlier than or equal to slot n+THARQ+3msNR slot length

-	and DTX is not observed by the SS outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length+Ninterruption+THARQNR slot length up to the end of T3,

-	Then the number of successes for the event "Deactivation" is increased by one. Otherwise, count a fail for the event "Deactivation".

9.	When T3 expires, or Activation in step 5 was not acknowledged, or a fail was counted for the event "Activation" in step 6, or Deactivation in step 7 was not acknowledged, the SS shall transmit an RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508[25] Table 4.6.1-8 to release NR PSCell. The UE shall transmit an RRCConnectionReconfigurationComplete message.

10.	Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

11.	The SS shall transmit an RRCConnectionReconfiguration message with condition MCG and SCG according to TS 36.508 [25] Table 4.6.1-8 to ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. The UE shall transmit an RRCConnectionReconfigurationComplete message.

12.	Repeat steps 2-11 until a test verdict has been achieved.

Each of the events "Activation" and "Deactivation" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

4.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with condition SCELL_CSI_ON_SPCELL with the following exceptions:

Table 4.5.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-7 with Condition Deactivated SCell |

Table 4.5.3.1.4.3-2: RRCReconfiguration in step 3: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition EN-DC_MEAS and EN-DC_SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig-SCell | Table 4.5.3.1.4.3-5 |  |
| measConfig | MeasConfig | Table 4.5.3.1.4.3-2A |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.3.1.4.3-2A: MeasConfig (Table 4.5.3.1.4.3-2)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR for SCell | entry 2 Table 4.5.3.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 4.5.3.1.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.3.1.4.3-3: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

Table 4.5.3.1.4.3-4: ReportConfigNR

| Derivation Path: Table H.3.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -30 | To ensure reporting can always be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.3.1.4.3-5: CellGroupConfig-SCell (Table 4.5.3.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex of NR SpCell |  |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 4.5.3.1.4.3-6 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 4.5.3.1.4.3-7 |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.3.1.4.3-6: ServingCellConfig-SpCell (Table 4.5.3.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 1 |  |  |
| } |  |  |  |

Table 4.5.3.1.4.3-7: ServingCellConfig-SCell (Table 4.5.3.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 2 |  |  |
| } |  |  |  |

4.5.3.1.5	Test requirement

Table 4.5.3.1.5-1 and Table 4.5.3.1.5-1A defines the primary level settings including test tolerances for all tests.

Table 4.5.3.1.5-1: Cell specific test parameters for for NR PSCell known FR1 SCell activation case,
160ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  |
| Duplex mode | Config 1,4 |  | FDD |  |  |
|  | Config 2,3,5,6 |  | TDD |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | Note 7 |  |  |
| BWoccupied | Config 1,2,4,5 | RB | 52 Note 5 |  |  |
|  | Config 3,6 |  | 106 Note 6 |  |  |
| DL initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |
| UL initial BWP configuration |  |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |  |
|  | Config 2,5 |  | SR.1.1 TDD |  |  |
|  | Config 3,6 |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |  |
|  | Config 2,5 |  | CR.1.1 TDD |  |  |
|  | Config 3,6 |  | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |  |  |
|  | Config 2,5 |  | CCR.1.1 TDD |  |  |
|  | Config 3,6 |  | CCR.2.1 TDD |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns | Config 1,2,4,5 |  | OP.1 Note 5 |  |  |
|  | Config 3,6 |  | OP.1 Note 6 |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |
|  | Config 3,6 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting | Config 1,4 |  | CSI-RS.1.1 FDD |  |  |
|  | Config 2,5 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3,6 |  | CSI-RS.2.1 TDD |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 |  |  |
|  | Config 3,6 |  | 30 |  |  |
| reportConfigType | Config 1-6 |  | periodic |  |  |
| reportQuantity | Config 1-6 |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1,2,4,5 | slot | 5 |  |  |
|  | Config 3,6 |  | 10 |  |  |
| CSI reporting offset | Config 1,2,4,5 | slot | 2 |  |  |
|  | Config 3,6 |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 |  |  |
| Note2 | Config 1,2,4,5 | dBm/SCS | -104 |  |  |
|  | Config 3,6 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRP Note3 | Config 1,2,4,5 | dBm/SCS | -87 |  |  |
|  | Config 3,6 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| IoNote3 | Config 1,2,4,5 | dBm/9.36MHz | -58.96 |  |  |
|  | Config 3,6 | dBm/38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna configuration |  |  | 2x2 Low |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |

Table 4.5.3.1.5-1A: Cell specific test parameters for NR SCell for known FR1 SCell activation case,
160ms SCell measurement cycle

| Parameter |  | Unit | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq2 |  |  |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not Applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | Note 7 |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |
| DL initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |
| UL initial BWP configuration |  |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | SR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel | ConfigSCell 1 |  | CR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel | ConfigSCell 1 |  | CCR.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CCR.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CCR.2.1 TDD |  |  |
| TRS configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |  |  |
|  | ConfigSCell 3 |  | OP.1 Note 6 |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| SSB configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |
| PDSCH/PDCCH subcarrier spacing | ConfigSCell 1,2 | kHz | 15 |  |  |
|  | ConfigSCell 3 |  | 30 |  |  |
| reportConfigType |  |  | N/A |  |  |
| reportQuantity |  |  | N/A |  |  |
| CSI reporting periodicity |  | slot | N/A |  |  |
| CSI reporting offset |  | slot | N/A |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -87 |  |  |
|  | ConfigSCell 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| IoNote3 | ConfigSCell 1,2 | dBm/9.36MHz | -58.96 |  |  |
|  | ConfigSCell 3 | dBm/38.16MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell no later than the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot m+THARQ+3msNR slot length. UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption. Whether CSI report in slot m+1+THARQ+3ms+TCSI_ReportingNR slot length  was interrupted or not is checked by monitoring ACK/NACK sent in PCell in slot m+1+THARQ+3ms+TCSI_ReportingNR slot length.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in TS 38.133 [6] section 8.3.

Figures 4.5.3.1.5-1 shows the deriviation of the Test procedure requreiment for DTX during T2, based on the core requirements for interruption.

Figure 4.5.3.1.5-1: Procedure derivation for Activation

1)	Activation command for SCell

2)	ACK for MAC-CE for SCell1 activation

3)	First CSI report timing (could be invalid CQI)

4)	First non-zero CSI report transmission

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot n+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

During T2 interruption of PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3ms+TXNR slot length+Ninterruption, and interruption of E-UTRA PCell during SCell activation shall not happen outside the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, as defined in TS 38.133 [6] section 8.3.

During T3 the starting point of interruption of PSCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3, and the starting point of interruption of E-UTRA PCell during SCell deactivation shall not happen outside the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length.

Figures 4.5.3.1.5-2 shows the deriviation of the Test procedure requreiment for NR PSCell DTX during T3, based on the core requirements for interruption.

Figure 4.5.3.1.5-2: Procedure derivation for Deactivation

1)	Deactivation command for SCell

2)	Latest slot stop sending CSI reports for Scell

The interruption of PSCell shall not be more than the values specified for EN-DC in TS 38.133 [6] clause 8.2.1.2.4.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90 %.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in TS 38.133 [6] section 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle

4.5.3.2.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements, when the SCell in FR1 is known by the UE at the time of activation.

4.5.3.2.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards and supporting 2DL CA in NR.

4.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 4.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.3.2.

4.5.3.2.4	Test description

4.5.3.2.4.1	Initial conditions

Same initial conditions as described in section 4.5.3.1.4.1 with following exception:

-	The listed parameter values in Tables 4.5.3.2.4.1-2 will replace the values of corresponding parameters in Tables 4.5.3.1.4.1-3.

Table 4.5.3.2.4.1-1: Void

Table 4.5.3.2.4.1-2: General test parameters for known FR1 SCell activation case, 640ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |

4.5.3.2.4.2	Test procedure

Same test procedure as described in section 4.5.3.1.4.2:

4.5.3.2.4.3	Message contents

Same message contents as described in section 4.5.3.1.4.3 with following exception:

- Table 4.5.3.1.4.3-3 is replaced by Table 4.5.3.2.4.3-1.

Table 4.5.3.2.4.3-1: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| } |  |  |  |

4.5.3.2.5	Test requirement

Same test requirement as described in section 4.5.3.1.5, except Tactivation_time will be replaced with the value TFirstSSB+5msTFirstSSB_MAX + Trs + 5ms.

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX

4.5.3.3.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements, when the SCell in FR1 is unknown by the UE at the time of activation.

4.5.3.3.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards and supporting 2DL CA in NR.

4.5.3.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 4.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.3.3.

4.5.3.3.4	Test description

4.5.3.3.4.1	Initial conditions

Same initial conditions as described in section 4.5.3.1.4.1 with following exception:

-	The listed parameter values in Tables 4.5.3.3.4.1-2 will replace the values of corresponding parameters in Tables 4.5.3.1.4.1-3.

Table 4.5.3.3.4.1-1: Void

Table 4.5.3.3.4.1-2: General test parameters for unknown FR1 SCell activation case,
160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

4.5.3.3.4.2	Test procedure

Same test procedure as described in section 4.5.3.1.4.2, except step 3and step 5 are replaced by following steps:

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 4.5.3.1.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. The SCell (Cell 3) shall be powered OFF till T2 starts.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m, power ON the SCell (Cell3), T2 starts in slot m. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

and:

-	Step 3a is removed.

4.5.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with condition SCELL_CSI_ON_SPCELL with the following exceptions:

Table 4.5.3.3.4.3-1: RRCReconfiguration in step 3: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition EN-DC_SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | Table 4.5.3.3.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.3.3.4.3-2: CellGroupConfig (Table 4.5.3.3.4.3-1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex of NR SpCell |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 4.5.3.3.4.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 4.5.3.3.4.3-4 |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.3.3.4.3-3: ServingCellConfig-SpCell (Table 4.5.3.3.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| } |  |  |  |

Table 4.5.3.3.4.3-4: ServingCellConfig-SCell (Table 4.5.3.3.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| } |  |  |  |

4.5.3.3.5	Test requirement

Same test requirement as described in section 4.5.3.1.5, except Tactivation_time will be replaced with the valueTFirstSSB+5msTFirstSSB_MAX+TSMTC_MAX+2*Trs+5ms TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms

<End of modified section 2>

<Start of modified section 3>

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle

6.5.3.1.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements in TS 38.133 [6] clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

6.5.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and supporting 2DL CA.

6.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.3.1.

6.5.3.1.4	Test description

6.5.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for NR PCell are shown in Table 6.5.3.1.4.1-1. Supported test configurations for NR SCell are shown in Table 6.5.3.1.4.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently.

Table 6.5.3.1.4.1-1: supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 6.5.3.1-1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 6.5.3.1-2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6.5.3.1-3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table 6.5.3.1.4.1-1A: Supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 6.5.3.1-1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 6.5.3.1-2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6.5.3.1-3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.3.1.4.1-2 and Table 6.5.3.1.4.1-3.

Table 6.5.3.1.4.1-2: Initial conditions for known FR1 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.3.1.5-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

Table 6.5.3.1.4.1-3: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | PCell Config 1: 2 PCell Config 2: 3 PCell Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. |

1.	Message contents are defined in clause 6.5.3.1.4.3.

2.	There are two NR carriers, each with one cell. Cell 1 is NR FR1 PCell and Cell 2 is the deactivated SCell. Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

6.5.3.1.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in TS 38.133 [6] clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3, where Ninterruption is the interruption length given in TS 38.133 [6] clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 6.5.3.1.4.1-3 and 6.5.3.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1 and provide measurement configurations.

3a.	The UE sends a MeasurementReport message.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n and T2 starts in slot n. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

6.	After at least one CSI-RS transmission occasion for channel measurement, the UE shall start sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell according to the following criteria:

-	If the first CSI report for SCell is received by the SS no later than slot sfirst_CSI=n+1+THARQ+3ms+TCSI_ReportingNR slot length,

-	or slot sfirst_CSIint=n+1+THARQ+3ms+TCSI_Reporting+TXNR slot length+Ninterruption+1 if the slot sfirst_CSI was subject to interruption,

-	or the next CSI report occasion if there is no CSI report occasion in slots sfirst_CSI or sfirst_CSIint,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot snonzero_CQI=n+THARQ+Tactivation_time+TCSI_ReportingNR slot length,

-	or slot snonzero_CQIint=maxsfirst_CSIint,snonzero_CQI if slot snonzero_CQI was subject to interruption,

-	or the next available CSI report occasion if there is no CSI report occasion for reporting the valid CSI in slot snonzero_CQI, or slot snonzero_CQIint if slot snonzero_CQI was subject to interruption,

-	and DTX is not observed by the SS outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 9.

7.	When T2 expires, the SS deactivate SCC by sending the deactivation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m and T3 starts in slot m. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 8, otherwise go to step 9.

8.	The UE shall stop sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell during SCell deactivation.

-	If the last CSI report is received by the SS earlier than or equal to slot m+THARQ+3msNR slot length

-	and DTX is not observed by the SS outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length+Ninterruption+THARQNR slot length up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

9.	When T3 expires, or Activation in step 5 was not acknowledged, or a fail was counted for the event “Activation” in step 6, or Deactivation in step 7 was not acknowledged, the SS shall transmit a RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

10.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

11.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

12.	Repeat steps 2-11 until a test verdict has been achieved.

Each of the events “Activation” and “Deactivation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

6.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with condition SCELL_CSI_ON_SPCELL with the following exceptions:

Table 6.5.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-7 with Condition Deactivated SCell; |

Table 6.5.3.1.4.3-2: RRCReconfiguration: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 6.5.3.1.4.3-2A |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCell | Table 6.5.3.1.4.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.3.1.4.3-2A: MeasConfig (Table 6.5.3.1.4.3-2)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR for SCell | entry 2 Table 6.5.3.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 6.5.3.1.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-3: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-4: ReportConfigNR

| Derivation Path: Table H.3.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -30 | To ensure reporting can always be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-5: CellGroupConfig-SCell (Table 6.5.3.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present | PCell always uses servCellIndex=0 |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 6.5.3.1.4.3-6 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 6.5.3.1.4.3-7 |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-6: ServingCellConfig-SpCell (Table 6.5.3.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 1 |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-7: ServingCellConfig-SCell (Table 6.5.3.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 2 |  |  |
| } |  |  |  |

6.5.3.1.5	Test requirement

Table 6.5.3.1.5-1 and Table 6.5.3.1.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.5.3.1.5-1: Cell specific test parameters for NR PCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | Note 7 |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1Note 5 |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting Note 8 | Config 1 |  | CSI-RS.1.1 FDD |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | periodic |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |
|  | Config 3 |  | 10 |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |
|  | Config 3 |  | 5 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | Config 1,2 | dBm/SCS | -104 |  |  |
|  | Config 3 |  | -101 |  |  |
| EsIot |  | dB | 17 |  |  |
| EsNoc |  | dB | 17 |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |
|  | Config 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |
| Correlation Matrix and Antenna configuration |  |  | 2x2 Low |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |

Table 6.5.3.1.5-2: Cell specific test parameters for NR SCell for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | Note 7 |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1Note 5 |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | N/A |  |  |
| reportQuantity |  |  | N/A |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |
| EsIot |  | dB | 17 |  |  |
| EsNoc |  | dB | 17 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -87 |  |  |
|  | ConfigSCell 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |  |  |
|  | ConfigSCell 3 | dBm/ 38.16MHz | -52.87 |  |  |
| Correlation Matrix and Antenna configuration |  | 2x2 Low |  |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell no later than the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+1+THARQ+3msNR slot length  UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption. Whether CSI report in slot n+1+THARQ+3ms+TCSI_ReportingNR slot length was interrupted or not is checked by monitoring ACK/NACK sent in PCell in slot n+1+THARQ+3ms+TCSI_ReportingNR slot length.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in TS 38.133 [6] section 8.3.

During T2 interruption of PCell/PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] section 8.3.

Figures 6.5.3.1.5-1 shows the derivation of the Test procedure requirement for DTX during T2, based on the core requirements for interruption.

Figure 6.5.3.1.5-1: Procedure derivation for Activation

1)	Activation command for SCell

2)	ACK for MAC-CE for SCell1 activation

3)	First CSI report timing (could be invalid CQI)

4) 	First non-zero CSI report transmission

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

Figures 6.5.3.1.5-2 shows the derivation of the Test procedure requirement for NR PSCell DTX during T3, based on the core requirements for interruption.

Figure 6.5.3.1.5-2: Procedure derivation for Deactivation

1)	Deactivation command for SCell

2)	Latest slot stop sending CSI reports for SCell

The interruption on any activated serving cell shall not be more than the values specified for SA in TS 38.133 [6] clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in TS 38.133 [6] section 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

<End of modified section 3>