3GPP TSG-RAN5 Meeting #102	R5-240708r1

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2928 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap RRM test case 16.6.5.3 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update test case in following aspects according to TT analysis: - Editor’s note is removed; - Test parameters; - To align with latest version of 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Test procedure is updated. The TE will release MG config before it sends CGI measurement config to avoid causing additional missed ACK/NACKs, which may fail conformant UE. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.6.5.3	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE

16.6.5.3.1	Test purpose

To verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in TS 38.133 clause 9.4A.4.

16.6.5.3.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting acquisition of CGI from neighbour E-UTRA cell using autonomous gap.

16.6.5.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.5.3.

16.6.5.3.4	Test description

The test scenario comprises of one NR carrier and an E-UTRA carrier and two cells. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively:

-	At the start of time duration T1, the UE does not have any timing information of Cell 2.

-	Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to setup.

-	The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

16.6.5.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.5.3.4.1-1.

Table 16.6.5.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.5.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 16.6.5.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.3-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 16.6.5.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 16.6.5.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.3-6 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| 16.6.5.3-7 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE FDD |
| 16.6.5.3-8 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.6.5.3.4.1-2

Table 16.6.5.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 16.6.5.3.4.3.

2.	The power levels and settings for NR PCell (Cell 1) and E-UTRA neighbour cell (Cell 2) are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 16.3.1.3.4.1-3 below.

Table 16.6.5.3.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.6.1.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | setup | As specified in clause 5.5.3.1 in TS 38.331[13]. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

16.6.5.3.4.2	Test procedure

Throughout the test, the UE shall be scheduled to perform DL reception on every slot suitable for DL.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 16.6.5.3.5-1 and Table 16.6.5.3.5-2. T1 starts.

3.	The SS transmits an RRCReconfiguration message to configure event B2 triggered measurement reporting on E-UTRA Cell 2.

4.	The UE transmits an RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.5.3.5-1 and 16.6.5.3.5-2, T2 starts.

6.	UE transmits a MeasurementReport message triggered by Event B2.

7.	After receiving MeasurementReport message in step 6, the SS transmits an RRCReconfiguration message to release the gap configuration.

8.	UE transmits an RRCReconfigurationComplete message.

9.	After receiving RRCReconfigurationComplete message in step 8, the SS transmits an RRCReconfiguration message to configure an autonomous gap based CGI measurement reporting on E-UTRA Cell 2.

10.	The start of T3 is the instant when the last TTI containing the RRCReconfiguration message in step 9 is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 16.6.5.3.5-1 and 16.6.5.3.5-2.

11.	The UE transmits an RRCReconfigurationComplete message.

12. If the UE

a)	transmits a MeasurementReport message containing CGI of E-UTRA Cell 2 less than 235 ms from the beginning of time period T3, and

b)	transmits ACK/NACKs no less than the minimum number of ACK/NACKs given in Table 16.6.5.3.5-5 from the start of T3 until 235 ms,

then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

13.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

14.	Repeat step 2-13 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.5.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 |

Table 16.6.1.1.4.3-2: MeasConfig (Step 3, 7 & 9, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-RAT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-B2 | Table 16.6.1.1.4.3-3 | Step 3 |
|  | ReportConfigInterRAT-CGI | Table 16.6.1.1.4.3-5 | Step 9 |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-DEFAULT specified in Table H.3.1-6 with condition gapUE and Pattern #0 |  | Step 3 |
| measGapConfig SEQUENCE { |  |  | Step 7 |
| gapUE CHOICE { |  |  |  |
| release | null | MG needs to be released to apply interruption requirements |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-3: ReportConfigInterRAT-B2 (Table 16.6.1.1.4.3-2)

| Derivation Path: Table H.3.1-4A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | 58 | Actual threshold = 58-156 = -98dBm/SCS | SCS15 |
|  | 61 | Actual threshold = 61-156 = -95dBm/SCS | SCS30 |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | 31 | Actual threshold = 31-140 = -109dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-4: MeasResults (Step 6, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultServingMOList | Not checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA { | 1 entry |  |  |
| MeasResultEUTRA[1] SEQUENCE { |  | Entry 1 |  |
| eutra-PhysCellId | EUTRA-PhysCellId of Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrp | (0..97) |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-5: ReportConfigInterRAT-CGI (Table 16.6.1.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-141 with condition CGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| reportCGI SEQUENCE { |  |  |  |
| cellForWhichToReportCGI | EUTRA-PhysCellId of Cell 2 |  |  |
| useAutonomousGaps-r16 | setup |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-6: MeasResults (Step 12, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultServingMOList | Not checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA { | 1 entry |  |  |
| MeasResultEUTRA[1] SEQUENCE { |  | Entry 1 |  |
| eutra-PhysCellId | EUTRA-PhysCellId of Cell 2 |  |  |
| measResult | Not checked |  |  |
| cgi-Info | Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.5.3.5	Test requirements

Table 16.6.5.3.4.1-3, Table 16.6.5.3.5-1 and Table 16.6.5.3.5-2 define the primary level settings including test tolerances for the test.

Table 16.6.5.3.5-3: NR cell specific test parameters (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,4 | FDD |  |  |
|  |  |  | 2,3,5,6 | TDD |  |  |
|  |  |  | 7,8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2,5 | TDDConf.1.1 |  |  |
|  |  |  | 3,6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1,2,4,5,7,8 | 10: NRB,c = 52 |  |  |
|  |  |  | 3,6 | 20: NRB,c = 51 |  |  |
| PDSCH reference measurement channel |  |  | 1,4,7,8 | SR.1.1 FDD |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,7,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |
| BWP configurations | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  | 1,2,3,4,5,6,7,8 | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  | 1,2,3,4,5,6,7,8 | ULBWP.1.1 |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 |  |  |
| SSB configuration |  |  | 1,2,4,5,7,8 | SSB.1 FR1 |  |  |
|  |  |  | 3,6 | SSB.1 RedCap FR1 |  |  |
| b2-Threshold1 |  | dBm/SCS | 1,2,4,5,7,8 | -98 |  |  |
|  |  |  | 3,6 | -95 |  |  |
| b2-Threshold2EUTRA |  | dBm/15kHz | 1,2,3,4,5,6,7,8 | -109 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -106.55 |  |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,7,8 | -106.55 |  |  |
|  |  |  | 3,6 | -103.55 |  |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | 18 | -2 | -2 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | 18 | -2 | -2 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88.55 | -108.55 | -108.55 |
|  |  |  | 3,6 | -85.55 | -105.55 | -105.55 |
| SSB_RPNote3 |  | dBm/SCS | 1,2,4,5,7,8 | -88.55 | -108.55 | -108.55 |
|  |  |  | 3,6 | -85.55 | -105.55 | -105.55 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,7,8 | -60.53 | -76.47 | -76.47 |
|  |  | dBm/18.36 MHz | 3,6 | -57.60 | -73.55 | -73.55 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration |  |  | 1,2,3,4,5,6,7,8 | 1x1 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 16.6.5.3.5-4: E-UTRA cell specific test parameters (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,7 | FDD |  |  |
|  |  | 4,5,6,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4,5,6,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4,5,6,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3,7 | 10 MHz: R.3 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1,2,3,7 | 10 MHz: R.6 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1,2,3,7 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4,5,6,8 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -106 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7.55 | 7.55 |
| Ês/IotNote5 | dB | 1,2,3,4,5,6,7,8 | -Infinity | 7.55 | 7.55 |
| RSRPNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -98.45 | -98.45 |
| SCH_RPNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -Infinity | -98.45 | -98.45 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration |  | 1,2,3,4,5,6,7,8 | 1x1 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within overall delay from the start of T3, where the overall delay is calculated as follows,

Overall delay = TRRC Processing+Tidentify_CGI_RedCap, E-UTRAN + additional margin + TTI insertion uncertainty

-	TRRC Processing = 10 ms, is the RRC procedure delay specified in 38.331 [13] clause 12;

-	Tidentify_CGI_RedCap, E-UTRAN = 190 ms, is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell;

-	TTI insertion uncertainty = 2 x TTIDCCH = 2ms, is the delay uncertainty when inserting the measurement report to the TTI of the uplink DCCH;

-	additional margin = 30 ms, is specified in 38.133 [6] clause 9.4A.4.2

Which gives overall delay = 10 + 190 + 30 + 2 = 232 ms, allow 235 ms.

From the start of T3 until 235 ms. The number of ACK/NACK being transmitted by the UE shall be no less than the minimum number of ACK/NACK given in Table 16.6.5.3.5-5.

The rate of correct events observed during repeated tests shall be at least 90%.

Table 16.6.5.3.5-5: Minimum number of ACK/NACKs transmitted by the UE during the overall delay

| Test configuration | Minimum number of transmitted ACK/NACKsNote 1 |
| --- | --- |
| 1,4,7,8 | 129 |
| 2,5 | 55 |
| 3,6 | 135 |
| NOTE 1: The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in 38.133 [6] Table 9.4A.4.3-1. Secondly, given that continuous DL data allocation, additional 45, 27 and 54 ACK/NACK shall be sent for FDD/HD-FDD 15 kHz, TDD 15 kHz and TDD 30 kHz, respectively, from the start of T3 until 235 ms excludes 190 ms for identifying the cell global identifier of cell 2. |  |

<End of modified section 1>