3GPP TSG-RAN5 Meeting #102	R5-240746r1

# Athens, Greece, Feb 26th – Mar 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0669 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for RedCap RRM test case 16.7.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To provide TT analysis for RedCap RRM TC 16.7.3.4.1. - To correct existing TT analysis for R15 legacy SS-SINR inter-frequency absolute accuracy TCs: - TT analysis for BG NR_FDD_FR1_F is still missing. It should be added. - SSB Es/Iot threshold should be -3dB for sub-test 1,2 and -4dB for sub-test 3 considering the accuracy requirements used in TT analysis. - SSB_RP threshold should be -118.5dB/SCS rather than -121.5dB/SCS for test config 3 according to 38.133 Table B.2.3-1. - Minimum Io threshold should be -117.5dBm/15kHz according to 38.133 Table 10.1.9.1.1-1; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TT analysis for RedCap RRM TC 16.7.3.4.1 is added to the attachement of TS 38.903. - Existing TT analysis for R15 legacy SS-SINR inter-frequency absolute accuracy TCs is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: .xlsx file is updated based the comments received during reviewing: - Redundant highlight is removed. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Remove the following .zip file from the attachements of TS 38.903:

“38.533 4.7.3.2.1+6.7.3.2.1 TT.zip”

Add the following .zip file to the attachements of TS 38.903:

“38.533 4.7.3.2.1+6.7.3.2.1+16.7.3.4.1 TT.zip”

<End of modified section 1>