3GPP TSG-RAN5 Meeting #102	R5-240756r1

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2953 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SDT RRM test case 6.2.1 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To align with latest version of 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2403525 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: TT related changes are reversed due to the ongoing RAN4 discussion. Test parameters are updated to align with R4-2403525. Coversheet is updated accordingly. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

Editor's Note:

- Some test parameters are still in brackets

6.2.1.1	Test Purpose

The purpose of this test is to partly verify that the UE properly perform TA validation for CG-SDT transmission in 38.133 [6] clause 5.5.3.

6.2.1.2	Test Applicability

This test applies to all types of NR UE release 17 and forward, supporting NR SA and CG-SDT.

6.2.1.3	Minimum Conformance Requirement

The minimum requirements are specified in clause 6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.2.1.

6.2.1.4	Test Description

The test includes two sub-tests, Sub-test#1 for testing valid TA where UE can initiate CG-SDT transmission, and Sub-test#2 for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 has passed. For each sub-test, UE is configured with CG-SDT configuration when entering RRC_INACTIVE state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3 and T4, respectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6, respectively. There is one cell, which is the active NR cell in FR1.  Figure 6.2.1.4-1 shows the variation of the RSRP over the duration of Sub-test#1, and Figure 6.2.1.4-2 shows the variation of the RSRP over the duration of Sub-test#2.

Figure 6.2.1.4-1: RSRP variation model for Sub-test#1 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

In Sub-test#1:

Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC_CONNECTED state.

Before starting the test at time point TA, test equipment configures RSRP to P0.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P2.

Test equipment triggers UL data arrival at UE lower layer at time point TF. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG, which is W3 after TF.

After time point TG, RRC status is changed from RRC INACTIVE to RRC CONNECTED.

Figure 6.2.1.4-2: RSRP variation model for Sub-test#2 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

In Sub-test#2:

Prior to the time point TA, the UE shall pass Sub-test#1 and have entered RRC connected mode. Otherwise, Sub-test#2 shall not be executed.

From time point TA to time point TD, RSRP is set to P2.

At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P2 to P0.

Test equipment triggers UL data arrival at UE lower layer at time point TF. TF is 3360ms after TD. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

W1 equals to 640ms and W2 equals to 640ms, based on requirements in clause 6.2.0. W3 is 860ms.

6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.2.1.4.1-1.

Table 6.2.1.4.1-1: NR configuration for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.2.1.4.1-2.

Table 6.2.1.4.1-2: Initial conditions for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 without fading | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.2.1.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 6.2.1.4.1-3. Supplementary timeline events are given in Table 6.2.1.4.1-4.

Table 6.2.1.4.1-3: General test parameters for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Unit | Test configuration |  | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 |  | N/A |  |
|  |  | 2 |  | TDDConf.1.1 |  |
|  |  | 3 |  | TDDConf.2.1 |  |
| BWchannel | MHz |  |  | 10: NRB,c = 52 |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  |
|  |  | 2 |  | SR.1.1 TDD |  |
|  |  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  |
|  |  | 2 |  | CR.1.1 TDD |  |
|  |  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  |
|  |  | 2 |  | CCR.1.1 TDD |  |
|  |  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2 |  | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1,2,3 |  | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 |  | SMTC.1 |  |
| DRX configuration | ms | 1,2,3 |  | 640 |  |
| T1 | s | 1,2,3 |  | [0.4] |  |
| T2 | s | 1,2,3 |  | [1.28] |  |
| T3 | s | 1,2,3 |  | [2.72] |  |
| T4 | s | 1,2,3 |  | [2.24] |  |
| T5 | s | 1,2,3 |  | [4.22] |  |
| T6 | s | 1,2,3 |  | [4.22] |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 |  | [14] |  |
| cg-SDT-RSRP-ThresholdSSB | dBm | 1,2,3 |  | -110 |  |
|  | dB | 3 |  | -119 | Modified by TT analysis |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 |  | infinity |  |
| CG-SDT resource period | ms |  |  | 320 | configuredGrantConfig Peridocity at 38.331 [13] Config 1,2 : sym320x14, config 3 : sym640x14 |
| Propagation condition |  | 1,2,3 |  | AWGN |  |

Table 6.2.1.4.1-4: Supplementary timeline event derivations for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Time Period | Unit | Test configuration | Value | Derivation |
| --- | --- | --- | --- | --- |
| T_delay_modeB | ms | 1,2,3 | [4000] | Test configuration for step 4 and step 9 |
| W1 | ms | 1,2,3 | 640 | Test configuration |
| W2 | ms | 1,2,3 | 640 | Test configuration |
| W3 | ms | 1,2,3 | 860 | Test configuration |
| TAtoTB | ms | 1,2,3 | [400] | Sub-test#1: TAtoTB = T1 Sub-test#2: TAtoTB = T5 – 2*W1 |
| TBtoTC | ms | 1,2,3 | [640] | TBtoTC = W1 |
| TCtoTD | ms | 1,2,3 | [640] | TCtoTD = W1 |
| TDtoTE | ms | 1,2,3 | [2720] | Sub-test#1: TDtoTE = T3 Sub-test#2: TDtoTE = T6 – W2 – W3 |
| TEtoTF | ms | 1,2,3 | [640] | TEtoTF = W2 |
| TFtoTG | ms | 1,2,3 | [860] | TFtoTG = W3 |
| Note 1: These timeline derivations are useful to understand different timeline events other than T1, T2, T3, T4, T5 and T6. Calculations are informative. |  |  |  |  |

6.2.1.4.2	Test Procedure

There is one cell in the test, the FR1 PCell, which is the active NR cell. The test parameters for the PCell are given in Table 6.2.1.4.1-1, Table 6.2.1.4.1-3, and Table 6.2.1.5-1, for Sub-test#1 and Sub-test#2, respectively. Prior to the start of each of the Sub-tests, the UE shall be fully synchronized to PCell (Cell 1). According to Table 6.2.1.5-1, in time intervals T1 to T6 the SS configures the SS-RSRP by fixing Noc and adjusting SNR correspondingly.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Loop Function On and Activate UE Test Loop Mode B according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters of NR Cell according to T1 in Table 6.2.1.5-1. Propagation conditions are set according to clause C.2.3. Sub-test#1 starts. T1 starts.

3.	When T1 expires the SS shall change the SS-RSRP value to T2 as specified in Table 6.2.1.5-1. T2 starts.

4.	W1=640ms seconds after the start of T2 (time event TC), the SS shall send CLOSE UE TEST LOOP message for UE Test Loop Mode B and configuring T_delay_modeB as per Table 6.2.1.4.3-5, and transmit one PDCP SDU (which will get buffered at the UE) immediately followed by an RRCRelease message containing CG-SDT configuration for the UE to change its RRC state to RRC_INACTIVE, according to section 6.2.1.4.3.

5.	When T2 expires the SS shall change the SS-RSRP value to T3 as specified in Table 6.2.1.5-1. T3 starts.

6.	When T3 expires the SS shall change the SS-RSRP value to T4 as specified in Table 6.2.1.5-1. T4 starts.

7.	T_delay_modeB expires after the start of T4, at time event TF. If the UE transmits PUSCH at the next CG-SDT resource no later than TG, which is W3 seconds after TF, then the number of successful tests for CG-SDT Sub-test#1 is increased by one. Otherwise, the number of failed tests for CG-SDT Sub-test#1 is increased by one and proceed with step 12.

8.	The SS shall transmit RRCRelease. Set the parameters of NR Cell according to T5 in Table 6.2.1.5-1. Propagation conditions are set according to clause C.2.3. Afterwards, the SS shall ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Loop Function On and Activate UE Test Loop Mode B according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell. Sub-test#2 starts. T5 starts.

9.	W1=640ms seconds before the end of T5 (time event TC), the SS shall send CLOSE UE TEST LOOP message for UE Test Loop Mode B and configuring T_delay_modeB as per Table 6.2.1.4.3-5, and transmit one PDCP SDU (which will get buffered at the UE) immediately followed by an RRCRelease message containing CG-SDT configuration for the UE to change its RRC state to RRC_INACTIVE, according to section 6.2.1.4.3.

10.	When T5 expires the SS shall change the SS-RSRP value to T6 as specified in Table 6.2.1.5-1. T6 starts.

11.	T_delay_modeB expires after the start of T6, at time event TF. If the UE transmits PUSCH at the next CG-SDT resource no later than TG, which is W3 seconds after TF, then the number of failed tests for CG-SDT Sub-test#2 is increased by one. Otherwise, the number of successful tests for CG-SDT Sub-test#2 is increased by one. Proceed with step 12.

12.	The SS shall switch off and then on the UE and then proceed with step 13.

13.	Repeat steps 1-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved for both Sub-tests. If Sub-test#1 fails, there is no need to achieve any confidence level for Sub-test#2.

6.2.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 6.2.1.4.3-1: Common Exception messages for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | 38.508-1 [14] Table 4.6.1-16 with conditions NR_RRC_INACTIVE and SDT |

Table 6.2.1.4.3-2: SDT-MAC-PHY-CG-Config-r17 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { | 1 entry |  |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |
| allowedCG-List-r17 | Not present |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup | BWP-UplinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 39 | -117dBm | Config 1,2 |
|  | 37 | -119dBm | Config 3 |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB14 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.1.4.3-3: BWP-UplinkDedicatedSDT-r17 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicatedSDT-r17::= SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig | 1 entry |  |  |
| } |  |  |  |

Table 6.2.1.4.3-4: ConfiguredGrantConfig for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| periodicity | sym320x14 |  | Config 1,2 |
|  | sym640x14 |  | Config 3 |
| } |  |  |  |

Table 6.2.1.4.3-5: CLOSE UE TEST LOOP for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.509 [16], clause 6.3.1 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| Protocol discriminator | 1 1 1 1 |  |
| Skip indicator | 0 0 0 0 |  |
| Message type | 1 0 0 0 0 0 0 0 |  |
| UE test loop mode | 0 0 0 0 0 0 0 1 | UE test loop mode B |
| UE test loop mode B LB setup |  |  |
| IP PDU delay | 0 0 0 0 0 1 0 0 | T_delay_modeB duration in seconds (4 seconds) |

6.2.1.5	Test Requirement

Table 6.2.1.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT).

Table 6.2.1.5-1: Cell-specific test parameters for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
|  | 1,2 | dBm/15kHz | -98.95 | -98.95 | -101 | -101 | -99.5 | -105 |
|  | 3 | dBm/15kHz | -98.2 | -98.2 | -101.25 | -101.25 | -103.7 | -107.95 |
|  | 1,2 | dBm/SSB SCS | -98.95 | -98.95 | -101 | -101 | -99.5 | -105 |
|  | 3 | dBm/SSB SCS | -95.2 | -95.2 | -98.25 | -98.25 | -100.7 | -104.95 |
|  | 1,2 | dB | [0] | [10] | [0] | [15] | [19.6] | [-0.95] |
|  | 3 | dB | [0] | [10.5] | [0] | [15.5] | [18] | [-2.8] |
| SS RSRP | 1,2 | dBm/SSB SCS | [-98.95] | [-88.95] | [-101] | [-86] | [-79.9] | [-105.95] |
|  | 3 | dBm/SSB SCS | [-95.2] | [-84.7] | [-98.25] | [-82.75] | [-82.7] | [-107.75] |
| Io | 1,2 | dBm/9.36MHz | [-67.99] | [-60.58] | [-70.04] | [-57.91] | [-51.9] | [-74.49] |
|  | 3 | dBm/38.16MHz | [-61.13] | [-53.27] | [-64.18] | [-51.57] | [-51.58] | [-72.06] |
|  | 1,2 | dB | [0] | [10] | [0] | [15] | [19.6] | [-0.95] |
|  | 1,2 | dB | [0] | [10.5] | [0] | [15.5] | [18] | [-2.8] |

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit PUSCH at CG-SDT resource within 860ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit PUSCH at CG-SDT resources after TF until the end of the test at time point TG.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

<End of modified section 1>