3GPP TSG-RAN5 Meeting #102	 R5-240892r1

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0594 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for additional band configurations with PC2 UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update for additional band configurations with PC2 UL |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the following tables for additional band configurations with PC2 UL: - Table A.4.3.2A.4.1-4: Inter-band CA within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities) - Table A.4.3.2A.4.2-4: Inter-band CA within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities Include the following new table: - Table A.4.3.2A.4.3-4: Inter-band CA within FR1 (four bands) PC2 UE RF Baseline Implementation Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related band configurations are not specified in the related section. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to work item code of NR_PC2_CA_R17_2BDL_2BUL-UEConTest as well |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st Revision: Resolve 3GU issue of Work item code by including information in Other comments on title page |  |  |  |  |  |  |  |  |

<Start of changes>

#### A.4.3.2A.4	NR Inter-band CA within FR1

##### A.4.3.2A.4.1	NR Inter-band CA within FR1 (two bands)

<skipped unchanged sections>

Table A.4.3.2A.4.1-4: Inter-band CA within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item | CA configuration | Inter-band CA within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CA_n1A-n78A | n1 band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n1A_n78A_PC2_Supp |  |
| A | CA_n2A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| x | CA_n2A-n77C | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 1B | CA_n3A-n41A | n3 band: 1710-1785 MHz (UL),1805- 1880 MHz (DL) n41 band: 2496-2690 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n3A_n41A_PC2_Supp |  |
| 2 | CA_n3A-n78A | n3 band: 1710-1785 MHz (UL),1805- 1880 MHz (DL) n78 band: 3300-3800 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n3A_n78A_PC2_Supp |  |
| 2A | CA_n5A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| x | CA_n5A-n77C | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 3 | CA_n28A-n41A | n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n41 band: 2496-2690 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n28A_n41A_PC2_Supp |  |
| 4 | CA_n28A-n79A | n28 band: 703-748 MHz (UL),758- 803 MHz (DL) n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n28A_n79A_PC2_Supp |  |
| 5 | CA_n41A-n79A | n41 band: 2496-2690 MHz n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n41A_n79A_PC2_Supp |  |
| 6 | CA_n66A-n77A | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| x | CA_n66A-n77C | n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 7 | CA_n78A-n79A | n78 band: 3300-3800 MHz n79 band: 4400-5000 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n78A_n79A_PC2_Supp |  |

<skipped unchanged sections>

Table A.4.3.2A.4.2-4: Inter-band CA within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities

| Item | CA configuration | Inter-band CA within FR1 (three bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| x | CA_n2A-n48A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n48 band: 3550-3700 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 1 | CA_n2A-n5A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2 | CA_n2A-n5A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 3 | CA_n2A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 4 | CA_n2A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| x | CA_n5A-n48A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n48 band: 3550-3700 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 5 | CA_n5A-n66A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 6 | CA_n5A-n66A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-17 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| x | CA_n48A-n66A-n77A | n48 band: 3550-3700 MHz n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |

<skipped unchanged sections>

Table A.4.3.2A.4.3-3: Supported configurations for NR Inter-band CA within FR1 and four bands

| NR FR1 Inter-band CA configuration / Item (Note 1, 7) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A | Rel-16 |  |  |  |
| CA_n2A-n5A-n48A-n66A | Rel-17 |  |  |  |
| CA_n2A-n5A-n48A-n77A | Rel-17 |  |  |  |
| CA_n2A-n5A-n66A-n77A | Rel-17 |  |  |  |
| CA_n2A-n48A-n66A-n77A | Rel-17 |  |  |  |
| CA_n5A-n48A-n66A-n77A | Rel-17 |  |  |  |
| Note 1: Notation used for inter-band CA Bands is according to TS 38.101-1 [23] Table 5.5A.3.3-1. Note 2: The UL CA capabilities as per Table A.4.3.2A.4.3-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.3.3-1. For this release of specification valid choices are ’N’, ‘nXA-nYA’, ‘nX(2A)’ and ‘nXC’, where both nX and nY are the NR bands. For example, for CA_nXA-nYA-nWA-nZA , ‘N‘ would mean only DL CA, ‘nXA-nYA’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.3.3-1. Note 4: Void. Note 5: See UL(table_index) in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 6: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_n66B and CA_n66(2A) in the current version of the specification as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 7: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |

Table A.4.3.2A.4.3-4: Inter-band CA within FR1 (four bands) PC2 UE RF Baseline Implementation Capabilities

| Item | CA configuration | Inter-band CA within FR1 (four bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CA_n2A-n5A-n48A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n48 band: 3550-3700 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 2 | CA_n2A-n5A-n48A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n48 band: 3550-3700 MHz n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 3 | CA_n2A-n5A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel- Rel-1818 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 4 | CA_n2A-n5A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 5 | CA_n2A-n5A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 6 | CA_n2A-n48A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n48 band: 3550-3700 MHz n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n2A_n77A_PC2_Supp |  |
| 7 | CA_n2A-n48A-n66A-n77A | n2 band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) n48 band: 3550-3700 MHz n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |
| 8 | CA_n5A-n48A-n66A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n48 band: 3550-3700 MHz n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n5A_n77A_PC2_Supp |  |
| 9 | CA_n5A-n48A-n66A-n77A | n5 band: 824-849 MHz (UL), 869-894 MHz (DL) n48 band: 3550-3700 MHz n66 band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) n77 band: 3300-4200 MHz | 38.101-1, 6.2A.1.3 | Rel-18 | pc_UL_inter_band_CA_n66A_n77A_PC2_Supp |  |

#### A.4.3.2A.5	NR Inter-band CA within FR2

<End of changes>