3GPP TSG-RAN5 Meeting # 102	R5-240972r1

# Athens, Greece, 26th February – 3rd March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3080 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Asymmetric channel bandwidths test frequencies updates for frequency bands n5 and n8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Nokia, Skyworks Solutions Inc., T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for asymmetric channel bandwidths for bands n5 and n8 are not considering Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex C.2.3.1 is updated so appropriate Tx-Rx carrier center frequency separation is used when calculating test frequencies for asymmetric channel bandwidths combinations impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 (refer to R5-240970 and R5-240971). Hence test frequencies for asymmetric channel bandwidths for frequency bands n5 and n8 impacted by Note 5 in table 7.3.2-2 in 3GPP TS 38.101-1 are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A device could potentially pass certain receiver test cases (REFSENS, ACS and blocking) that would fail if correct Tx-Rx carrier center frequency separation were used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.5, 4.3.1.1.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on RAN5 discussion paper R5-240970 and CR R5-240971. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Values corrected in tables 4.3.1.1.1.5-2, 4.3.1.1.1. 8-1, 4.3.1.1.1.8-2. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

###### 4.3.1.1.1.5	Reference test frequencies for NR operating band n5

Table 4.3.1.1.1.5-1: Test frequencies for NR operating band n5, SCS 15 kHz and ΔFRaster 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 860.89 | 172178 | 102 |  | 2203 | 176210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 743.53 | 148706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 874 | 174800 | 869.32 | 173864 | 0 | 15 | 2179 | 174290 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 858.46 | 171692 | 102 |  | 2197 | 175730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 829 | 165800 | 824.32 | 164864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 741.1 | 148220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 876.5 | 175300 | 869.39 | 173878 | 0 | 15 | 2177 | 174250 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 856.03 | 171206 | 102 |  | 2191 | 175250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.39 | 164878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 738.67 | 147734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 879 | 175800 | 869.46 | 173892 | 0 | 15 | 2178 | 174270 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 853.6 | 170720 | 102 |  | 2185 | 174770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 834 | 166800 | 824.46 | 164892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 736.24 | 147248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 881.5 | 176300 | 869.53 | 173906 | 0 | 15 | 2179 | 174290 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 836.5 | 167300 | 826.96 | 165392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.5-2: Test frequencies for NR operating band n5, SCS 30 kHz and ΔFRaster 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 874 | 174800 | 869.68 | 173936 | 0 | 30 | 2185 | 174770 | 14 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 840.46 | 168092 | 102 |  | 2203 | 176210 | 18 | 0 | 0 (0) | 204 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low | 829 | 165800 | 824.68 | 164936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 650.74 | 130148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 876.5 | 175300 | 869.66 | 173932 | 0 | 30 | 2183 | 174730 | 2 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 837.94 | 167588 | 102 |  | 2197 | 175730 | 2 | 0 | 1 (1) | 206 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.66 | 164932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 648.22 | 129644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 879 | 175800 | 869.82 | 173964 | 0 | 30 | 2184 | 174750 | 22 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 835.6 | 167120 | 102 |  | 2191 | 175250 | 22 | 0 | 0 (0) | 204 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 834 | 166800 | 824.82 | 164964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 645.88 | 129176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 881.5 | 176300 | 869.8 | 173960 | 0 | 30 | 2185 | 174770 | 6 | 0 | 1 (1) | 2 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 836.54 | 167300 | 827.32 | 165464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

###### 4.3.1.1.1.8	Reference test frequencies for NR operating band n8

Table 4.3.1.1.1.8-1: Test frequencies for NR operating band n8 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 927.5 | 185500 | 925.25 | 185050 | 0 | 15 | 2318 | 185530 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 921.89 | 184378 | 102 |  | 2354 | 188410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 957.5 | 191500 | 864.53 | 172906 | 504 |  | 2393 | 191530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 930 | 186000 | 925.32 | 185064 | 0 | 15 | 2319 | 185550 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 919.46 | 183892 | 102 |  | 2348 | 187930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 955 | 191000 | 859.6 | 171920 | 504 |  | 2383 | 190610 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2344 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2344 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 52 | Uplink | Low | 892.5 | 178500 | 887.82 | 177564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.74 | 179348 | 6 |  | - | - | - | - | - | - |
| 10/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 897.5 | 179500 | 892.82 | 178564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 79 | Downlink | Low | 932.5 | 186500 | 925.39 | 185078 | 0 | 15 | 2320 | 185570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 917.03 | 183406 | 102 |  | 2342 | 187450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 952.5 | 190500 | 854.67 | 170934 | 504 |  | 2370 | 189630 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 15/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 79 | Uplink | Low | 892.5 | 178500 | 885.39 | 177078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 894.31 | 178862 | 6 |  | - | - | - | - | - | - |
| 15/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 | Uplink | Low | 897.58 | 179500 | 890.398 | 178078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 106 | Downlink | Low | 935 | 187000 | 925.46 | 185092 | 0 | 15 | 2318 | 185530 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 914.6 | 182920 | 102 |  | 2336 | 186970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 950 | 190000 | 849.74 | 169948 | 504 |  | 2357 | 188650 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 106 | Uplink | Low | 892.5 | 178500 | 882.96 | 176592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.88 | 178376 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 897.5 | 179500 | 887.96 | 177592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.8-2: Test frequencies for NR operating band n8 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 930 | 186000 | 925.68 | 185136 | 0 | 15 | 2325 | 186030 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 901.46 | 180292 | 102 |  | 2354 | 188410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 955 | 191000 | 769.24 | 153848 | 504 |  | 2389 | 191090 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 24 | Uplink | Low | 892.5 | 178500 | 888.18 | 177636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.02 | 179204 | 6 |  | - | - | - | - | - | - |
| 10/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 | Uplink | Low | 897.56 | 179500 | 893.18 | 178636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 38 | Downlink | Low | 932.5 | 186500 | 925.66 | 185132 | 0 | 15 | 2326 | 186050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 898.94 | 179788 | 102 |  | 2348 | 187930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 952.5 | 190500 | 764.22 | 152844 | 504 |  | 2373 | 189870 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 15/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 38 | Uplink | Low | 892.5 | 178500 | 885.66 | 177132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 893.5 | 178700 | 6 |  | - | - | - | - | - | - |
| 15/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | Uplink | Low | 897.5 | 179500 | 890.66 | 178132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 51 | Downlink | Low | 935 | 187000 | 925.82 | 185164 | 0 | 15 | 2324 | 186010 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 896.6 | 179320 | 102 |  | 2342 | 187450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 950 | 190000 | 759.38 | 151876 | 504 |  | 2360 | 188890 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 51 | Uplink | Low | 892.5 | 178500 | 883.32 | 176664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.16 | 178232 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 897.5 | 179500 | 888.32 | 177664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES 2>>>