3GPP TSG-WG5 Meeting #102	R5-241001r1

# Athens, Greece, Feburary 26, 2024 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2678 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding AMPR test cases for V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The AMPR test cases for V2X are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding AMRP test cases for V2X single antenna, SL-MIMO and con-current operation. - The measurement period of PSFCH and S-SSB is set to cumulative 1ms - The MU/TT reuse that of NR test case - For con-current operation, no test detail is specified as there is no exceptional requirement. The test could be done by NR testing and V2X testing respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases couldn’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2E.3.1 (new), 6.2E.3.1D (new), 6.2E.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Change the numbering of newly added reference specification to avoid conflict with R5-240049. |  |  |  |  |  |  |  |  |

<Start of Changes>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

For a specific reference, subsequent revisions do not apply.

For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: " NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: " NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment ".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[7]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[8]	3GPP TS 38.211: "NR; Physical channels and modulation".

[9]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[10]	3GPP TR 38.903: "NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance tests".

[11]	3GPP TR 38.905: "NR; Derivation of test points for radio transmission and reception conformance test cases".

[12]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[13]	3GPP TS 38.521-2: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[14]	3GPP TS 38.521-3: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[15]	3GPP TS 38.521-4: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”.

[16]	3GPP TS 38.522: "NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases".

[17]	3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases".

[18]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[19]	3GPP TS 38.133: "NR; Requirements for support of radio resource management ".

[20]	3GPP TS 38.215: "NR; Physical layer measurements".

[21]	3GPP TS 36.521-1: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing”.

[22]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain".

[23]	3GPP TS 38.307: "NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[24]	3GPP TS 37.213: “Physical layer procedures for shared spectrum channel access”.

[25]	3GPP TS 36.508: "Common test environments for User Equipment (UE)"

[26]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[28]	3GPP TS 36.101: “NR; User Equipment (UE) radio access capabilities”.

<Unchanged Texk Skipped>

6.2E.2.2.5	Test requirement

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.2.5-1.

Table 6.2E.2.2.5-1: UE MPR test requirement for inter-band con-current NR V2X operation

|  | PPowerClass,NR (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,c, NR (dBm) | PCMAX_H,c, NR (dB) | PPowerClass,V2X (dBm) | PCMAX_H,f,c,V2X (dBm) | PCMAX_L (dBm) | PCMAX_H (dBm) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 2 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |
| 3 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 4 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |

Table 6.2E.2.2.5-2: Test Tolerance (inter-band con-current NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |

6.2E.3	UE additional maximum output power reduction for V2X

6.2E.3.0	Minimum conformance requirements

6.2E.3.0.1	General

For the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in clause 6.2E.2.0 and A-MPR requirements specified in current clause.

Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2E.2.0. Outer and inner allocation notation used in clause 6.2E.3.0.2 is defined in clause 6.2E.2.0. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2E.3.0.1-1: Additional Maximum Power Reduction (A-MPR) for PC3 NR V2X

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2E.1-1 | 10, 20, 30, 40 | Table 5.3.2-1 | N/A |
| NS_33 | 6.5E.2.3.0.1 (A-SEM) 6.5E.3.3.0 (A-SE) | n47 | 10 | Clause 6.2E.3.0.2 |  |
| NS_52 | 6.5E.2.3.0.2 (A-SEM) | n47 | 40 | Clause 6.2E.3.0.3 |  |

Table 6.2E.3.0.1-2: Mapping of network signaling label

| NR V2X operating bands | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n47 | NS_01 | NS_33 | NS_52 |  |  |  |  |  |
| NOTE: [additionalSpectrumEmission] corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

For UE with two transmit antenna connectors, the A-MPR values specified in clause 6.2E.3.0.2 and 6.2E.3.0.3 shall apply to the maximum output power specified in Table 6.2E.1.0.1-1. The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2E.4.0 apply.

6.2E.3.0.2	A-MPR for Power class 3 V2X UE by NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2E.3.0.2-1, 6.2E.3.0.2-2 is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in annex I in [28]. The A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector.

For the contiguous PSSCH and PSCCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 6.2E.3.0.2-1: A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 24 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤19 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 and ≤ 38 | ≤6 |  |  |
|  | ≥ 10 and ≤ 15 | ≥38 | ≤ 6 |  |  |
|  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤9.5 |  |  |
|  |  | ≥ 20 and ≤ 25 | ≤8.0 |  |  |
|  | > 15 and < 25 | ≥ 25 | ≤ 8 |  |  |
|  | ≥ 10 and < 40 | ≥ 4 and ≤7 | ≤ 16 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and < 40 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | ≥ 25 and < 40 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  | ≥ 22 and ≤ 27 | ≤ 8.0 |  |  |
|  | ≥ 24 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  | ≥ 2 and ≤ 5 | ≤ 16 |  |  |
|  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  | >45 | ≥ 0 | ≤ 16 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 6.2E.3.0.2-2: A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.2 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

For the simultaneous PSFCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 6.2E.3.0.2-3: A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860 | NRB =1 | 19.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

For the S-SSB transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 6.2E.3.0.2-4: A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

6.2E.3.0.3	A-MPR for Power class 3 V2X UE by NS_52

When NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPR

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

For the contiguous PSSCH and PSCCH transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 6.2E.3.0.3-1: A-MPR for PSSCH/PSCCH by NS_52

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Region 1 | Region 2 | Region 3 |
| 5885 | QPSK | ≤ 15 | ≤ 8.0 | ≤ 5.5 |
|  | 16QAM |  | ≤ 8.0 | ≤ 5.5 |
|  | 64QAM |  | ≤ 8.5 | ≤ 5.5 |
|  | 256QAM |  | ≤ 8.5 | ≤ 6.0 |
| Note1: Void. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Region1, Region2 and Region3 according to RB allocations:

Table 6.2E.3.0.3-1a: A-MPR Region definitions for PSSCH/PSCCH by NS_52

| Channel Bandwidth, MHz | Carrier frequency (MHz) | A-MPR parameters for region definitions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart or RBend | LCRB |  |
| 40 | 5885 | RBstart ≤ floor(NRB*0.2) or RBend ≥ NRB - floor(NRB*0.2) | LCRB ≤floor(NRB*0.2) | Region 1 |
|  |  | The RB allocation is in Region 2 allocation for all other allocations which are not a Region1 or Region3 allocation. |  | Region 2 |
|  |  | floor(NRB /3.5) ≤ RBstart ≤ NRB –floor(NRB /3.5) – LCRB | LCRB ≤ceil(NRB/3.5) | Region 3 |

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

For the simultaneous PSFCH transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 6.2E.3.0.3-2: A-MPR for simultaneous PSFCH by NS_52

| Channel Bandwidth [MHz] | Carrier frequency [MHz] | A-MPR (dB) |
| --- | --- | --- |
| 40 MHz | 5885 | 23.5 |

For the S-SSB transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 6.2E.3.0.2-3: A-MPR for S-SSB transmission by NS_52

| Carrier Frequency [MHz] | RBStart * 12*SCS [MHz] | A-MPR (dB) |
| --- | --- | --- |
| 5885 | ≤ 7 | ≤ 16 |
|  | > 7 and ≤ 12 | ≤ 10.5 |
|  | > 12 and ≤ 19 | ≤ 4.0 |
|  | > 19 and ≤ 25 | ≤ 10.5 |
|  | > 25 | ≤ 16 |

6.2E.3.0.4	A-MPR for power class 3 V2X con-current operation

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in clause 6.2.3.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in clause 6.2E.3.0.2 and 6.2E.3.0.3 apply for NR sidelink operation in Band n47.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2E.3.

6.2E.3.1	UE additional maximum output power reduction for V2X / non-concurrent operation

6.2E.3.1.1	Test purpose

To verify that the reduction of UE transmitted power when additional emission requirements are signalle by the network in the field additionalSpectrumEmission.

6.2E.3.1.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5E.2.3.1 are executed.

6.2E.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.3.0.

6.2E.3.1.4	Test description

6.2E.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.3.1.4.1-1 to 6.2E.3.1.4.1-3. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.2E.3.1.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation for NS_33

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | 10MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  | (MHz) | Modulation |  | PSCCH and PSSCH RB allocation |
| 1 | 5860 | 64QAM |  | 15@0 |
| 2 | 5860 | 64QAM |  | 15@1 |
| 3 | 5860 | 64QAM |  | 15@26 |
| 4 | 5860 | 64QAM |  | 12@38 |
| 5 | 5860 | 64QAM |  | 20@12 |
| 6 | 5860 | 64QAM |  | 20@15 |
| 7 | 5860 | 64QAM |  | 20@20 |
| 8 | 5860 | 64QAM |  | 24@25 |
| 9 | 5860 | 64QAM |  | 36@4 |
| 10 | 5860 | 64QAM |  | 36@8 |
| 11 | 5860 | 64QAM |  | 36@0 |
| 12 | 5860 | 64QAM |  | 36@16 |
| 13 | 5860 | 64QAM |  | 36@22 |
| 14 | 5860 | 64QAM |  | 40@12 |
| 15 | 5860 | 64QAM |  | 45@0 |
| 16 | 5860 | 64QAM |  | 45@2 |
| 17 | 5860 | 64QAM |  | 45@6 |
| 18 | 5860 | 64QAM |  | 50@0 |
| 19 | 5920 | 16QAM |  | Inner_Full (Note 1) |
| 20 | 5920 | 64QAM |  | Inner_Full (Note 1) |
| 21 | 5920 | 256QAM |  | Inner_Full (Note 1) |
| 22 | 5920 | 16QAM |  | Outer_Full (Note 1) |
| 23 | 5920 | 64QAM |  | Outer_Full (Note 1) |
| 24 | 5920 | 256QAM |  | Outer_Full (Note 1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.2E.3.1.4.1-2: Test Configuration Table for PSFCH for NS_33

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | 10MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  | (MHz) | PSFCH RB allocation |  |  |
| 1 | 5860 | PSFCH_1RB_Left (Note 1) |  |  |
| 2 | 5860 | PSFCH_Max_Gap (Note 1) |  |  |
| 3 | 5920 | 1@49 |  |  |
| 4 | 5920 | 1@0 |  | 1@7 |
| 5 | 5920 | 1@0 |  | 1@15 |
| 6 | 5920 | PSFCH_Max_Gap (Note 1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-2. |  |  |  |  |

Table 6.2E.3.1.4.1-3: Test Configuration Table for S-SSB for NS_33

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | (See Freq column) |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | 10MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  | (MHz) | S-SSB RB allocation |  |
| 1 | 5860 | 11@0 |  |
| 2 | 5860 | 11@6 |  |
| 3 | 5860 | 11@12 |  |
| 4 | 5860 | 11@19 |  |
| 5 | 5860 | 11@21 |  |
| 6 | 5920 | 11@5 |  |
| 7 | 5920 | 11@8 |  |
| 8 | 5920 | 11@14 |  |
| 9 | 5920 | 11@18 |  |
| 10 | 5920 | 11@24 |  |
| 11 | 5920 | 11@41 |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.3.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.2E.3.1.4.1-1 to Table 6.2E.3.1.4.1-3.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.2E.3.1.4.2	Test procedure

Subtest 1: PSCCH/PSSCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from Table 6.2E.3.1.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

Subtest 2: PSFCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Cast type Unicast, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink reception according to SL-PreconfigurationNR.

3.	The UE’s PSFCH transmission occasion is on slot n according to Table 6.2E.3.1.4.1-2. SS transmits PSSCH on combination of slot and subchannel as below:

a) Test ID 1: slot n-6, Lowest sub-channel

b) Test ID 2, 4, 5 and 6: slot n-6, Lowest sub-channel and slot n-3, Highest sub-channel

c) Test ID 3: slot n-3, Highest sub-channel

4.	Measure the mean power of the UE on slot n in the channel bandwidth according to the test configuration from Table 6.2E.3.1.4.1-2. The cumulative measurement period is at least 1ms.

Subtest 3: S-SSB

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE transmits PSBCH according SL-PreconfigurationNR.

3.	Measure the mean power of the S-SSB in the channel bandwidth according to the test configuration from Table 6.2E.3.1.4.1-3. The cumulative measurement period is at least 1ms.

6.2E.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

6.2E.3.1.4.3.1	Message contents exceptions (network signalling value “NS_33”)

Table 6.2E.3.1.4.3.1-1: Network signalling value "NS_33"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_33) |  |  |

Table 6.2E.3.1.4.3.1-2: SL-ResourcePool for PSCCH/PSSCH Testing for NS_33

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | 10 |  |  |
| sl-FreqResourcePSCCH-r16 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | 10 |  | Test ID 5, 6, 7, 14, 18 |
|  | 12 |  | Test ID 4, 8, 9, 10, 11, 12, 13 |
|  | 15 |  | Test ID 1, 2, 3, 15, 16, 17 |
| sl-StartRB-Subchannel-r16 | 0 |  | Test ID 1, 11, 15, 18 |
|  | 1 |  | Test ID 2 |
|  | 26 |  | Test ID 3 |
|  | 38 |  | Test ID 4 |
|  | 12 |  | Test ID 5, 14 |
|  | 15 |  | Test ID 6 |
|  | 20 |  | Test ID 7 |
|  | 25 |  | Test ID 8 |
|  | 4 |  | Test ID 9 |
|  | 8 |  | Test ID 10 |
|  | 16 |  | Test ID 12 |
|  | 22 |  | Test ID 13 |
|  | 2 |  | Test ID 16 |
|  | 6 |  | Test ID 17 |
| sl-NumSubchannel-r16 | 1 |  | Test ID 1, 2, 3, 4 |
|  | 2 |  | Test ID 5, 6, 7, 8, |
|  | 3 |  | Test ID 9, 10, 11, 12, 13, 15, 16, 17 |
|  | 4 |  | Test ID 14 |
|  | 5 |  | Test ID 18 |
| } |  |  |  |

Table 6.2E.3.1.4.3.1-3: SL-ResourcePool for PSFCH Testing for NS_33

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-PSFCH-Period-r16 | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  | Test ID 1, 2, 3, 6 |
|  | Bitstring of length 50, The leftmost 8 bits are set to "1" and the rest are set to "0" |  | Test ID 4 |
|  | Bitstring of length 50, The leftmost 4 bits and bits 12~15 are set to "1" and the rest are set to "0" |  | Test ID 5 |
| sl-NumMuxCS-Pair-r16 | n1 |  |  |
| sl-MinTimeGapPSFCH-r16 | sl3 |  |  |
| sl-PSFCH-HopID-r16 | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 | startSubCH |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | 25 |  |  |
| sl-StartRB-Subchannel-r16 | 0 |  |  |
| sl-NumSubchannel-r16 | 2 |  |  |
| } |  |  |  |

Table 6.2E.3.1.4.3.1-4: SL-FreqConfigCommon for S-SSB Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-FreqConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-AbsoluteFrequencySSB-r16 | According to section 4.3.1.8 of TS 38.508-1 [5] |  |  |
| } |  |  |  |

6.2E.3.1.5	Test requirement

The maximum output power, derived in step 3 of Subtest 1, step 4 of Subtest 2 and step 3 of Subtest 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.3.1.5-1 to Table 6.2E.3.1.5-3.

Table 6.2E.3.1.5-0: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7dB | 1.0dB |

Table 6.2E.3.1.5-1: UE AMPR test requirement for contiguous PSCCH/PSSCH for NS_33

| Test ID | PPowerClass (dBm) | MPR (dB) | AMPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 23 | 4.5 | 24 | 0 | -1 | 7 | 25.0 + TT | -8-TT |
| 22 | 23 | 4.5 | 19 | 0 | 4 | 7 | 25.0 + TT | -3-TT |
| 32 | 23 | 4.5 | 6 | 0 | 17 | 5 | 25.0 + TT | 13-TT |
| 42 | 23 | 4.5 | 6 | 0 | 17 | 5 | 25.0 + TT | 13-TT |
| 52 | 23 | 4.5 | 11 | 0 | 12 | 6 | 25.0 + TT | 6-TT |
| 62 | 23 | 4.5 | 9.5 | 0 | 13.5 | 6 | 25.0 + TT | 7.5-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 6 | 25.0 + TT | 9-TT |
| 82 | 23 | 4.5 | 8 | 0 | 15 | 6 | 25.0 + TT | 9-TT |
| 92 | 23 | 4.5 | 16 | 0 | 7 | 7 | 25.0 + TT | 0-TT |
| 102 | 23 | 4.5 | 13.5 | 0 | 9.5 | 7 | 25.0 + TT | 2.5-TT |
| 112 | 23 | 4.5 | 22 | 0 | 1 | 7 | 25.0 + TT | -6-TT |
| 122 | 23 | 4.5 | 9.5 | 0 | 13.5 | 6 | 25.0 + TT | 7.5-TT |
| 132 | 23 | 4.5 | 8 | 0 | 15 | 6 | 25.0 + TT | 9-TT |
| 142 | 23 | 4.5 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| 152 | 23 | 4.5 | 19 | 0 | 4 | 7 | 25.0 + TT | -3-TT |
| 162 | 23 | 4.5 | 16 | 0 | 7 | 7 | 25.0 + TT | 0-TT |
| 172 | 23 | 4.5 | 13.5 | 0 | 9.5 | 7 | 25.0 + TT | 2.5-TT |
| 182 | 23 | 4.5 | 16 | 0 | 7 | 7 | 25.0 + TT | 0-TT |
| 19 | 23 | 2.5 | 3 | 0 | 20 | 6 | 25.0 + TT | 14-TT |
| 20 | 23 | 4.5 | 5 | 0 | 18 | 5 | 25.0 + TT | 13-TT |
| 21 | 23 | 7.0 | 6 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| 22 | 23 | 4.5 | 4.5 | 0 | 18.5 | 5 | 25.0 + TT | 13.5-TT |
| 23 | 23 | 4.5 | 5 | 0 | 18 | 5 | 25.0 + TT | 13-TT |
| 24 | 23 | 7.0 | 6 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.3.1.5-0. NOTE 2: If the supported post antenna connector gain Gpost connector is declared by the UE, the lower limit is further lower down by Gpost connector* A-MPRStep. A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart. |  |  |  |  |  |  |  |  |

Table 6.2E.3.1.5-2: UE MPR test requirement for PSFCH for NS_33

| Test ID | PPowerClass (dBm) | MPR (dB) | AMPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3.5 | 19 | 0 | 4 | 7 | 25.0 + TT | -3-TT |
| 2 | 23 | 12 | 22 | 0 | 1 | 7 | 25.0 + TT | -6-TT |
| 3 | 23 | 3.5 | 5 | 0 | 18 | 5 | 25.0 + TT | 13-TT |
| 4 | 23 | 7.5 | 14 | 0 | 9 | 7 | 25.0 + TT | 2-TT |
| 5 | 23 | 7.5 | 7 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| 6 | 23 | 12 | 18.5 | 0 | 4.5 | 7 | 25.0 + TT | -2.5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.3.1.5-0. |  |  |  |  |  |  |  |  |

Table 6.2E.3.1.5-3: UE MPR test requirement for S-SSB for NS_33

| Test ID | PPowerClass (dBm) | MPR (dB) | AMPR (dBm) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 6 | 25 | 0 | -2 | 7 | 25.0 + TT | -9-TT |
| 2 | 23 | 2.5 | 19 | 0 | 4 | 7 | 25.0 + TT | -3-TT |
| 3 | 23 | 2.5 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| 4 | 23 | 2.5 | 10 | 0 | 13 | 6 | 25.0 + TT | 7-TT |
| 5 | 23 | 2.5 | 9 | 0 | 14 | 6 | 25.0 + TT | 8-TT |
| 6 | 23 | 6 | 7 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| 7 | 23 | 2.5 | 6.5 | 0 | 16.5 | 5 | 25.0 + TT | 11.5-TT |
| 8 | 23 | 2.5 | 5.8 | 0 | 17.2 | 5 | 25.0 + TT | 12.2-TT |
| 9 | 23 | 2.5 | 4.5 | 0 | 18.5 | 5 | 25.0 + TT | 13.5-TT |
| 10 | 23 | 2.5 | 5.5 | 0 | 17.5 | 5 | 25.0 + TT | 12.5-TT |
| 11 | 23 | 6 | 6.5 | 0 | 16.5 | 5 | 25.0 + TT | 11.5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.3.1.5-0. |  |  |  |  |  |  |  |  |

6.2E.3.1D	UE additional maximum output power reduction for V2X / non-concurrent operation / SL-MIMO

6.2E.3.1D.1	Test purpose

Same test purpose as in 6.2E.3.1.

6.2E.3.1D.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5E.2.3.1D are executed.

6.2E.3.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.3.0.

6.2E.3.1D.4	Test description

6.2E.3.1D.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2E.1-1 and table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.3.1D.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.2E.3.1D.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation for NS_33

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | 10MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  | (MHz) | Modulation |  | PSCCH and PSSCH RB allocation |
| 1 | 5860 | 64QAM |  | 15@0 |
| 2 | 5860 | 64QAM |  | 15@1 |
| 3 | 5860 | 64QAM |  | 15@26 |
| 4 | 5860 | 64QAM |  | 12@38 |
| 5 | 5860 | 64QAM |  | 20@12 |
| 6 | 5860 | 64QAM |  | 20@15 |
| 7 | 5860 | 64QAM |  | 20@20 |
| 8 | 5860 | 64QAM |  | 24@25 |
| 9 | 5860 | 64QAM |  | 36@4 |
| 10 | 5860 | 64QAM |  | 36@8 |
| 11 | 5860 | 64QAM |  | 36@0 |
| 12 | 5860 | 64QAM |  | 36@16 |
| 13 | 5860 | 64QAM |  | 36@22 |
| 14 | 5860 | 64QAM |  | 40@12 |
| 15 | 5860 | 64QAM |  | 45@0 |
| 16 | 5860 | 64QAM |  | 45@2 |
| 17 | 5860 | 64QAM |  | 45@6 |
| 18 | 5860 | 64QAM |  | 50@0 |
| 19 | 5920 | 16QAM |  | Inner_Full (Note 1) |
| 20 | 5920 | 64QAM |  | Inner_Full (Note 1) |
| 21 | 5920 | 256QAM |  | Inner_Full (Note 1) |
| 22 | 5920 | 16QAM |  | Outer_Full (Note 1) |
| 23 | 5920 | 64QAM |  | Outer_Full (Note 1) |
| 24 | 5920 | 256QAM |  | Outer_Full (Note 1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.3.1D.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.2E.3.1D.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO.

6.2E.3.1D.4.2	Test procedure

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure the sum of mean power of the UE at each transmit antenna connector in the channel bandwidth according to the test configuration from Table 6.2E.3.1D.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.3.1D.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.2E.3.1D.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing for NS_33

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | 10 |  |  |
| sl-FreqResourcePSCCH-r16 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | 10 |  | Test ID 5, 6, 7, 14, 18 |
|  | 12 |  | Test ID 4, 8, 9, 10, 11, 12, 13 |
|  | 15 |  | Test ID 1, 2, 3, 15, 16, 17 |
| sl-StartRB-Subchannel-r16 | 0 |  | Test ID 1, 11, 15, 18 |
|  | 1 |  | Test ID 2 |
|  | 26 |  | Test ID 3 |
|  | 38 |  | Test ID 4 |
|  | 12 |  | Test ID 5, 14 |
|  | 15 |  | Test ID 6 |
|  | 20 |  | Test ID 7 |
|  | 25 |  | Test ID 8 |
|  | 4 |  | Test ID 9 |
|  | 8 |  | Test ID 10 |
|  | 16 |  | Test ID 12 |
|  | 22 |  | Test ID 13 |
|  | 2 |  | Test ID 16 |
|  | 6 |  | Test ID 17 |
| sl-NumSubchannel-r16 | 1 |  | Test ID 1, 2, 3, 4 |
|  | 2 |  | Test ID 5, 6, 7, 8, |
|  | 3 |  | Test ID 9, 10, 11, 12, 13, 15, 16, 17 |
|  | 4 |  | Test ID 14 |
|  | 5 |  | Test ID 18 |
| } |  |  |  |

6.2E.3.1D.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.3.1D.5-1.

Table 6.2E.3.1D.5-0: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7dB | 1.0dB |

Table 6.2E.3.1D.5-1: UE MPR test requirement for contiguous PSCCH/PSSCH for NS_33

| Test ID | PPowerClass (dBm) | MPR (dB) | AMPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 23 | 4.5 | 24 | 0 | -1 | 7 | 25.0 + TT | -8-TT |
| 22 | 23 | 4.5 | 19 | 0 | 4 | 7 | 25.0 + TT | -3-TT |
| 32 | 23 | 4.5 | 6 | 0 | 17 | 5 | 25.0 + TT | 13-TT |
| 42 | 23 | 4.5 | 6 | 0 | 17 | 5 | 25.0 + TT | 13-TT |
| 52 | 23 | 4.5 | 11 | 0 | 12 | 6 | 25.0 + TT | 6-TT |
| 62 | 23 | 4.5 | 9.5 | 0 | 13.5 | 6 | 25.0 + TT | 7.5-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 6 | 25.0 + TT | 9-TT |
| 82 | 23 | 4.5 | 8 | 0 | 15 | 6 | 25.0 + TT | 9-TT |
| 92 | 23 | 4.5 | 16 | 0 | 7 | 7 | 25.0 + TT | 0-TT |
| 102 | 23 | 4.5 | 13.5 | 0 | 9.5 | 7 | 25.0 + TT | 2.5-TT |
| 112 | 23 | 4.5 | 22 | 0 | 1 | 7 | 25.0 + TT | -6-TT |
| 122 | 23 | 4.5 | 9.5 | 0 | 13.5 | 6 | 25.0 + TT | 7.5-TT |
| 132 | 23 | 4.5 | 8 | 0 | 15 | 6 | 25.0 + TT | 9-TT |
| 142 | 23 | 4.5 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| 152 | 23 | 4.5 | 19 | 0 | 4 | 7 | 25.0 + TT | -3-TT |
| 162 | 23 | 4.5 | 16 | 0 | 7 | 7 | 25.0 + TT | 0-TT |
| 172 | 23 | 4.5 | 13.5 | 0 | 9.5 | 7 | 25.0 + TT | 2.5-TT |
| 182 | 23 | 4.5 | 16 | 0 | 7 | 7 | 25.0 + TT | 0-TT |
| 19 | 23 | 2.5 | 3 | 0 | 20 | 6 | 25.0 + TT | 14-TT |
| 20 | 23 | 4.5 | 5 | 0 | 18 | 5 | 25.0 + TT | 13-TT |
| 21 | 23 | 7.0 | 6 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| 22 | 23 | 4.5 | 4.5 | 0 | 18.5 | 5 | 25.0 + TT | 13.5-TT |
| 23 | 23 | 4.5 | 5 | 0 | 18 | 5 | 25.0 + TT | 13-TT |
| 24 | 23 | 7.0 | 6 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.3.1D.5-0. NOTE 2: If the supported post antenna connector gain Gpost connector is declared by the UE, the lower limit is further lower down by Gpost connector* A-MPRStep. A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart. |  |  |  |  |  |  |  |  |

6.2E.3.2	UE maximum output power reduction for V2X / concurrent operation

6.2E.3.2.1	Test purpose

Same test purpose as in 6.2E.3.1 for concurrent operation.

6.2E.3.2.2	Test applicability

No test case details are specified. The AMPR requirements for concurrent operation apply and are tested in clauses 6.2.3 for NR Uu operation and 6.2E.3.1 for NR sidelink option.

< End of Changes >