3GPP TSG-RAN5 Meeting #102	R5-241027r2

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2700 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.3C.3.4 1Tx-2Tx UL switching between two bands in SUL configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has introduced time mask requirements for 1Tx-2Tx UL switching between two bands in SUL configuration, when non-SUL band is configured with intra-band contiguous CA. RAN5 testing is still missing.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test case of time mask requirements for 1Tx-2Tx UL switching between two bands in SUL configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1Tx-2Tx UL switching between two bands in SUL configuration couldn’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3C.3.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correting test case title to align with WP Removing redundant message exception of ‘PUSCH-Config’ R2: Adding MU/TT |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3C.3.2.4.3	Test requirement

The requirement for the power of carrier 1 measured in step 1.5, 1.8, 2.5, 2.8 of the test procedure and the power of carrier 2 measured in step 1.6 and 2.6 shall not exceed the values specified in table 6.3C.3.2.4.3-1.

Table 6.3C.3.2.4.3-1: General SUL Time mask for switching between two uplink carriers (On power)

|  | Measured output power |
| --- | --- |
| Transmit ON power | Same as table 6.2.1.5-1 for NUL carrier and table 6.2C.3.5-1 for SUL carrier |
| NOTE 1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2.1.5-3 |  |

6.3C.3.4	General transmit ON/OFF time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

6.3C.3.4.1	Test purpose

To verify that the time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors meets the requirements given in 6.3C.3.0.4 when the capability uplinkTxSwitchingPeriod is present.

6.3C.3.4.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL configuration with intra-band contiguous CA and dynamic UL 1Tx-2Tx switching.

6.3C.3.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3C.3.0.4.

6.3C.3.4.4	Test description

6.3C.3.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR SUL configuration specified in 5.5C. All of these configurations shall be tested with applicable test parameters for each SUL configuration, and are shown in table 6.3C.3.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3C.3.4.4.1-1: Test Configuration Table for SUL Tx switching

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for SUL carrier Mid range for Non-SUL aggregate carriers |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Highest for SUL carrier Highest NRB_agg for Non-SUL aggregate carriers |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier, lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  | SUL Configuration |  |
| 1 | N/A | Modulation | RB allocation (NOTE 3) |  | Modulation | RB allocation (NOTE 3) |
|  |  | CP-OFDM QPSK | Inner Full |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: SUL carrier is configured as Carrier 1 and Non-SUL carrier is configured as Carrier2. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 4: The specific configuration of each RB allocation is defined in Table 6.1A-1a. |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.6 for TE diagram and section A.3.2.3.11 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

6.3C.3.4.4.2	Test procedure

In this test case, one SUL carrier and two contiguous aggregated non-SUL carriers are configured. The SUL carrier is configured as Carrier 1, and both non-SUL carriers are configured as Carrer 2.

1.	Sub test 1: Switching period located in Carrier 1 (SUL carrier)

1.1 Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

1.2 The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.4.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier1 and FALSE on carrier 2.

1.3 SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

1.4 SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.5	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.4.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.6	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.4.4.1-1 on carrier 2 starting on slot n and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

1.7	Measure the output power of UE PUSCH transmission for carrier 1 during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

1.8	Measure the sum of output power of UE PUSCH transmission on carrier 2 over all antenna connectors and both component carriers during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n and in the end of slot m.

1.9	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.4.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.10	Measure the output power of UE PUSCH transmission for carrier 1 during slot m+1 excluding a switching period X and a transient period of 10 µs in the beginning of slot m+1. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

2.	Sub test 2: Switching period located in Carrier 2 (Non-SUL carriers)

2.1 Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.2 The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.4.4.3 with uplinkTxSwitchingPeriodLocation-r16 configured TRUE on carrier1 and FALSE on carrier 2.

2.3 SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

2.4	SS send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.5	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.4.4.1-1 on carrier 1 on slot n-1, where slot n is an uplink slot for carrier 2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.6	The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.4.4.1-1 on carrier 2 starting on slot n and slot m, with both slot n and slot m being uplink slots for carrier 2 and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.7	Measure the output power of UE PUSCH transmission for carrier 1 during slot n-1 excluding a transient period of 10 µs in the end of slot n-1.

2.8	Measure the sum of output power of UE PUSCH transmission on carrier 2 over all antenna connectors and both component carriers during slot n and slot m excluding a transient period of 10 µs and a Switching period X µs in the beginning of slot n and in the end of slot m. The length of uplink switching period X is indicated by UE capability uplinkTxSwitchingPeriod.

2.9	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3C.3.4.4.1-1 on carrier 1 on slot m+1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.10	Measure the output power of UE PUSCH transmission for carrier 1 during slot m+1 excluding a transient period of 10 µs in the beginning of slot m+1.

6.3C.3.4.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and following exceptions:

Table 6.3C.3.4.4.3-1: ServingCellConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkTxSwitching-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingPeriodLocation-r16 | TRUE |  | PL |
|  | FALSE |  | noPL |
| uplinkTxSwitchingCarrier-r16 | carrier1 |  | 1TxCC |
|  | carrier2 |  | 2TxCC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PL | The location of UL Tx switching period is configured in this carrier |
| noPL | The location of UL Tx switching period is not configured in this carrier |
| 1TxCC | The carrier is capable of one transmit antenna connector |
| 2TxCC | The carrier is capable of two transmit antenna connectors |

Table 6.3C.3.4.4.3-2: P-Max

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

6.3C.3.4.4.3	Test requirement

The requirement for the power of carrier 1 measured in step 1.7, 1.10, 2.7, 2.10 of the test procedures and the power of carrier 2 measured in step 1.8 and 2.8 shall not exceed the values specified in table 6.3C.3.4.4.3-1.

Table 6.3C.3.4.4.3-1: SUL Time mask for 2Tx-2Tx switching between two bands (On power)

|  | Measured output power |
| --- | --- |
| Transmit ON power | NUL: Same as Test ID 1 of Table 6.2H.1.2.5-1 or 6.2H.1.2.5-2 SUL: Same as Test ID 19 of Table 6.2C.4.5-1 to Table 6.2C.4.5-4 |
| NOTE 1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2.1.5-3 |  |

6.3C.4	Power control for SUL

6.3C.4.1	Absolute power tolerance for SUL

6.3C.4.1.1	Test purpose

Same test purpose as in clause 6.3.4.2.1

6.3C.4.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.4.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.4.2.

<Unchanged Text Skipped >

F.1.2	Measurement of transmitter

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.1 UE maximum output power | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2.2 Maximum Power Reduction (MPR) | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2.3 UE additional maximum output power reduction | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2.4 Configured transmitted power | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2A.1.1 UE maximum output power for CA (2UL CA) | For Inter-band CA MAX (MUCC1, MUCC2) | MUCCX is MU of each UL CC specified in single UL case 6.2.1. |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | For Inter-band CA MAX (MUCC1, MUCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD | MUCCX is MU of each UL CC specified in single UL case 6.2.2. |
| 6.2A.3.1 UE additional maximum output power reduction CA (2UL CA) | For Inter-band CA MAX (MUCC1, MUCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD | MUCCX is MU of each UL CC specified in single UL case 6.2.3. |
| 6.2A.4.1 Configured transmitted power for CA (2UL CA) | For Inter-band CA MAX (MUCC1, MUCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | MUCCX is MU of each UL CC specified in single UL case 6.2.4. |
| 6.2C.1 Configured transmitted power for SUL | Same as 6.2.4 |  |
| 6.2C.3 UE maximum output power for SUL | Same as 6.2.1 |  |
| 6.2C.4 UE maximum output power reduction for SUL | Same as 6.2.2 |  |
| 6.2C.5 UE additional maximum output power reduction for SUL | Same as 6.2.3 |  |
| 6.2D.1 UE maximum output power for UL MIMO | Same as 6.2.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2D.1_1 UE maximum output power for SUL with UL MIMO | Same as 6.2D.1 | Same as 6.2D.1 |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Same as 6.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2D.2_1 UE maximum output power reduction for SUL with UL MIMO | Same as 6.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2D.3 UE additional maximum output power reduction for UL MIMO | Same as 6.2.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2D.3_1 UE additional maximum output power reduction for SUL with UL MIMO | Same as 6.2.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2D.4 Configured transmitted power for UL MIMO | Same as 6.2.4 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.4 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2D.4_1 Configured transmitted power for SUL with UL MIMO | Same as 6.2D.4 | Same as 6.2D.4 |
| 6.2F.1 UE maximum output power for shared spectrum channel access | 4.2GHz < f ≤ 7.125GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2F.2 UE maximum output power reduction for shared spectrum access | Same as 6.2F.1 |  |
| 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Same as 6.2F.1 |  |
| 6.2F.4 Configured transmitted power for shared spectrum access | FFS |  |
| 6.2G.1 UE maximum output power for Tx Diversity | Same as 6.2.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2G.2 UE maximum output power reduction for Tx Diversity | Same as 6.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2G.3 UE additional maximum output power reduction for Tx Diversity | Same as 6.2.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2G.4 Configured transmitted power for Tx Diversity | Same as 6.2.4 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.4 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2H.1.1 UE maximum output power for intra-band UL contiguous CA with UL MIMO | Aggregated BW ≤ 100M: same as 6.2.1 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD |  |
| 6.2H.1.2 UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO | Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD |  |
| 6.2H.1.4 Configured transmitted power for intra-band UL contiguous CA with UL MIMO | Aggregated BW ≤ 100M: same as 6.2.4 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD |  |
| 6.2I.1 UE maximum output power for RedCap | Same as 6.2.1 for BW ≤ 20MHz |  |
| 6.3.1 Minimum output power | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.3 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.5 dB, BW ≤ 40MHz ±1.8 dB, 40MHz < BW ≤ 100MHz |  |
| 6.3.2 Transmit OFF power | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.3.2 General ON/OFF time mask | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.3.4 PRACH time mask | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.3.6 SRS time mask | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.4.2 Absolute power tolerance | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.4 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 40MHz ±2.2 dB, 80MHz < BW ≤ 100MHz | Test System uncertainty = SQRT (UL Meas Uncer2 + DL Meas Uncer2) |
| 6.3.4.3 Relative power tolerance | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2.1 same as 6.2.1. Absolute Uplink power measurement for step 1.1 same as 6.3.1. |  |
| 6.3.4.4 Aggregate power tolerance | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | Same as 6.3.1 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.3.1 Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD |  |
| 6.3A.3.1 Transmit ON/OFF time mask for CA (2UL CA) | Same as 6.3.3.2 for each CC |  |
| 6.3A.3.2 Time mask for switching between two uplink carriers | Same as 6.3.3.2 for each CC |  |
| 6.3A.3.3 Time mask for switching between two uplink carriers with two transmit antenna connectors | Same as 6.3.3.2 for each CC |  |
| 6.3A.4.1.1 Absolute power tolerance for CA (2UL CA) | Same as 6.3.4.2 for each CC |  |
| 6.3A.4.2.1 Power Control Relative power tolerance for CA (2UL CA) | Same as 6.3.4.3 for each CC |  |
| 6.3A.4.3.1 Aggregate power tolerance for CA (2UL CA) | Same as 6.3.4.4 for each CC |  |
| 6.3C.3.4 General transmit ON/OFF time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | ON power: Same as 6.3.3.2 for SUL carrier and for sum of power at each of UE antenna connector on NUL carriers |  |
| 6.3D.1 Minimum output power for UL MIMO | Same as 6.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3D.1_1 Minimum output power for SUL with UL MIMO | Same as 6.3D.1 | Same as 6.3D.1 |
| 6.3D.2 Transmit OFF power for UL MIMO | Same as 6.3.2 for each antenna |  |
| 6.3D.2_1 Transmit OFF power for SUL with UL MIMO | Same as 6.3D.2 |  |
| 6.3D.3 Transmit ON/OFF time mask for UL MIMO | ON power: Same as 6.2D.1 OFF power: Same as 6.3D.2 |  |
| 6.3D.3_1 Transmit ON/OFF time mask for SUL with UL MIMO | Same as 6.3D.3 |  |
| 6.3D.4.1 Absolute Power tolerance | Same as 6.3.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3D.4.1_1 Absolute power tolerance for SUL with UL MIMO | Same as 6.3D.4.1 | Same as 6.3D.4.1 |
| 6.3D.4.2 Relative Power tolerance | ±0.9 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < f ≤ 100MHz Absolute Uplink power measurement for step 2.1 same as 6.2.1. Absolute Uplink power measurement for step 1.1 same as 6.3.1. | MU is for the sum of power at each of UE antenna connector |
| 6.3D.4.2_1 Relative power tolerance for SUL with UL MIMO | Same as 6.3D.4.2 | Same as 6.3D.4.2 |
| 6.3D.4.3 Aggregate Power tolerance | Same as 6.3.4.4 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.4 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3D.4.3_1 Aggregate power tolerance for SUL with UL MIMO | Same as 6.3D.4.3 | Same as 6.3D.4.3 |
| 6.3F.1 Minimum output power for shared spectrum channel access | 4.2GHz < f ≤ 7.125GHz ±1.5 dB, BW ≤ 40MHz ±1.8 dB, 40MHz < BW ≤ 100MHz |  |
| 6.3F.2 Transmit OFF power for shared spectrum channel access | 4.2GHz < f ≤ 7.125GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3F.3.2 General ON/OFF time mask for shared spectrum channel access | 4.2GHz < f ≤ 5.925GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 5.925GHz < f ≤ 7.125GHz TBD |  |
| 6.3F.4.2 Absolute power tolerance for shared spectrum access | 4.2GHz < f ≤ 7.125GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 40MHz ±2.2 dB, 80MHz < BW ≤ 100MHz | Test System uncertainty = SQRT (UL Meas Uncer2 + DL Meas Uncer2) |
| 6.3G.1 Minimum output power for Tx Diversity | Same as 6.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3G.2 Transmit OFF power for Tx Diversity | Same as 6.3.2 for each antenna |  |
| 6.3G.3.1 General ON/OFF time mask for Tx Diversity | ON power: Same as 6.2G.1 OFF power: Same as 6.3G.2 |  |
| 6.3G.3.2 PRACH time mask for Tx Diversity | Same as 6.3.3.4 for each antenna |  |
| 6.3G.3.3 SRS time mask for Tx Diversity | Same as 6.3.3.6 for each antenna |  |
| 6.3G.4.1 Absolute power tolerance for Tx Diversity | Same as 6.3.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3G.4.2 Relative power tolerance for Tx Diversity | Same as 6.3.4.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3G.4.3 Aggregate power tolerance for Tx Diversity | Same as 6.3.4.4 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.4 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3H.1.1 Minimum output power for intra-band UL contiguous CA with UL MIMO | For each CC, same as 6.3.1 for the sum of power at each of UE antenna connector |  |
| 6.3H.1.2 Transmit OFF power for intra-band UL contiguous CA with UL MIMO | For each CC, same as 6.3.2 for each antenna |  |
| 6.3H.1.3 Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO | ON power: Same as 6.2H.1.2 OFF power: Same as 6.3H.1.2 |  |
| 6.4.1 Frequency Error | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz DL Signal level: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.4.2.1 Error Vector Magnitude | For up to 256QAM: f ≤ 6.0GHz, BW ≤ 100MHz 15 dBm < PUL PUSCH, PUCCH, PRACH: ±1.5 % -25 dBm < PUL ≤ 15 dBm PUSCH, PUCCH, PRACH: ±2.5 % -40dBm ≤ PUL ≤ -25dBm PUSCH, PUCCH, PRACH: ±3.0 % Absolute Uplink power measurement same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | FFS |  |
| 6.4.2.2 Carrier Leakage | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2 and step 4 same as 6.2.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4.2.3 In-band emissions | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for steps 1.2, 1.4, 2.2, and 2.4 same as 6.2.1. Absolute Uplink power measurement for steps 1.6, 1.8, 2.6, and 2.8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4.2.4 EVM equalizer spectrum flatness | ±1.4 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK | Same as 6.4.2.4 |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | For inter-band CA: same as 6.4.1 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.1 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD |  |
| 6.4A.2.1.1 Error Vector Magnitude for CA (2UL CA) | For inter-band CA: same as 6.4.2.1 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.2.1 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD Absolute Uplink power measurement same as 6.3A.1.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | For inter-band CA: same as 6.4.2.2 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.2.2 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD Uplink power measurement for step 5 and step 7 same as 6.2A.1.1. Absolute Uplink power measurement for step 9 and step 11 same as 6.3A.1.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4A.2.3.1 In-band emission for CA (2UL CA) | For inter-band CA: same as 6.4.2.3 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.2.3 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD Absolute Uplink power measurement for step 5 and step 7 same as 6.2A.1.1. Absolute Uplink power measurement for step 9 and step 11 same as 6.3A.1.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4C.1 Frequency error for SUL | Same as 6.4.1 |  |
| 6.4C.2.1 Error Vector Magnitude for SUL | Same as 6.4.2.1 |  |
| 6.4C.2.2 Carrier leakage for SUL | Same as 6.4.2.2 |  |
| 6.4C.2.3 In-band emissions for SUL | Same as 6.4.2.3 |  |
| 6.4C.2.4 EVM equalizer spectrum flatness for SUL | Same as 6.4.2.4 |  |
| 6.4D.1 Frequency error for UL MIMO | Same as 6.4.1 for each antenna |  |
| 6.4D.1_1 Frequency error for SUL with UL MIMO | Same as 6.4D.1 |  |
| 6.4D.2.1 Error Vector Magnitude for UL MIMO | Same as 6.4.2.1 for each antenna Absolute Uplink power measurement same as 6.3D.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4D.2.1_1 Error Vector Magnitude for SUL with UL MIMO | Same as 6.4D.2.1 |  |
| 6.4D.2.2 Carrier leakage for UL MIMO | Same as 6.4.2.2 for each antenna Absolute Uplink power measurement for step 2 and step 4 same as 6.2D.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3D.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4D.2.2_1 Carrier leakage for SUL with UL MIMO | Same as 6.4D.2.2 |  |
| 6.4D.2.3 In-band emissions for UL MIMO | Same as 6.4.2.3 for each antenna Absolute Uplink power measurement for steps 1.2 and 1.4 same as 6.2D.1. Absolute Uplink power measurement for steps 1.6 and 1.8 same as 6.3D.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4D.2.3_1 In-band emissions for SUL with UL MIMO | Same as 6.4D.2.3 |  |
| 6.4D.2.4 EVM equalizer spectrum flatness for UL MIMO | Same as 6.4.2.4 for each antenna |  |
| 6.4D.2.4_1 EVM equalizer spectrum flatness for SUL with UL MIMO | Same as 6.4.2.4 for each antenna |  |
| 6.4D.3 Time alignment error for UL MIMO | ±25ns |  |
| 6.4D.3_1 Time alignment error for SUL with UL MIMO | ±25ns |  |
| 6.4D.4 Requirements for Coherent UL MIMO | FFS |  |
| 6.4F.1 Frequency Error for shared spectrum access | ±36 Hz, f > 3.0GHz DL Signal level: ±1.5 dB, 4.2GHz < f ≤ 7.125GHz |  |
| 6.4F.2.1 Error Vector Magnitude for shared spectrum access | Same as 6.4.2.1 for f ≤ 5.925GHz TBD for f > 5.925GHz |  |
| 6.4F.2.2 Carrier Leakage | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 7.125GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2 and step 4 same as 6.2.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4F.2.3 In-band emissions | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 7.125GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for steps 1.2, 1.4, 2.2, and 2.4 same as 6.2.1. Absolute Uplink power measurement for steps 1.6, 1.8, 2.6, and 2.8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4F.2.4 EVM equalizer spectrum flatness | ±1.4 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.4G.1 Frequency Error for Tx Diversity | Same as 6.4.1 for each antenna |  |
| 6.4G.2.1 Error Vector Magnitude for Tx Diversity | FFS |  |
| 6.4G.2.2 Carrier Leakage for Tx Diversity | Same as 6.4.2.2 for each antenna Absolute Uplink power measurement for step 2 and step 4 same as 6.2G.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3G.1. Relative Uplink power measurement same as 6.3G.4.3. |  |
| 6.4G.2.3 In-band emissions for Tx Diversity | Same as 6.4.2.3 for each antenna Absolute Uplink power measurement for steps 1.2 and 1.4 same as 6.2G.1. Absolute Uplink power measurement for steps 1.6 and 1.8 same as 6.3G.1. Relative Uplink power measurement same as 6.3G.4.3. |  |
| 6.4G.2.4 EVM equalizer spectrum flatness for Tx Diversity | ±1.4 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.5.1 Occupied bandwidth | 1.5% of channel bandwidth |  |
| 6.5.2.2 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.5.2.3 Additional spectrum emission mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.5.2.4.1 NR ACLR | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 6.0GHz |  |
| 6.5.2.4.2 UTRA ACLR | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 6.0GHz |  |
| 6.5.3.1 General spurious emissions | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 6.5.3.2 Spurious emission for UE co-existence | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 6.5.3.3 Additional spurious emissions | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 6.5.4 Transmit intermodulation | f ≤ 3.0GHz ±2.7 dB, BW ≤ 40MHz ±3.1 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±3.7 dB, BW ≤ 40MHz ±4.0 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±5.1 dB, BW ≤ 40MHz ±5.3 dB, 40MHz < BW ≤ 100MHz | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) | For inter-band CA: same as 6.5.1 for each CC For intra-band CA: Aggregated BW ≤ 100M: same as 6.5.1 for aggregated channel bandwidth Aggregated BW > 100M: TBD |  |
| 6.5A.2.2.1 Spectrum emission mask for CA (2UL CA) | For inter-band CA: same as 6.5.2.2 for each CC For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.2.2 Aggregated BW > 100M: TBD |  |
| 6.5A.2.3.1 Additional Spectrum emission mask for CA (2UL CA) | For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.2.3 Aggregated BW > 100M: TBD |  |
| 6.5A.2.4.1.1 NR ACLR for CA (2UL CA) | For inter-band CA: same as 6.5.2.4.1 for each CC For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.2.4.1 Aggregated BW > 100M: TBD |  |
| 6.5A.2.4.2.1 URTA ACLR for CA (2UL CA) | For inter-band CA: same as 6.5.2.4.2 for each CC |  |
| 6.5A.3.1.1 General spurious emissions for CA (2UL CA) | For inter-band CA: same as 6.5.3.1 for each CC For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.3.1 Aggregated BW > 100M: TBD |  |
| 6.5A.3.2.1 Spurious emission for UE co-existence for CA (2UL CA) | For inter-band CA: same as 6.5.3.2 for each CC For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.3.2 Aggregated BW > 100M: TBD |  |
| 6.5A.3.3.1 Additional Spurious emission for CA (2UL CA) | For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.3.2 Aggregated BW > 100M: TBD |  |
| 6.5A.4.1 Transmit intermodulation for CA (2UL CA) | For inter-band CA: same as 6.5.4, for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: same as 6.5.4, for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD |  |
| 6.5C.1 Occupied bandwidth for SUL | Same as 6.5.1 |  |
| 6.5C.2.2 Spectrum Emission Mask for SUL | Same as 6.5.2.2 |  |
| 6.5C.2.3 Additional spectrum emission mask for SUL | Same as 6.5.2.3 |  |
| 6.5C.2.4.1 NR ACLR for SUL | Same 6.5.2.4.1 |  |
| 6.5C.2.4.2 UTRA ACLR for SUL | Same as 6.5.2.4.2 |  |
| 6.5C.3.1 General spurious emissions for SUL | Same as 6.5.3.1 |  |
| 6.5C.3.2 Spurious emission for UE co-existence for SUL | Same as 6.5.3.2 |  |
| 6.5C.3.3 Additional spurious emissions for SUL | Same as 6.5.3.3 |  |
| 6.5C.4 Transmit intermodulation for SUL | Same as 6.5.4 |  |
| 6.5D.1 Occupied bandwidth for UL MIMO | Same as 6.5.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.1_2 Occupied bandwidth for SUL with UL MIMO | Same as 6.5D.1 |  |
| 6.5D.2.2 Spectrum emission mask for UL MIMO | Same as 6.5.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.2.2_1 Spectrum emission mask for SUL with UL MIMO | Same as 6.5D.2.2 |  |
| ]6.5D.2.3 Additional spectrum emission mask for UL MIMO | Same as 6.5.2.3 for each antenna |  |
| 6.5D.2.4.1 NR ACLR for UL MIMO | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.2.4.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.2.4.1_1 NR ACLR for SUL with UL MIMO | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.2.4.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.2.4.2 UTRA ACLR for UL MIMO | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.2.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.2.4.2_1 UTRA ACLR for SUL with UL MIMO | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.2.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.3.1 General spurious emissions for UL MIMO | Same as 6.5.3.1 for each antenna |  |
| 6.5D.3.2 Spurious emissions for UE co-existence for UL MIMO | Same as 6.5.3.2 for each antenna |  |
| 6.5D.3.3 Additional spurious emissions for UL MIMO | Same as 6.5.3.3 for each antenna |  |
| 6.5D.3_1.1 General spurious emissions for UL MIMO (Rel-16 onward) | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.3_1.2 Spurious emissions for UE co-existence for UL MIMO (Rel-16 onward) | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.3.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.3_1.3 Additional spurious emissions for UL MIMO (Rel-16 onward) | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.3.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.3_2.1 General spurious emissions for SUL with UL MIMO | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.3_2.2 Spurious emissions for UE co-existence for SUL with UL MIMO | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.3.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.3_2.3 Additional spurious emissions for SUL with UL MIMO | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.5.3.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5D.4 Transmit intermodulation for UL MIMO | Same as 6.5.4 for each antenna |  |
| 6.5D.4_1 Transmit intermodulation for SUL with UL MIMO | Same as 6.5.4 for each antenna |  |
| 6.5F.1 Occupied bandwidth for shared spectrum channel access | Same as 6.5.1 for f ≤ 5.925GHz TBD for f > 5.925GHz |  |
| 6.5F.2.2 Spectrum emission mask for operation with shared spectrum channel access | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 7.125GHz |  |
| 6.5F.2.4.1 NR ACLR | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 7.125GHz |  |
| 6.5F.2.4.2 Shared spectrum channel access ACLR with additional requirement for NS_29 | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 7.125GHz |  |
| 6.5F.3.1 General spurious emissions | Same as in 6.5.3.1 |  |
| 6.5F.4 Transmit intermodulation for shared spectrum channel access | Same as 6.5.4 for each antenna with exception: 4.2GHz < f ≤ 7.125GHz ±5.1 dB, BW ≤ 40MHz ±5.3 dB, 40MHz < BW ≤ 100MHz |  |
| 6.5G.1 Occupied bandwidth for Tx Diversity | Same as 6.5.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.1 Spectrum emission mask for Tx Diversity | Same as 6.5.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.2 Additional spectrum emission mask for Tx Diversity | Same as 6.5.2.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.3.1 NR ACLR for Tx Diversity | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.3.2 UTRA ACLR for Tx Diversity | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.3.1 General spurious emissions for Tx Diversity | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.3.2 Spurious emissions for UE co-existence for Tx Diversity | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.3.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.3.3 Additional spurious emissions for Tx Diversity | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.3.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.4 Transmit intermodulation for Tx Diversity | Same as 6.5.4 for each antenna |  |
| 6.5H.1.2.1 Spectrum emission mask for intra-band UL contiguous CA for UL MIMO | Aggregated BW ≤ 100M: same as 6.5.2.2 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD |  |
| 6.5H.1.2.3 NR ACLR for intra-band UL contiguous CA for UL MIMO | Aggregated BW ≤ 100M: same as 6.5.2.4.1 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD |  |

F.1.3	Measurement of receiver

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |
| 7.3A Reference sensitivity for CA (Same MU apply to all subsections including 7.3A.1, 7.3A.1_1, 7.3A.2, 7.3A.3, 7.3A.4, etc.) | Same as 7.3.2 for each component carrier |  |
| 7.3C.2 Reference sensitivity power level | Same as 7.3.2 |  |
| 7.3D Reference sensitivity for MIMO | Same as 7.3.2 |  |
| 7.3D.2_1 Reference sensitivity power level for SUL with UL MIMO | Same as 7.3D |  |
| 7.3F.2 Reference sensitivity power level | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 7.125GHz |  |
| 7.3I.2 Reference sensitivity power level | Same as 7.3.2 |  |
| 7.4 Maximum input level | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement same as 6.2.1 |  |
| 7.4A Maximum input level for CA (Same MU apply to all subsections including 7.4A.1, 7.4A.2, 7.4A.3, 7.4A.4, etc.) | Same as 7.4 for each component carrier |  |
| 7.4D Maximum input level for UL MIMO | Downlink power same as 7.4 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.4D_1 Maximum input level for SUL with UL MIMO | Downlink power same as 7.4D Uplink power measurement same as 6.2D.1_1 | Same as 7.4D |
| 7.5 Adjacent channel selectivity | ACS value ±1.6 dB, f ≤ 3.0GHz ±2.3 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement same as 6.2.1 | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. |
| 7.5A Adjacent channel selectivity for CA (Same MU apply to all subsections including 7.5A.1, 7.5A.2, 7.5A.3, 7.5A.4, etc.) | Same as 7.5 for each component carrier | Same as 7.5 The wanted signal level uncertainty applies for each CC. Overall ACS uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |
| 7.5D Adjacent channel selectivity for UL MIMO | ACS value same as 7.5 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.5D_1 Adjacent channel selectivity for SUL with UL MIMO | ACS value same as 7.5D Uplink power measurement same as 6.2D.1_1 | Same as 7.5D |
| 7.5F.1 Adjacent channel selectivity for shared spectrum channel access | ACS value ±3.0 dB, 4.2GHz < f ≤ 7.125GHz Uplink power measurement same as 6.2F.1 | Same as 7.5 |
| 7.6.2 Inband Blocking | Blocking ±1.6 dB, f ≤ 3.0GHz ±2.3 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement same as 6.2.1 | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: Broadband noise not applicable |
| 7.6.3 Out-of-band blocking | Wanted signal, f ≤ 3.0GHz ±2.0 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±3.9 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.0 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Wanted signal, 4.2GHz < f ≤ 6GHz ±2.6 dB, Blocking, 1MHz < finterferer ≤ 3GHz ±4.2 dB, Blocking, 3GHz < finterferer ≤ 12.75GHz Uplink power measurement same as 6.2.1 | Out of band blocking, using CW interferer: Interferer ACLR not applicable Impact of interferer Broadband noise 0.8dB Figures are combined to give Test System uncertainty, using formula given for 7.6.2 |
| 7.6.4 Narrow band blocking | Blocking ± 2.0dB, f ≤ 3.0GHz ± 2.4dB, 3.0GHz < f ≤ 4.2GHz ± 3.1dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement same as 6.2.1 | Narrow band blocking, using CW interferer: Interferer ACLR not applicable Impact of interferer Broadband noise 0.8dB Figures are combined to give Test System uncertainty, using formula given for 7.6.2 |
| 7.6A.2 Inband Blocking for CA (Same MU apply to all subsections including 7.6A.2.1, 7.6A.2.2, 7.6A.2.3, 7.6A.2.4, etc.) | Same as 7.6.2 for each component carrier | Same as 7.6.2 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |
| 7.6A.3 Out-of-band Blocking for CA (Same MU apply to all subsections including 7.6A.3.1, 7.6A.3.2, 7.6A.3.3, 7.6A.3.4, etc.) | Same as 7.6.3 for each component carrier | Same as 7.6.3 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |
| 7.6A.4 Narrow band Blocking for CA (Same MU apply to all subsections including 7.6A.4.1, 7.6A.4.2, 7.6A.4.3, 7.6A.4.4, etc.) | Same as 7.6.4 for each component carrier | Same as 7.6.4 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |
| 7.6C.2 Inband Blocking for SUL | Same as 7.6.2 | Same as 7.6.2 |
| 7.6C.2_1.1 Inband Blocking for SUL with 2 DL CA | Same as 7.6A.2 | Same as 7.6A.2 |
| 7.6D.2 Inband blocking for UL MIMO | Blocking same as 7.6.2 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.6D.2_1 In-band blocking for SUL with UL MIMO | Blocking same as 7.6.2 Uplink power measurement same as 6.2D.1_1 |  |
| 7.6D.3 Out-of-band blocking for UL MIMO | Wanted signal same as 7.6.3 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.6D.3_1 Out-of-band blocking for SUL with UL MIMO | Wanted signal same as 7.6D.3 Uplink power measurement same as 6.2D.1_1 | Same as 7.6D.3 |
| 7.6D.4 Narrow-band blocking for UL MIMO | Blocking same as 7.6.4 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.6F.2.1 In-band blocking for shared spectrum channel access | Blocking ±3.0 dB, 4.2GHz < f ≤ 7.125GHz Uplink power measurement same as 6.2F.1 | Same as 7.6.2 |
| 7.7 Spurious response | Same as 7.6.3 | Same as 7.6.3 |
| 7.7A Spurious response for CA (Same MU apply to all subsections including 7.7A.1, 7.7A.2, 7.7A.3, etc.) | Same as 7.6A.3 | Same as 7.6A.3 |
| 7.7D Spurious response for UL MIMO | Same as 7.7 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.7D_1 Spurious response for SUL with UL MIMO | Same as 7.7D Uplink power measurement same as 6.2D.1_1 | Same as 7.7D |
| 7.7F.1 Spurious response for shared spectrum channel access | Same as 7.6.3 | Same as 7.6.3 |
| 7.8.2 Wide band Intermodulation | Intermodulation ± 2.3dB, f ≤ 3.0GHz ± 3.1dB, 3.0GHz < f ≤ 4.2GHz ± 4.3dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement same as 6.2.1 | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] |
| 7.8A.2 Wide band Intermodulation for CA (Same MU apply to all subsections including 7.8A.2.1, 7.8A.2.2, 7.8A.2.3, etc.) | Same as 7.8.2 for each component carrier | Same as 7.8.2 The wanted signal level uncertainty applies for each CC. Overall intermodulation uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |
| 7.8D.2 Intermodulation characteristics for UL MIMO | Intermodulation same as 7.8.2 Uplink power measurement same as 6.2D.1 | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 7.8D.2_1 Wide band Intermodulation for SUL with UL MIMO | Intermodulation same as 7.8D.2 Uplink power measurement same as 6.2D.1_1 | Same as 7.8D.2 |
| 7.8F.2 Wide band Intermodulation for shared spectrum channel access | Intermodulation ± 2.3dB, f ≤ 3.0GHz ± 3.1dB, 3.0GHz < f ≤ 4.2GHz ± 4.3dB, 4.2GHz < f ≤ 7.125GHz Uplink power measurement same as 6.2F.1 |  |
| 7.9 Spurious emissions | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 7.9A.1 Spurious emissions for CA (2DL CA) | Same as 7.9 |  |

F.2	Interpretation of measurement results (normative)

The measurement results returned by the Test System are compared – without any modification – against the Test Requirements. The Test Requirement is defined as a threshold considered in a test to assess compliance of the device; it might be either equal (“Shared Risk” principle) or relaxed (“Never fail a good DUT” principle) compared to the corresponding core specification value by an amount defined in Annex F.3 as Test Tolerance.

The “Shared Risk” and the “Never fail a good DUT” principles are defined in Rec. ITU-R M.1545.

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

F.3	Test Tolerance and Derivation of Test Requirements (informative)

The Test Requirements in the present document have been calculated by relaxing the Minimum Requirements of the core specification using the Test Tolerances defined in this clause. When the Test Tolerance is zero, the Test Requirement will be the same as the Minimum Requirement. When the Test Tolerance is non-zero, the Test Requirements will differ from the Minimum Requirements, and the formula used for the relaxation is given in this clause.

The Test Tolerances are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the Test Tolerances may sometimes be set to zero.

The test tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

The downlink Test Tolerances apply at each receiver antenna connector.

F.3.1	Measurement of test environments

The UE test environments are set to the values defined in TS 38.508-1 subclause 4.1, without any relaxation. The applied Test Tolerance is therefore zero.

F.3.2	Measurement of transmitter

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1 UE maximum output power | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2.2 Maximum Power Reduction (MPR) | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2.3 UE additional maximum output power reduction | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2.4 Configured transmitted power | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2A.1.1 UE maximum output power for CA (2UL CA) | For Inter-band CA MAX (TTCC1, TTCC2) | TTCCX is TT of each UL CC specified in single UL case 6.2.1. |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | For Inter-band CA MAX (TTCC1, TTCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD | TTCCX is TT of each UL CC specified in single UL case 6.2.2. |
| 6.2A.3.1 UE additional maximum output power reduction CA (2UL CA) | For Inter-band CA MAX (TTCC1, TTCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD | TTCCX is TT of each UL CC specified in single UL case 6.2.3. |
| 6.2A.4.1 Configured transmitted power for CA (2UL CA) | For Inter-band CA MAX (TTCC1, TTCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.4 for sum of powers of all CCs Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | TTCCX is TT of each UL CC specified in single UL case 6.2.4. |
| 6.2C.1 Configured transmitted power for SUL | Same as 6.2.4 | Same as 6.2.4 |
| 6.2C.3 UE maximum output power for SUL | Same as 6.2.1 | Same as 6.2.1 |
| 6.2C.4 UE maximum output power reduction for SUL | Same as 6.2.2 | Same as 6.2.2 |
| 6.2C.5 UE additional maximum output power reduction for SUL | Same as 6.2.3 | Same as 6.2.3 |
| 6.2D.1 UE maximum output power for UL MIMO | Same as 6.2.1 for the sum of power at each of UE antenna connector | Same as 6.2.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2D.1_1 UE maximum output power for SUL with UL MIMO | Same as 6.2D.1 | Same as 6.2D.1 |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Same as 6.2.2 for the sum of power at each of UE antenna connector | Same as 6.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2D.2_1 UE maximum output power reduction for SUL with UL MIMO | Same as 6.2.2 for the sum of power at each of UE antenna connector | Same as 6.2.2 |
| 6.2D.3 UE additional maximum output power reduction for UL MIMO | Same as 6.2.3 for the sum of power at each of UE antenna connector | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2D.3_1 UE additional maximum output power reduction for SUL with UL MIMO | Same as 6.2.3 for the sum of power at each of UE antenna connector | Same as 6.2.3 |
| 6.2D.4 Configured transmitted power for UL MIMO | Same as 6.2.4 for the sum of power at each of UE antenna connector | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2D.4_1 Configured transmitted power for SUL with UL MIMO | Same as 6.2D.4 | Same as 6.2D.4 |
| 6.2F.1 UE maximum output power for shared spectrum channel access | 3.0GHz < f ≤ 7.125GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2F.2 UE maximum output power reduction for shared spectrum access | Same as 6.2F.1 | Same as 6.2F. |
| 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Same as 6.2F.1 | Same as 6.2F. |
| 6.2F.4 Configured transmitted power for shared spectrum access | FFS | FFS |
| 6.2G.1 UE maximum output power for Tx Diversity | Same as 6.2.1 for the sum of power at each of UE antenna connector | Same as 6.2.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2G.2 UE maximum output power reduction for Tx Diversity | Same as 6.2.2 for the sum of power at each of UE antenna connector | Same as 6.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2G.3 UE additional maximum output power reduction for Tx Diversity | Same as 6.2.3 for the sum of power at each of UE antenna connector | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2G.4 Configured transmitted power for Tx Diversity | Same as 6.2.4 for the sum of power at each of UE antenna connector | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2H.1.1 UE maximum output power for intra-band UL contiguous CA with UL MIMO | Aggregated BW ≤ 100M: same as 6.2.1 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD | Same as 6.2.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2H.1.2 UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO | Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD | Same as 6.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2H.1.4 Configured transmitted power for intra-band UL contiguous CA with UL MIMO | Aggregated BW ≤ 100M: same as 6.2.4 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.2I.1 UE maximum output power for RedCap | Same as 6.2.1 for BW ≤ 20MHz | Same as 6.2.1 |
| 6.3.1 Minimum output power | f ≤ 3.0GHz 1.0 dB, BW ≤ 40MHz 1.3 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.3 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.3.2 Transmit OFF power | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.3.3.2 General ON/OFF time mask | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: –Same as 6.2.1 |
| 6.3.3.4 PRACH time mask | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: Upper limit + TT, Lower limit - TT |
| 6.3.3.6 SRS time mask | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: Upper limit + TT, Lower limit - TT |
| 6.3.4.2 Absolute power tolerance | UL Power ≥ 0dBm f ≤ 3.0GHz 1.0 dB, BW ≤ 40MHz 1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.4 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| 6.3.4.3 Relative power tolerance | 0.7 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| 6.3.4.4 Aggregate power tolerance | 0.7 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | Same as 6.3.1 | Minimum requirement + TT |
| 6.3A.3.1 Transmit ON/OFF time mask for CA (2UL CA) | Same as 6.3.3.2 For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.3.3.2 Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | Minimum requirement + TT |
| 6.3A.3.2 Time mask for switching between two uplink carriers | ON power: same as 6.2A.2.1 for inter-band CA | Same as 6.2A.2.1 for inter-band CA |
| 6.3A.3.3 Time mask for switching between two uplink carriers with two transmit antenna connectors | ON power: same as 6.2A.2.1 for inter-band CA | Same as 6.2A.2.1 for inter-band CA |
| 6.3A.4.1.1 Absolute power tolerance for CA (2UL CA) | Same as 6.3.4.2 for each CC | Upper limit + TT, Lower limit – TT |
| 6.3A.4.2.1 Power Control Relative power tolerance for CA (2UL CA) | Same as 6.3.4.3 for each CC | Upper limit + TT, Lower limit – TT |
| 6.3A.4.3.1 Aggregate power tolerance for CA (2UL CA) | Same as 6.3.4.4 for each CC | Upper limit + TT, Lower limit – TT |
| 6.3C.1 Minimum output power for SUL | Same as 6.3.1 | Same as 6.3.1 |
| 6.3C.2 Transmit OFF power for SUL | Same as 6.3.2 | Same as 6.3.2 |
| 6.3C.3.1 Transmit ON/OFF time mask for SUL | Same as 6.3.3.2 | Same as 6.3.3.2 |
| 6.3C.3.2 General transmit ON/OFF time mask for switching between two uplink carriers | ON power: Same as 6.3.3.2 | ON power: Same as 6.3.3.2 |
| 6.3C.3.4 General transmit ON/OFF time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | ON power: Same as 6.3.3.2 for SUL carrier and for sum of power at each of UE antenna connector on NUL carriers | ON power: Same as 6.3.3.2 |
| 6.3C.4.1 Absolute power tolerance for SUL | Same as 6.3.4.2 | Same as 6.3.4.2 |
| 6.3C.4.2 Power Control Relative power tolerance for SUL | Same as 6.3.4.3 | Same as 6.3.4.3 |
| 6.3C.4.3 Aggregate power tolerance for SUL | Same as 6.3.4.4 | Same as 6.3.4.4 |
| 6.3D.1 Minimum output power for UL MIMO | Same as 6.3.1 for the sum of power at each of UE antenna connector | Same as 6.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3D.1_1 Minimum output power for SUL with UL MIMO | Same as 6.3D.1 | Same as 6.3D.1 |
| 6.3D.2 Transmit OFF power for UL MIMO | Same as 6.3.2 for each antenna | Same as 6.3.2 Uplink power measurement applies to each Tx antenna connector |
| 6.3D.2_1 Transmit OFF power for SUL with UL MIMO | Same as 6.3D.2 | Same as 6.3D.2 |
| 6.3D.3 Transmit ON/OFF time mask for UL MIMO | ON power: Same as 6.2D.1 OFF power: Same as 6.3D.2 | ON power: Same as 6.2D.1 OFF power: Same as 6.3D.2 |
| 6.3D.3_1 Transmit ON/OFF time mask for SUL with UL MIMO | Same as 6.3D.3 | Same as 6.3D.3 |
| 6.3D.4.1 Absolute Power tolerance | Same as 6.3.4.2 for the sum of power at each of UE antenna connector | Same as 6.3.4.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3D.4.1_1 Absolute power tolerance for SUL with UL MIMO | Same as 6.3D.4.1 | Same as 6.3D.4.1 |
| 6.3D.4.2 Relative Power tolerance | Same as 6.3.4.3 for the sum of power at each of UE antenna connector | Same as 6.3.4.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3D.4.2_1 Relative power tolerance for SUL with UL MIMO | Same as 6.3D.4.2 | Same as 6.3D.4.2 |
| 6.3D.4.3 Aggregate Power tolerance | Same as 6.3.4.4 for the sum of power at each of UE antenna connector | Same as 6.3.4.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3D.4.3_1 Aggregate power tolerance for SUL with UL MIMO | Same as 6.3D.4.3 | Same as 6.3D.4.3 |
| 6.3F.1 Minimum output power | 3.0GHz < f ≤ 7.125GHz 1.3 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.3F.2 Transmit OFF power | 3.0GHz < f ≤ 7.125GHz 1.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.3F.3.2 General ON/OFF time mask | 3.0GHz < f ≤ 7.125GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: Upper limit + TT, Lower limit - TT |
| 6.3F.4.2 Absolute power tolerance for shared spectrum access | UL Power ≥ 0dBm 4.2GHz < f ≤ 7.125GHz 1.4 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| 6.3G.1 Minimum output power for Tx Diversity | Same as 6.3.1 for the sum of power at each of UE antenna connector | Same as 6.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3G.2 Transmit OFF power for Tx Diversity | Same as 6.3.2 for each antenna | Same as 6.3.2 Uplink power measurement applies to each Tx antenna connector |
| 6.3G.3.1 General ON/OFF time mask for Tx Diversity | ON power: Same as 6.2G.1 OFF power: Same as 6.3G.2 | ON power: Same as 6.2G.1 OFF power: Same as 6.3G.2 |
| 6.3G.3.2 PRACH time mask for Tx Diversity | Same as 6.3.3.4 for each antenna | Same as 6.3.3.4 Uplink power measurement applies to each Tx antenna connector |
| 6.3G.3.3 SRS time mask for Tx Diversity | Same as 6.3.3.6 for each antenna | Same as 6.3.3.6 Uplink power measurement applies to each Tx antenna connector |
| 6.3G.4.1 Absolute power tolerance for Tx Diversity | Same as 6.3.4.2 for the sum of power at each of UE antenna connector | Same as 6.3.4.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3G.4.2 Relative power tolerance for Tx Diversity | Same as 6.3.4.3 for the sum of power at each of UE antenna connector | Same as 6.3.4.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3G.4.3 Aggregate power tolerance for Tx Diversity | Same as 6.3.4.4 for the sum of power at each of UE antenna connector | Same as 6.3.4.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3H.1.1 Minimum output power for intra-band UL contiguous CA with UL MIMO | For each CC, same as 6.3.1 for the sum of power at each of UE antenna connector | Same as 6.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3H.1.2 Transmit OFF power for intra-band UL contiguous CA with UL MIMO | For each CC, same as 6.3.2 for each antenna | Same as 6.3.2 Uplink power measurement applies to each Tx antenna connector |
| 6.3H.1.3 Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO | ON power: Same as 6.2H.1.2 OFF power: Same as 6.3H.1.2 | ON power: Same as 6.2H.1.2 OFF power: Same as 6.3H.1.2 |
| 6.4.1 Frequency Error | 15 Hz | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.4.2.1 Error Vector Magnitude | For up to 64QAM 0% For 256QAM f ≤ 6.0GHz, BW ≤ 100MHz 0.3%, 15dBm < PUL 0.8%, -25dBm < PUL ≤ 15dBm, 1.1%, -40dBm ≤ PUL ≤ -25dBm | Minimum requirement + TT |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | FFS | Same as 6.4.2.1 |
| 6.4.2.2 Carrier Leakage | 0.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.4.2.3 In-band emissions | 0.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.4.2.4 EVM equalizer spectrum flatness | 1.4 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK | Same as 6.4.2.4 | Minimum requirement + TT |
| 6.4A.1.1 Frequency error for CA (2UL CA) | For inter-band CA: same as 6.4.1 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.1 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | Modulated carrier frequency: Upper limit + TT, Lower limit – TT |
| 6.4A.2.1.1 Error Vector Magnitude for CA (2UL CA) | For up to 64QAM 0% For 256QAM For inter-band CA: same as 6.4.2.1 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.2.1 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | Minimum requirement + TT |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | For inter-band CA: same as 6.4.2.2 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.2.2 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | Minimum requirement + TT |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) | For inter-band CA: same as 6.4.2.3 for each CC For intra-band contiguous UL CA: Aggregated BW ≤ 100M: Same as 6.4.2.3 for each CC Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | Minimum requirement + TT |
| 6.4C.1 Frequency error for SUL | Same as 6.4.1 | Minimum requirement + TT |
| 6.4C.2.1 Error Vector Magnitude for SUL | Same as 6.4.2.1 | Minimum requirement + TT |
| 6.4C.2.2 Carrier leakage for SUL | Same as 6.4.2.2 | Minimum requirement + TT |
| 6.4C.2.3 In-band emissions for SUL | Same as 6.4.2.3 | Minimum requirement + TT |
| 6.4C.2.4 EVM equalizer spectrum flatness for SUL | Same as 6.4.2.4 | Minimum requirement + TT |
| 6.4D.1 Frequency error for UL MIMO | Same as 6.4.1 for each antenna | Same as 6.4.1 |
| 6.4D.1_1 Frequency error for SUL with UL MIMO | Same as 6.4D.1 | Same as 6.4D.1 |
| 6.4D.2.1 Error Vector Magnitude for UL MIMO | Same as 6.4.2.1 for each antenna | Same as 6.4.2.1 |
| 6.4D.2.1_1 Error Vector Magnitude for SUL with UL MIMO | Same as 6.4D.2.1 | Same as 6.4D.2.1 |
| 6.4D.2.2 Carrier leakage for UL MIMO | Same as 6.4.2.2 for each antenna | Same as 6.4.2.2 |
| 6.4D.2.2_1 Carrier leakage for SUL with UL MIMO | Same as 6.4.2.2 for each antenna | Same as 6.4.2.2 |
| 6.4D.2.3 In-band emissions for UL MIMO | Same as 6.4.2.3 for each antenna | Same as 6.4.2.3 |
| 6.4D.2.3_1 In-band emissions for SUL with UL MIMO | Same as 6.4.2.3 for each antenna | Same as 6.4.2.3 |
| 6.4D.2.4 EVM equalizer spectrum flatness for UL MIMO | Same as 6.4.2.4 for each antenna | Same as 6.4.2.4 |
| 6.4D.2.4_1 EVM equalizer spectrum flatness for SUL with UL MIMO | Same as 6.4.2.4 for each antenna | Same as 6.4.2.4 |
| 6.4D.3 Time alignment error for UL MIMO | 25ns | Minimum Requirement + TT |
| 6.4D.3_1 Time alignment error for SUL with UL MIMO | 25ns | Minimum Requirement + TT |
| 6.4D.4 Requirements for Coherent UL MIMO | FFS | FFS |
| 6.4F.1 Frequency Error | 15 Hz | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.4F.2.1 Error Vector Magnitude for shared spectrum access | Same as 6.4.2.1 for f ≤ 5.925GHz TBD for f > 5.925GHz | Minimum requirement + TT |
| 6.4F.2.2 Carrier Leakage | Same as 6.4.2.2 | Same as 6.4.2.2 |
| 6.4F.2.3 In-band emissions | Same as 6.4.2.3 | Same as 6.4.2.3 |
| 6.4F.2.4 EVM equalizer spectrum flatness | Same as 6.4.2.4 | Same as 6.4.2.4 |
| 6.4G.1 Frequency Error for Tx Diversity | Same as 6.4.1 for each antenna | Same as 6.4.1 |
| 6.4G.2.1 Error Vector Magnitude for Tx Diversity | FFS | FFS |
| 6.4G.2.2 Carrier Leakage for Tx Diversity | Same as 6.4.2.2 for each antenna | Same as 6.4.2.2 |
| 6.4G.2.3 In-band emissions for Tx Diversity | Same as 6.4.2.3 for each antenna | Same as 6.4.2.3 |
| 6.4G.2.4 EVM equalizer spectrum flatness for Tx Diversity | Same as 6.4.2.4 for each antenna | Same as 6.4.2.4 |
| 6.5.1 Occupied bandwidth | 0 kHz | Minimum requirement + TT |
| 6.5.2.2 Spectrum Emission Mask | 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6.0GHz | Minimum requirement + TT |
| 6.5.2.3 Additional spectrum emission mask | 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6.0GHz | Minimum requirement + TT |
| 6.5.2.4.1 NR ACLR | Absolute requirement 0 dB Relative requirement 0.8 dB | Absolute requirement ACLR Minimum Requirement + TT Relative requirement ACLR Minimum Requirement - TT |
| 6.5.2.4.2 UTRA ACLR | Same as 6.5.2.4.1 | Same as 6.5.2.4.1 |
| 6.5.3.1 General spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.3.2 Spurious emission for UE co-existence | 0 dB | Minimum requirement + TT |
| 6.5.3.3 Additional spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.4 Transmit intermodulation | 0 dB | CW interferer Minimum Requirement - TT |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) | For inter-band CA: same as 6.5.1 for each CC For intra-band CA: Aggregated BW ≤ 100M: same as 6.5.1 for aggregated channel bandwidth Aggregated BW > 100M: TBD |  |
| 6.5A.2.2.1 Spectrum emission mask for CA (2UL CA) | For inter-band CA: same as 6.5.2.2 for each CC For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.2.2 Aggregated BW > 100M: TBD | Minimum requirement + TT |
| 6.5A.2.3.1 Additional Spectrum emission mask for CA (2UL CA) | For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.2.3 Aggregated BW > 100M: TBD |  |
| 6.5A.2.4.1.1 NR ACLR for CA (2UL CA) | For inter-band CA: same as 6.5.2.4.1 for each CC For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.5.2.4.1 Aggregated BW > 100M: TBD | Same as 6.5.2.4.1 |
| 6.5A.2.4.2.1 UTRA ACLR for CA (2UL CA) | For inter-band CA: same as 6.5.2.4.2 for each CC | Same as 6.5.2.4.2 |
| 6.5A.3.1.1 General spurious emissions for CA (2UL CA) | 0 dB | Minimum requirement + TT |
| 6.5A.3.2.1 Spurious emissions for UE co-existence for CA (2UL CA) | 0 dB | Minimum requirement + TT |
| 6.5A.3.3.1 Additional Spurious emission for CA (2UL CA) | 0dB |  |
| 6.5A.4.1 Transmit intermodulation for CA (2UL CA) | 0 dB For intra-band contiguous UL CA: Aggregated BW ≤ 100M: 0 dB Aggregated BW > 100M: TBD For intra-band non-contiguous CA: TBD | CW interferer Minimum Requirement - TT |
| 6.5C.1 Occupied bandwidth for SUL | Same as 6.5.1 | Same as 6.5.1 |
| 6.5C.2.2 Spectrum Emission Mask for SUL | Same as 6.5.2.2 | Same as 6.5.2.2 |
| 6.5C.2.3 Additional spectrum emission mask for SUL | Same as 6.5.2.3 | Same as 6.5.2.3 |
| 6.5C.2.4.1 NR ACLR for SUL | Same as 6.5.2.4.1 | Same as 6.5.2.4.1 |
| 6.5C.2.4.2 UTRA ACLR for SUL | Same as 6.5.2.4.2 | Same as 6.5.2.4.2 |
| 6.5C.3.1 General spurious emissions for SUL | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5C.3.2 Spurious emission for UE co-existence for SUL | Same as 6.5.3.2 | Same as 6.5.3.2 |
| 6.5C.3.3 Additional spurious emissions for SUL | Same as 6.5.3.3 | Same as 6.5.3.3 |
| 6.5C.4 Transmit intermodulation for SUL | Same as 6.5.4 | Same as 6.5.4 |
| 6.5D.1 Occupied bandwidth for UL MIMO | Same as 6.5.1 for the sum of power at each of UE antenna connector | Same as 6.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.1_2 Occupied bandwidth for SUL with UL MIMO | Same as 6.5D.1 | Same as 6.5D.1 |
| 6.5D.2.2 Spectrum emission mask for UL MIMO | Same as 6.5.2.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.2.2_1 Spectrum emission mask for SUL with UL MIMO | Same as 6.5D.2.2 | Same as 6.5D.2.2 |
| 6.5D.2.3 Additional spectrum emission mask for UL MIMO | Same as 6.5.2.3 for each antenna | Same as 6.5.2.3 |
| 6.5D.2.4.1 NR ACLR for UL MIMO | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.2.4.1_1 NR ACLR for SUL with UL MIMO | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.1 |
| 6.5D.2.4.2 UTRA ACLR for UL MIMO | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.2.4.2_1 UTRA ACLR for SUL with UL MIMO | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.2 |
| 6.5D.3.1 General spurious emissions for UL MIMO | Same as 6.5.3.1 for each antenna | Same as 6.5.3.1 |
| 6.5D.3.2 Spurious emissions for UE co-existence for UL MIMO | Same as 6.5.3.2 for each antenna | Same as 6.5.3.2 |
| 6.5D.3.3 Additional spurious emissions for UL MIMO | Same as 6.5.3.3 for each antenna | Same as 6.5.3.3 |
| 6.5D.3_1.1 General spurious emissions for UL MIMO (Rel-16 onward) | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | Same as 6.5.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.3_1.2 Spurious emissions for UE co-existence for UL MIMO (Rel-16 onward) | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | Same as 6.5.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.3_1.3 Additional spurious emissions for UL MIMO (Rel-16 onward) | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | Same as 6.5.3.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.3_2.1 General spurious emissions for SUL with UL MIMO | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | Same as 6.5.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.3_2.2 Spurious emissions for UE co-existence for SUL with UL MIMO | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | Same as 6.5.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.3_2.3 Additional spurious emissions for SUL with UL MIMO | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | Same as 6.5.3.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.5D.4 Transmit intermodulation for UL MIMO | Same as 6.5.4 for each antenna | Same as 6.5.4 |
| 6.5D.4_1 Transmit intermodulation for SUL with UL MIMO | Same as 6.5.4 for each antenna | Same as 6.5.4 |
| 6.5F.1 Occupied bandwidth for shared spectrum channel access | Same as 6.5.1 for f ≤ 5.925GHz TBD for f > 5.925GHz | Same as 6.5.1 for f ≤ 5.925GHz TBD for f > 5.925GHz |
| 6.5F.2.2 Spectrum emission mask for operation with shared spectrum channel access | 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 7.125GHz | Minimum requirement + TT |
| 6.5F.2.4.1 NR ACLR | Absolute requirement 0 dB Relative requirement 0.8 dB | Absolute requirement ACLR Minimum Requirement + TT Relative requirement ACLR Minimum Requirement - TT |
| 6.5F.2.4.2 Shared spectrum channel access ACLR with additional requirement for NS_29 | Same as 6.5F.2.4.1 | Same as 6.5F.2.4.1 |
| 6.5F.3.1 General spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5F.4 Transmit intermodulation for shared spectrum channel access | 0 dB | CW interferer Minimum Requirement - TT |
| 6.5G.1 Occupied bandwidth for Tx Diversity | Same as 6.5.1 for the sum of power at each of UE antenna connector | Same as 6.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.1 Spectrum emission mask for Tx Diversity | Same as 6.5.2.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.2 Additional spectrum emission mask for Tx Diversity | Same as 6.5.2.3 for the sum of power at each of UE antenna connector | Same as 6.5.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.3.1 NR ACLR for Tx Diversity | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.3.2 UTRA ACLR for Tx Diversity | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.3.1 General spurious emissions for Tx Diversity | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | Same as 6.5.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.3.2 Spurious emissions for UE co-existence for Tx Diversity | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | Same as 6.5.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.3.3 Additional spurious emissions for Tx Diversity | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | Same as 6.5.3.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.4 Transmit intermodulation for Tx Diversity | Same as 6.5.4 for each antenna | Same as 6.5.4 |
| 6.5H.1.2.1 Spectrum emission mask for intra-band UL contiguous CA for UL MIMO | Aggregated BW ≤ 100M: same as 6.5.2.2 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD | Same as 6.5.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5H.1.2.3 NR ACLR for intra-band UL contiguous CA for UL MIMO | Aggregated BW ≤ 100M: same as 6.5.2.4.1 for sum of powers of all CCs and both antennas Aggregated BW > 100M: TBD | Same as 6.5.2.4.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |

F.3.3	Measurement of receiver

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 6.0GHz | Reference sensitivity power level + TT T-put limit unchanged |
| 7.3A Reference sensitivity for CA (Same TT apply to all subsections including 7.3A.1, 7.3A.1_1, 7.3A.2, 7.3A.3, 7.3A.4, etc.) | Same as 7.3.2 for each component carrier | Same as 7.3.2 for each component carrier |
| 7.3C.2 Reference sensitivity power level | Same as 7.3.2 | Same as 7.3.2 |
| 7.3D Reference sensitivity for MIMO | Same as 7.3.2 | Same as 7.3.2 |
| 7.3D.2_1 Reference sensitivity power level for SUL with UL MIMO | Same as 7.3D | Same as 7.3D |
| 7.3F Reference sensitivity for shared spectrum channel access | 1.0 dB, 3.0GHz < f ≤ 7.125GHz | Same as 7.3.2 |
| 7.3I.2 Reference sensitivity power level | Same as 7.3.2 | Same as 7.3.2 |
| 7.4 Maximum input level | 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 6.0GHz | Maximum input level - TT |
| 7.4A Maximum input level for CA (Same TT apply to all subsections including 7.4A.1, 7.4A.2, 7.4A.3, 7.4A.4, etc.) | Same as 7.4 for each component carrier | Same as 7.4 for each component carrier |
| 7.4D Maximum input level for UL MIMO | Same as 7.4 | Same as 7.4 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.4D_1 Maximum input level for SUL with UL MIMO | Same as 7.4D | Same as 7.4D |
| 7.5 Adjacent channel selectivity | 0 dB | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.5A Adjacent channel selectivity for CA (Same TT apply to all subsections including 7.5A.1, 7.5A.2, 7.5A.3, 7.5A.4, etc.) | Same as 7.5 for each component carrier | Same as 7.5 for each component carrier |
| 7.5D Adjacent channel selectivity for UL MIMO | Same as 7.5 | Same as 7.5 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.5D_1 Adjacent channel selectivity for SUL with UL MIMO | Same as 7.5D | Same as 7.5D |
| 7.5F.1 Adjacent channel selectivity for shared spectrum channel access | Same as 7.5 | Same as 7.5 |
| 7.6.2 Inband Blocking | 0 dB | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.3 Out-of-band blocking | 0 dB | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.4 Narrow band blocking | 0 dB | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6A.2 Inband Blocking for CA (Same TT apply to all subsections including 7.6A.2.1, 7.6A.2.2, 7.6A.2.3, 7.6A.2.4, etc.) | Same as 7.6.2 for each component carrier | Same as 7.6.2 for each component carrier |
| 7.6A.3 Out-of-band Blocking for CA (Same TT apply to all subsections including 7.6A.3.1, 7.6A.3.2, 7.6A.3.3, 7.6A.3.4, etc.) | Same as 7.6.3 for each component carrier | Same as 7.6.3 for each component carrier |
| 7.6A.4 Narrow band Blocking for CA (Same TT apply to all subsections including 7.6A.4.1, 7.6A.4.2, 7.6A.4.3, 7.6A.4.4, etc.) | Same as 7.6.4 for each component carrier | Same as 7.6.4 for each component carrier |
| 7.6C.2 Inband Blocking for SUL | Same as 7.6.2 | Same as 7.6.2 |
| 7.6C.2_1.1 Inband Blocking for SUL with 2 DL CA | Same as 7.6A.2 | Same as 7.6A.2 |
| 7.6C.3 Out-of-band blocking for SUL | Same as 7.6.3 | Same as 7.6.3 |
| 7.6D.2 Inband blocking for UL MIMO | Same as 7.6.2 | Same as 7.6.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6D.2_1 In-band blocking for SUL with UL MIMO | Same as 7.6D.2 | Same as 7.6D.2 |
| 7.6D.3 Out-of-band blocking for UL MIMO | Same as 7.6.3 | Same as 7.6.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6D.3_1 Out-of-band blocking for SUL with UL MIMO | Same as 7.6D.3 | Same as 7.6D.3 |
| 7.6D.4 Narrow-band blocking for UL MIMO | Same as 7.6.4 | Same as 7.6.4 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6F.2.1 In-band blocking for shared spectrum channel access | Same as 7.6.2 | Same as 7.6.2 |
| 7.6F.3.1 Out-of-band blocking for shared spectrum channel access | Same as 7.6.3 | Same as 7.6.3 |
| 7.7 Spurious response | 0 dB | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7A Spurious response for CA (Same TT apply to all subsections including 7.7A.1, 7.7A.2, 7.7A.3, etc.) | Same as 7.7 for each component carrier | Same as 7.7 for each component carrier |
| 7.7D Spurious response for UL MIMO | Same as 7.7 | Same as 7.7 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.7D_1 Spurious response for SUL with UL MIMO | Same as 7.7D | Same as 7.7D |
| 7.7F.1 Spurious response for shared spectrum channel access | Same as 7.7 | Same as 7.7 |
| 7.8.2 Wide band Intermodulation | 0 dB | Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |
| 7.8A.2 Wide band Intermodulation for CA (Same TT apply to all subsections including 7.8A.2.1, 7.8A.2.2, 7.8A.2.3, etc.) | Same as 7.8.2 for each component carrier | Same as 7.8.2 for each component carrier |
| 7.8D.2 Intermodulation characteristics for UL MIMO | Same as 7.8.2 | Same as 7.8.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.8D.2_1 Wide band Intermodulation for SUL with UL MIMO | Same as 7.8D.2 | Same as 7.8D.2 |
| 7.8F Intermodulation characteristics for shared spectrum channel access | Same as 7.8.2 | Same as 7.8.2 |
| 7.9 Spurious emissions | 0 dB | Minimum requirement + TT |
| 7.9A.1 Spurious emissions for CA (2DL CA) | Same as 7.9 | Same as 7.9 |

< End of Changes >