3GPP TSG-RAN5 Meeting #102	R5-241041r1

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5292 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UAS EPS test case 10.10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 10.10.4 [UAS / UE requested PDN connection establishment / UUAA / Authorization failure of C2 Communication] shall be added as specified in UAS EPS WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 10.10.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAS EPS WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.4 (New) |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1443 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-241041r1: 1. Added spec name in derivation path of specific message contents Table 10.10.4.3.3-1 |  |  |  |  |  |  |  |  |

<Start of modified section>

### 10.10.4	UAS / UE requested PDN connection establishment / UUAA / Authorization failure of C2 Communication

10.10.4.1	Test Purpose (TP)

(1)

with { UE having peformed successful UUAA-SM procedure for USS communication establishes PDN connection with service-level-AA container including CAA-level UAV ID for C2 communication }

ensure that {

when { UE receives MODIFY EPS BEARER CONTEXT REQUEST message with service-level-AA container including service-level-AA response parameter with the C2AR field set to "C2 authorization was not successful or C2 authorization is revoked" }

then { the UE transmits MODIFY EPS BEARER CONTEXT ACCEPT message }

}

10.10.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 6.3.13.3, 6.4.3.1, 6.4.3.2 and 6.4.3.3. Unless otherwise stated these are Rel-17 requirements.

[TS 24.301, clause 6.3.13.3]

The network supports C2 communication authorization for pairing of UAV and UAV-C. The pairing of UAV and UAV-C needs to be authorized by USS successfully before the user plane connectivity for C2 communication (over Uu or over NR-PC5) is enabled. The UE supporting UAS services may provide the network with an identification information of UAV-C to pair with, if available, via the protocol configuration options as follows:

-	If the UE uses a common PDN connectivity for both USS communication and C2 communication with a UAV-C, the C2 communication with the UAV-C can be authorized using UUAA-SM procedure during the PDN connectivity procedure or during the bearer resource modification procedure. If the pairing of UAV and UAV-C is revoked, the network shall disable C2 communication for the PDN connection.

NOTE 1:	The network can disable C2 communication for the PDN connection e.g., by removing the packet filter(s) allocated for C2 communication during EPS bearer context modification procedure as specified in clause 6.4.3 or by deactivating the EPS bearer context for C2 communication during EPS bearer context deactivation procedure as specified in clause 6.4.4.

-	If the UE uses separate PDN connectivity for, respectively, USS communication and C2 communication with a UAV-C, the C2 communication with the UAV-C is authorized using UUAA-SM during the PDN connectivity procedure. If the pairing of UAV and UAV-C is revoked, the PDN connectivity or C2 communication shall be released by the network.

The authorization of direct C2 communication can be performed during the C2 communication authorization procedure.

NOTE 2:	The C2 authorization payload in the service-level-AA payload, sent to the network via the protocol configuration options, can include an indication of the request for direct C2 communication and pairing information for direct C2 communication (see subclauses 6.4.3.3 and 6.5.1.2).

The authorization of UAV flight can also be performed during the C2 communication authorization procedure. The UE supporting UAS services provides flight authorization information to the network via the protocol configuration options if the flight authorization information is already available in the UE.

[TS 24.301, clause 6.4.3.1]

The purpose of the EPS bearer context modification procedure is to modify an EPS bearer context with a specific QoS and TFT, or re-negotiate header compression configuration associated to an EPS bearer context. The EPS bearer context modification procedure is initiated by the network, but it may also be initiated as part of the UE requested bearer resource allocation procedure or the UE requested bearer resource modification procedure.

…

[TS 24.301, clause 6.4.3.2]

The MME shall initiate the EPS bearer context modification procedure by sending a MODIFY EPS BEARER CONTEXT REQUEST message to the UE, starting the timer T3486, and entering the state BEARER CONTEXT MODIFY PENDING (see example in figure 6.4.3.2.1).

The MME shall include an EPS bearer identity that identifies the EPS bearer context to be modified in the MODIFY EPS BEARER CONTEXT REQUEST message.

…

Figure 6.4.3.2.1: EPS bearer context modification procedure

[TS 24.301, clause 6.4.3.3]

Upon receipt of the MODIFY EPS BEARER CONTEXT REQUEST message, if the UE provided an APN for the establishment of the PDN connection, the UE shall stop timer T3396, if it is running for the APN provided by the UE.

…

If the UE receives the MODIFY EPS BEARER CONTEXT REQUEST message containing the Uplink data allowed parameter in the Extended protocol configuration options IE, then the UE may start transmitting uplink user data over EPS bearer context(s) of the corresponding PDN connection.

The MODIFY EPS BEARER CONTEXT REQUEST message as a part of authorization procedure for the C2 communication, can include an Extended protocol configuration options IE containing the service-level-AA container with the length of two octets. The service-level-AA container with the length of two octets:

a)	contains the service-level-AA response with the C2AR field set to the C2 authorization result informed by the UAS NF;

b)	can contain the service-level-AA payload parameter set to the C2 authorization payload and the service-level-AA payload type parameter set to "C2 authorization payload"; and

c)	can contain the service-level device ID with the value set to a new CAA-level UAV ID.

NOTE 1:	The C2 authorization payload in the service-level-AA payload can include one, some or all of the pairing information for C2 communication, the C2 session security information, and the pairing information for direct C2 communication.

If the EPS bearer context being modified is associated with a PDN connection for UAS services and the MODIFY EPS BEARER CONTEXT REQUEST message includes the service-level-AA container with the length of two octets in the Extended protocol configuration options IE, the UE supporting UAS services shall forward the contents of the service-level-AA container with the length of two octets to the upper layers.

10.10.4.3	Test description

10.10.4.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 of TS 36.508 [18].

UE:

-	The UE is capable of UAS operation and provisioned with CAA-level UAV ID

Preamble:

-	The UE has successfully performed generic test procedure for USS UAV Authorization/Authentication - Session Management in EPS according to [18] subclause 4.5A.31 by executing steps 1 to 9.

10.10.4.3.2	Test procedure sequence

Table 10.10.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Configure the UE to request transport of C2 authorization parameters (see Note 1) | - | - | - | - |
| 2 | Cause the UE to request connectivity to an additional PDN for C2 communication. (see Note 2) | - | - | - | - |
| 3 | Check: Does the UE transmit a PDN CONNECTIVITY REQUEST message with service-level-AA container including service-level device ID parameter set to the UE's CAA-level UAV ID, service-level-AA payload type parameter set to "C2 authorization payload" and service-level-AA payload parameter set to the C2 authorization payload? | --> | PDN CONNECTIVITY REQUEST | - | - |
| 4 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message with IE EPS Bearer Identity set to new EPS bearer context. Note: the SS allocates a PDN address of a PDN type which is compliant with the PDN type requested by the UE. | <-- | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: In parallel to the event described in step 5 below, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| 5 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified. | --> | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 6 | The SS transmits a MODIFY EPS BEARER CONTEXT REQUEST message including the EPS bearer identity of the default EPS bearer established at step 4, including service-level-AA container including service-level-AA response parameter with the C2AR field set to " C2 authorization was not successful or C2 authorization is revoked ". | <-- | MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 7 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | MODIFY EPS BEARER CONTEXT ACCEPT | 1 | P |
| 8 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST including the EPS bearer identity of the default EPS bearer to the additional PDN established for C2 communication. | <-- | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 9 | Check: Does the UE transmit a DEACTIVATE EPS BEARER CONTEXT ACCEPT? | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |
| 10 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The request for transport of C2 authorization parameters may be performed by MMI or AT command. Note 2: The request of connectivity to an additional PDN for UAS services may be performed by MMI or AT command. |  |  |  |  |  |

10.10.4.3.3	Specific message contents

Table 10.10.4.3.3-1: Message PDN CONNECTIVITY REQUEST (step 3, Table 10.10.4.3.2-1)

| Derivation Path: 36.508 table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol configuration options |  |  |  |
| Service-level-AA container | ‘0041’H |  |  |
| Length of Service-level-AA container contents | ‘00000000 00010100’B | 20 octets |  |
| Service-level-AA container contents |  |  |  |
| Service-level-AA parameter |  | Service-level device ID |  |
| Service-level device ID IEI | ‘10’H |  |  |
| Service-level device ID length | ‘08’H | 8 octets |  |
| Service-level device ID | ‘AABBCCDDEEFFGGHH’H | configured using AT command |  |
| Service-level-AA parameter |  | Service-level-AA payload type |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘10’H | C2 authorization payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0004’H | 4 octets |  |
| Service-level-AA payload | ‘ABCDEFGH’H | configured using AT command |  |

Table 10.10.4.3.3-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 4, Table 10.10.4.3.2-1)

| Derivation Path: 36.508 table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | 8 |  |  |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present additional PDN connectivity request procedure. |  |
| Access point name | UAS C2 APN | SS re-uses the particular APN defined by UE for this present additional PDN connectivity request procedure |  |

Table 10.10.4.3.3-3: Message MODIFY EPS BEARER CONTEXT REQUEST (steps 6, Table 10.10.4.3.2-1)

| Derivation Path: 36.508 table 4.7.3-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | 8 | SS assigns the current default EPS bearer context. |  |
| Extended protocol configuration options |  |  |  |
| Service-level-AA container | ‘0041’H |  |  |
| Length of Service-level-AA container contents | ‘00000000 00010111’B | 23 octets |  |
| Service-level-AA container contents |  |  |  |
| Service-level-AA parameter |  | Service-level-AA response |  |
| Service-level-AA response IEI | ‘30’H |  |  |
| Service-level-AA response length | ‘01’H | 1 octet |  |
| Service-level-AA response | ‘00000100’B | C2 authorization was not successful or C2 authorization is revoked |  |
| Service-level-AA parameter |  | Service-level device ID |  |
| Service-level device ID IEI | ‘10’H |  |  |
| Service-level device ID length | ‘08’H | 8 octets |  |
| Service-level device ID | ‘AABBCCDDEEFFGGHH’H | SS re-uses the Service-level device ID sent by UE for this present additional PDN connectivity request procedure |  |
| Service-level-AA parameter |  | Service-level-AA payload type |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘10’H | C2 authorization payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0004’H | 4 octets |  |
| Service-level-AA payload | ‘ABCDEFGH’H | SS re-uses the Service-level-AA payload sent by UE for this present additional PDN connectivity request procedure | Step 6 |

Table 10.10.4.3.3-4: Message MODIFY EPS BEARER CONTEXT ACCEPT (steps 7, Table 10.10.4.3.2-1)

| Derivation path: 36.508 table 4.7.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | 8 | Same value as in MODIFY EPS BEARER CONTEXT REQUEST |  |
| Extended protocol configuration options |  |  |  |
| Service-level-AA container | ‘0041’H |  |  |
| Length of Service-level-AA container contents | ‘00000000 00001010’B | 10 octets |  |
| Service-level-AA container contents |  |  |  |
| Service-level-AA parameter |  | Service-level-AA payload type |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘10’H | C2 authorization payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0004’H | 4 octets |  |
| Service-level-AA payload | ‘ABCDEFGH’H |  | Step 7 |

Table 10.10.4.3.3-5: Message DEACTIVATE EPS BEARER CONTEXT REQUEST (step 8, Table 10.10.4.3.2-1)

| Derivation path: 36.508 table 4.7.3-12 and table 4.6.1-8 with condition DRB-REL(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | 8 | Same value as in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST of step 4 |  |
| ESM cause | 00100100 | regular deactivation |  |

Table 10.10.4.3.3-6: Message DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 9, Table 10.10.4.3.2-1)

| Derivation path: 36.508 table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | 8 | Same value as in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST of step 4 |  |

<End of modified section>