3GPP TSG-RAN5 Meeting #102	R5-241065r2

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5294 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 11.2.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2024-1-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As discussion paper(R5-241063) mentioned, the UE is allowed to make a emergency call on the subsstate “EMM-DEREGISTERED.ATTACH-NEEDED” TS24.301 Rel-18.5 == 5.1.3.2.3.7 EMM-DEREGISTERED.ATTACH-NEEDED Valid subscriber data are available for the UE and for some reason an attach must be performed as soon as possible. This substate can be entered if the access class is blocked due to access class control, or if the network rejects the NAS signalling connection establishment. == 5.2.2.3.6 ATTACH-NEEDED The UE shall initiate the attach or combined attach procedure, if still needed, as soon as the access is allowed in the selected cell for one of the access classes of the UE. The UE may initiate attach for emergency bearer services. The UE may initiate attach for access to RLOS. == So, the UE can establish the RRC connection with the RRC establishmentCause set to ‘emergency’ and transmits an ATTACH REQUEST message to attach for emergency bearer services as well as “EMM-DEREGISTERED. LIMITED-SERVICE”. According to current TS36.523-1[2], there is no test case which checks whether the UE performs an emargencycall from the substate “EMM-DEREGISTERED.ATTACH-NEEDED” although there is a test case which checks whether the UE performs an emargencycall from the substate “EMM-DEREGISTERED. LIMITED-SERVICE” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of test case 11.2.13 “Emergency bearer services / Normal cell / ATTACH-NEEDED / Attach / PDN connect” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior of emergency call from substates “ATTACH-NEEDED” will not be quaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-2 CR 1444 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 Modified following points. Add “Rohde & Schwarz” as co-source R2 Add the tdoc number of discussion papar Modify the unnecessary description related to different TAC on pre-test condition Modify the step numbers on Main behaviour Add 60seconds to camp on the cell as select cell on step2 on Main behaviour. |  |  |  |  |  |  |  |  |

[Start of change]

### 11.2.13	Emergency bearer services / Normal cell / ATTACH-NEEDED / Attach / PDN connect

11.2.13.1	Test Purpose (TP)

(1)

with { UE is switched on and UE did not camp on the cell due to Access is “barred” for signalling in the cell and the UE is in EMM-DEREGISTERED.ATTACH-NEEDED state }

ensure that {

when { UE is requested to make an outgoing emergency call }

then { UE establishes the RRC connection with the RRC establishmentCause set to ‘emergency’ and transmits an ATTACH REQUEST message to attach for emergency bearer services together with a PDN CONNECTIVITY REQUEST message for emergency bearer services }

}

11.2.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 4.3, 5.2.3.1 and TS 36.331 clauses 5.3.3.2 and 5.3.3.3, and, TS 24.301 clauses 5.1.3.2.3.7, 5.2.2.3.6, 5.5.1.2.2, 5.5.2.1 and 6.5.1.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.304, clause 4.3]

This clause defines the level of service that may be provided by the network to a UE in Idle mode.

The action of camping on a cell is necessary to get access to some services. Three levels of services are defined for UE:

-	Limited service (emergency calls, ETWS and CMAS on an acceptable cell)

-	Normal service (for public use on a suitable cell)

-	Operator service (for operators only on a reserved cell)

Furthermore, the cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in a E-UTRAN network:

-	The cell is not barred, see subclause 5.3.1;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements.

The cell is part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

-	For a CSG cell, the cell is a CSG member cell for the UE;

According to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming", which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

barred cell:

A cell is barred if it is so indicated in the system information [3].

[TS 36.304, clause 5.2.3.1]

The UE shall use one of the following two cell selection procedures:

a)	Initial Cell Selection

This procedure requires no prior knowledge of which RF channels are E-UTRA carriers. The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find a suitable cell. On each carrier frequency, the UE need only search for the strongest cell. Once a suitable cell is found this cell shall be selected.

b)	Stored Information Cell Selection

This procedure requires stored information of carrier frequencies and optionally also information on cell parameters, from previously received measurement control information elements or from previously detected cells. Once the UE has found a suitable cell the UE shall select it. If no suitable cell is found the Initial Cell Selection procedure shall be started.

NOTE: Priorities between different frequencies or RATs provided to the UE by system information or dedicated signalling are not used in the cell selection process.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that EAB is applicable, upon which the procedure ends;

1>	if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile terminating calls is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to TRUE:

4>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as barred;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

3>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls is applicable, upon which the procedure ends;

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

1>	else (the UE is establishing the RRC connection for mobile originating CS fallback):

2>	if SystemInformationBlockType2 includes ac-BarringForCSFB:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForCSFB as "AC barring parameter";

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating CS fallback is applicable, due to ac-BarringForCSFB, upon which the procedure ends;

2>	else:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

3>	if access to the cell is barred:

4>	if timer T303 is not running, start T303 with the timer value of T306;

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating CS fallback and mobile originating calls is applicable, due to ac-BarringForMO-Data, upon which the procedure ends;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

1>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else:

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

[TS 36.331, clause 6.2.2]

SystemInformationBlockType1 field descriptions

…

ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode.

[TS 24.301, clause 5.1.3.2.3.7]

Valid subscriber data are available for the UE and for some reason an attach must be performed as soon as possible. This substate can be entered if the access class is blocked due to access class control, or if the network rejects the NAS signalling connection establishment.

[TS 24.301, clause 5.2.2.3.6]

The UE shall initiate the attach or combined attach procedure, if still needed, as soon as the access is allowed in the selected cell for one of the access classes of the UE.

The UE may initiate attach for emergency bearer services.

[TS 24.301, clause 5.5.1.2.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

If the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17] and the selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs, the UE shall include the IMSI in the EPS mobile identity IE in the ATTACH REQUEST message.

For all other cases, the UE shall handle the EPS mobile identity IE in the ATTACH REQUEST message as follows:

If the UE supports neither A/Gb mode nor Iu mode:

-	the UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. In addition, the UE shall include Old GUTI type IE with GUTI type set to "native GUTI". If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

If the UE supports A/Gb mode or Iu mode or both:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and the RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI".

-	If the TIN is deleted and

-	the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI";

-	the UE does not hold a valid GUTI but holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE; or

-	the UE does not hold a valid GUTI, P-TMSI or RAI, the UE shall include the IMSI in the EPS mobile identity IE.

-	Otherwise the UE shall include the IMSI in the EPS mobile identity IE.

If the UE is attaching for emergency bearer services and does not hold a valid GUTI, P-TMSI or IMSI as described above, the IMEI shall be included in the EPS mobile identity IE.

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container information element to request PDN connectivity.

If UE supports A/Gb mode or Iu mode or if the UE wants to indicate its UE specific DRX parameter to the network, the UE shall include the UE specific DRX parameter in the DRX parameter IE in the ATTACH REQUEST message.

If a valid NAS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message. When the UE does not have a valid NAS security context, the ATTACH REQUEST message combined with the PDN CONNECTIVITY REQUEST message is not integrity protected.

If the UE supports SRVCC to GERAN/UTRAN, the UE shall set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported".

If the UE supports vSRVCC from S1 mode to Iu mode, then the UE shall set the H.245 after handover capability bit to "H.245 after SRVCC handover capability supported" and additionally set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported" in the ATTACH REQUEST message.

Figure 5.5.1.2.2.1: Attach procedure and combined attach procedure

[TS 24.301, clause 5.5.2.1]

When upper layers indicates that emergency bearer services are no longer required, the UE if still attached for emergency bearer services, may perform a detach followed by a re-attach to regain normal services, if the UE is in or moves to a suitable cell.

[TS 24.301, clause 6.5.1.2]

When the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message, the UE shall not include the APN.

11.2.13.3	Test description

11.2.13.3.1	Pre-test conditions

System Simulator:

- Cell A as specified in  TS 36.508 clause 4.4.1.2 is configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

- Cell A is configured according to table 6.3.2.2-1 in TS 36.508 [18].

UE:

-	The UE is in Automatic PLMN selection mode.

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

11.2.13.3.2	Test procedure sequence

Table 11.2.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts SystemInformationBlockType2 parameters. | - | - | - | - |
| 2 | Power on the UE and waits for 60 seconds to camp on the cell as select cell. | - | - | - | - |
| 3 | Make the UE attempt an IMS emergency call. | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? | --> | RRCConnectionRequest | 1 | P |
| 5 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message for emergency bearer services? | --> | ATTACH REQUEST PDN CONNECTIVITY REQUEST | 1 | P |
| 7-21 | Steps 5 to 19 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1). | - | - | - | - |
| 22 | Release IMS Call (see Note 1). | - | - | - | - |
| - | EXCEPTION: Step 23a1 to 23b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 23a1 | The SS starts timerT1 = 5 seconds. | - | - | - | - |
| 23a2 | The UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 23a3 | The SS responds a DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 23b1 | Timer T1 expires. | - | - | - | - |
| 24 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The IMS Call is released using the generic procedure in TS 34.229-1 [35] subclause C.32. |  |  |  |  |  |

11.2.13.3.3	Specific message contents

Table 11.2.13.3.3-1: SystemInformationBlockType2 for Cell A (step1, Table 11.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| accessBarringInformation SEQUENCE { |  |  |  |
| accessBarringForEmergencyCalls | FALSE |  |  |
| accessBarringForSignalling SEQUENCE { |  |  |  |
| accessProbabilityFactor | p00 |  |  |
| accessBarringTime | s4 |  |  |
| accessClassBarringList SEQUENCE (SIZE (maxAC)) OF SEQUENCE { | 5 entries |  |  |
| accessClassBarring[1] | TRUE |  |  |
| accessClassBarring[2] | TRUE |  |  |
| accessClassBarring[3] | TRUE |  |  |
| accessClassBarring[4] | TRUE |  |  |
| accessClassBarring[5] | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| accessBarringForOriginatingCalls | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2.13.3.3-2: RRCConnectionRequest (step 4, Table 11.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | emergency |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2.13.3.3-3: ATTACH REQUEST (Step 6, Table 11.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach type | '0110'B | EPS emergency attach |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to an emergency PDN |  |  |

Table 11.2.13.3.3-4: PDN CONNECTIVITY REQUEST (Step 6, Table 11.2.13.3.2-1)

| Derivation Path: 36.508, Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | '0100'B | emergency |  |

[End of change]