3GPP TSG-RAN5 Meeting #102	R5-241071r1

# Athens, Greece, 26th Feb– 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4292 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 6.1.2.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-1-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There were some editorial mistakes in the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of test case 6.1.2.14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will make a misreading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 Modified following points. Current version of TS38.323-1 on cover sheets to 17.5.1 |  |  |  |  |  |  |  |  |

[Start of change]

#### 6.1.2.14	Cell reselection using cell status and cell reservations / Access Identity 11 or 15 - cellReservedForOperatorUse

6.1.2.14.1	Test Purpose (TP)

(1)

with { UE camped normally in state NR RRC_IDLE operating in their HPLMN/EHPLMN and fitted with a USIM with access Identities set: 11 or 15}

ensure that {

when { a higher ranked cell is found "reserved" for Operator use }

then { UE re-selects to the higher ranked cell }

}

6.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.304, clause 5.3.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.304, clause 5.3.1]

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of three fields:

-	cellBarred (IE type: "barred" or "not barred")
Indicated in MIB message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "true" for other use,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN and not "true" for other use,

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to an Access Identity 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

6.1.2.14.3	Test description

6.1.2.14.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 3 in different tracking areas.

-	NR Cell 1 and NR Cell 3 are HPLMN.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cells.

UE:

-	The UE is equipped with a USIM containing default values and USIM Configuration 17.

Preamble:

-	The UE is in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.14.3.2	Test procedure sequence

Table 6.1.2.14.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T0" is applied for Preamble. Configurations marked "T1 is applied at the points indicated in the Main behaviour description in Table 6.1.2.14.3.2-3.

Table 6.1.2.14.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -80 | The power level values are assigned to satisfy RNRCell 1 < RNRCell 3 |
|  | cellReservedForOperatorUse | - | - | Reserved |  |

Table 6.1.2.14.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | The power level values are assigned to satisfy RNRCell 1 < RNRCell 3 |
|  | cellReservedForOperatorUse | - | - | Reserved |  |

Table 6.1.2.14.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.14.3.2-1/2. | - | - | - | - |
| 2 | Check: Does the test result of test steps 1 to 5 of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 3? | - | - | 1 | P |

6.1.2.14.3.3	Specific message contents

Table 6.1.2.14.3.3-1: SIB1 for NR Cell 3 (Preamble, Table 6.1.2.14.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| cellReservedForOperatorUse | reserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.14.3.3-2: SERVICE REQUEST (Preamble, Table 6.1.2.14.3.2-3; step 4 ,Table 4.5.2.2-4, TS 38.508-1 [4])

| Derivation Path: Table 4.7.1-16. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service type | ‘0101’B | high priority access |  |

[End of change]