3GPP TSG-RAN5 Meeting #102	R5-241074r4

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4294 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 8.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC., TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-1-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to current test case, there were some editorial points on Table 8.1.1.2.4.3.3-2 as follows. -SIB2 -sib21-v1700 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of test case 8.1.1.2.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will make a misreading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 Modified following points. Current version of TS38.323-1 on cover sheets to 17.5.1 R2 Keep it original sentence based on TF160 to be more readable R3 Clarify the parameter in the SystemInformation Table 8.1.1.2.4.3.3-2 because the entry of SIBs is not clear Add TF160 as co-source R4 modify editorial point on Table 8.1.1.2.4.3.3-2 |  |  |  |  |  |  |  |  |

[Start of change]

##### 8.1.1.2.4	RRC connection establishment / Extended value, spare fields and non critical extensions in SI

8.1.1.2.4.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { UE receives extended value that is not defined in the version of the transfer syntax supported by the UE and non critical extensions }

then { UE consider the value as not comprehended and treat the message as if the field were absent and UE establishes an RRC connection }

}

8.1.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3.1, 10.1, 10.3 and A3.3. Unless otherwise stated, these are Rel-15 requirements.

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

NOTE:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

[TS 38.331, clause 10.1]

The generic error handling defined in the subsequent clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE;

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved field.

[TS 38.331 clause 10.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with clause 10.4.

[TS 38.331 clause A3.3]

Each PDU (message) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfiguration ::=    SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

c1                                  CHOICE{

rrcConnectionReconfiguration-r8     RRCConnectionReconfiguration-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

-- Enter the IEs here.

...

}

-- ASN1STOP

Hooks for critical and non-critical extension should normally be included in the PDU type specification. How these hooks are used is further described in clause A.4.

Critical extensions are characterised by a redefinition of the PDU contents and need to be governed by a mechanism for protocol version agreement between the encoder and the decoder of the PDU, such that the encoder is prevented from sending a critically extended version of the PDU type, which is not comprehended by the decoder.

Critical extension of a PDU type is facilitated by a two-level CHOICE structure, where the alternative PDU contents are alternatives within the inner level c1 CHOICE. Spare alternatives (i.e., spare3 down to spare1 in this case) may be included within the c1 CHOICE. The number of spare alternatives to be included in the original PDU specification should be decided case by case, based on the expected rate of critical extension in the future releases of the protocol.

Further critical extension, when the spare alternatives from the original specifications are used up, is facilitated using the criticalExtensionsFuture in the outer level CHOICE.

In PDU types where critical extension is not expected in the future releases of the protocol, the inner level c1 CHOICE and the spare alternatives may be excluded, as shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

-- Enter the fields here.

...

}

-- ASN1STOP

Non-critical extensions are characterised by the addition of new information to the original specification of the PDU type. If not comprehended, a non-critical extension may be skipped by the decoder, whilst the decoder is still able to complete the decoding of the comprehended parts of the PDU contents.

Non-critical extensions at locations other than the end of the message or other than at the end of a field contained in a BIT or OCTET STRING are facilitated by use of the ASN.1 extension marker "...". The original specification of a PDU type should normally include the extension marker at the end of the sequence of information elements contained.

Non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING may be facilitated by use of an empty sequence that is marked OPTIONAL e.g. as shown in the following example:

-- /example/ ASN1START

RRCMessage-r8-IEs ::=                   SEQUENCE {

field1                                  InformationElement1,

field2                                  InformationElement2,

nonCriticalExtension                    SEQUENCE {}                     OPTIONAL

}

-- ASN1STOP

The ASN.1 section specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.

%PDU-TypeIdentifier% field descriptions

%field identifier% Field description.

%field identifier% Field description.

The field description table has one column. The header row shall contain the ASN.1 type identifier of the PDU type.

The following rows are used to provide field descriptions. Each row shall include a first paragraph with a field identifier (in bold and italic font style) referring to the part of the PDU to which it applies. The following paragraphs at the same row may include (in regular font style), e.g., semantic description, references to other specifications and/or specification of value units, which are relevant for the particular part of the PDU.

The parts of the PDU contents that do not require a field description shall be omitted from the field description table.

8.1.1.2.4.3	Test description

8.1.1.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

None.

Preamble:

-	The UE is switched off

8.1.1.2.4.3.2	Test procedure sequence

Table 8.1.1.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. (Note 1) | <-- | NR RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCSetupRequest message on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 4 | The SS transmits an RRCSetup message on NR Cell 1. | <-- | NR RRC: RRCSetup | - | - |
| 5 | Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and initiates the registration procedure by including the REGISTRATION REQUEST message? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 6-21a1 | Steps 5 to 20a1 of the registration procedure described in TS 38.508 Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| Note 1: The NR-2 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] is used with SIB1 according to Table 8.1.1.2.4.3.3-2A and SystemInformation according to Table 8.1.1.2.4.3.3-2. |  |  |  |  |  |

8.1.1.2.4.3.3	Specific message contents

Table 8.1.1.2.4.3.3-1: MIB (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MIB ::= SEQUENCE { |  |  |  |
| spare | ‘1’B | In Rel-15 and Rel-16, the lowest bit is considered a spare according to TS 38.331 [12]. | Rel-15 to Rel-16 |
| } |  |  |  |

Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1, Table 4.6.1-29 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { | 2 entries |  |  |
| SIB2 | SIB2 | entry 1 |  |
| sib21-v1700 | SIB21 (Table 8.1.1.2.4.3.3-4) | entry 2 In Rel-15 and Rel-16, SIB21 is considered as a non-critical extension according to TS 38.331 [12]. |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2.4.3.3-2A: SIB1 (step 1, Table 8.1.1.2.4.3.2-1)

| Derivation Path: TS 38.508-1, Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| si-SchedulingInfo::= SEQUENCE { | SI-SchedulingInfo |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |
| schedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| si-BroadcastStatus | broadcasting |  |  |
| si-Periodicity | rf32 |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 2 entries |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType2 |  |  |
| valueTag | 0 |  |  |
| areaScope | Not present |  |  |
| } |  |  |  |
| SIB-TypeInfo[2] SEQUENCE { |  | entry 2 |  |
| type | spare3 | To simulate an extended SI that is undefined for a Rel-15 and Rel-16 UE. |  |
| valueTag | 0 |  |  |
| areaScope | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  | SIB1-v1610-IEs |  |
| nonCriticalExtension ::= SEQUENCE { |  | SIB1-v1630-IEs |  |
| nonCriticalExtension ::= SEQUENCE { |  | SIB1-v1700-IEs |  |
| hsdn-Cell-r17 | true | To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2.4.3.3-3: Void

Table 8.1.1.2.4.3.3-4: SIB21 (Table 8.1.1.2.4.3.3-2)

| Derivation Path: TS 38.508-1, Table 4.6.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |
| mbs-FSAI-IntraFreq-r17 | Not Present |  |  |
| mbs-FSAI-InterFreqList-r17 | Not Present |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| } |  |  |  |

[End of change]