3GPP TSG-RAN5 Meeting #102	R5-241138r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0807 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Es for 1024QAM scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_DL1024QAM_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 introduced requirements for Demodulation with MCS Table 4 (1024QAM) assuming maximum TxEVM=2.5%. Es value for EVM 2.5% needs to be computed accordingly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated FR1 Es value considering EVM 2.5% for DL1024QAM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Es value for DL1024QAM will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency R4-2400635 R4-2403072 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated as per latest RAN4 agreed CR |  |  |  |  |  |  |  |  |

### <<Start of change>>

### 4.4.4	Es

#### 4.4.4.1	Introduction

This clause describes the Es power level for Mode 2 conditions conducted testing of demodulation and CSI requirements.

Unless otherwise stated for CA and EN-DC testing, the same Es level shall be provided on different component carriers.

#### 4.4.4.2	Es for NR operating bands in FR1

The Es power spectrum density shall be larger or equal to the minimum Es power level for each operating band supported by the UE as defined in Clause 4.4.4.2.1.

Unless otherwise stated, a fixed Es power level of -112 dBm/Hz shall be used for all operating bands.

##### 4.4.4.2.1	Derivation of Es values for NR operating bands in FR1

The minimum Es power level for an operating band, subcarrier spacing and channel bandwidth is derived based on the following equation:

EsBand_X, SCS_Y, CBW_Z = REFSENSBand_X, SCS_Y, CBW_Z – 10*log10(12*SCS_Y*nPRB) + D – SNRREFSENS + dBEVM +∆thermal

where:

-	REFSENSBand_X, SCS_Y, CBW_Z is the REFSENS value in dBm for Band X, SCS Y and CBW Z specified in Table 7.3.2-1 of TS 38.101-1 [2]

-	12 is the number of subcarriers in a PRB

-	SCS Y is the subcarrier spacing associated with the REFSENS value

-	nPRB is the maximum number of PRB for SCS Y and CBW Z associated with the REFSENS value, and is specified in Table 5.3.2-1 of TS 38.101-1 [2]

-	D is diversity gain equal to 3 dB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	dBEVM is the SNR of the applied signal due to EVM impairment on the wanted Es. An allowed EVM of 3% gives a dBEVM of 30.5dB, derived as 20*log10(1/0.03). An allowed EVM of 2.5% gives a dBEVM of 32dB, derived as 20*log10(1/0.025).

-	∆thermal is the amount of dB that the impairment due to EVM on the wanted Es is set above UE thermal noise, giving a defined rise in total impairment. ∆thermal = 7.6dB, giving a rise in total impairment of 0.7dB, regarded as acceptable.

For an allowed EVM of 3%, the calculated Es value for the baseline of Band n12, 15kHz SCS, 15MHz CBW is -113.5 dBm/Hz.

For an allowed EVM of 2.5%, the calculated Es value for the baseline of Band n12, 15kHz SCS, 15MHz CBW is -112 dBm/Hz.

An allowance of 1.5dB is made for CA and for future bands, giving an Es power level of -112 dBm/Hz for EVM of 3%, and -110.5 dBm/Hz for EVM of 2.5%.

### <<several sections skipped>>

5.5.1.3	Minimum conformance requirements

The requirements in this clause are applicable to the FR1 single carrier case.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the channel bandwidth with the downlink physical channel setup according to Annex C.3.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5.1.3-1. The parameters specified in Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in Table 5.5.1.3-3 are applicable for tests on TDD bands.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5.1.3-1: Common test parameters for FDD and TDD bands

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Select channel bandwidth as per test parameter selection clause 5.5.1.3.1 |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5.1.3-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 30 kHz / 10 MHz and 30 kHz / 20 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of Tx=1: No precoding;For number of Tx=2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder indices 0 and 2 For number of Tx=4: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | WB |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | For number of Tx=1: ‘No CDM’ For number of Tx>1: 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of Tx=1: No precoding; For number of Tx>1: Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot with equal probability of each applicable i1, i2 combination |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Es |  |  | dBm/kHz | -112 for MCS indexes in Table 5.5.1.3-5: -110.5 For 1024QAM MCS indexes in Table 5.5.2.3-1: |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

### <<End of change>>