3GPP TSG-RAN5 Meeting #102	R5-241164r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4299 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to title of 8.1.5.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test title has RACS as well as UL Segmentation. There are two different PICS defined for RACS(pc_5GC_RACS) and UL segmentation(pc_NR_UL_Segmentation) but RACS is not used in applicability ie 8.1.5.9.1 applicability only uses UL segmentation(pc_NR_UL_Segmentation).Therefore, having RACS in title is misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the RACS from the title |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Application of the test case will be unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Update title to align with 8.1.5.9.2 and 8.1.5.9.3 ie RRC Message followed by UL segment transfer |  |  |  |  |  |  |  |  |

<Start of  Change>

##### 8.1.5.9.1	UECapabilityInformation / UL segment transfer

8.1.5.9.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives UECapabilityEnquiry message with rrc-segAllowed-r16 enabled }

then { UE successfully segments UECapabilityInformation message at RRC layer and sends UlDedicatedMessageSegment messages with rrc-MessageSegmentType set accordingly for the first and last segment and rrc-MessageSegmentContainer containing the segments of UECapabilityInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state and UE having received UECapabilityEnquiry message with rrc-segAllowed-r16 enabled and having transmitted the firstSegment of UECapabilityInformation message within the ULDedicatedMessageSegment message }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE successfully completes RRC re-establishment procedure and discards the remaining segments of UECapabilityInformation message }

}

8.1.5.9.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clause 5.6.1.3, 5.7.7.2, 5.7.7.3 and 5.3.7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.7.7.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

[TS 38.331, clause 5.7.7.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	F or each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

[TS 38.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

…

1>	if UE is not configured with conditionalReconfiguration:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, except SRB0;

8.1.5.9.1.3	Test description

8.1.5.9.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.9.1.3.2	Test procedure sequence

Table 8.1.5.9.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |

Table 8.1.5.9.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2. |  |  |  |

Table 8.1.5.9.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| - | EXCEPTION: Steps 2a1 to 2b7 describe behaviour that depends on the size of UEs capability container; the "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 to 2b7 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the ULDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | NR RRC: ULDedicatedMessageSegment | 1 | P |
| 2b1 | Check: Does the UE send UECapabilityInformation Message? | --> | NR RRC: UECapabilityInformation | - | - |
| 3a1-4a13 | Void | - | - | - | - |
| 2b2 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b3 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 2b4 | The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| - | EXCEPTION: Step 2b5 is repeated a maximum of 2 times. | - | - | - | - |
| 2b5 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | NR RRC: ULDedicatedMessageSegment | 1 | P |
| 2b6 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b7 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 3 | SS stops regular grant transmission upon scheduling request. | - | - | - | - |
| 4 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 5 | The SS allocates 15 UL grants from (80∙2μ+1+TDL) slots after step 4 at an interval of 10ms to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. (Note 1, Note 2, Note 3) | - | - | - | - |
| 6 | The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | NR RRC: ULDedicatedMessageSegment | - | - |
| 7 | The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.9.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded. | - | - | - | - |
| 8 | Wait for T310 to expire | - | - | - | - |
| 9 | The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. | - | - | - | - |
| 10 | The UE sends an RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCReestablishment message to resume SRB1 operation on NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 12 | The UE transmits an RRCReestablishmentComplete message on NR Cell 1? | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 13 | SS resumes regular UL grant transmission. | - | - | - | - |
| 14 | Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and does not send any ULDedicatedMessageSegment message in the next 5 seconds? | - | - | 2 | P |
| Note 1: TDL is the delay in slots to the next available DL slot suitable for the transmission of a DCI. Note 2: μ is the SCS index. μ = 0/1/2/3 for SCS = 15kHz/30kHz/60kHz/120kHz respectively. Note 3: 15 UL grants of default size (624 bytes, using LRBs & IMCS as per 38.523-3[3] Table 7.1.2.3.3-1) are chosen to enable UE to transmit only one ulDedicatedMessageSegment as maximum PDCP SDU size 9000 bytes. PDCP is segmented into several RLC Segments. Each MAC TB contains 3 bytes of MAC header + 2 bytes of Short BSR + 3 bytes of RLC Header + Payload of Segmented RLC SDU. Additional 8 bytes are required in each RLC segment. 15 grants provide (15*624=9360) which is sufficient to accommodate 9000 bytes and RLC/MAC header and MAC CE. |  |  |  |  |  |

8.1.5.9.1.3.3	Specific message contents

Table 8.1.5.9.1.3.3-0: CellGroupConfig (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlf-TimersAndConstants SEQUENCE { |  |  |  |
| t311-v1530 | ms10000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-1: UECapabilityEnquiry (step 1, 2b4 and step 4, Table 8.1.5.9.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request { | 2 entries |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | eutra-nr |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ue-CapabilityEnquiryExt | OCTET STRING Containing UECapabilityEnquiry-v1560-IEs |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-2: UE-CapabilityRequestFilterNR (Table 8.1.5.9.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-47 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRequestFilterNR ::= SEQUENCE { |  |  |  |
| frequencyBandListFilter | FreqBandList |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| srs-SwitchingTimeRequest | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| srs-SwitchingTimeRequest | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-3: FreqBandList (Table 8.1.5.9.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.4-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandList::= SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | 43 entries | FreqBandIndicatorNR = n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n70, n71, n74, n75, n76, n77, n78, n79, n80, n81, n82, n83, n84, n86, n257, n258, n260, n261. |  |
| FreqBandInformation[n] CHOICE { |  | entry n |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not present |  |  |
| maxBandwidthRequestedUL | Not present |  |  |
| maxCarriersRequestedDL | Not present |  |  |
| maxCarriersRequestedUL | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-4: UECapabilityEnquiry-v1560-IEs (Table 8.1.5.9.1.3.3-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UECapabilityEnquiry-v1560-IEs ::= SEQUENCE { |  |  |  |
| capabilityRequestFilterCommon | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-5: ULDedicatedMessageSegment (step 2a1, 2b5 and step 6, Table 8.1.5.9.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING Includes a segment of the encoded UL DCCH message. The size of the included segment in this container should be small enough so the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-6: UECapabilityInformation (step 2b1, Table 8.1.5.1.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF UE-CapabilityRAT-Container { | 2 entries |  |  |
| UE-CapabilityRAT- Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| UE-CapabilityRAT- Container[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | eutra-nr |  |  |
| ueCapabilityRAT-Container | UE-MRDC-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Change>