3GPP TSG-RAN5 Meeting #102	R5-241206r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.561 | CR | 0002 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on EIS search interpolation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_FR1_TRP_TRS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As captured in the editor’s note in clause A.4.2.6, further definition of EIS search with interpolation is expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The TRS test procedure was ammended to include the alternate case of EIS search with interpolation. Description of the two options for EIS interpolation (linear and polynomial) were included, as well as the Measurement Uncertainty considerations for each option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EIS search procedure required for TRS measurements will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.1.1.4.2, A.3.3.3, A.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Clauses affected were missing in original coverpage. |  |  |  |  |  |  |  |  |

< Unchanged Text Deleted >

< Beginning of Changes >

7.2.1.1.1.4.2	Test procedure

For TRS measurement, the evaluations shall be performed at maximum transmit power. The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the UE positioning guidelines defined in Annex B.3.1.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.3-2. For any additional RMC parameters beyond Table 5.3-2 refer to Table 7.3.2.4.1-1 of TS 38.521-1 [11]. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL parameters defined in Table 5.3-2 and any additional parameters from Table 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

4.	Ensure the DUT transmits with its maximum power by continuously sending uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

5.	The TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. Determine each EIS by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

6.	The measurement is performed with a constant sampling step of in both theta () and phi () axes using any of the measurement grids and quadratures options outlined and applicable to TRS in Table A.4.2.12-1. Any of the measurement grids in Table A.4.2.12-1 could be used for testing. Measurements at theta = 0 and 180 degrees only require one measurement each. For test systems that cannot measure 180º EIS, an extrapolation approach shall be adopted when generating the 3D antenna pattern and calculating TRS.

7.	All of the measured power values at each position or measurement angle will be integrated to TRS, as defined in Annex A.3.5.2.

8.	The reported TRS value shall be corrected by -0.5·output level step resolution of the final power step search and note the correction in the test report. If the alternate EIS search procedure is applied as described in A.3.3.3.1 to assess the EIS at each grid point, no correction of TRS shall be applied.

< Unchanged Text Deleted >

### A.3.3.3	TRS Test procedure

The TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step in both theta () and phi () axes using any of the measurement grids and quadratures options outlined and applicable to TRS in Table A.4.2.12-1.

EIS, or receiver sensitivity measurements, is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC) (the maximum throughput is per Appendix A of TS 38.521-1 [11]).

For TRS measurement, the evaluations shall be performed at maximum transmit power.

The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the positioning guideline defined in Clause 6.

2.	Connect the SS with the DUT through the measurement antenna.

3.	Follow steps 1 through 4 in section 7.3.2.4.2 of TS 38.521-1 [11], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

The TRS value is calculated using the equation outlined in Clause A.3.5.2. The reported TRS value shall be corrected by -0.5·output level step resolution of the final power step search and note the correction in the test report. If an alternate EIS search procedure, detailed in A.3.3.3.1, is applied to assess the EIS at each grid point, no correction of TRS shall be applied.

This TRS test procedure is common to both SA and EN-DC measurements. The detailed UE configurations for TRS test in SA and EN-DC mode are specified in Clause A.2.

#### A.3.3.3.1	Alternate EIS Search Procedure with Linearization

Either of the following alternate EIS search procedure methods can be used given that the corresponding uncertainty term as described in A.4.2.6 is considered.

##### A.3.3.3.1.1	EIS linear interpolation

The simplest approach to approximate the sensitivity threshold is to calculate the linear interpolation between the two power level steps providing the closest result to the 95% throughput and separated by the power step size:

y=y1-y0x1-x0·x-x0+y0

Where:

x = DL power in dBm,

x1 = last DL power providing a throughput ≥95%,

x0 = first DL power providing a throughput <95%;

y = Throughput in %,

y1 = last throughput ≥95%,

y0 = first throughput <95%.

##### A.3.3.3.1.2	EIS polynomial interpolation

A more accurate approach to approximate the sensitivity threshold is to calculate the polynomial interpolation based on the DUT sensitivity response, which is characterized with a fine step size at a reference position following this procedure:

1.	At the first grid point, or at a reference initial position, an EIS search is performed with a very fine power step size (e.g. 0.1dB) capturing the DL power vs. Throughput for a sufficient DL power range (e.g. 1dB) around the sensitivity threshold.

2.	Based on the results in step 1, curve fitting techniques are used to obtain the best fit n degree polynomial characterizing the DUT sensitivity response in the form of:

y=cn·xn+cn-1·xn-1+…+c2·x2+c1·x+c0

In case the DL power is treated as variable (Option 2A), then:

y = Throughput in %

x = DL power in dBm

ci = polynomial coefficients

n = polynomial degree

In case the Throughput is treated as variable (Option 2B), then:

y = DL power in dBm

x = Throughput in %

ci = polynomial coefficients

n = polynomial degree

3.	The EIS per grid point measurement is performed as described in clause 7.2.1.1.1.4.2 using the maximum power step size allowed (i.e. 0.5dB), but the final EIS value per grid point is interpolated using the polynomial coefficients calculated in step 2.

In case of Option 2A, i.e. using the DL power as variable for the polynomial, the EIS estimation at a grid point θi,ϕi is based on the first DL Power providing a TP < 100%, and calculated with respect to the polynomial obtained at the reference position using the following equation:

EISθi,ϕi=DLPoweri-DLPowerrefy=TPi+DLPowerrefy=95%

Where:

DLPoweri = DL power providing a TP lower than 100% at grid point θi,ϕi

TPi = TP (lower than 100%) corresponding to DLPoweri at grid point θi,ϕi

DLPowerrefy=TPi = polynomial solved to obtain the DLPowerref=x corresponding to TPi

DLPowerrefy=95% = polynomial solved to obtain the DLPowerref=x corresponding to 95% Throughput threshold.

In case of Option 2B, i.e. using the Throughput as variable for the polynomial, the EIS estimation at a grid point θi,ϕi point is based on the first DL Power providing a TP lower than 100%, and calculated with respect to the polynomial obtained at the reference position using the following equation:

EISθi,ϕi=DLPoweri-yTPi+y95%

Where:

DLPoweri = DL power providing a TP lower than 100% at grid point θi,ϕi

TPi = TP (lower than 100%) corresponding to DLPoweri at grid point θi,ϕi

yTPi = polynomial evaluated for TPi

y95% = polynomial evaluated for the fixed sensitivity threshold at 95% Throughput.

4.	All of the resulting EIS values per grid point, calculated as described in step 3, are integrated to TRS as defined in Annex A.4.5.2.

< Unchanged Text Deleted >

### A.4.2.6	Sensitivity measurement: output level step resolution

When output power of the communication tester is swept to reach the throughput target that defines the sensitivity threshold, the final power step resolution represents an asymmetric uncertainty contribution that can be corrected since this uncertainty is device and test system independent. The lab shall correct the TRS results by -0.5·output level step resolution of the final power step search and note the correction in the test report.

Alternatively, if the alternate EIS search procedure is applied as described in A.3.3.3.1 to assess the EIS at each grid point, no correction of TRS shall be applied and the following uncertainty contribution must be considered depending on the linearization method used.

#### A.4.2.6.1	EIS linear interpolation

The residual error for the EIS linear interpolation as described in A.3.3.3.1.1 is single sided, it has normal distribution and can be bounded by 0.5·output level step resolution. In order to simplify the treatment of this error term and the combined uncertainty, it is recommended to consider this contributor as double sided with normal distribution.

#### A.4.2.6.2	EIS polynomial interpolation

The residual error for the EIS polynomial interpolation as described in A.3.3.3.1.2 is double sided, it has normal distribution and can be bounded by 0.5·fine threshold search step resolution.

< End of Changes >