3GPP TSG-RAN5 Meeting #102	R5-241349r2

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2728 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 4DL CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test configuration for Inter-band CA with four bands is missing in Refsens test case 7.3A.3. Table reference in Note 5 in Table 7.4A.3.4.1-1 is incorrect. Test repetition and measurement configuration of Maximum input level for CA in Table 7.4A.3.4.2-1, of ACS for CA in Table 7.5A.3.4.2-1, of in-band blocking Table 7.6A.2.3.4.2-1, and of Out-of-band blocking in Table 7.6A.3.3.4.1-1 do not consider inter-band CA with four bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test configuration and test requirement have been added for Inter-band CA with four bands to enable testing of - CA_n2A-n5A-n48A-n66A - CA_n2A-n5A-n48A-n77A - CA_n2A-n5A-n66A-n77A - CA_n2A-n48A-n66A-n77A - CA_n5A-n48A-n66A-n77A Table reference has been corrected. Test repetition and measurement configuration have been updated for Maximum input power and ACS test cases. Editor's note updated in 7.3A.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 4DL CA configurations cannot be tested with Rx test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3, 7.3A.3.4.1, 7.3A.3.5, 7.4A.3.4.1, 7.4A.3.4.2, 7.5A.3.4.2, 7.6A.2.3.4.2, 7.6A.3.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0877 TS 38.522 CR 0399 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated update of test point selection in CR R5-241351 and of applicability in CR R5-241350. r1: removed updates to Table 7.6A.2.3.4.1-1 in order to resolve overlap with CR R5-241196. updated editor's note in clause 7.3A.3 and merged default frequency in Table 7.3A.3.4.1-1 from CR R5-240457. corrected some band notations. Reverted replacement of whole Table 7.3A.3.5-1 in order to resolve conflict with R5-240463. r2: Added Verizon as co-sourcing company Remove rows with exceptions in Refsens TC, add same exceptions to header of Table as for 3CC. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.3	Reference sensitivity power level for 4DL CA

NOTE: Intraband contiguous  4DL CA is FFS

7.3A.3.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 4DL CA.

7.3A.3.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.3.4	Test description

7.3A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.3.4.1-1: Test Configuration Table for 4DL CA

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For Inter-band CA: CA_nXA-nYA-nZA-nRA: Mid range for PCC and SCC with exceptions CA_nX(2A)-nYA-nZA: For nY and nZ bands, Mid range for PCC and SCC with exceptions. CA_nXC-nYA-nZA and CA_nXB-nYA-nZA : Low range, High Range for nXC and nXB, mid range for nYA for PCC and SCC with exceptions. Exceptions for CA configurations containing the following band combinations: CA_n1-n77: Mid in band n1 and Low in band n77 CA_n2-n77: UL and DL Mid in band n2 and band n77 at 3850MHz CA_n3-n77: TBD in band n3 and TBD in band n77. CA_n3-n78: Mid in band n3 and High in band n78. CA_n8-nX: Low range for PCC in Band n8 CA_n70-n71: High range for PCC in band n71 These exceptions apply only for nY and nZ bands in the case of CA_nX(2A)-nYA-nZA. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0) |  |  |
|  | CA configuration |  |  |  |  |  |  |  |  |  |  | PCC |  | SCC1 | SCC2 | SCC3 | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  | Wgap3 | SCC3 |  |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nX | default | N/A | nY | default | N/A | nZ | default | Highest NRB_agg |  | Highest NRB_agg | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | default | N/A | nX | default | N/A | nZ | default | Highest |  | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nX | default | N/A | nX | default | N/A | nY | default | Highest |  | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0) |  |  |
|  | CA configuration |  |  |  |  |  |  |  |  |  |  | PCC | SCC1 | SCC2 | SCC3 | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  | Wgap3 | SCC3 |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nY | N/A | N/A | nZ | default | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nZ | default | Highest | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | Ny | default | Highest | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nY | CC1 | Max (NOTE 7) | nY | CC2 | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | CC1 | Max (NOTE 7) | nY | CC2 | N/A | nX | CC1 | Max (NOTE 7) | nX | CC2 | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for CA_nXA-nYA-nZA-nRA Configurations (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nY | default | N/A | nZ | default | N/A | nR | default | Highest | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | default | N/A | nZ | default | N/A | nR | default | Highest | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nX | default | N/A | nY | default | N/A | nR | default | Highest | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 4 | nR | default | N/A | nX | default | N/A | nY | default | N/A | nZ | default | Highest | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2.0: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: Inter-band: nX, nY, nZ, nR correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n28A-n78A, nX=n1, nY=n3, nZ=n28, nR=n78. Note 4: Intra-band contiguous + Inter-band: nX, nY, nZ correspond to the different bands in the CA Configuration, e.g. for CA_n1C-n3A-n8A, nX=n1, nY=n3, nZ = n8 Note 5: Intra-band non-contiguous + Inter-band: nX, nY and nZ correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A-n28A, nX=n1, nY =8, nZ = 28. Note 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 7: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration for Intra-band non-contiguous Note 8: For band combinations including operating bands without uplink band (as noted in Table 5.2-1), only the CA configurations where PCC band has uplink band shall be tested Note 9: The fallback configurations including CA_nXA-nYA for 4CA configurations nXC-nYA-nZA and nXB-nYA-nZA do not need to be tested even if the test frequency differs. 7.3A.1_1 shall be tested for all nXA-nYA combinations including exceptions. Note 10: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Table 7.3A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.2.1.4.3.

7.3A.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.3.1.4.3.

3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1-1 for C_RNTI to transmit the DL RMC according to Table 7.3A.3.4.1-1 on both PCC and SCC.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3A.3.4.1-1 on PCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3.2.5-1 and 7.3.2.5-2 as appropriate. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.3A.3.5	Test requirement

For 4DL carrier aggregation, test parameters are specified in table 7.3A.3.4.1-1. For the CA configurations listed in table 7.3A.3.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference sensitivity power level specified in table 7.3.2.5-1 for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for each non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC  given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.3.5-1: Reference sensitivity requirement for 4DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Inter-band 4DL CA | CA_n1A-n3A-n28A-n78A | - |
|  | CA_n26A-n66(2A)-n70A | - |
|  | CA_n48A-n66A-n71(2A) | - |
|  | CA_n48A-n66(2A)-n70A | - |
|  | CA_n48A-n66(2A)-n71A | - |
|  | CA_n48A-n70A-n71(2A) | - |
|  | CA_n48B-n66A-n70A | - |
|  | CA_n48B-n66A-n71A | - |
|  | CA_n48B-n70A-n71A | - |
|  | CA_n48(2A)-n66A-n70A | - |
|  | CA_n48(2A)-n66A-n71A | - |
|  | CA_n48(2A)-n66(2A) | - |
|  | CA_n48(2A)-n70A-n71A | - |
|  | CA_n48(2A)-n71(2A) | - |
|  | CA_n66A-n70A-n71(2A) | - |
|  | CA_n66B-n70A-n71A | - |
|  | CA_n66(2A)-n70A-n71A | - |
|  | CA_n66(2A)-n71(2A) | - |
|  | CA_n2A-n5A-n48A-n66A | - |
|  | CA_n2A-n5A-n48A-n77A | - |
|  | CA_n2A-n5A-n66A-n77A | - |
|  | CA_n2A-n48A-n66A-n77A | - |
|  | CA_n5A-n48A-n66A-n77A | - |
| SDL configuration | CA_n29A-n66B-n70A | - |
|  | CA_n29A-n66(2A)-n70 | - |

< Unchanged sections omitted >

### 7.4A.3	Maximum input level for CA (4DL CA)

7.4A.3.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.4A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.3.4	Test description

7.4A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 6 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 5 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 by default |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA-nRA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intraband non-contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: nX, nY, nZ,nR correspond to the different bands in the CA Configuration. E.g. for CA_n66A-n70A-n71A-n77A, nX=n66, nY=n70, nZ=n71 ,nR=n77; Intra-band contiguous + Inter-band: nX, nY correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, nX=n1, nY=n3; Intra-band non-contiguous + Inter-band: nX and nY correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, nX=n1, nY =n8. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test CC Combination setting is set as defined in Table 7.3A.3.4.1-1. If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 6: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.3.4.1-1. NOTE 7: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4A.3.4.3.

7.4A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4A.3.4.1-1 as appropriate for PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.4A.3.4.1-1. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value as defined in Table 7.4A.3.5-1 and Table 7.4A.3.5-2 according to the type of CA. Send uplink power control commands to the UE using 1dB step size to ensure that the PCC output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4A.3.5-1 or Table 7.4A.3.5-2 as appropriate for at least the duration of the Throughput measurement ,where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

7.	Measure the average throughput for the component carrier(s) indicated in table 7.4A.3.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.3.4.2-1.

Table 7.4A.3.4.2-1: Test repetition and measurement configuration

| CA configuration | Test ID (NOTE1) | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- | --- |
| Intra-band contiguous | 1, 2, 3 | 15 | PCC,SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
| Inter-band | 1, 2, 3 | 12, 22, 32, 42 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
| Intra-band contiguous + Inter-band | 1, 2, 3 | 13 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 23 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
|  |  | 33 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
| Intra-band non-contiguous + Inter-band | 1, 2, 3 | 14 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 24 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 34 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
| Intra-band non-contiguous + Intra-band non-contiguous | 1, 2, 3 | 1 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 2 | SCC2, SCC3 | Table 7.4A.3.5-2 |
| NOTE 1: Refers to Test IDs in Table 7.4A.3.4.1-1 NOTE 2: CA configuration ID as defined in table 7.3A.3.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 5: CA configuration ID as defined in “Default Test Settings for a CA_nXE Configuration (Intra-band contiguous)” in table 7.3A.3.4.1-1. NOTE 6: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous)” in table 7.3A.3.4.1-1. NOTE 7: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |

7.4A.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

The following exception applies to 1024QAM testing:

Table 7.4A.3.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table-r17 | qam1024 |  |  |
| } |  |  |  |

7.4A.3.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.3.5-1 and Table 7.4A.3.5-2 as applicable.

Table 7.4A.3.5-1: Maximum input level for 4DL CA (Intra-band contiguous)

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253, 4-TT | -253, 4-TT | -273, 4-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 5: TT for each frequency is specified in Table 7.4A.3.5-3 for each CC. |  |  |  |  |  |

Table 7.4A.3.5-2: Maximum input level for 4DL CA (Intra-band non-contiguous, Inter-band), per CC

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252-TT |  |  |  | -242-TT | -232-TT | -222-TT | -212-TT | -202-TT |  |  |  |  |
|  |  | -273-TT |  |  |  | -263,4-TT | -253,4-TT | -243,4-TT | -233,4-TT | -223-TT |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 5: TT for each frequency is specified in Table 7.4A.3.5-3 for each CC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.4A.3.5-3: Test Tolerance (Maximum input level), per CC

| f ≤ 3.0GHz | 3.0GHz < f ≤6.0GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

< Unchanged sections omitted >

### 7.5A.3	Adjacent channel selectivity for CA (4DL CA)

Editor’s note: - part content of Table 7.5A.3.4.2-1 is FFS.

7.5A.3.1	Test Purpose

Adjacent channel selectivity for 4DL CA verifies the receiver's ability to receive a wanted 4DL carrier aggregated at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

7.5A.3.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 4DL CA.

7.5A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.5A.0.

7.5A.3.4	Test Description

7.5A.3.4.1	Initial Conditions

Same as in clause 7.5A.1.4.1 with following exceptions:

-	Instead of Table 7.5A.1.4.1-1 use Table 7.5A.3.4.1-1.

-	Instead of Table 7.5A.1.4.1-2 use Table 7.5A.3.4.1-2.

-	Instead of Table 7.5A.1.4.1-3 use Table 7.5A.3.4.1-3.

Table 7.5A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band CA: NOTE 1, NOTE 5 Inter-band + Intra-band contiguous : NOTE 1, NOTE 5 Inter-band + Intra-band non-contiguous : NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.3.4.1-1(NOTE 5) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Intra-band contiguous: Lowest NRB_agg, Highest NRB_agg Inter-band: Highest NRB_agg Inter-band + Intra-band contiguous : Highest NRB_agg Inter-band + Intra-band non-contiguous : Highest NRB_agg NOTE 6 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation |  | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (intra-band contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA-nRA Configuration (Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYB-nZA, CA_nXA-nYA-nZB, CA_nXD-nYA, CA_nXC-nYB, CA_nXC-nYC and CA_nXB-nYB Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZ(2A), CA_nXA-nY(3A) and CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(4A) Configuration intra-band non-contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: nX, nY, nZ,nR correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n7A-n28A, nX=n1, nY=n3, nZ=n7,nR=n28; Intra-band contiguous + Inter-band: nX, nY, nZ correspond to the different bands in the CA Configuration, E.g. for CA_n3A-n7B-n28A, nX=n1, nY=n3, nz=n28; Intra-band non-contiguous + Inter-band: nX, nY, nZ correspond to the different bands in the CA Configuration. E.g. for CA_n2A-n66A-n77(2A), nX=n2, nY=n66, nZ=n77； NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.3.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

Table 7.5A.3.4.1-2: Void

Table 7.5A.3.4.1-3: Void

7.5A.3.4.2	Test Procedure

1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.1.1. Message contents are defined in clause 7.5A.3.4.3.

3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.5A.3.4.1-1 on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.5A.3.4.2-1 on PCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level according to Table 7.5A.3.4.2-1 as appropriate (Case 1). Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.5A.3.4.2-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

7.	Set the Interferer signal level to the value as defined in Table 7.5A.3.4.2-1 as appropriate (Case 1) and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D.

8.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

9.	Repeat steps from 6 to 8, using an interfering signal above the wanted signal in Case 1 at step 7.

10.	Set the Downlink signal level according to Table 7.5A.3.4.2-1 as appropriate (Case 2). Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.5A.3.4.2-1 for at least the duration of the Throughput measurement, where MU and Uplink power control window size are defined above.

11.	Set the Interferer signal level to the value as defined in Table 7.5A.3.4.2-1 as appropriate (Case 2) and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D.

12.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

13.	Repeat steps from 10 to 12, using an interfering signal above the wanted signal in Case 2 at step 11.

14.	Repeat for applicable channel bandwidths and operating band combinations in both Case 1 and Case 2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

Table 7.5A.3.4.2-1: Test repetition and measurement configuration

| CA configuration | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- |
| Intra-band contiguous | 14 | PCC, SCC1, SCC2,SCC3 | TBD |
| Inter-band | 11 | SCC1, SCC2,SCC3 | 7.5A.3.5-4, 7.5A.3.5-5, 7.5A.3.5-6, 7.5A.3.5-7, 7.5A.3.5-8, 7.5A.3.5-9 |
|  | 21 |  |  |
|  | 31 |  |  |
|  | 41 |  |  |
| Intra-band contiguous + Inter-band | 12 | TBD | TBD |
|  | 22 |  |  |
|  | 32 |  |  |
| Intra-band non-contiguous + Inter-band | 13 | TBD | TBD |
|  | 23 |  |  |
|  | 33 |  |  |
| Intra-band non-contiguous | 15 | TBD | TBD |
| NOTE 1: CA configuration ID as defined in table 7.3A.3.4.1-1. NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_nXA-nYB-nZA, CA_nXA-nYA-nZB, CA_nXD-nYA, CA_nXC-nYB, CA_nXC-nYC and CA_nXB-nYB Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nXA-nYA-nZ(2A), CA_nXA-nY(3A) and CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nXE Configuration (Intra-band contiguous)” in table 7.3A.3.4.1-1. NOTE 5: CA configuration ID as defined in “Default Test Settings for a CA_nX(4A) Configuration intra-band non-contiguous)” in table 7.3A.3.4.1-1. |  |  |  |

7.5A.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.5A.3.5	Test Requirement

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.3.5-2, 7.5A.3.5-2a, 7.5A.3.5-3, 7.5A.3.5-3a.

Table 7.5A.3.5-1: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D |  |
| ACS | dB | 26.0 | 33.0 | 25.2 |  |

Table 7.5A.3.5-1a: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

|  |  | NR CA bandwidth class |  |
| --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 20.0 | 17.0 |

Table 7.5A.3.5-2: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
| PInterferer | dBm | Aggregated power + 24.5 dB | Aggregated power + 31.5 dB | Aggregated power + 23.7 dB |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 - Foffset | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

Table 7.5A.3.5-2a: Test parameters for intra-band contiguous CA with FDL_low<2700 MHz and FUL_low<2700 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 18.5 dB | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.3.5-3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49.5 + 10log(NRB,c/NRB_agg) | -56.5 | -48.7 + 10log(NRB,c/NRB_agg) |  |
| PInterferer | dBm | -25 | -25 | -25 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 -Foffset | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

Table 7.5A.3.5-3a: Test parameters for intra-band contiguous CA with FDL_low <2700 MHz and FUL_low<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -43.5 + 10log(NRB,c/NRB_agg) | -40.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

For NR PCC and SCCs of inter-band and intra-band non-contiguous CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.3.5-5 and 7.5A.3.5-6.

Table 7.5A.3.5-4: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| ACS | dB | 33 | 33 | 30 | 27 | 26 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| ACS | dB | 25.5 | 24 | 23 | 22.5 | 21 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 20.5 | 20 |  |  |  |

Table 7.5A.3.5-5: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1 (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 42.5 dB | REFSENS for SCC + 39.5 dB | REFSENS for SCC + 38.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 38 dB | REFSENS for SCC + 36.5 dB | REFSENS for SCC + 35.5 dB | REFSENS for SCC + 35 dB | REFSENS for SCC + 33.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 33 dB | REFSENS for SCC + 32.5 dB |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset from SCC) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5A.3.5-6: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2 (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -49.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49 | -47 | -46.5 | -46 | -44.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -44 | -43.5 |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset from SCC) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

For NR PCC and SCCs of inter-band and intra-band non-contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.3.5-8 and 7.5A.3.5-9.

Table 7.5A.3.5-7: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| ACS | dB | 33 | 33 | 33 | 33 |  |

Table 7.5A.3.5-8: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1 (inter-band,intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset from SCC) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset from SCC) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5A.3.5-9: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2 (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset from SCC) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset from SCC) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.2.3	In-band Blocking for CA (4DL CA)

7.6A.2.3.1	Test purpose

Same test purpose as in clause 7.6A.2.1.

7.6A.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support 4DL CA.

7.6A.2.3.3	Minimum conformance requirements

Minimum requirements are defined in clause 7.6A.2.0.

7.6A.2.3.4	Test description

7.6A.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.2.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.2.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 6 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg, NOTE 1, NOTE 5 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous CA) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-ZA and CA_nXB-nYA-ZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-ZA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configurations (Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 6: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.3.4.1-1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.7 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.6A.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.6A.2.2.4.3.

7.6A.2.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.6A.2.3.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to Table 7.6A.2.3.4.1-1 on both SCC and PCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 7.6A.2.3.4.1-1 on PCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6A.2.3.4.2-1.

7	Set the downlink signal level according to the Table 7.6A.2.3.4.2-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in table 7.6A.2.2.4.2-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

8	Measure the average throughput for the carrier(s) indicated in Table 7.6A.2.3.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

9	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to Table 7.6A.2.3.4.2-1 in Case 1 at step 6.

10	Repeat steps from 6 to 9, using interfering signals in Case 2 at step 6 and 9. The ranges of case 2 are covered in steps equal to the interferer bandwidth.

11.	Repeat steps 1 to 10 for all component carriers listed in Table 7.6A.2.3.4.2-1.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

Table 7.6A.2.3.4.2-1: Test repetition and measurement configuration

| CA configuration | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2, SCC3 | 7.6A.2.3.5-3 7.6A.2.3.5-3a 7.6A.2.3.5-4 7.6A.2.3.5-4a |
| Inter-band | 11 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 21 |  |  |
|  | 31 |  |  |
|  | 41 |  |  |
| Intra-band contiguous + Inter-band | 12 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 22 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-3 7.6A.2.3.5-3a 7.6A.2.3.5-4 7.6A.2.3.5-4a |
|  | 32 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-3 7.6A.2.3.5-3a 7.6A.2.3.5-4 7.6A.2.3.5-4a |
| Intra-band non-contiguous + Inter-band | 13 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 23 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 33 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-2 7.6A.2.3.5-2a |
| Intra-band non-contiguous + Intra-band non-contiguous | 14 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 24 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-2 7.6A.2.3.5-2a |
| NOTE 1: CA configuration ID as defined in table 7.3A.3.4.1-1. NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous)” in table 7.3A.2.4.1-1. |  |  |  |

7.6A.2.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.6A.2.3.5	Test requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables below, according to the type of CA.

Table 7.6A.2.3.5-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 11.5 | 12 | 12.5 | 13 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 14 | 14.5 | 15 | 15.5 | 16 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. |  |  |  |  |  |  |

Table 7.6A.2.3.5-1a: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | - BWChannel_CA/2 – FIoffset, case 1 and BWChannel_CA/2 + FIoffset, case 1 | ≤ - BWChannel_CA/2 – FIoffset, case 2 and ≥BWChannel_CA/2 + FIoffset, case 2 |  | - BWChannel_CA/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n26, n28,n34, n38,n39, n40, n41, n483, n50, n51, n53, n65, n66, n70, n74, n75, n76 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel_CA/2 – FIoffset, case 1; b: BWChannel_CA/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

7.6A.2.3.5-1b: In-band blocking for additional NR operating bands for carrier aggregation with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
|  | Finterferer (offset) | MHz | - BWChannel_CA/2 – FIoffset, case 1 and BWChannel_CA/2 + FIoffset, case 1 | ≤ - BWChannel_CA/2 – FIoffset, case 2 and ≥BWChannel_CA/2 + FIoffset, case 2 |
| n29 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel_CA/2 – FIoffset, case 1; b: BWChannel_CA/2 + FIoffset, case 1 NOTE 3: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal NOTE 4: BWChannel_CA denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.3.5-2: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| FIoffset, case 1 | MHz | 15 | 22.5 | 30 | 37.5 | 45 |
| FIoffset, case 2 | MHz | 25 | 37.5 | 50 | 62.5 | 75 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| FIoffset, case 1 | MHz | 60 | 75 | 90 | 105 | 120 |
| FIoffset, case 2 | MHz | 100 | 125 | 150 | 175 | 200 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| FIoffset, case 1 | MHz | 135 | 150 |  |  |  |
| FIoffset, case 2 | MHz | 225 | 250 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |

Table 7.6A.2.3.5-2a: In-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | - BWChannel_CA/2 – FIoffset, case 1 and BWChannel_CA/2 + FIoffset, case 1 | ≤ - BWChannel_CA/2 – FIoffset, case 2 and ≥BWChannel_CA/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3* BWChannel_CA to FDL_high + 3* BWChannel_CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel_CA/2 – FIoffset, case 1; b: BWChannel_CA/2 + FIoffset, case 1 NOTE 3: BWChannel_CA denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.3.5-3: In-band blocking parameters with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (intra-band contiguous CA)

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  |  | 10.0 | 6 | 13.8 |  |
| BWInterferer | MHz | 20 | BWChannel_CA | 50 |  |
| FIoffset, case 1 | MHz | 30 | BWChannel_CA+ BWChannel_CA/2 | 75 |  |
| FIoffset, case 2 | MHz | 50 | BWInterferer + FIoffset, case 1 | 125 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |

Table 7.6A.2.3.5-3a: In-band blocking parameters with FDL_low < 2700 MHz and FUL_low < 2700 MHz (intra-band contiguous CA)

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below |  |
|  |  | 16.0 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table7.6A.2.3.5-4: In-band blocking with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (intra-band contiguous CA)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel_CA/2 –FIoffset, case 1 and BWchannel_CA/2 +FIoffset, case 1 | ≤ -BWChannel_CA/2 –FIoffset, case 2 and ≥ BWChannel_CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3*BWChannel_CA to FDL_high + 3*BWChannel_CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel_CA/2 – FIoffset, case 1; b: BWChannel_CA/2 + FIoffset, case 1 NOTE 3: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.3.5-4a: In-band blocking with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz (intra-band contiguous CA)

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n41, n66, n484,n40 | Finterferer (offset) | MHz | -BWChannel CA/2 –FIoffset, case 1 and BWChannel CA/2 +FIoffset, case 1 | ≤ -BWChannel CA/2 –FIoffset, case 2 and ≥ BWChannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel_CA/2 – FIoffset, case 1; b: BWchannel_CA/2 + FIoffset, case 1 NOTE 3: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.3-1, Pinterferer power defined in Table 7.6A.2.3.5-1a and Table 7.6A.2.3.5-2a is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.3-1.

< Unchanged sections omitted >

#### 7.6A.3.3	Out-of-band blocking for CA (4DL CA)

7.6A.3.3.1	Test purpose

Out-of-band band blocking for CA is defined for an unwanted CW interfering signal falling more than 15 MHz or 3*BWChannel_CA below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz or 3*BWChannel_CA below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5A and sub-clause 7.6A.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.6A.3.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.3.0.

7.6A.3.3.4	Test description

7.6A.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.6A.3.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.3.3.4.1-1: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Inter-band : NOTE 5 Intra-band contiguous + Inter-band: NOTE 5 Intra-band non-contiguous + Inter-band: MaxWGap for Intra-band non-contiguous (NOTE 5) Intra-band non-contiguous + Intra-band non-contiguous: MaxWGap for Intra-band non-contiguous (NOTE 5) |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.3.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.7 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.6A.3.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.6A.3.3.4.3.

7.6A.3.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.6A.3.3.4.3.

3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6A.3.3.4.1-1 on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6A.3.3.4.1-1 on PCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

6.	For the CCs the throughput is measured on according to Table 7.6A.3.3.4.2-1, set the parameters of the CW signal generator for an interfering signal below each SCC’s operating band for inter-band CA according to Table 7.6A.3.3.5.1-2 or 7.6A.3.3.5.1-4. The frequency step size is  MHz.

If CW interferer falls in a gap between FDL_High(j) and FDL_Low(j+1) where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

For the UE which supports inter-band CA configuration in Table 7.3A.0.3.2.1-1, Pinterferer power defined in Table 7.6A.3.3.5.1-2 and 7.6A.3.3.5.1-4 is increased by the amount given by ΔRIB,c in Table 7.3A.0.3.2.1-1. Use the highest RIB,c among CA bands for Pinterferer calculation.

For inter-band CA combination listed in Table 7.6A.3.3.5.1-5, exceptions to the requirement specified in Table 7.6A.3.3.5.1-6 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

7.	Set the downlink signal level according to the Table 7.6A.3.3.5.1-1, or 7.6A.3.3.5.1-3 for all carriers. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6A.3.3.5.1-1, or 7.6A.3.3.5.1-3 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

8.	Measure the average throughput for the carrier(s) indicated in Table 7.6A.3.3.4.2-1 for a duration sufficient to achieve statistical significance according to Annex H.2A for inter-band CA.

9.	Record the frequencies for which the throughput doesn't meet the requirements and for each frequency, the carriers for which the throughput was not met.

10.	Repeat steps 6 to 8 for each recorded frequency-carrier pair, with exception of pairs for which RIB,c is the same as RIB used in Step 6. In Step 6 use only recorded frequencies for interferer placement and use RIB,c relevant to recorded carrier for Pinterferer calculation. Remove the frequency-carrier pairs that meet the throughput requirements from the record.

11.	Repeat steps from 6 to 10, using an interfering signal above each SCC’s operating band for inter-band CA at step 6.

12.	For Inter-band CA: Switch the SCell into PCell and repeat steps 1 to 11 for the CA configuration IDs in REFSENS indicated in Table 7.6A.3.3.4.2-1, except for operating bands without uplink band.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

Table 7.6A.3.3.4.2-1: Test repetition and measurement configuration

| CA configuration | CA configuration ID in REFSENS | Throughput measured on |
| --- | --- | --- |
| Inter-band | 11 | SCC1, SCC2, SCC3 |
|  | 21 |  |
|  | 31 |  |
|  | 41 |  |
| Intra-band contiguous + Inter-band | 12 | SCC2, SCC3 |
|  | 22 | SCC1, SCC2, SCC3 |
|  | 32 | SCC1, SCC2, SCC3 |
| Intra-band non-contiguous + Inter-band | 13 | SCC2, SCC3 |
|  | 23 | SCC1, SCC2, SCC3 |
|  | 33 | SCC1, SCC2, SCC3 |
| Intra-band non-contiguous + Intra-band non-contiguous | 14 | SCC2, SCC3 |
|  | 24 | SCC2, SCC3 |
| NOTE 1: CA configuration ID as defined in Table 7.3A.3.4.1-1. NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band)” in Table 7.3A.3.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band)” in Table 7.3A.3.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous)” in Table 7.3A.2.4.1-1. |  |  |

7.6A.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.6A.3.3.5	Test requirement

7.6A.3.3.5.1	Out-of-band blocking for Inter-band CA

Except for the spurious response frequencies recorded in step 9 and step 10 of test procedure, the throughput measurement derived in the test procedure of SCCs shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6A.3.3.5.1-1 and 7.6A.3.3.5.1-2 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and Tables 7.6A.3.3.5.1-3 and 7.6A.3.3.5.1-4 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

The number of spurious response frequencies recorded in step 9 and step 10 of test procedure shall not exceed   in each assigned frequency channel when measured using a  MHz step size. For these exceptions the requirements of clause 7.7A Spurious Response are applicable.

Table 7.6A.3.3.5.1-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 11.5 | 12 | 12.5 | 13 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 14 | 14.5 | 15 | 15.5 | 16 |
| NOTE: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.6A.3.3.5.1-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n20, n25, n28, n34, n38, n39, n40, n41, n50, n51, n66, n70, n71, n74, n75, n76 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 6: Void. NOTE 7: For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. |  |  |  |  |  |

Table 7.6A.3.3.5.1-3: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| NOTE: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.6A.3.3.5.1-4: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3*BWChannel or 3*BWChannel ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel) or FDL_high + MAX(200,3*BWChannel) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel from the band edge. For BWChannel larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. |  |  |  |  |  |

If CW interferer falls in a gap between FDL_High(j) and FDL_Low(j+1) where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

For the UE which supports inter-band CA configuration in Table 7.3A.3.5.1.3-5, Pinterferer power defined in Table 7.6A.3.3.5.1-2 and 7.6A.3.3.5.1-4 is increased by the amount given by ΔRIB,c in Table 7.3A.3.5.1.3-1.

For inter-band CA combination listed in Table 7.6A.3.3.5.1-5, exceptions to the requirement specified in Table 7.6A.3.3.5.1-6 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6A.3.3.5.1-5: CA band combination with exceptions allowed

CA band combination

CA_n8-n78

CA_n28-n78

Table 7.6A.3.3.5.1-6: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

< End of changes >