3GPP TSG-RAN5 Meeting #102	R5-241361r2

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0028 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Splitting the NR NTN frequency error test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Pre-configuration inside the test procedure breaks test case automatization. In order to allow grouping of test cases with same pre-configuration in order to execute them without manual interaction, splitting of the NTN frequency error test case is required. For details refer to the discussion in R5-241360. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NTN frequency error test case 6.4.1 has been split into 6.4.1 with GSO ephemeris and 6.4.1a with NGSO ephemeris. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case automatization remains prevented and test time cannot be reduced due to manual interaction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.1.2, 6.4.1.4.1, 6.4.1.4.2, 6.4.1.4.3, 6.4.1a (new) F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0400 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper in R5-241360, applicability update in CR R5-241362 r1: Added changes to split the test case. r2: Adding updates to Annex F |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 6.4.1	Frequency error with GSO ephemeris

Editor's Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Verification of UE frequency pre-compensation is to be updated

#### 6.4.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

#### 6.4.1.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation and GSO.

#### 6.4.1.3	Minimum conformance requirements

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

The normative reference for this requirement is TS 38.101-5 [11] clause 6.4.1.

#### 6.4.1.4	Test description

##### 6.4.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 Annex C.2.

Table 6.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, in Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to clauses C.0, C.1, C.2, and uplink signals according to clauses G.0, G.1, G.2, G.3.0.

4.	The DL and UL Reference Measurement channels are set according to Table 6.4.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location for GSO according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

##### 6.4.1.4.2	Test procedure

1.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508 [12] table 5.6.2.1-1. Test system shall send same SIB19 information during the duration of this frequency error measurement as defined in TS 38.508-1 [12] clause 5.6.3.1.

2.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.4.1.4.3.

3.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 6.4.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3.2.5-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test. Allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

7.	Repeat from test procedure steps 1-6 with ephemeris values for maximum positive Doppler replacing ephemeris in step 1 by Table 6.4.1.4.3-1a. Test system shall send same SIB19 information during the duration of this frequency error measurement.

8.	Repeat from test procedure steps 1-6 with ephemeris values for maximum negative Doppler replacing ephemeris in step 1 by Table 6.4.1.4.3-2a. Test system shall send same SIB19 information during the duration of each frequency error measurement.

9.	Repeat from test procedure steps 1-6 with ephemeris values for half of maximum positive Doppler replacing ephemeris in step1 by Table 6.4.1.4.3-3a. Test system shall send same SIB19 information during the duration of this frequency error measurement.

##### 6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and subclause 5.6.2.1 with the following exceptions:

Table 6.4.1.4.3-1a: EphemerisInfo- NR NTN Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -17104941 |  |  |
| positionY-r17 | 27550229 |  |  |
| positionZ-r17 | -607219 |  |  |
| velocityVX-r17 | 258 |  |  |
| velocityVY-r17 | 299 |  |  |
| velocityVZ-r17 | 6277 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 26.15 degrees, one-way delay equal to 129.93 ms and Doppler equal to 0.17 ppm |  |  |  |

Table 6.4.1.4.3-1b: Void.

Table 6.4.1.4.3-2a: EphemerisInfo -NR NTN Ephemeris Information for GSO satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -17061001 |  |  |
| positionY-r17 | 27582763 |  |  |
| positionZ-r17 | -276165 |  |  |
| velocityVX-r17 | 361 |  |  |
| velocityVY-r17 | 160 |  |  |
| velocityVZ-r17 | -6335 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 26.78 degrees, one-way delay equal to 129.74 ms and Doppler equal to -0.17 ppm. |  |  |  |

Table 6.4.1.4.3-2b: Void.

Table 6.4.1.4.3-3a: EphemerisInfo -NR NTN Ephemeris Information for GSO  satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -17062164 |  |  |
| positionY-r17 | 27354696 |  |  |
| positionZ-r17 | -3544856 |  |  |
| velocityVX-r17 | -360 |  |  |
| velocityVY-r17 | 164 |  |  |
| velocityVZ-r17 | 2993 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 20.61 degrees, one-way delay equal to 131.70 ms and Doppler equal to -0.085 ppm. |  |  |  |

Table 6.4.1.4.3-3b: Void.

#### 6.4.1.5	Test requirement

The frequency error Δf shall fulfil the test requirement:

|Δf| ≤ (0.1 PPM + [15 Hz])

The above requirement shall be verified for at least two cases of which one has zero Doppler conditions.

### 6.4.1a	Frequency error with NGSO ephemeris

Editor's Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Verification of UE frequency pre-compensation is to be updated

#### 6.4.1a.1	Test purpose

#### Same test purpose as in clause 6.4.1.1.

#### 6.4.1a.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation and NGSO.

#### 6.4.1a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.4.1.3.

#### 6.4.1a.4	Test description

##### 6.4.1a.4.1	Initial condition

Same initial conditions as in clause 6.4.1.4.1 with the following exception:

-	In step 6, instead of UE location for GSO satellite  use UE location for NGSO satellite

##### 6.4.1a.4.2	Test procedure

Same test procedure as in clause 6.4.1.4.2 with the following exceptions:

-	In step 1, instead of TS 38.508-1 [12] Table 5.6.2.1-1  use TS 38.508-1 [12] Table 5.6.2.1-3 (ephemeris for NGSO LEO 1200).

-	Instead of Tables 6.4.1.4.3-1a, 6.4.1.4.3-2a, and 6.4.1.4.3-3a  use Tables 6.4.1a.4.3-1, 6.4.1a.4.3-2, and 6.4.1a.4.3-3, respectively.

-	If the UE supports GSO and NGSO, skip steps 1 to 6 of the test procedure and start with step 7.

##### 6.4.1a.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and subclause 5.6.2.1 with the following exceptions

Table 6.4.1a.4.3-1: EphemerisInfo- NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.2.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 |  |  |
| positionY-r17 | 4550419 |  |  |
| positionZ-r17 | 852799 |  |  |
| velocityVX-r17 | 6164 |  |  |
| velocityVY-r17 | -19424 |  |  |
| velocityVZ-r17 | 124281 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1a.4.3-2: EphemerisInfo -NR NTN NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.2.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 |  |  |
| positionY-r17 | 3404229 |  |  |
| positionZ-r17 | 3535794 |  |  |
| velocityVX-r17 | 35394 |  |  |
| velocityVY-r17 | -74414 |  |  |
| velocityVZ-r17 | 94682 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1a.4.3-3: EphemerisInfo -NR NTN Ephemeris Information for NGSO (LEO-600)  satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.2.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 |  |  |
| positionY-r17 | 4245650 |  |  |
| positionZ-r17 | 2024520 |  |  |
| velocityVX-r17 | 19359 |  |  |
| velocityVY-r17 | -43278 |  |  |
| velocityVZ-r17 | 116553 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

#### 6.4.1a.5	Test requirement

The frequency error Δf shall fulfil the test requirement:

|Δf| ≤ (0.1 PPM + [15 Hz])

The above requirement shall be verified for at least two cases of which one has zero Doppler conditions.

## 6.5	Output RF spectrum emissions

< Unchanged sections omitted >

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.3.1 Minimum output power | Same as clause 6.3.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.3.2 Transmit OFF power | Same as clause 6.3.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.4.1 Frequency error with GSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.4.1a Frequency error with NGSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |

< Unchanged sections omitted >

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.3.1 Minimum output power | Same as clause 6.3.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Minimum requirement + TT |
| 6.3.2 Transmit OFF power | Same as clause 6.3.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Minimum requirement + TT |
| 6.4.1 Frequency error with GSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.4.1a Frequency error with NGSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |

< End of changes >