3GPP TSG-RAN5 Meeting #102	R5-241378r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4308 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrrection to NR5GC CAG test case 6.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specific message contents in Table 6.5.2.1.3.3-3 are not in accordance with the Note specified at Steps 5-22. The note says that REGISTRATION ACCEPT message is specified to contain PLMN 2 which corresponds to cell 4 and CAG ID 2 while in the specific message contents it contains the parameters for PLMN 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the Note at at Steps 5-22 to align with specific message contents in Table 6.5.2.1.3.3-3. Also made some editorial changes in Table 6.5.2.1.3.2-2 and in the test procedure Table 6.5.2.1.3.2-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1->Aligned the note with specific message contents |  |  |  |  |  |  |  |  |

#### 6.5.2.1	CAG Selection in Manual Mode

6.5.2.1.1	Test Purpose (TP)

(1)

with { UE in Manual network selection mode and provided with CAG information list }

ensure that {

when { there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" }

then { UE manually selects the CAG cell }

}

(2)

with { UE in Manual network selection mode and PLMN ID of the PLMN in the "CAG information list" and the CAG-ID are not included in the "Allowed CAG list" }

ensure that {

when { the available CAG cell broadcasts that the PLMN allows a user to manually select the CAG-ID }

then { UE manually selects the CAG cell that is outside the UE's Allowed CAG list }

}

(3)

with { UE in Manual network selection mode }

ensure that {

when { there exists an entry for the presented PLMN in the "CAG information list" and the entry includes an "indication that the MS is only allowed to access 5GS via CAG cells" }

then { UE manually selects the CAG cell }

}

6.5.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.1.2. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 4.4.3.1.2]

The MS indicates whether there are any PLMNs, which are available using all supported access technologies. This includes PLMNs in the "forbidden PLMNs" list, "forbidden PLMNs for GPRS service" list and PLMNs which only offer services not supported by the MS. An MS which supports GSM COMPACT shall also indicate GSM COMPACT PLMNs (which use PBCCH).

If displayed, PLMNs meeting the criteria above are presented in the following order:

i)-	either the HPLMN (if the EHPLMN list is not present or is empty) or, if one or more of the EHPLMNs are available then based on an optional data field on the SIM either only the highest priority available EHPLMN is to be presented to the user or all available EHPLMNs are presented to the user in priority order. If the data field is not present on the SIM, then only the highest priority available EHPLMN is presented;

ii)-	PLMN/access technology combinations contained in the " User Controlled PLMN Selector with Access Technology " data file in the SIM (in priority order);

iii)- PLMN/access technology combinations contained in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order);

iv)- other PLMN/access technology combinations with received high quality signal in random order;

v)-	other PLMN/access technology combinations in order of decreasing signal quality.

…

In i to v, if the MS supports CAG, for each PLMN/access technology combination of NG-RAN access technology, the MS shall present to the user:

a)	the PLMN/access technology combination and a list of CAG-IDs composed of one or more CAG-IDs such that for each CAG-ID:

1)	there is an available CAG cell which broadcasts the CAG-ID for the PLMN; and

2)	the following is true:

i)	there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry; or

ii)	the available CAG cell broadcasting the CAG-ID for the PLMN also broadcasts that the PLMN allows a user to manually select the CAG-ID.

For each of the presented CAG-ID, the MS may indicate to the user whether the CAG-ID is present in the "Allowed CAG list" stored in the UE; and

b)	the PLMN/access technology combination without a list of CAG-IDs, if there is an available NG-RAN cell which is not a CAG cell for the PLMN. If there exists an entry for the presented PLMN in the "CAG information list" and the entry includes an "indication that the MS is only allowed to access 5GS via CAG cells", the MS may indicate to the user that the MS is only allowed to access the PLMN via CAG cells.

6.5.2.1.3	Test description

6.5.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 4.

-	NR Cell 2 and NR Cell 4 are CAG cells.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR Cell 1, NR Cell 2 and NR Cell 4.

-	The PLMNs are identified in the test by the identifiers in Table 6.5.2.1.3.1-1 and the PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.5.2.1.3.1–1: PLMN and CAG identifiers

| NR Cell | PLMN name | CAG IDs |
| --- | --- | --- |
| 1 | PLMN 1 | - |
| 2 | PLMN 3 | 1 |
| 4 | PLMN 2 | 2 |

UE:

None.

Preamble:

-	The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1.

-    The UE is set to Manual PLMN selection mode before it is in state Switched OFF (State 0N-B) as per TS 38.508-1 [4] Table 4.4A.2-0.

6.5.2.1.3.2	Test procedure sequence

Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2 illustrate the downlink power to be applied for the cells at various time instants of the test execution. The configuration T0 indicates the initial conditions for preamble. Configurations marked "T1", "T2" and "T3"are applied at the points indicated in the Main behaviour description in Table 6.5.2.1.3.2-3.

Table 6.5.2.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | -88 | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -115 | "Off" | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |

Table 6.5.2.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | ”Off” | ”Off” | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.2.2-2 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | -82 | ”Off” | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.2.2-2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | ”Off” | -82 | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.2.2-2 |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | -82 | ”Off” | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.2.2-2 |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -82 | ”Off” | ”Off” | Power level "Off" is defined in TS 38.508-1 [4] Table 6.2.2.2.2-2 |

Table 6.5.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes cell power levels according to row "T1" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | The UE is made to perform manual CAG selection and selects NR Cell 2. | - | - | - | - |
| 4 | Check: Does the UE send a RRCSetupRequest on NR Cell 2? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 5-22 | Steps 3-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 2. Note: The UE performs initial registration procedure with REGISTRATION ACCEPT message including CAG information list with PLMN ID 3 and CAG only indication and the RRC connection is released. | - | - | - | - |
| 23 | The SS changes cell power levels according to row "T2" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 24 | The UE is made to perform manual CAG selection and select NR Cell 4. | - | - | - | - |
| 25 | Void | - | - | - | - |
| 26-27A | Steps 1 to 3 of the test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 are performed on NR Cell 4. | - | - | 2 | P |
| 28-36 | Void | - | - | - | - |
| 37 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 38-39 | Steps 5 to 6a1 of the test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 are performed on NR Cell 4. | - | - | - | - |
| 40-46 | Void | - | - | - | - |
| 46A | The SS changes cell power levels according to row "T3" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 47 | The UE is made to perform manual CAG selection and select NR Cell 2. | - | - | - | - |
| 48 | Void | - | - | - | - |
| 49-53A | Steps 1-6a1 of the test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 are performed on NR Cell 2. | - | - | 3 | P |
| 54 | The SS changes cell power levels according to row "T4" in Table 6.5.2.1.3.2-1 for FR1 and Table 6.5.2.1.3.2-2 for FR2. | - | - | - | - |
| 55 | The user sets the UE in Automatic PLMN selection mode. | - | - | - | - |
| 56 | The UE performs the Registration procedure for mobility registration update as specified in TS 38.508-1 [4] subclause 4.9.5 on NR Cell 1. | - | - | - | - |

6.5.2.1.3.3	Specific message contents

Table 6.5.2.1.3.3-1: REGISTRATION REQUEST (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type |  |  |  |
| 5GS registration type value | '001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| CAG | ‘1’B | CAG supported |  |

Table 6.5.2.1.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 003; MNC : 21; CAG only : 1; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See Table 6.5.2.1.3.1–1 |  |
| MNC | MNC of NR Cell 2 | See Table 6.5.2.1.3.1–1 |  |
| CAG only | ‘0’B |  |  |
| CAG-ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.1.3.3-3: REGISTRATION ACCEPT (Step 16, Table 6.5.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : ; MNC : 21; CAG only : 1; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See Table 6.5.2.1.3.1–1 |  |
| MNC | MNC of NR Cell 2 | See Table 6.5.2.1.3.1–1 |  |
| CAG only | ‘1’B |  |  |
| CAG-ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.1.3.3-4: SIB1 for NR Cell 4 (all steps in T2, Table 6.5.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 002 | PLMN2 MCC |  |
| mnc | 11 | PLMN2 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entr 1 |  |
| cag-Identity-r16 | 2 | CAG ID is coded as a 32 bit BITSTRING |  |
| manualCAGselectionAllowed-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 4 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 4 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1.3.3-5: SIB1 for NR Cell 2 (all steps in Table 6.5.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition CAG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| npn-IdentityInfoList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxPLMN)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] CHOICE { |  | entry 1 |  |
| pni-npn-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc | 003 | PLMN3 MCC |  |
| mnc | 21 | PLMN3 MNC |  |
| } |  |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| CAG-IdentityInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| cag-Identity-r16 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |
| manualCAGselectionAllowed-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 2 |  |  |
| cellIdentity-r16 | Cell Identity of NR Cell 2 |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |