3GPP TSG-RAN WG5 Meeting #102	R5-241381r2

Athens, Greece, 26th February – 3rd March 2024

Title:	IoT NTN RRM Ephemeris: definition for neighbour cells and updates for serving cells

Source:	Keysight Technologies UK Ltd, Thales

Agenda item:	5.3.22.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN4 has been working on the definition of core and performance requirements for IoT NTN User Equipment (UE) supporting satellite access operation as part of the work item defined in [1] during Rel-18. One of this work item outcomes has been the definition of IoT NTN RRM test cases defined in 3GPP TS 36.133 Annex A.13 (applicable to UE Category NB1) and A.14 (applicable to UE category M1).

Some of these RRM test cases require the definition of satellite neighbour cells:

Table 1. NB-IoT NTN RRM test cases requiring a neighbour cell according to TS 36.133[2]

| TC ID# | TC Title |
| --- | --- |
| A.13.1.1.1 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |
| A.13.1.1.2 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation |
| A.13.1.1.3 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |
| A.13.3.1.1 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |
| A.13.3.1.2 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |

Table 2. eMTC NTN RRM test cases requiring a neighbour cell according to TS 36.133[2]

| TC ID# | TC Title |
| --- | --- |
| A.14.1.1.1 | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
| A.14.1.1.2 | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |
| A.14.1.1.3 | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation |
| A.14.1.1.4 | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation |
| A.14.2.1.1 | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition |
| A.14.2.1.2 | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition |
| A.14.2.1.3 | E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA |
| A.14.2.1.4 | E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA |
| A.14.3.1.1 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access |
| A.14.3.1.2 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |
| A.14.5.1.1 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |
| A.14.5.1.2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |
| A.14.5.1.3 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |
| A.14.5.1.4 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |
| A.14.6.1.1 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |
| A.14.6.1.2 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |

Additionally, RAN5 is working in the corresponding work item [3] to define conformance test cases for IoT NTN.

As part of this work, RAN5 defined the ephemeris of the serving cell for all type of test cases in [4]. At that point, ephemeris for GSO (7º inclination) and NGSO were defined. However, there was no ephemeris defined for GEO satellites for the serving cell as required in test cases listed in Tables 1 and 2 above.

Additionally, the definition of the ephemeris for the neighbour cell required in RRM test cases listed in Tables 1 and 2 is still pending according to the latest work plan [5].

This document analyses the IoT NTN RRM scenarios where a neighbour cell is required and makes proposals for the definition of its ephemeris. Additionally, it makes some proposals to define the ephemeris for serving cell for GEO scenarios.

# 2. 	Discussion

## 2.1 Scenarios

Analysing the list of test cases included in Tables 1 and 2 and its content in 3GPP TS 36.133 [2], it can be seen that:

- For NB-IoT NTN RRM test cases requiring neighbour cells:

  - All test cases are applicable only to GEO satellites.

  - Only one neighbour cell is required.

  - Neighbour cell is defined as intra-frequency.

- For eMTC NTN RRM test cases requiring neighbour cells:

  - Cell re-selection, E-UTRAN handover, Intra-frequency measurements and RSRP test cases are assumed to be applicable only to GSO satellites (explicitly stated for test cases A.14.1.1.1/2/3, A.14.2.1.1/2/3/4, A.14.5.1.1/2/3/4 and A.14.6.1.1/2 but missing for test case A.14.1.1.4).

  - RRC re-establishment test cases (A.14.3.1/2) are applicable to both GSO and NGSO satellites.

  - Only one neighbour cell is required.

  - Neighbour cell is defined as intra-frequency.

Observation 1: According to 3GPP TS 36.133 [2], for NB-IoT NTN RRM test cases requiring neighbour cells, only one neighbour intra-frequency cell is required for GEO satellites for Rel-17 UEs.

Observation 2: According to 3GPP TS 36.133 [2], for eMTC NTN RRM test cases requiring neighbour cells, only one neighbour intra-frequency cell is required for GSO and NGSO satellites.

However, RAN2 decided not to introduce satellite assistance information of neighbour cells in system information in Rel-17 IoT NTN but in Rel-18 (refer to LS response in [6] to corresponding LS in [7] Neighbour cell ephemeris information has been defined in SIB33 for eMTC NTN and SIB33-NB for NB-IoT NTN in Rel-18.

As a consequence, RAN4 decided to define test cases IoT NTNT RRM test cases requiring neighbour cells only for intra-frequency scenarios for GEO satellites applicable to Rel-17 devices (refer to agreement in [8] under Topic#2 Issue 3, agreements in [9] under Topic#1 Issue 1 and Topic#2 Issue 7 in [9]) and a clarification was added to 3GPP TS 36.307 in [10] as shown below to indicate that requirements defined were applicable even in no satellite assistance information is provided for neighbour cells:

Proposal 1: Define ephemeris for an intra-frequency neighbour cell for NB-IoT NTN RRM test cases requiring neighbour cells for GEO satellites scenario.

As it is stated in current specifications [2] and [11], it seems that eMTC NTN requirements requiring neighbour cells are applicable to GEO satellites only for Rel-17 UEs while applicable to both GSO and NGSO for Rel-18 UEs. It could be worthy to double check with RAN4 via an LS if this is the correct understanding or just some maintenance work is missing in 3GPP TS 36.133 eMTC NTN test cases. This LS could also raise the missing satellite supported configurations for test A.14.1.1.4.

Proposal 2: Define ephemeris for an intra-frequency neighbour cell for eMTC NTN RRM test cases requiring neighbour cells for GEO satellites scenario at this stage.

Proposal 3: Send an LS to RAN4 to clarify supported test configurations (satellites scenarios) for eMTC NTN requirements involving a neighbour cell for either Rel-17 and Rel-18 UEs (highlight that such supported test configuration is totally missing in test A.14.1.1.4). If the outcome of such LS indicates that for Rel-18 UEs, the mentioned test cases are applicable to GSO and NGSO scenarios, the ephemeris for neighbour cells for GSO and NGSO will be defined in the future as part of maintenance activities.

## 2.2 RAN4 assumptions for neighbour cells

For IoT NTN RRM test cases, RAN4 agreed in [12] as part of general aspects of this work item (Issue 1-1-3) that RRM requirements apply provided that serving cell and neighbour satellites on the same layer are of same satellite type (LEO or GEO):

This agreement was leveraged from NR NTN RRM discussions as per Issue 1-9-1 in [13].

Regarding the altitude for serving and neighbour cells for NGSO satellite scenarios, it was previously agreed in [14] for NR NTN RRM Intra-NTN for both RRC Connected and Idle/Inactive modes and (new) SMTC and Measurement Gap based requirements, that same altitude will be considered for both serving and neighbour cells in this case.

Keysight’s understanding is that when leveraging satellites of the same type, same altitude for NGSO satellites is also leveraged.

Proposal 4: When defining ephemeris for neighbour cell for IoT NTN, assume that neighbour cell satellite type is same as serving cell, including its altitude.

Another important aspect that has been analysed is if RAN4 discussed whether both serving and neighbour cells are on the same satellite or in different satellites. As per [14], RAN4 considered both scenarios, i.e. serving and neighbour cells within a satellite or belonging to different satellites. Even when this discussion was held under NR NTN work item, its conclusion can be leveraged in IoT NTN. To be noticed that, according to commercial deployments, there will be scenarios including regions where more than one GEO satellites are visible.

Observation 3: RAN4 requirements/test cases considered both scenarios, i.e. serving and neighbour cells within a satellite or belonging to different satellites.

## 2.3 Criteria to define ephemeris for neighbour cell in GEO satellites scenarios

### 2.3.1 Same vs different satellites and Doppler for serving and neighbour cells in NTN

First criterion that RAN5 needs to decide upon is whether serving and neighbour cells for testing will be located in same or different satellites.

For the GEO use case, there could be not much difference in terms of Doppler or one-way-delay between 2 different GEO satellites once the UE location has been fixed. However, this might not be the case for GSO satellites (with inclination different from zero) or NGSO satellites (to be defined in the future) depending on UE location.

Test cases will have a wider coverage if they require UEs to handle 2 different satellites (with different Doppler and delays) than just one satellite while maximizing the Doppler difference between them.

Hence, in order to have a future-proof common criterion for all types of satellites and a wider test coverage, it is proposed to use different satellites for serving and neighbour cells and maximize the Doppler difference between them.

Proposal 5: Use different satellites for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells.

### 2.3.2 Elevation angle

According to 3GPP TS 36.133 [2], Annex B.8:

As it is not specified that highlighted statement applies only to serving cell, the assumption is that the minimum elevation angle relative to the UE position shall be equal or higher than 30 degrees during the entire test time for both serving and neighbour cells.

Observation 4: Minimum elevation angle shall be 30 degrees for both serving and neighbour cells in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells.

Regarding the difference in elevation angles perceived by the UE for serving and neighbour cell satellites, it could be thought that test cases coverage will be broader if the difference in elevation angles between the serving and neighbour cells can be maximized.

Proposal 6:  When defining ephemeris for GEO neighbour cells, consider satellites in different orbits so there is some difference in elevation angles for the satellite in charge of serving cell and the one in charge of the neighbour cell.

## 2.4 Serving cell ephemeris definition for GEO satellites

As described in section 1, RAN5 defined the ephemeris of the serving cell for all type of test cases in [4]. At that point, ephemeris for GSO (7º inclination) and NGSO were defined. However, there was no ephemeris defined for GEO satellites for the serving cell as required in test cases listed in Tables 1 and 2 above.

Proposal 7: Define ephemeris information for GEO satellites for serving cells in TS 36.508 clauses 7.6.2.1 and 8.4.6.2.1.

As Rel-17 is only covering the testing of static satellite conditions, GSO satellite orbits could be used to gather GEO ephemeris configuration for serving and neighbour cell choosing elevation angles higher than 30º corresponding to zero Doppler conditions while trying to maximize elevation angle difference for serving and neighbour cells and delays. Refer to Figure 1 for further details on this.

| Serving cell | Serving cell |
| --- | --- |
| (a) |  |
| Neighbour cell | Neighbour cell |
| (b) |  |
| Neighbour cell | Neighbour cell |
| (c) |  |

Figure 1. Doppler for 3 GSO satellite orbits (7º inclination) with 55 UE latitude and 121.56 UE longitude:

a) Satellite longitude equal to UE longitude for serving cell,

b) +20º shifted satellite longitude regarding UE longitude for neighbour cell

c) -20º shifted satellite longitude regarding UE longitude for neighbour cell.

Red crosses indicate GEO satellite configuration proposed.

Proposal 8: Leverage GSO orbits to gather ephemeris for serving and neighbour cell for GEO satellites (using zero Doppler conditions within the GSO orbits) as shown in Figure 1.

# 3. 	Conclusion

This document analyses the IoT NTN RRM scenarios where a neighbour cell is required and makes proposals for the definition of its ephemeris. Additionally, it makes some proposals to define the ephemeris for serving cell for GEO scenarios.

The following observations and proposals are made:

Observation 1: According to 3GPP TS 36.133 [2], for NB-IoT NTN RRM test cases requiring neighbour cells, only one neighbour intra-frequency cell is required for GEO satellites for Rel-17 UEs.

Observation 2: According to 3GPP TS 36.133 [2], for eMTC NTN RRM test cases requiring neighbour cells, only one neighbour intra-frequency cell is required for GSO and NGSO satellites.

Proposal 1: Define ephemeris for an intra-frequency neighbour cell for NB-IoT NTN RRM test cases requiring neighbour cells for GEO satellites scenario.

Proposal 2: Define ephemeris for an intra-frequency neighbour cell for eMTC NTN RRM test cases requiring neighbour cells for GEO satellites scenario at this stage.

Proposal 3: Send an LS to RAN4 to clarify supported test configurations (satellites scenarios) for eMTC NTN requirements involving a neighbour cell for either Rel-17 and Rel-18 UEs (highlight that such supported test configuration is totally missing in test A.14.1.1.4). If the outcome of such LS indicates that for Rel-18 UEs, the mentioned test cases are applicable to GSO and NGSO scenarios, the ephemeris for neighbour cells for GSO and NGSO will be defined in the future as part of maintenance activities.

Proposal 4: When defining ephemeris for neighbour cell for IoT NTN, assume that neighbour cell satellite type is same as serving cell, including its altitude.

Observation 3: RAN4 requirements/test cases considered both scenarios, i.e. serving and neighbour cells within a satellite or belonging to different satellites.

Proposal 5: Use different satellites for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells.

Proposal 6:  When defining ephemeris for GEO neighbour cells, consider satellites in different orbits so there is some difference in elevation angles for the satellite in charge of serving cell and the one in charge of the neighbour cell.

Proposal 7: Define ephemeris information for GEO satellites for serving cells in TS 36.508 clauses 7.6.2.1 and 8.4.6.2.1

Proposal 8: Leverage GSO orbits to gather ephemeris for serving and neighbour cell for GEO satellites (using zero Doppler conditions within the GSO orbits) as shown in Figure 1.

These proposals are submitted for approval in CR ([15-16]).

# 4. 	References

- RP-223437, “WID on NB-IoT/eMTC core & performance requirements for NTN”, MediaTek Inc., RAN#98-e.

- 3GPP TS 36.133 v18.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”.

- RP-223064, “WID on UE Conformance - NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN)”, CMCC, MediaTek Inc., RAN#98-e.

- R5-237936, “Common test environment for IoT NTN RF/RRM testing”, Keysight UK Ltd, Thales, MediaTek, Qualcomm.

- R5-236642, “WP Rel-18 NB-IoT/eMTC NTN after RAN#101”, CMCC, RAN5#101.

- R4-2300014, “Reply LS on information for neighbor/target cell in IoT NTN”, RAN2.

- R4-2217265, “LS on information for neighbor/target cell in IoT NTN”, RAN4.

- R4-2303311, “WF on NB-IoT/eMTC RRM requirements for NTN”, MediaTek inc., RAN4#106.

- R4-2306370, “WF on NB-IoT/eMTC RRM requirements for NTN “, MediaTek inc., RAN4#106bis-e.

- R4-2306372, “CR to TS 36.307 (Rel-17): release independence RRM requirements for IoT NTN”, Sony, MediaTek inc., RAN4#106bis.

- 3GPP TS 36.307, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) Supporting a release-independent frequency band”.

- R4-2214350, “WF on LTE IoT NTN RRM requirements”, MediaTek inc., RAN4#104-e.

- R4-2210610, “WF on NR NTN RRM requirements”, Qualcomm Incorporated, RAN4#103-e.

- R4-2103682, “WF on NTN RRM measurement requirements”, Qualcomm Incorporated, RAN4#98-e.

- R5-241382, “Updates to Test environment for eMTC NTN RRM testing”, Keysight UK Ltd, RAN5#102.

- R5-241383, “Updates to Test environment for NB-IoT NTN RRM testing”, Keysight UK Ltd, RAN5#102.