3GPP TSG-RAN5 Meeting # 102	R5-241384r2

# Athens, Greece, 26th February – 3rd March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0015 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL RMCs updates for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UL scheduling patterns described in notes below Table A.2.4-1 are not viable considering the long RTT delay introduced in NTN test scenario. Typically, for GEO scenario, the RTT delay would be hundreds of milli-seconds, the gateway has to wait for the NPUSCH it scheduled by the first UL grant (at least the RTT delay) before it can send another UL grant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete notes below Table A.2.4-1 (not leveraged from core specifications but from legacy NB-IoT) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on RAN4 R4-2402907. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - Clause renumbered - References updated - Extended Overview of UL reference measurement channels (eMTC NTN) - Added Overview of UL reference measurement channels (HD-FDD, NB-IoT NTN) - RAN4 dependency added to the cover page Revision 2: - Table A.2.2.2.1-1 and Table A.2.2.2.2-1 have been corrected to remove non applicable channel bandwidths and not applicable RMC. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

# A.2	UL reference measurement channels

## A.2.1	General

### A.2.1.1	Applicability and common parameters

The following sections define the UL signal applicable to the Transmitter Characteristics (clause 6) and for the Receiver Characteristics (clause 7) where the UL signal is relevant.

The Reference channels in this section assume transmission of PUSCH and Demodulation Reference signal only. The following conditions apply:

-	1 HARQ transmission

-	Cyclic Prefix normal

-	PUSCH hopping off

-	Link adaptation off

-	Demodulation Reference signal as per TS 36.211 [3] clause 5.5.2.1.2.

Where ACK/NACK is transmitted, it is assumed to be multiplexed on PUSCH as per TS 36.212 [19] subclause 5.2.2.6.

-	ACK/NACK 1 bit

-	ACK/NACK mapping adjacent to Demodulation Reference symbol

-	ACK/NACK resources punctured into data

-	Max number of resources for ACK/NACK: 4 SC-FDMA symbols per subframe

-	No CQI transmitted, no RI transmitted

### A.2.1.2	Determination of payload size

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB:

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a)	A is a valid TB size according to clause 7.1.7 of TS 36.213 [20] assuming an allocation of NRB resource blocks.

b)	C is the number of Code Blocks calculated according to section 5.1.2 of TS 36.212 [19].

c)	For RMC-s, which at the nominal target coding rate do not cover all the possible UE categories for the given modulation, reduce the target coding rate gradually (within the same modulation), until the maximal possible number of UE categories is covered.

3.	If there is more than one A that minimises the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

### A.2.1.3	Overview of UL reference measurement channels

In Table A.2.1.3-1 are listed the UL reference measurement channels specified in Annexes A.2.2 and A.2.3 of this release of TS 36.521-4. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for testing are annexes A.2.2 and A.2.3 as appropriate.

Table A.2.1.3-1: Overview of UL reference measurement channels

| Duplex | Table | Name |  | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD, Full RB allocation, QPSK |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.2.2.1.1-1 |  |  | 1.4 | QPSK | 1/3 | 6 |  | M1 |  |
| FDD, Full RB allocation, 16-QAM |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.2.2.1.2-1 |  |  | 1.4 | 16QAM | 1/3 | 6 |  | M1 |  |
| FDD, Partial RB allocation, QPSK |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 2 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 3 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 4 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 5 |  | M1 |  |
| FDD, Partial RB allocation, 16-QAM |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 1/2 | 1 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 1/2 | 2 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 1/2 | 3 |  | M1 |  |
| FDD / HD-FDD |  | Table A.2.2.2.2-1 |  | 1.4 |  | 16QAM | 2/5 | 4 |  | M1 |  |
| FDD, SubPRB allocation |  |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.4.2.3-1 |  | 1.4 |  | π/2 BPSK | 1/3 | 1 |  | M1 | 2 out of 3 subcarriers |
| FDD / HD-FDD |  | Table A.4.2.3-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 | 3 subcarriers |
| FDD / HD-FDD |  | Table A.4.2.3-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 | 6 subcarriers |

Table A.2.1.3-2: Overview of UL reference measurement channels (HD-FDD, NB-IoT)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HD-FDD | Table A.2.3-1 |  | 0.2 | π/2 BPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.2.3-1 |  | 0.2 | π/4 QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.2.3-1 |  | 0.2 | π/2 BPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.2.3-1 |  | 0.2 | π/4 QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.2.3-1 |  | 0.2 | QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.2.3-1 |  | 0.2 | QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.2.3-1 |  | 0.2 | QPSK | 1/3 | 1 |  | NB1 |  |

## A.2.2	Reference measurement channels for FDD

### A.2.2.1	Full RB allocation

#### A.2.2.1.1	QPSK

Table A.2.2.1.1-1: Reference Channels for QPSK with full/maximum RB allocation for UE category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. is total number of absolute subframes a PUSCH with repetition spans [4]. NOTE 3: For HD-FDD UE with , MPDCCH are scheduled at 0th DL subframe every +5 subframes (starting from the 0th subframe). The associated PUSCH is scheduled at the 4th to (+3)-th UL subframes every +5 subframes. Information bit payload is available if uplink subframe is scheduled. |  |  |

#### A.2.2.1.2	16-QAM

Table A.2.2.1.2-1: Reference Channels for 16-QAM with maximum RB allocation for UE category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | 16QAM |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 872 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 2880 |
| Total symbols per Sub-Frame |  | 720 |
| UE Category |  | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |

### A.2.2.2	Partial RB allocation

For each channel bandwidth, various partial RB allocations are specified. The number of allocated RBs is chosen according to values specified in the Tx and Rx requirements. The single allocated RB case is included.

The allocated RBs are contiguous and start from one end of the channel bandwidth. A single allocated RB is at one end of the channel bandwidth.

#### A.2.2.2.1	QPSK

Table A.2.2.2.1-1: Reference Channels for QK with partial RB allocation for UE category M1

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | M1 |
|  | 1.4 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | M1 |
|  | 1.4 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | M1 |
|  | 1.4 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | M1 |
|  | 1.4 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | M1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: . For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.2	16-QAM

Table A.2.2.2.2-1: Reference Channels for 16-QAM with partial RB allocation for UE category M1

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 | 1 | 12 | 16QAM | 1/2 | 256 | 24 | 1 | 576 | 144 | M1 |
|  | 1.4 | 2 | 12 | 16QAM | 1/2 | 552 | 24 | 1 | 1152 | 288 | M1 |
|  | 1.4 | 3 | 12 | 16QAM | 1/2 | 840 | 24 | 1 | 1728 | 432 | M1 |
|  | 1.4 | 4 | 12 | 16QAM | 2/5 | 904 | 24 | 1 | 2304 | 576 | M1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.3	subPRB allocation

The location of allocated RB for subPRB allocation is chosen according to values specified in the Tx requirements.

Table A.2.2.3-1: Reference Channels for SubPRB allocation

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 |
| Allocated resource blocks |  | 1 | 1 | 1 |
| Number of subcarriers |  | 2 out of 3 | 3 | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 |
| Modulation |  | π/2 BPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 32 | 72 | 72 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Number of code blocks |  | 1 | 1 | 1 |
| Total number of bits per resource unit | Bits | 192 | 288 | 288 |
| Total symbols per resource unit |  | 192 | 144 | 144 |
| Tx time | ms | 8 | 4 | 2 |
| UE UL Category |  | M1 | M1 | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |

## A.2.3	Reference measurement channels for category NB1

Table A.2.3-1 Reference Channels for category NB1

| Parameter | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 3.75 | 3.75 | 15 | 15 | 15 | 15 | 15 |
| Number of tone | 1 | 1 | 1 | 1 | 3 | 6 | 12 |
| Modulation | π/2 BPSK | π/4 QPSK | π/2 BPSK | π/4 QPSK | QPSK | QPSK | QPSK |
| Number of NPUSCH repetition (NOTE 5) | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| IMCS / ITBS | 0 / 0 | 3 / 3 | 0 / 0 | 3 / 3 | 5 / 5 | 5 / 5 | 5 / 5 |
| Payload size (bits) | 32 | 40 | 32 | 40 | 72 | 72 | 72 |
| Allocated resource unit | 2 | 1 | 2 | 1 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.33 | 0.29 | 0.33 | 0.33 | 0.33 | 0.33 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 192 | 96 | 192 | 288 | 288 | 288 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 144 | 144 | 144 |
| Tx time (ms) | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: Parameters related to NPUSCH format 1 scheduling are defined in Table A.2.4-2. NOTE 3: NPDCCH is not transmitted in the subframes used for transmission of SI messages. NOTE 4: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. NOTE 5: Number of repetition NRep as defined in table 16.5.1.1-3 in TS 36.213 [20]. |  |  |  |  |  |  |  |

Table A.2.3-2: NPDCCH configuration for NPUSCH format 1 scheduling

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N0 |
| NPDCCH format |  | 1 |
| Scheduling delay () |  | 0 |
| DCI subframe repetition number |  | 00 |
| (npdcch-NumRepetitions) |  | 1 |
| G (NPDCCH-startSF-USS) |  | 8 |
| (npdcch-Offset-USS) |  | 1/4 |

# A.3	DL reference measurement channels

## <<< END OF CHANGES 1>>>