3GPP TSG-RAN5 Meeting #102	R5-241411r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0337 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating applicability for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Some RedCap devices might have a different form factor than regular devices therefore they might not have a keyboard to type short messages or a display to receive short messages. - As per 26.114 support of video codec is only required if the device supports video but at the moment the status is “m”. Additionally requirements for “constrained terminals” (many RedCap devices will be such terminals) is different: “ Constrained terminal: UE that is (i) operating in radio access capability category series "M" capable of supporting conversational services, and/or (ii) a wearable device which is constrained in size, weight or power consumption (e.g. connected watches), excluding smartphones and feature phones. ... MTSI clients in terminals offering speech communication shall support narrowband, wideband and super-wideband communication. The only exception to this requirement is for the MTSI client in constrained terminal offering speech communication, in which case the MTSI client in constrained terminal shall support narrowband and wideband, and should support super-wideband communication. ... MTSI clients in terminals offering video communication shall support: - H.264 (AVC) [24] Constrained Baseline Profile (CBP) Level 1.2; - H.265 (HEVC) [119] Main Profile, Main Tier, Level 3.1. The only exception to this requirement is for the MTSI client in constrained terminal offering video communication, in which case the MTSI client in constrained terminal should support H.265 (HEVC) Main Profile, Main Tier, Level 3.1. In addition they should support: - H.264 (AVC) [24] Constrained High Profile (CHP) Level 3.1. For backwards compatibility to previous releases, if H.264 (AVC) [24] Constrained High Profile Level 3.1 is supported, then H.264 (AVC) [24] Constrained Baseline Profile (CBP) Level 3.1 should also be offered.” - There was a note added to NG.114/video as a feature but no similar note was added to other Video fearue tag |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update status of SM-over-IP sender and SM-over-IP receiver for RedCap devices - Corrected status for video codec to align it with TS 26.114. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Status for RedCap devices will not reflect market requirements - Status for 26.114 codecs will be incorrect. - Adding note to the video feature tag |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.1, A.4.5, A.4.7, A.4.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-241411->R5-241411r1 - added NOTE 1 to video feature tag |  |  |  |  |  |  |  |  |

### A.4.1	Roles

Table A.2: Roles

| Item | UE roles | Ref. | Status | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | User agent | 24.229 [10], A.1.3 RFC 3261 [15] | m | Rel-5 |  |

Table A.3A: UE roles specific to additional capabilities

| Item | UE roles | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | Multimedia telephony service participant | 24.173 [55] | o | Rel-8 | pc_MultimediaTelephonyService |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 61 | SM-over-IP sender | 24.341 [66] | o | Rel-8 | pc_SMS_IP_MO |  |
|  |  | IR.92 [83], 2.5 | m |  |  |  |
|  |  | NG.114 [96] NOTE 1 | m | Rel-15 |  |  |
| 62 | SM-over-IP receiver | 24.341 [66] | o | Rel-8 | pc_SMS_IP_MT |  |
|  |  | IR.92 [83], 2.5 | m |  |  |  |
|  |  | NG.114 [96] NOTE 1 | m | Rel-15 |  |  |
| 63 | Void |  |  |  |  |  |
| 64 | Concatenated SM-over-IP sender | 23.040 [94], 9.2.3.24.1 | o | Rel-8 | pc_Concatenated_SMS_IP_MO |  |
| 65 | Concatenated SM-over-IP receiver | 23.040 [94], 9.2.3.24.1 | o | Rel-8 | pc_Concatenated_SMS_IP_MT |  |
| NOTE 1: As an exception to NG.114 support of SM-over-IP sender/Receiver is optional for RedCap devices. |  |  |  |  |  |  |

[SECTION SKIPPED]

### A.4.5	Additional information

Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |  |
| 2 | Void |  |  |  |  |  |
| 3 | Void |  |  |  |  |  |
| 4 | UE capable of being configured to initiate Dedicated PDP Context | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiatesDedicatedPDPContext |  |
| 5 | UE capable of being configured to initiate P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaPCO |  |
| 6 | Void |  |  |  |  |  |
| 7 | UE capable of being configured to initiate P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv6 |  |
| 8 | UE supports P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaPCO |  |
| 9 | Void |  |  |  |  |  |
| 10 | UE supports P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv6 |  |
| 11 | Void |  |  |  |  |  |
| 12 | UE capable of initiating a bidirectional voice session over IMS | 24.229 [10], 5.1.6 | o | Rel-5 | pc_BidirecVoiceOverIMS |  |
| 13 | Void |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |
| 15 | Void |  |  |  |  |  |
| 16 | UE Supports " IPv6 address with embedded IPv4 address" in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 17 | UE Supports IPv4 address in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 18 | Void |  |  |  |  |  |
| 19 | UE supports UI capable of showing user notification for Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 |  |  |
| 20 | Void |  |  |  |  |  |
| 21 | Void |  |  |  |  |  |
| 22 | Void |  |  |  |  |  |
| 23 | Void |  |  |  |  |  |
| 24 | UE supports no reply timer setting | 24.604 [68], 4.9.1.4 | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.3.8 | m |  |  |  |
| 25 | UE supports sending DTMF events over RTP | 26.114 [56], Annex G | o | Rel-7 |  |  |
|  |  | IR.92 [83], 3.3 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 26 | Void |  |  |  |  |  |
| 27 | UE is capable of obtain ing location information | 24.229 [10] 4.7, 36.509 [79] 4.1, 5.5.2, 34.109 [78] 5.4.2 | o | Rel-9 | pc_IMS_Geolocation |  |
| 28 | Void |  |  |  |  |  |
| 29 | UE supports emergency speech call over 1xRTT | C.S0005-E [74] | o |  | pc_CS_Em_Call_in_1xRTT |  |
| 30 | Void |  |  |  |  |  |
| 31 | UE supports end-to-access-edge media security using SDES | 24.229 [10], 4.2.B.2 | o | Rel-9 |  |  |
| 32 | UE supports video media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | o | Rel-13 | pc_IMS_Video_FeatureTag |  |
|  |  | IR.94 [75], 2.2 | m |  |  |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 33 | UE supports Communication Hold during emergency call | 23.167 [76], 7.1.1 | o | Rel-9 | pc_CommunicationHold_DuringEmergencyCall |  |
| 34 | UE indicates g.3gpp.srvcc-alerting media feature tag in INVITE request or 180 (Ringing) response | 24.237 [77], 12.2.3B.1 | o | Rel-10 | pc_IMS_SRVCCAlert |  |
| 35 | Void |  |  |  |  |  |
| 36 | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request | 24.237 [77] 6A.2.2.2 | o | Rel-12 | pc_BSRVCC |  |
| 37 | UE indicates OMA-TS-XDM_MO-V1_1-20080627-A.doc, section 5.2.8 "Node: /<X>/ AAUTHNAME" is configured | 24.623 [81], Annex B.2 | o | Rel-8 | pc_XCAP_UsernameIsConfiguredInUE |  |
| 38 | The UE use the default public user identity received in P-Associated -URI header in 200 OK for REGISTER as XCAP User Identity (XUI) | IR.92 [83] | m |  | pc_XCAP_XUIisDefaultPublicUserId |  |
|  |  | 24.623 [81] | o | Rel-8 |  |  |
| 39 | UE has the method that support IMS deregistration | 24.229 [10], 5.1.1.6 | o | Rel-5 | pc_IMS_Deregistration |  |
| 40 | UE supports Cs to PS SRVCC | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCC |  |
| 41 | UE supports Cs to PS SRVCC in alerting state | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCAlert |  |
| 42 | UE supports Cs to PS SRVCC and the MSC server assisted mid-call feature | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCMidCall |  |
| 43 | Void |  |  |  |  |  |
| 44 | Void |  |  |  |  |  |
| 45 | UE supports early media | 24.229 [10], 5.1.3 | o | Rel-8 | pc_EarlyMedia |  |
| 46 | UE indicates g.3gpp.accesstype media feature tag in REGISTER | 24.237 [77], 6.2.2 | c1 | Rel-11 |  |  |
| 47 | Void |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |
| 49 | The UE uses XCAP without PDN connection in WLAN | IR.51 [84], 4.6 | o | Rel-11 | pc_WLAN_XCAP_without_PDN | If PICS is true, Wi-Fi access without PDN connection applies. Otherwise, EPC-integrated WLAN applies |
| 50 | The UE uses XCAP in fixed broadband access |  | o | Rel-12 | pc_fixed_broadband_XCAP | In the context of the present specification, a UE supporting fixed broadband, A.18/3, shall set this PICS to true. |
| 51 | The UE uses same public user Identity for From/To as in initial REGISTER | 24.229 [10] | o | Rel-8 | pc_Same_IMPU |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 52 | UE reattempts IMS emergency service without registration after rejection of emergency registration. (For Rel-14, this is applicable only when the rejection indicates an anonymous emergency call support) | 23.167 [76], 4.1 | o | Rel-9 | pc_IMS_after_EmReg_rejection |  |
|  |  |  | m | Rel-14 |  |  |
| 53 | Void |  |  |  |  |  |
| 54 | UE supports IMS eCall type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Only |  |
| 56 | UE supports audio media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | m | Rel-13 | pc_IMS_Audio_FeatureTag |  |
|  |  | IR.92 [83], 2.2.1, 2.2.4 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 57 | UE supports Session Timer | 24.229 [10], 5.1.2A, 5.2.7.2, 5.2.7.3, 5.4.5.3 | o | Rel-6 | pc_IMS_Session_Timer |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 58 | UE supports enabling/disabling SMS over IMS | 27.007 [97], 8.72 | o | Rel-15 | pc_Enable_Disable_SMSoverIMS |  |
| 59 | UE supports disabling preconditions | 24.229 [10], 5.1.5A | o | Rel-15 | pc_IMS_Disable_Preconditions |  |
| 60 | UE uses Session Timer for its benefit alone | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_own_benefit |  |
|  |  | NG.114 [96] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 61 | UE supports usage of re-INVITE for session refresh | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_reINVITE |  |
|  |  | NG.114 [96] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 62 | Void |  |  |  |  |  |
| 63 | UE supports IMS eCall type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Capable |  |
| 64 | UE supports IMS eCall Only type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Only |  |
| 65 | UE supports IMS emergency call over WLAN | 24.229 [10], 5.1.6, R.2.2.6 | o | Rel-13 | pc_WLAN_IMS_EmergencyCall |  |
| Conditions/Options |  |  |  |  |  |  |
| c1: IF [73] A.4.4-1/32 THEN m ELSE o - - SC UE indicates accesstype. |  |  |  |  |  |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. |  |  |  |  |  |  |

[SECTION SKIPPED]

### A.4.7	MTSI media

Table A.15: MTSI media

| Item | Media | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Speech | 26.114 [56], 5.2.1 | o | Rel-7 | pc_MTSI_Speech |  |
|  |  | IR.92 [83], 3.2 | m |  |  |  |
|  |  | IR 51 [84], 3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 2 | Speech, AMR wideband | 26.114 [56], 5.2.1 | o | Rel-7 | pc_MTSI_Speech_AMRWB |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
|  |  | IR.92 [83], 3.2 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 3 | Video | 26.114 [56], 5.2.2 | o | Rel-7 | pc_MTSI_Video |  |
|  |  | IR.94 [75], 3.3 | m |  |  |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 4 | Video, H.263 Profile 3 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 5 | Video, MPEG-4 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 6 | Video, H.264 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 7 | Void |  |  |  |  |  |
| 8 | Void |  |  |  |  |  |
| 9 | Video codec H.264 CBP Level 1.2 | 26.114 [56], 5.2.2 | o | Rel-8 | pc_VideoCodecH264CBP |  |
|  |  | IR.94 [75], 3.3 | m |  |  |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
| 10 | Speech, EVS | 26.114 [56] | o | Rel-12 | pc_MTSI_Speech_EVS |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 11 | Video codec H.265 (HEVC), Main Profile, Main Tier, Level 3.1 | 26.114 [56], 5.2.2 | c1 | Rel-15 | pc_VideoCodecH265Lv31 |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 12 | Video codec H.264 (AVC) CHP Level 3.1 | 26.114 [56], 5.2.2 | o | Rel-15 | pc_VideoCodecH264CHPLv31 |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 13 | Video codec H.264 (AVC) CBP Level 3.1 | 26.114 [56], 5.2.2 | o | Rel-15 | pc_VideoCodecH264CBPLv31 |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. c1: IF NOT A.15/3 OR [93]A.4.3.12-1/2 THEN o ELSE m |  |  |  |  |  |  |

[SECTION SKIPPED]

### A.4.14	NG.114 capabilities

Table A.22: NG.114 capabilities

| Item |  | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | mmtel ICSI value | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_mmtel |  |
| 2 | audio feature tag | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_audio |  |
| 3 | video feature tag | NG.114 v1.0 [96] | m NOTE 1 | n/a | pc_NG114_v1_0_video |  |
| 4 | Void |  |  |  |  |  |
| 5 | Void |  |  |  |  |  |
| 6 | Void |  |  |  |  |  |
| 7 | Void |  |  |  |  |  |
| 8 | Annex C.3 default configuration for voice exempt | NG.114 v2.0 [98] | m | n/a | pc_NG114_v1_0_default_config_voice_exempt |  |
| 9 | Annex C.3 default configuration for video exempt | NG.114 v2.0 [98] | m | n/a | pc_NG114_v2_0_default_config_video_exempt |  |
| 10 | Annex C.3 default configuration for EVS/Br | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_default_config_evs_br |  |
| 11 | Annex C.3 default configuration for EVS/Bw | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_default_config_evs_bw |  |
| 12 | Void |  |  |  |  |  |
| 13 | Void |  |  |  |  |  |
| 14 | Forking behavior | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_forking |  |
| 15 | EVS Configuration B0 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_B0 |  |
| 16 | EVS Configuration B1 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_B1 |  |
| 17 | EVS Configuration B2 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_B2 |  |
| 18 | EVS Configuration A1 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_A1 |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. |  |  |  |  |  |  |