3GPP TSG-RAN WG5 Meeting #102                                                       R5-241412r1

# Athens, Greece, 26th Feb – 1st Mar 2024

Source:	MediaTek Inc.

Title:		Discussion paper for IMS test cases over Redcap Only UE

Agenda item:	6.3.6.7

# Document for: Discussion and Decision

Agenda Item:

## Introduction

## At RAN5#100 [1] “Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE” was agreed that introduces the way forward on how to handle Rel-16 and Rel-17 protocol (TS 38.523-1) legacy test cases applicability to Redcap UEs.

At RAN5#101 [2] “Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE” has been agreed that all Rel-17 test cases should be upgraded and only a date needs to be decided.

Rel-15 IMS test cases have been already updated to run on RedCap devices (there is no NOTE stating otherwise is added to TS 34.229-2).

## Discussion

Given the different form factors used by different RedCap devices sending and/or receiving short messages might not be possible.

At the moment support of these features is mandated by NG.114 as per TS 34.29-2 Table A.3A:

“

Table A.3A: UE roles specific to additional capabilities

| Item | UE roles | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | Multimedia telephony service participant | 24.173 [55] | o | Rel-8 | pc_MultimediaTelephonyService |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 61 | SM-over-IP sender | 24.341 [66] | o | Rel-8 | pc_SMS_IP_MO |  |
|  |  | IR.92 [83], 2.5 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 62 | SM-over-IP receiver | 24.341 [66] | o | Rel-8 | pc_SMS_IP_MT |  |
|  |  | IR.92 [83], 2.5 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |

“

## 3.	Proposal

Proposal 1:	Clarify that SM-over-IP sender/receiver is optional for RedCap devices although NG.114 mandates it.

## 4.	References

[1]	R5-235348: Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE

[2]	R5-237399: Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE

[3]	R5-241411: Updating applicability for RedCap