3GPP TSG-RAN5 Meeting #102	R5-241422r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.561 | CR | 0004 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TT within TRP and TRS tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_FR1_TRP_TRS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TT is not yet defined for the TRP and TRS tests and is pending completion |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test requirements with TT values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Tolerance will be missing in the test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.1, 7.2.1.1.1, Annex A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with Discussion Paper R5-241423 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 correctly adds a new normative Annex to capture the Maximum Test System Uncertainty and Test Tolerances. Annex structure is updated accordingly. |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

Table 6.2.1.1.1.5.1-1	Handheld PC2 UE TRP test requirements for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n28 | 20 | HL and HR |  |  |
| n41 | 100 | HL and HR |  | 12.5-TT |
| n78 | 100 | HL and HR |  | 13-TT |
| n79 | 100 | HL and HR |  |  |

Table 6.2.1.1.1.3.1-3: Test Tolerance (NR FR1 TRP)

| Operating band | Test Tolerance (dB) |
| --- | --- |
| n41 | 1.02 |
| n78 | 1.07 |

<Unchanged sections skipped>

Table 7.2.1.1.1.5-3: Test Tolerance (NR FR1 TRS)

| Operating band | Test Tolerance (dB) |
| --- | --- |
| n41 | 1.28 |
| n78 | 1.32 |

<Unchanged sections skipped>

## C.2.2	Voltage

All test cases shall be performed with the DUT operated in stand-alone battery powered mode. It is preferable if the UE is fully charged in the beginning of the test.

Annex D (normative): Maximum uncertainty of test system and test tolerance

# D.1	Maximum uncertainty of test system

- The maximum acceptable uncertainty of the Test System is specified in Table D.1-1 for each test, where appropriate. The Test System shall enable each test to be measured with an uncertainty not exceeding the specified values. All ranges and uncertainties are absolute values and are valid for a confidence level of 95 %.

- In Annex A.4, the estimation of measurement uncertainty is defined.

- Table D.1-1: Maximum Test System uncertainties for for TRP hand only (browsing mode) tests using anechoic chamber method for NR FR1 bands

| - Clause | - Maximum Test System Uncertainty | - Derivation of Test System Uncertainty |
| --- | --- | --- |
| - 6.2.1.1.1 Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | - 1.64 dB : < 3 GHz - 1.73 dB >= 3 GHz | - Refer to Annex A.4 |
| - 7.2.1.1.1 Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | - 2.06 dB : < 3 GHz - 2.13 dB >= 3 GHz | - Refer to Annex A.4 |

### D.2	Test Tolerances

### Test tolerances in Table D.2-1 are used to relax the Minimum Requirements in the present document and to derive the Test Requirements.

- Table D.2-1: Test Tolerances for TRP hand only (browsing mode) tests using anechoic chamber method for NR FR1 bands

| - Clause | - Test Tolerance |
| --- | --- |
| - 6.2.1.1.1 Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | 1.02 dB : < 3 GHz 1.07 dB >= 3 GHz |
| - 7.2.1.1.1 Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | 1.28 dB : < 3 GHz - 1.32 dB >= 3 GHz |

Annex E (informative): Change History

<Unchanged sections skipped>