Title:	NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA

Source:	CATT

# 1. Introduction

This document relates to NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in TS 37.571-1. The test cases are:

- 14.3.3, NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA

- 14.3.8, NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v16.19.0.

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer

<< Some clauses skipped >>

Table A.7.7.10.1.1-2: RSTD accuracy test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 200: NRB,c = 132 |  | 200: NRB,c = 132 |  |
| Measurement gap |  | GP#24 or GP#13 |  | GP#24 or GP#13 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| PRS configuration |  | PRS.1.1 FR2 | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 |
| PRS Resource slot offset | slot | 0 | 4 | 0 | 4 |
| Expected RSTD | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | s | N/A | 5 | N/A | 5 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| EPRE ratio of PRS to SSS | dB | 0 | 0 | 0 | 0 |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. |  |  |  |  |  |

Table A.7.7.10.1.1-3: RSTD accuracy OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 5 |  | Rough |  | Rough |  |
| Note1 | dBm/SCSNote3 | -89 |  | -89 |  |
| PRS | dB | -5.7 | -11.9 | -5.7 | -11.9 |
| PRPNote2 | dBm/SCS | -94.7 | -100.9 | -94.7 | -100.9 |
| PRS | dB | -6 | -13 | -6 | -13 |
| IoNote2 | dBm/190.08 MHz Note3 | -55.75 | -55.75 | -55.75 | -55.75 |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PRP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

For RSTD, the core requirements for RSTD Measurements are specified in TS 38.133 clause 10.1.23, with side conditions in Annex B.2.14.

# 3. Discussion

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR2.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in TS 38.133 clause 9.9.2.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are selectively copied from Table A.7.7.10.1.1-2 and Table A.7.7.10.1.1-3 in TS 38.133, which are subcarrier spacing, channel bandwidth, RBs in channel bandwidth, Noc, Cell 1 PRS Es/Noc and Cell 2 PRS Es/Noc. All other parameters such as Io, PRS Es/Iot and PRS-RSRP are derived, and not independently settable by the Test Equipment.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1.

c) Uncertainties

The SS provides AWGN and 2 intra-frequency cells to the UE under AWGN conditions in FR2 in standalone scenario. We propose to control the following parameters:

- AWGN absolute power of RF channel 2, Noc averaged over BWConfig ±5.65 dB

- AWGN absolute power of RF channel 2, Noc averaged over measurement PRBs ±5.65 dB

- Ratio of cell 1 PRS signal / AWGN, PRS Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 1 PRS signal / AWGN Es1/Noc averaged over measurement PRBs ±0.3 dB

- Ratio of cell 2 PRS signal / AWGN, PRS Ês2 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 2 PRS signal / AWGN Es2/Noc averaged over measurement PRBs ±0.3 dB

- LPP Response Time, 0.3 seconds

The LPP Response Time uncertainty has been chosen to be in-line with all other LPP test cases defined in TS 37.571-1 and there is no necessity to derive the test tolerance in the spreadsheet. LPP Response Time is as defined in TS 37.571-1 clause 4.15.3.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1 Each Angle of Arrival is considered separately

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

The stimulus requirements defined in TS 38.133 clause 10.1.23 are reproduced below. The stimulus requirements defined in TS 38.133 clause 10.1.23 indicate that the NR RSTD measurement accuracy for all measured cells and the Io range.

<< Extracts from TS 38.133 v16.19.0 >>

10.1.23	RSTD Measurements

<< Some text skipped >>

10.1.23.2	Measurement Accuracy Requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y+Z) Tc.

X is defined in Table 10.1.23.2-1 for AWGN channel and Table 10.1.23.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

X is defined in Table 10.1.23.2-2 for AWGN channel and Table 10.1.23.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

Note: The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

When UE measures RSTD on PRS resources belonging different PFLs, Y=256 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 1280 ms.

Z is defined in Table 10.1.23.2-5 for FR1 and Table 10.1.23.2-6 for FR2, respectively.

<< Some text skipped >>

Table 10.1.23.2-2: RSTD absolute accuracy in FR2 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 35 +ΔNote 6 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| 24 +Δ |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 11 +Δ |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 24 +Δ |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| 13 +Δ |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 6 +Δ |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ= 0 for single PFL, Δis defined in Table 10.1.23.2-6a for dual PFL. |  |  |  |  |  |  |

<< Some text skipped >>

Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 72 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 16 |
| N/A | ≥ 128 | 12 |

Table 10.1.23.2-6a: Margin Δ for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 32 |
| ≥ 64 | ≥ 32 | 16 |
| ≥ 132 | ≥ 64 | 8 |
| N/A | ≥ 128 | 4 |

The side conditions are defined in TS 38.133 Annex B.2.14 and are reproduced below.

B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

<< Some text skipped >>

Table B.2.14-2: Conditions for NR PRS-based measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum PRP1,2 Note 2, Note 3 |  |  |  |  | PRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSPRS |  |  |  |  | dB |
|  |  |  | SCSPRS = 120 kHz |  |  |  | SCSPRS = 60 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 Note4 -13 Note5 -3 Note6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 Note4 -13 Note5 -3 Note6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum PRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. NOTE 4: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement and UE Rx-Tx time difference measurement. NOTE 6: PRS Ês/Iot for PRS-RSRP measurement and UE Rx-Tx time difference measurement. |  |  |  |  |  |  |  |  |

Band n259 has the tighest PRP requirement, which is -108.5 dBm/120kHz for UE PC3 including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

And UE gain G variation is defined in 38.133 Annex B.2.1.6:

B.2.1.6	Gain to PRS-RSRP measurement point for FR2

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.28 of TS 38.215 [4] PRS-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.6.1-1: Gain and Reference point for requirement parameters

The gain range for each power class is specified in Table B.2.1.61-1.

Table B.2.1.6.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Minimum, dBi | FFS | FFS | -10 | FFS |
| Maximum, dBi | FFS | FFS | +20 | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

So the UE gain variation can be expressed as (maximum UE gain - minimum UE gain)/2 according to 38.903 annex A, For UE PC3, it's 15dB.

In summary, the following conditions shall be satisfied:

-	PRS Ês/Iot of Cell 1 at baseband shall be no less than -6dB.

-	PRS Ês/Iot of Cell 2 at baseband shall be no less than -13dB,

-	PRP of Cell 1 shall be no less than -108.5 dBm/120kHz, including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

-	PRP of Cell 2 shall be no less than -108.5 dBm/120kHz, including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

-	Io shall be no less than -108.5dBm/Ch BW and no larger than -50dBm/Ch BW.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE reporting delay (RSTD measurements)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For Cell 1, they are the PRS Es/Iot range and the PRP power range for the reference cell over which the UE is expected to provide RSTD. For Cell 2, they are the PRS Es/Iot range and the PRP power range for the neighbour cell over which the UE is expected to provide RSTD measurements. The Io power shall also be within the range over which the UE is expected to provide RSTD measurements.

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 PRP, so the sensitivity factor is 1.000 for all tests. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases the sensitivity factor value between 0 and 1 result. It is important to calculate these correctly to obtain the overall uncertainty.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

We observe that the PRS Es/Iot of Cell 1 and PRS Es/Iot of Cell 2 are outside the allowed range, so the PRS Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve Cell 1 PRS Es/Iot >= -6dB. PRS Es/Noc of Cell 2 is therefore increased by an amount sufficient to achieve Cell 2 PRS Es/Iot >= -13dB. For Noc is decreased by an amount sufficient to achieve Io<=-50 dBm/Ch BW.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances.

The PRS Es/Noc of Cell 1 is increased by an amount sufficient to achieve PRS Es/Iot >=-6dB. The actual offset required is 0.53dB for cell 1.

The PRS Es/Noc of Cell 2 is increased by an amount sufficient to achieve PRS Es/Iot >=-13dB. The actual offset required is 0.54dB for cell 2.

For Noc is decreased by an amount sufficient to achieve Io<=-50 dBm/Ch BW. The actual offset required is -0.1dB.

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

# 5. Treatment of other configurations within the same test case

TS 37.571-1 test case 14.3.3 also contains other sub-test with different PRS configurations. The calculations use the same values, a separate spreadsheet tab is provided.

# 6. Treatment of other test cases 14.3.8

TS 37.571-1 also contains similar test case 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA with different PRS configurations. The calculations use the same values, a separate spreadsheet tab is not provided.