Title:	UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA

Source:	CATT

# 1. Introduction

This document relates to UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA in TS 37.571-1. The test case is:

- 15.2.4, UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v16.19.0.

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA

<< Some clauses skipped >>

Table A.7.6.11.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1 | 1 | For Cell 1 |
| RF Channel Number |  | 1 | 2 | For Cell 2 |
| BWchannel | MHz | 1 | 200: NRB,c = 132 |  |
| SSB configuration |  | 1 | SSB.2 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 1 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 | Synchronous cells |
| Expected RSTD | s | 1 | 3 |  |
| Expected RSTD uncertainty | s | 1 | 5 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 20 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |  |

Table A.7.6.11.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  | PRS.1.1 FR2 |  |
| PRS muting info |  | 1 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.3 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -92 | -Infinity | -102 |
| Io | dBm/190.08 MHz | 1 | N/A | -55.24 | N/A | -56.79 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. Note 8: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

For UE Rx-Tx time difference, the core requirements for UE Rx-Tx Time Difference Measurements are specified in TS 38.133 clause 10.1.25, with side conditions in Annex B.2.14.

# 3. Discussion

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the different RF channel in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request. The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources. The UE is configured to transmit SRS during T2.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause TS 38.133 clause 10.1.25.3.1.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are selectively copied from Table A.7.6.11.2.1-2 and Table A.7.6.11.2.1-3 in TS 38.133, which are subcarrier spacing, channel bandwidth, RBs in channel bandwidth, Noc, Cell 1 PRS Es/Noc and Cell 2 PRS Es/Noc. All other parameters such as Io, PRS Es/Iot and PRS-RSRP are derived, and not independently settable by the Test Equipment.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1.

c) Uncertainties

The SS provides AWGN and 2 inter-frequency cells to the UE under AWGN conditions in FR2 in standalone scenario. We propose to control the following parameters:

- AWGN absolute power on frequency 1, Noc averaged over BWConfig ±5.65 dB

- AWGN absolute power on frequency 1, Noc averaged over measurement PRBs ±5.65 dB

- AWGN absolute power on frequency 2, Noc averaged over BWConfig ±5.65 dB

- AWGN absolute power on frequency 2, Noc averaged over measurement PRBs ±5.65 dB

- Ratio of cell 1 PRS signal / AWGN, PRS Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 1 PRS signal / AWGN Es1/Noc averaged over measurement PRBs ±0.3 dB

- Ratio of cell 2 PRS signal / AWGN, PRS Ês2 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 2 PRS signal / AWGN Es2/Noc averaged over measurement PRBs ±0.3 dB

- LPP Response Time, 0.3 seconds

The LPP Response Time uncertainty has been chosen to be in-line with all other LPP test cases defined in TS 37.571-1 and there is no necessity to derive the test tolerance in the spreadsheet. LPP Response Time is as defined in TS 37.571-1 clause 4.14.3.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- Absolute accuracy of FR2 PRS-RSRP measurement for normal test environment, ±6 dB.

- Relative accuracy of FR2 PRS-RSRP measurement for normal test environment, ±7.5 dB.

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1 Each Angle of Arrival is considered separately

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

The stimulus requirements defined in TS 38.133 clause 10.1.25 are reproduced below. The stimulus requirements defined in TS 38.133 clause 10.1.25 indicate that the UE Rx-Tx Time Difference measurement accuracy for all measured cells and the Io range.

<< Extracts from TS 38.133 v16.19.0 >>

10.1.25	UE Rx-Tx Time Difference Measurements

<< Some text skipped >>

The accuracy requirements in Table 10.1.25.2-3 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-3: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 22+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 12+ |  | ≥32 | 120 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 7+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 35+ | -13 | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 14+ |  | ≥32 | 120 | ≥4 | NOTE 6 | NOTE 6 |
| ± 9+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-6. |  |  |  |  |  |  |

<< Some text skipped >>

Table 10.1.25.2-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

The side conditions are defined in TS 38.133 Annex B.2.14 and are reproduced below.

B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

<< Some text skipped >>

Table B.2.14-2: Conditions for NR PRS-based measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum PRP1,2 Note 2, Note 3 |  |  |  |  | PRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSPRS |  |  |  |  | dB |
|  |  |  | SCSPRS = 120 kHz |  |  |  | SCSPRS = 60 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 Note4 -13 Note5 -3 Note6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 Note4 -13 Note5 -3 Note6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum PRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. NOTE 4: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement and UE Rx-Tx time difference measurement. NOTE 6: PRS Ês/Iot for PRS-RSRP measurement and UE Rx-Tx time difference measurement. |  |  |  |  |  |  |  |  |

Band n259 has the tighest PRP requirement, which is -108.5 dBm/120kHz for UE PC3 including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

And UE gain G variation is defined in 38.133 Annex B.2.1.6:

B.2.1.6	Gain to PRS-RSRP measurement point for FR2

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.28 of TS 38.215 [4] PRS-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.6.1-1: Gain and Reference point for requirement parameters

The gain range for each power class is specified in Table B.2.1.61-1.

Table B.2.1.6.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Minimum, dBi | FFS | FFS | -10 | FFS |
| Maximum, dBi | FFS | FFS | +20 | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

So the UE gain variation can be expressed as (maximum UE gain - minimum UE gain)/2 according to 38.903 annex A, For UE PC3, it's 15dB.

In summary, during T2 the following conditions shall be satisfied:

-	PRS Ês/Iot of Cell 1 at baseband shall be no less than -3dB.

-	PRS Ês/Iot of Cell 2 at baseband shall be no less than -13dB,

-	PRP of Cell 1 shall be no less than -108.5 dBm/120kHz, including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

-	PRP of Cell 2 shall be no less than -108.5 dBm/120kHz, including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

-	Io on frequency 1 shall be no less than -108.5dBm/Ch BW and no larger than -50dBm/Ch BW.

-	Io on frequency 2 shall be no less than -108.5dBm/Ch BW and no larger than -50dBm/Ch BW.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly inter-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE reporting delay (UE Rx-Tx time difference measurements)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For Cell 1, they are the PRS Es/Iot range and the PRP power range for the reference cell over which the UE is expected to provide UE Rx-Tx time difference measurements. For Cell 2, they are the PRS Es/Iot range and the PRP power range for the neighbour cell over which the UE is expected to provide UE Rx-Tx time difference measurements. The Io power shall also be within the range over which the UE is expected to provide UE Rx-Tx time difference measurements.

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 PRP, so the sensitivity factor is 1.000 for all tests. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

We observe that the PRS Es/Iot of Cell 1 is outside the allowed range, so the PRS Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve Cell 1 PRS Es/Iot >= -3dB.

Similarly, the PRS Es/Iot of Cells 2 is outside the allowed range. Increasing the PRS Es/Noc of any of the cells will drop a little lower for the other cells. Therefore, several iterations of adjustment during step g) are needed to reach to a set of values that fulfils all conditions.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances.

The PRS Es/Noc of Cell 1 is increased by an amount sufficient to achieve PRS Es/Iot >=-3dB. The actual offset required is 0.57dB for cell 1 in T2.

The PRS Es/Noc of Cell 2 is increased by an amount sufficient to achieve PRS Es/Iot >=-13dB.The PRS Es/Noc of Cells 2 is increased by 1.18dB.

The Noc on Freq 1 is decreased by -0.65 dB sufficient to achieve Io<=-50 dBm/Ch BW.

These values are found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

# 5. Calculation of the response time

The response time is determined by the core requirements in the following extract from TS 38.133:

9.9.4.5	Measurements Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i.

where i is the index of positioning frequency layer,

TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in positioning frequency layer i as further defined in this clause,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where

CSSFi is the carrier-specific scaling factor for NR PRS-based measurement in the positioning frequency layer i as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and NRxBeam,i=8 if positioning frequency layer i is in FR2,

Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3  of TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

MGRPi is the measurement gap repetition periodicity in positioning frequency layer i.

TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting, TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive TPRS,i, where

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TUERxTx,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TUERxTx,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

Editor’s Note: FFS the start of measurement period for deferred MT-LR with “Periodic Location”.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TUERxTx,Total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

Since the test case is for single positioning frequency layer, i = 2 and L = 2, so TUERxTx, Total=TUERxTx, 1+TUERxTx, 2+Max(Teffect,i).

Analysing the equation for TUERxTx,i:

CSSFPRS,i=1

NRxBeam,i = 8

NPRS,islot=1

LavailablePRS,i=1080*1*14*4*2=0.071ms

Nsample= 4.

N is the parameter durationOfPRS-ProcessingSysmbols from TS 37.355

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

N’ is the parameter maxNumOfDL-PRS-ResProcessedPerSlot from TS 37.355

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...

},

Therefore, the first part of the equation:

1*8*1N'0.071N*4-1

Based on the value of N’:

| N’ | 1N' |
| --- | --- |
| >=n1 | 1 |

Based on the value of N:

| N | 0.071N |
| --- | --- |
| >=nDot125 | 1 |

Tlast,i = Ti + Tavailable_PRS,i

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,MGRPi ; where Tprs = 160 ms, and MGRP is 80 (for GP#24) or 40 (for GP#13) depending on UE capabilities. Therefore, Tavailable_PRS,i=160

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz

Table A.3.31.2.1-1: PRS.1 FR2: PRS Pattern 1 for SCS=120 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.3 FR2 |  | PRS.1.4 FR2 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 120kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW | 0-31 | 0-127 | 0-31 |  | 0-127 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

Ti depends on UE parameter durationOfPRS-ProcessingSymbolsInEveryTms from TS 37.355:

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280}

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i  and Tlast,i = Ti + Tavailable_PRS,itake following values depending on Ti:

| durationOfPRS-ProcessingSymbolsInEveryTms | Teffect | Tlast |
| --- | --- | --- |
| n8 | 160 | 168 |
| n16 | 160 | 176 |
| n20 | 160 | 180 |
| n30 | 160 | 190 |
| n40 | 160 | 200 |
| n80 | 160 | 240 |
| n160 | 160 | 320 |
| n320 | 320 | 480 |
| n640 | 640 | 800 |
| n1280 | 1280 | 1440 |

Finally, it results in the following equation:

TUERxTx, i=8*1N'0.071N*4-1*Teffect+Tlast

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

Solving the equation for worst-case results in: (32-1)*1280+1440= 41120ms.

Solving the equation for best-case results in: (32-1)*160+168=5128ms.

Therefore, worst-case TUERxTx, Total=83520 ms and best-case TUERxTx, Total=10416ms

Given the wide range of response time possibilities, it is not possible to conclude on a unique value for the response time. The SS will have to calculate the response time based on the UE capabilities.

Example of the response time calculation:

For the best-case result of 10.416s, the SS should round up to the next integer (11 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 11.3s.

For the worst-case result of 83.52s, the SS should round up to the next integer (84 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 84.3s.