3GPP TSG-RAN5 Meeting #103	R5-242277

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4350 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update URLLC TC 7.1.1.4.2.6 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RedCap UE use 12bit SN for AM RLC and PDCP, so the RLC header is 2 bytes and PDCP header is 2 bytes. - Maximum channel bandwidth for FR1 RedCap UE is 20MHz, FR2 RedCap UE is 100MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Table 7.1.1.4.2.6.3.2-2AA to define the Number of uplink PDCP SDUs and PDCP SDU size used as test data for RedCap UE. - Limit the maximum test channel bandwidth for FR1 to 20Mhz for RedCap UE. - Limit the maximum test channel bandwidth for FR2 to 100Mhz for RedCap UE. - Correct typo in Table 7.1.1.4.2.6.3.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

###### 7.1.1.4.2.6	UL-SCH Transport Block Size selection / DCI format 0_2

7.1.1.4.2.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives on PDCCH DCI format 0_2 indicating a resource block assignment correspondent to physical resource blocks , Time domain resource assignment and modulation and coding}

then { UE transmits MAC PDU on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.3, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.1, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.212, clause 7.3.1.1.3]

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-0-2, as defined in Clause 10.1 of [5, TS38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs nBWP, RRC configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	nBWP=nBWP, RRC+1 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP=nBWP, RRC, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214]

-	 bits if only resource allocation type 1 is configured, or  bits if resourceAllocationDCI-0-2-r16 is configured as 'dynamicSwitch', where  NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter resourceAllocationType1GranularityDCI-0-2. If the higher layer parameter resourceAllocationType1GranularityDCI-0-2 is not configured, K1 is equal to 1.

-	If resourceAllocationDCI-0-2-r16 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetListsDCI-0-2 contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetListsDCI-0-2 contains four offset values

-	 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-0-2-r16 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as  bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-2 if the higher layer parameter is configured, or I is the number of entries in the higher layer parameter PUSCH-TimeDomainResourceAllocationList if the higher layer parameter PUSCH-TimeDomainResourceAllocationList is configured and the higher layer parameter pusch-TimeDomainAllocationListDCI-0-2 is not configured; otherwise I is the number of entries in the default table.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if the higher layer parameter frequencyHoppingDCI-0-2 is not configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme –5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-0-2

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

-	HARQ process number – 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2

-	Downlink assignment index – 0, 1, 2 or 4 bits

-	0 bit if the higher layer parameter downlinkAssignmentIndexDCI-0-2 is not configured;

-	1, 2 or 4 bits otherwise,

-	1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

-	2nd downlink assignment index – 0 or 2 bits

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the Downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	SRS resource indicator –  or bits, where NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook',

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook' and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2, and codebookSubsetDCI-0-2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in an SRS resource set with usage set to 'codebook' and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in an SRS resource set with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

-	Antenna ports – number of bits determined by the following:

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-0-2 is not configured;

-	2, 3, 4, or 5 bits otherwise,

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by 7.3.1.1.2-6A, if transform precoder is enabled, and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled, and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 and dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 and is configured with antennaPortsFieldPresenceDCI-0-2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-0-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23.

-	SRS request – 0, 1, 2 or 3 bits

-	0 bit if the higher layer parameter srs-RequestDCI-0-2 is not configured;

-	1 bit as defined by Table 7.3.1.1.3-1 if higher layer parameter srs-RequestDCI-0-2 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	2 bits if higher layer parameter srs-RequestDCI-0-2 = 1 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second bit is defined by Table 7.3.1.1.3-1;

-	2 bits as defined by Table 7.3.1.1.2-24 if higher layer parameter srs-RequestDCI-0-2 = 2 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	3 bits if higher layer parameter srs-RequestDCI-0-2 = 2 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24;

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSizeDCI-0-2.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

-	UL-SCH indicator – 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. Except for DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_2 with UL-SCH indicator of "0" and CSI request of all zero(s).

-	Open-loop power control parameter set indication – 0 or 1 or 2 bits.

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator is present in the DCI format 0_2;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-2 if SRS resource indicator is not present in the DCI format 0_2;

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-0-2 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Invalid symbol pattern indicator – 0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-2 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214].

A UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_2 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_2 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell.

Table 7.3.1.1.3-1: 1 bit SRS request in DCI format 0_2 and DCI format 1_2

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_2 and 1_2 |
| --- | --- |
| 0 | No aperiodic SRS resource set triggered |
| 1 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 |

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberOfRepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where  are the corresponding list entries of the higher layer parameter

-	reportSlotOffsetListDCI-0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 is configured;

-	reportSlotOffsetListDCI-0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListDCI-0-1 is configured;

-	reportSlotOffsetList, otherwise;

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks =, otherwise, and where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and ??offset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and ??offset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211], and

-	for PUSCH scheduled by DCI format 0_1, if pusch-RepTypeIndicatorDCI-0-1 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

-	For PUSCH repetition Type A, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	For PUSCH repetition Type B, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are provided by startSymbol and length of the indexed row of the resource allocation table, respectively.

-	For PUSCH repetition Type A, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

-	For PUSCH repetition Type B, the PUSCH mapping type is set to Type B.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor;

-	otherwise K=1.

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI except for a PUSCH transmission scheduled by a RAR UL grant or fallbackRAR UL grant, in which case the frequency domain resource allocation is determined according to clause 8.3 of [6, 38.213] or a MsgA PUSCH transmission with frequency domain resource allocation determined according to clause 8.1A of [6, 38.213]. Three uplink resource allocation schemes type 0, type 1 and type 2 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 and type 2 are supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the 'Frequency domain resource' assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to 'dynamicSwitch', for DCI format 0_1 or setting a higher layer parameter resourceAllocationDCI-0-2 in pusch-Config to 'dynamicSwitch' for DCI format 0_2, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation for DCI format 0_1 or the higher layer parameter resourceAllocationDCI-0-2 for DCI format 0_2. The UE shall assume that when the scheduling PDCCH is received with DCI format 0_1 and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, uplink type 2 resource allocation is used.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used, except when any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured in which case uplink resource allocation type 2 is used.

The UE expects that either none or both of useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured.

If a bandwidth part indicator field is not configured in the scheduling DCI or the UE does not support active bandwidth part change via DCI, the RB indexing for uplink type 0, type 1 and type 2 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI and the UE supports active bandwidth part change via DCI, the RB indexing for uplink type 0, type 1, type 2 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part. RB numbering starts from the lowest RB in the determined uplink bandwidth part.

[38.214 clause 6.1.2.2.1]

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1.

Table 6.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

[38.214 clause 6.1.2.2.2]

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in any common search space in which case the size of the initial UL bandwidth part  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

When the DCI size for DCI format 0_0 in USS is derived from the initial UL BWP with size  but applied to another active BWP with size of , an uplink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks .

The resource indication value is defined by

if  then

else

where, and where shall not exceed .

If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.

When the scheduling grant is received with DCI format 0_2, an uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0, 1, …, NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=1, …, NRBG, where the resource block groups are defined as in 6.1.2.2.1 with P defined by resourceAllocationType1GranularityDCI-0-2 if the UE is configured with higher layer parameter resourceAllocationType1GranularityDCI-0-2, and P=1 otherwise. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

[TS 38.214, clause 6.1.4.1]

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by a fallbackRAR UL grant or

for a MsgA PUSCH transmission, or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or

for a PUSCH with configured grant using CS-RNTI, and

if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3

-	if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel;

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

[TS 38.214, clause 5.1.3.1]

Table 5.1.3.1-3: MCS index table 3 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 30 | 0.0586 |
| 1 | 2 | 40 | 0.0781 |
| 2 | 2 | 50 | 0.0977 |
| 3 | 2 | 64 | 0.1250 |
| 4 | 2 | 78 | 0.1523 |
| 5 | 2 | 99 | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 4 | 340 | 1.3281 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 6 | 438 | 2.5664 |
| 22 | 6 | 466 | 2.7305 |
| 23 | 6 | 517 | 3.0293 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 719 | 4.2129 |
| 28 | 6 | 772 | 4.5234 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 6.1.4.2]

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by fallbackRAR UL grant or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or

for a PUSCH transmission with configured grant, or

for a MsgA PUSCH transmission,

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  is the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH,  is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 in Clause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 6, 12, or 18), the  is assumed to be 0. For Msg3 or MsgA PUSCH transmission the  is always set to 0. In case of PUSCH repetition Type B,  is determined assuming a nominal repetition with the duration of L symbols without segmentation.

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

-	For a PUSCH scheduled by fallbackRAR UL grant, UE assumes the TB size determined by the UL grant in the fallbackRAR shall be the same as the TB size used in the corresponding MsgA PUSCH transmission.

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant,

-	the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH.

-	the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 PUSCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant,

-	the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH.

-	the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 PUSCH.

[TS 38.214, clause 5.1.3.2]

2)	Unquantized intermediate variable (Ninfo) is obtained by .

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-1 find the closest TBS that is not less than .

Table 5.1.3.2-1: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.6.3	Test description

7.1.1.4.2.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value) and Short_DCI condition is applied in NR Serving cell configuration.

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

NOTE:	If pc_supportOfRedCap_r17=true, the FR1 test channel bandwidth is MIN {20MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]} and the FR2 test channel bandwidth is MIN {100MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]}

7.1.1.4.2.6.3.2	Test procedure sequence

Table 7.1.1.4.2.6.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.6.3.2-2: Number of uplink PDCP SDUs and PDCP SDU size used as test data for non-RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 108705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| TBS> 144920 | 13 | 8*FLOOR((TBS – 992)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.2.6.3.2-2A: Void

Table 7.1.1.4.2.6.3.2-2AA: Number of uplink PDCP SDUs and PDCP SDU size used as test data for RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 120 ≤ TBS ≤ 12112 note 2 | 1 | 8*FLOOR((TBS – 112)/8) |
| 12113 ≤ TBS ≤ 24168 | 2 | 8*FLOOR((TBS – 168)/16) |
| 24169 ≤ TBS ≤ 36224 | 3 | 8*FLOOR((TBS – 224)/24) |
| 36225 ≤ TBS ≤ 48280 | 4 | 8*FLOOR((TBS – 280)/32) |
| 48281≤ TBS ≤ 60336 | 5 | 8*FLOOR((TBS – 336)/40) |
| 60337 ≤ TBS ≤ 72392 | 6 | 8*FLOOR((TBS – 392)/48) |
| 72393 ≤ TBS ≤ 84448 | 7 | 8*FLOOR((TBS – 448)/56) |
| 84449 ≤ TBS ≤ 96504 | 8 | 8*FLOOR((TBS – 504)/64) |
| 96505 < TBS ≤ 108560 | 9 | 8*FLOOR((TBS –560)/72) |
| 108561 ≤ TBS ≤ 120616 | 10 | 8*FLOOR((TBS – 616)/80) |
| 120617 ≤ TBS ≤ 132672 | 11 | 8*FLOOR((TBS –672)/88) |
| 132673 ≤ TBS ≤ 144728 | 12 | 8*FLOOR((TBS – 728)/96) |
| 144729 ≤ TBS ≤ 156784 | 13 | 8*FLOOR((TBS – 784)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is 16 bits with 12 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR ((TBS – N*16- N*16 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 120 bits. |  |  |

Table 7.1.1.4.2.6.3.2-3: Specific Parameter

| Parameter | Value | Comment | Condition |
| --- | --- | --- | --- |
| mcs-TableDCI-0-2-r16 | Not Present | qam64 per default |  |
| rbg-Size | Not present | configuration 1 applicable |  |
| NstartBWP | 0 |  |  |

Table 7.1.1.4.2.6.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per Table 7.1.1.4.2.0-1 and from 0 to 28. Skip the execution of steps for = 28 and < 4. (Note 1) | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.6.3.2-1 and larger than or equal to X bits as specified in Table 7.1.1.4.2.6.3.2-2/2AA(Note 3). Skip the execution of steps 2 to 5 for which the TBS size equal to 3824 or 3840. (Note 2) | - | - | - | - |
| 2 | The SS creates one or more PDCP SDUs, depending on TBS, in accordance with Table 7.1.1.4.2.6.3.2-2/2AA(Note 4). | - | - | - | - |
| 3 | After 300ms, the SS transmits all PDCP SDUs (NSDUs) as created in step 2 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 4 | After 60ms of step 3, SS transmits UL Grant DCI 0_2, and values of S, L,and nPRB. | <-- | (UL Grant) (DCI Format 0_2, S, L,and nPRB.) | - | - |
| 5 | Check: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | MAC PDU (N x PDCP SDU) | 1 | P |
| Note 1: For = 28 and < 4, the resulting TBS is very small leading to CRC errors in decoding UL data. Note 2: There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 [15]. Note 3: For pc_supportOfRedCap_r17=false, Table 7.1.1.4.2.6.3.2-2 is used and X =136. For pc_supportOfRedCap_r17=true, Table 7.1.1.4.2.6.3.2-2AA is used and X=120. Note 4: For pc_supportOfRedCap_r17=false, Table 7.1.1.4.2.6.3.2-2 is used. For pc_supportOfRedCap_r17=true, Table 7.1.1.4.2.6.3.2-2AA is used. |  |  |  |  |  |

7.1.1.4.2.6.3.3	Specific message contents

Table 7.1.1.4.2.6.3.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [4], table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| harq-ProcessNumberSizeDCI-0-2-r16 | 4 | nrofHARQ-ProcessesForPUSCH is 16 |  |
| numberOfBitsForRV-DCI-0-2-r16 | 2 |  |  |
| resourceAllocationDCI-0-2-r16 | resourceAllocationType1 |  |  |
| } |  |  |  |

<End of change>