3GPP TSG-RAN5 Meeting #103	R5-242278

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4351 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update coverage enhancement TC 7.1.1.4.2.7 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RedCap UE use 12bit SN for AM RLC and PDCP, so the RLC header is 2 bytes and PDCP header is 2 bytes. - Maximum channel bandwidth for FR1 RedCap UE is 20MHz, FR2 RedCap UE is 100MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Table 7.1.1.4.2.7.3.2-2A to define the Number of uplink PDCP SDUs and PDCP SDU size used as test data for RedCap UE. - Limit the maximum test channel bandwidth for FR1 to 20Mhz for RedCap UE. - Limit the maximum test channel bandwidth for FR2 to 100Mhz for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

###### 7.1.1.4.2.7	UL-SCH Transport Block Size selection / TBoMS procedure

7.1.1.4.2.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting TBoMS }

ensure that {

when { UE has pending data for transmission and receives on PDCCH DCI format 0_1 indicating a resource block assignment correspondent to physical resource blocks , Time domain resource assignment including numberOfSlotsTBoMS and modulation and coding }

then { UE transmits MAC PDU on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.1, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

…

Otherwise, all the remaining fields are set as follows:

…

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured

…

-	bits if only resource allocation type 1 is configured, or  bits if resourceAllocation is configured as 'dynamicSwitch'.

…

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

…

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

…

-	Time domain resource assignment – 0, 1, 2, 3, 4, 5, or 6 bits

…

-	If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is configured or if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 or pusch-TimeDomainAllocationListForMultiPUSCH;

…

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

…

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report by a DCI or by a RAR UL grant or fallbackRAR UL grant, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the 'Time domain resource assignment' field value m of the DCI or the PUSCH time resource allocation field value m of the RAR UL grant or of the fallbackRAR UL grant provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, the number of slots used for TBS determination (if numberOfSlotsTBoMS is present in the resource allocation table), and the number of repetitions (if numberOfRepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

…

-	for PUSCH scheduled by DCI format 0_1 or DCI format 0_2, if numberOfSlotsTBoMS is present and larger than 1, the UE applies TB processing over multiple slots procedure when determining the time domain resource allocation.

-	For PUSCH repetition Type A and TB processing over multiple slots, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

…

-	For PUSCH repetition Type A and TB processing over multiple slots, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

…

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

For TB processing over multiple slots, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1,

-	the number of slots used for TBS determination N is indicated by numberOfSlotsTBoMS.

-	the number of repetitions K of the number of slots N used for TBS determination is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	otherwise, K=1.

-	when the UE supports repetition of TB processing over multiple slots, the UE does not expect that Na??K is larger than 32.

…

For unpaired spectrum:

…

-	The UE determines Na??K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2.

-	A slot is not counted in the number of Na??K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

…

For paired spectrum and SUL band:

-	The UE determines Na??K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2.

…

If a UE would transmit a PUSCH of PUSCH repetition Type A when AvailableSlotCounting is enabled and K>1 or a TB processing over multiple slots over Na??K slots, and the UE does not transmit the PUSCH of a TB processing over multiple slots or the PUSCH repetition Type A in a slot from the Na??K slots, according to Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], the UE counts the slots in the number of Na??K slots.

…

For TB processing over multiple slots:

-	For unpaired spectrum, the same symbol allocation is applied across the Na??K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the Na??K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the  Na??K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the Na??K consecutive slots applying the same symbol allocation in each slot.

-	For the case of reduced capability half-duplex UE, the same symbol allocation is applied across the Na??K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the Na??K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

…

For PUSCH repetition Type A and TB processing over multiple slots, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

…

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI except for a PUSCH transmission scheduled by a RAR UL grant or fallbackRAR UL grant, in which case the frequency domain resource allocation is determined according to clause 8.3 of [6, 38.213] or a MsgA PUSCH transmission with frequency domain resource allocation determined according to clause 8.1A of [6, 38.213]. Three uplink resource allocation schemes type 0, type 1 and type 2 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 and type 2 are supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the 'Frequency domain resource' assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to 'dynamicSwitch', for DCI format 0_1 or setting a higher layer parameter resourceAllocationDCI-0-2 in pusch-Config to 'dynamicSwitch' for DCI format 0_2, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation for DCI format 0_1 or the higher layer parameter resourceAllocationDCI-0-2 for DCI format 0_2. The UE shall assume that when the scheduling PDCCH is received with DCI format 0_1 and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, uplink type 2 resource allocation is used.

…

[38.214 clause 6.1.2.2.2]

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in the Type0-PDCCH common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

[TS 38.214, clause 6.1.4.1]

…

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or

…

if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3

…

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

…

[TS 38.214, clause 5.1.3.1]

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

[TS 38.214, clause 6.1.4.2]

…

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or

…

if

…

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  is the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH,  is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 in Clause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 6, 12, or 18), the  is assumed to be 0. For Msg3 or MsgA PUSCH transmission the  is always set to 0. In case of PUSCH repetition Type B,  is determined assuming a nominal repetition with the duration of L symbols without segmentation.

-	A UE determines the total number of REs allocated for PUSCH  as follows

-	For TB processing over multiple slots,  where  is the total number of allocated PRBs for the UE and N is the number of slots used for TBS determination indicated by numberOfSlotsTBoMS.

…

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

…

[TS 38.214, clause 5.1.3.2]

2	Intermediate number of information bits (Ninfo) is obtained by .

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.7.3	Test description

7.1.1.4.2.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value) and Short_DCI condition is applied in NR Serving cell configuration.

Test frequency NRf1 is as specified in clause 4.3.1 of TS 38.508-1 [4] using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in clause 6.2.3.1 of TS 38.508-1 [4].

NOTE:	If pc_supportOfRedCap_r17=true, the FR1 test channel bandwidth is MIN {20MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]} and the FR2 test channel bandwidth is MIN {100MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]}

7.1.1.4.2.7.3.2	Test procedure sequence

Table 7.1.1.4.2.7.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.7.3.2-2: Number of uplink PDCP SDUs and PDCP SDU size used as test data for non-RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) | Number of MAC PDUs | MAC PDU size [bits] (Note 2) |
| --- | --- | --- | --- | --- |
| 136 ≤ TBS ≤ 12128 (note 2) | 1 | 8*FLOOR((TBS – 128)/8) | 1 | PDCP SDU size + 128 |
| 12129 < TBS ≤ 24200 | 2 | 8*FLOOR((TBS – 200)/16) | 1 | 2*PDCP SDU size + 200 |
| 24201 < TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) | 1 | 3*PDCP SDU size + 272 |
| 36273 < TBS ≤ 48344 | 4 | 8*FLOOR((TBS – 344)/32) | 1 | 4*PDCP SDU size + 344 |
| 48345 <TBS ≤ 60416 | 5 | 8*FLOOR((TBS – 416)/40) | 1 | 5*PDCP SDU size + 416 |
| 60417 < TBS ≤ 72488 | 6 | 8*FLOOR((TBS – 488)/48) | 1 | 6*PDCP SDU size + 488 |
| 72489 < TBS ≤ 84560 | 7 | 8*FLOOR((TBS – 560)/56) | 1 | 7*PDCP SDU size + 560 |
| 84561 < TBS ≤ 96632 | 8 | 8*FLOOR((TBS – 632)/64) | 1 | 8*PDCP SDU size + 632 |
| 96633 < TBS ≤ 108704 | 9 | 8*FLOOR((TBS – 704)/72) | 1 | 9*PDCP SDU size + 704 |
| 108705 < TBS ≤ 120776 | 10 | 8*FLOOR((TBS – 776)/80) | 1 | 10*PDCP SDU size + 776 |
| 120777 < TBS ≤ 132848 | 11 | 8*FLOOR((TBS –848)/88) | 1 | 11*PDCP SDU size + 848 |
| 132849 < TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) | 1 | 12*PDCP SDU size + 920 |
| 144921 < TBS ≤ 156992 | 13 | 8*FLOOR((TBS – 992)/104) | 1 | 13*PDCP SDU size + 992 |
| 156993 < TBS ≤ 169064 | 14 | 8*FLOOR((TBS – 1064)/112) | 1 | 14*PDCP SDU size + 1064 |
| 169065 < TBS ≤ 181136 | 15 | 8*FLOOR((TBS – 1136)/120) | 1 | 15*PDCP SDU size + 1136 |
| 181137 < TBS ≤ 193208 | 16 | 8*FLOOR((TBS – 1208)/128) | 1 | 16*PDCP SDU size + 1208 |
| 193209 < TBS ≤ 205280 | 17 | 8*FLOOR((TBS – 1280)/136) | 1 | 17*PDCP SDU size + 1280 |
| 205281 < TBS ≤ 217352 | 18 | 8*FLOOR((TBS – 1352)/144) | 1 | 18*PDCP SDU size + 1352 |
| 217353 < TBS ≤ 229424 | 19 | 8*FLOOR((TBS – 1424)/152) | 1 | 19*PDCP SDU size + 1424 |
| 229424 < TBS ≤ 241496 | 20 | 8*FLOOR((TBS – 1496)/160) | 1 | 20*PDCP SDU size + 1496 |
| 241496 < TBS ≤ 253568 | 21 | 8*FLOOR((TBS – 1568)/168) | 2 | 10.5*PDCP SDU size + 836 |
| 253568 < TBS ≤ 265640 | 22 | 8*FLOOR((TBS – 1640)/176) | 2 | 11*PDCP SDU size + 848 |
| 265640 < TBS ≤ 277712 | 23 | 8*FLOOR((TBS – 1712)/184) | 2 | 11.5*PDCP SDU size + 908 |
| 277712 < TBS ≤ 289784 | 24 | 8*FLOOR((TBS – 1784)/192) | 2 | 12*PDCP SDU size + 920 |
| 289784 < TBS ≤ 301856 | 25 | 8*FLOOR((TBS – 1856)/200) | 2 | 12.5*PDCP SDU size + 980 |
| 301856 < TBS ≤ 313928 | 26 | 8*FLOOR((TBS – 1928)/208) | 2 | 13*PDCP SDU size + 992 |
| 313928 < TBS ≤ 326000 | 27 | 8*FLOOR((TBS – 2000)/216) | 2 | 13.5*PDCP SDU size + 1052 |
| 326000 < TBS ≤ 338072 | 28 | 8*FLOOR((TBS – 2072)/224) | 2 | 14*PDCP SDU size + 1064 |
| 338072 < TBS ≤ 350144 | 29 | 8*FLOOR((TBS – 2144)/232) | 2 | 14.5*PDCP SDU size + 1124 |
| 350144 < TBS ≤ 362216 | 30 | 8*FLOOR((TBS – 2216)/240) | 2 | 15*PDCP SDU size + 1136 |
| 362216 < TBS ≤ 374288 | 31 | 8*FLOOR((TBS – 2288)/248) | 2 | 15.5*PDCP SDU size + 1196 |
| 374288 < TBS ≤ 386360 | 32 | 8*FLOOR((TBS – 2360)/256) | 2 | 16*PDCP SDU size + 1208 |
| 386360 < TBS ≤ 398432 | 33 | 8*FLOOR((TBS – 2432)/264) | 2 | 16.5*PDCP SDU size + 1268 |
| 398432 < TBS ≤ 410504 | 34 | 8*FLOOR((TBS – 2504)/272) | 2 | 17*PDCP SDU size + 1280 |
| 410504 < TBS ≤ 422576 | 35 | 8*FLOOR((TBS – 2576)/280) | 2 | 17.5*PDCP SDU size + 1340 |
| 422576 < TBS ≤ 434648 | 36 | 8*FLOOR((TBS – 2648)/288) | 2 | 18*PDCP SDU size + 1352 |
| 434648 < TBS ≤ 446720 | 37 | 8*FLOOR((TBS – 2720)/296) | 2 | 18.5*PDCP SDU size + 1412 |
| 446720 < TBS ≤ 458792 | 38 | 8*FLOOR((TBS – 2792)/304) | 2 | 19*PDCP SDU size + 1424 |
| 458792 < TBS ≤ 470864 | 39 | 8*FLOOR((TBS – 2864)/312) | 2 | 19.5*PDCP SDU size + 1484 |
| TBS > 470864 | 40 | 8*FLOOR((TBS – 2936)/320) | 2 | 20*PDCP SDU size + 1496 |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC subheader size –Timing Advance Command MAC CE size – MAC subheader for MAC CE size – RLC Status PDU size - MAC subheader for RLC Status PDU size) / N, where PDCP header size = 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size = 24 bits for 18 bit SN case; MAC header size for AMD PDU = 16 or 24 bits depending on L= 8 or 16 bits. Worst case 24 is taken. Timing Advance Command MAC CE size + MAC subheader for MAC CE size = 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size + MAC subheader for RLC Status PDU size = 40 bits for 1 ACK_SN case. This gives: PDCP SDU size = 8*FLOOR((TBS –72*N -56 )/(8*N)) bits. Note 2: M MAC PDUs are transmitted. Each MAC PDU consists ceil(N/M) RLC PDUs or RLC PDU segments. The MAC PDU size of each MAC PDU is MAC PDU size = (N*PDCP SDU size + N*PDCP header size) / M + CEIL(N / M)*(AMD PDU header size + MAC header size for AMD PDU ) + Timing Advance Command MAC CE size + MAC subheader for MAC CE size + RLC Status PDU size + MAC subheader for RLC Status PDU size, where N, PDCP SDU size, PDCP header size, AMD PDU header size, MAC header size for AMD PDU, Timing Advance Command MAC CE size, MAC subheader for MAC CE size, RLC Status PDU size, MAC subheader for RLC Status PDU size are same as in Note 1. This gives: MAC PDU size = (N/M)*PDCP SDU size + 24*N/M + 48*CEIL(N/M) + 56 Note 3: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |  |  |

Table 7.1.1.4.2.7.3.2-2A: Number of uplink PDCP SDUs and PDCP SDU size used as test data for RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 120 ≤ TBS ≤ 12112 note 2 | 1 | 8*FLOOR((TBS – 112)/8) |
| 12113 ≤ TBS ≤ 24168 | 2 | 8*FLOOR((TBS – 168)/16) |
| 24169 ≤ TBS ≤ 36224 | 3 | 8*FLOOR((TBS – 224)/24) |
| 36225 ≤ TBS ≤ 48280 | 4 | 8*FLOOR((TBS – 280)/32) |
| 48281≤ TBS ≤ 60336 | 5 | 8*FLOOR((TBS – 336)/40) |
| 60337 ≤ TBS ≤ 72392 | 6 | 8*FLOOR((TBS – 392)/48) |
| 72393 ≤ TBS ≤ 84448 | 7 | 8*FLOOR((TBS – 448)/56) |
| 84449 ≤ TBS ≤ 96504 | 8 | 8*FLOOR((TBS – 504)/64) |
| 96505 < TBS ≤ 108560 | 9 | 8*FLOOR((TBS –560)/72) |
| 108561 ≤ TBS ≤ 120616 | 10 | 8*FLOOR((TBS – 616)/80) |
| 120617 ≤ TBS ≤ 132672 | 11 | 8*FLOOR((TBS –672)/88) |
| 132673 ≤ TBS ≤ 144728 | 12 | 8*FLOOR((TBS – 728)/96) |
| 144729 ≤ TBS ≤ 156784 | 13 | 8*FLOOR((TBS – 784)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is 16 bits with 12 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR ((TBS – N*16- N*16 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 120 bits. |  |  |

Table 7.1.1.4.2.7.3.2-3: Specific Parameters

| Parameter | Value | Comment |
| --- | --- | --- |
| number of layers (ʋ) | 1 | TBoMS only supports 1 layer transmission |
| mcs-Table | qam64 |  |

Table 7.1.1.4.2.7.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1-5 are repeated for allowed values of NPRB 1 to NRBUL,BWP in BWP, time domain resource as per Table 7.1.1.4.2.7.3.3-2 and IMCS from 0 to 28. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] clause 6.1.4.2 based on the value of S, L, IMCS and NPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is no larger than UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.7.3.2-1 and no less than X bits as specified in Table 7.1.1.4.2.7.3.2-2/2A(Note 4). Skip the execution of steps 2 to 5 for which the TBS size equal to 3824 or 3840. (Note 2) Skip the execution of steps for IMCS > 27 and NPRB < 5. (Note 1) | - | - | - | - |
| 2 | The SS creates N PDCP SDUs, depending on TBS, in accordance with Table 7.1.1.4.2.7.3.2-2/2A(Note 5). | - | - | - | - |
| 3 | The SS transmits all PDCP SDUs as created in step 2 in M MAC PDUs in accordance with Table 7.1.1.4.2.7.3.2-2. (Note 3) | <-- | M MAC PDUs (N PDCP SDUs) | - | - |
| 4 | After the reception of 2 Scheduling Request, SS transmits UL Grant DCI 0_1 providing values of S, L, IMCS and NPRB. | <-- | DCI format 0_1(S, L, IMCS, NPRB) | - | - |
| 5 | Check: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | MAC PDU (N PDCP SDUs) | 1 | P |
| Note 1: For IMCS> 27 and NPRB < 5, the combination results in higher coding rate and therefore leading to CRC errors in decoding UL data. Note 2: There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 [15]. Note 3: For TBoMS, TBS is calculated based on all available resources in numberOfSlotsTBoMS slots. Then the TBS of UL-SCH may exceed the maximum TBS of DL-SCH. Note 4: For pc_supportOfRedCap_r17=false, Table 7.1.1.4.2.7.3.2-2 is used and X =136. For pc_supportOfRedCap_r17=true, Table 7.1.1.4.2.7.3.2-2A is used and X=120. Note 5: For pc_supportOfRedCap_r17=false, Table 7.1.1.4.2.7.3.2-2 is used. For pc_supportOfRedCap_r17=true, Table 7.1.1.4.2.7.3.2-2A is used. |  |  |  |  |  |

<End of change>