3GPP TSG-RAN5 Meeting #103	R5-242312

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0107 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of testcase 6.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Condition LOCATION_REPORT has been removed from 36.579-1 Table 5.5.2.7.1-1 - Table Table 6.9.1.3.3-8: LOCATION-INFO is used instead of LOCATION_REPORT but Request URI has been missing - 36.579-1 Table 5.5.3.4.1-1 (Location-info) does not have any conditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 6.9.1.3.3-6, Table 6.9.1.3.3-14: LOCATION_REPORT replaced by LOCATION-INFO; Request URI added - Request URI added to Table 6.9.1.3.3-8 - LOCATION-INFO removed from Table 6.9.1.3.3-7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.579-1 CR 0354 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.9.1	On-network / Location/ Location Information Request

6.9.1.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client)registered and authorized for MCVideo service and not in an emergency state and having received a location information configuration message providing configuration parameters for both NonEmergencyLocationInformation and EmergencyLocationInformation }

ensure that {

when { UE (MCVideo Client) receives a location information request before the minimumReportInterval timer has expired  }

then { UE (MCVideo Client)does not wait for the minimumReportInterval timer to expire, and,  sends"immediately"  a SIP MESSAGE containing location information report including information in accordance with the NonEmergencyLocationInformation configuration and the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request, and, resets the minimumReportInterval timer }

}

(2)

with { UE (MCVideo Client)registered and authorized for MCVideo service and not in an emergency state and having received a location information configuration message }

ensure that {

when { UE (MCVideo Client) receives a new location information configuration message and ConfigScope set to 'Full' }

then { UE (MCVideo Client) stores the contents of the <Configuration> elements thereby overriding the previous configuration }

}

6.9.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281, clauses 18.3.1, 18.3.2, 18.3.3, 18.3.4.2. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.281 clause 18.3.1]

The MCVideo client sends a location report when one of the trigger criteria is fulfilled or when it receives a request from the participating MCVideo function to send a location report. To send the location report the MCVideo client can use an appropriate SIP message that it needs to send for other reasons, or it can include the location report in a SIP MESSAGE request.

To send a location report, the MCVideo client includes in the SIP MESSAGE request an application/vnd.3gpp.mcvideo-location-info+xml MIME body as specified in clause F.3. The MCVideo client populates the elements in accordance with its reporting configuration. Further location information may also be included in the P-Access-Network-Info header field.

[TS 24.281 clause 18.3.2]

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcvideo";

2)	a Content-Type header field set to "application/vnd.3gpp.mcvideo-location-info+xml"; and

3)	an application/vnd.3gpp.mcvideo-location-info+xml MIME body with a <Configuration> root element included in the <location-info> root element;

then the MCVideo client:

1)	shall store the contents of the <Configuration> elements;

2)	shall set the location reporting triggers accordingly; and

3)	shall start the minimumReportInterval timer.

[TS  24.281 clause 18.3.3]

Upon receiving a SIP MESSAGE request containing

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcvideo";

2)	a Content-Type header field set to "application/vnd.3gpp.mcvideo-location-info+xml"; and

3)	an application/vnd.3gpp.mcvideo-location-info+xml MIME body with a <Request> element included in the <location-info> root element;

then the MCVideo client:

1)	shall send a location report as specified in clause 18.3.4; and

2)	shall reset the minimumReportInterval timer.

[TS 24.281 clause 18.3.4.2]

If the MCVideo client does not need to send a SIP request for other reasons, the MCVideo client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [11] and IETF RFC 3428 [17]. The MCVideo client;

1)	shall include in the Request-URI, the SIP URI received in the P-Asserted-Identity header field in the received SIP MESSAGE request for location report configuration;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.mcvideo-location-info+xml";

3)	shall include an application/vnd.3gpp.mcvideo-location-info+xml MIME body and in the <location-info> root element include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	a <TriggerId> child element set to the value of each <Trigger-Id> value of the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcvideo along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [20];

5)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer;

6)	shall reset all triggers; and

7)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [11].

6.9.1.3	Test description

6.9.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 and Cell 2 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	The UE has performed procedure 'MCVideo UE registration' as specified in TS 36.579-1 [2] clause 5.4.2A.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client

6.9.1.3.2	Test procedure sequence

Table 6.9.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-3 | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to configure location reporting with minimumReportInterval timer set to 20s and PeriodicReport reporting interval set to 15s. NOTE: The handling of the PeriodicReport is not tested in this test. It is added here for the purpose of verifying that the UE restarts the minimumReportInterval – when the Periodic report trigger fires before the expiration of the minimumReportInterval the UE needs to wait for its expiry before sending the periodic report. | - | - | - | - |
| 4 | Start timer t_Wait=10s (NOTE 1) | - | - | - | - |
| 5 | Start timer t_PeriodicReport=15s (PeriodicReport timer set in step 1). | - | - | - | - |
| - | EXCEPTION: The SS waits 2 seconds before it releases the RRC connection | - | - | - | - |
| 6 | t_Wait expires. | - | - | - | - |
| 7-9 | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to request a location report. | - | - | - | - |
| 10-11 | Check: Does the UE (MCVideo Client) correctly perform steps 2-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 before t_PeriodicReport expires to provide location information for the NonEmergencyLocationInformation configured at step 2? | - | - | 1 | P |
| 12 | Start timer t_MinimumReportInterval=20s | - | - | - | - |
| 13 | Stop timer t_PeriodicReport | - | - | - | - |
| - | EXCEPTION: The SS waits 2 seconds before it releases the RRC connection | - | - | - | - |
| 14 | t_MinimumReportInterval expires | - | - | - | - |
| 15-17 | Check: Does the UE (MCVideo Client) correctly perform steps 1a1-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to provide periodic location information? | - | - | 1 | P |
| 18-19 | Steps 2-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to remove the periodic trigger criteria form the location reporting configuration. | - | - | - | - |
| 20-21 | Steps 2-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to request a location report. | - | - | - | - |
| 22-23 | Check: Does the UE (MCVideo Client) correctly perform steps 2-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to provide location information for the configuration done at steps 4-4A? | - | - | 2 | P |
| 24 | Check: Does the UE (MCVideo Client) send any SIP MESSAGE in the next 25s? | --> | SIP MESSAGE | 2 | F |
| - | EXCEPTION: The SS waits 2 seconds before it releases the RRC connection | - | - | - | - |
| NOTE 1: The value of 10s is arbitrary chosen. However, it needs to be long enough to ensure that if the UE did not reset the minimumReportInterval timer after it sent the message in step 3 the UE will send the SIP MESSAGE for the periodic reporting before step 3D and will fail, and, it needs to be short enough so that step 2 happens reasonably before t_PeriodicReport expires. |  |  |  |  |  |

6.9.1.3.3	Specific message contents

Table 6.9.1.3.3-1: SIP MESSAGE from the SS (Step 2, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-info as described in Table 6.9.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-3 |  |  |  |

Table 6.9.1.3.3-2: MCVideo-Info in SIP MESSAGE (Table 6.9.1.3.3-1, 6.9.1.3.3-4, 6.9.1.3.3-10, 6.9.1.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-calling-user-id | not present |  |  |  |

Table 6.9.1.3.3-3: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| GeographicalCoordinate | present | Provision of <GeographicalCoordinate> in the report is requested. |  |  |
| minimumIntervalLength | "20" |  |  |  |
| TriggeringCriteria |  |  |  |  |
| PeriodicReport |  |  |  |  |
| extension base | "15" | periodicity of the reports (in seconds) |  |  |
| TiggerID | "PERIODIC" |  |  |  |

Table 6.9.1.3.3-4: SIP MESSAGE from the SS (Step 8, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-info as described in Table 6.9.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-5 |  |  |  |

Table 6.9.1.3.3-5: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.3-2

Table 6.9.1.3.3-6: SIP MESSAGE from the UE (Step 10, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-URI | tsc_MCVideo_PublicServiceId_PF_A | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCVideo-Info |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-7 |  |  |  |

Table 6.9.1.3.3-7: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | "1" | The same as the ID in the request message. |  |  |
| ReportType attribute | "NonEmergency" |  |  |  |
| CurrentLocation |  | Information corresponding to the location reporting configuration of table 6.9.1.3.3-3 |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |

Table 6.9.1.3.3-8: SIP MESSAGE from the UE (Step 16, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-URI | tsc_MCVideo_PublicServiceId_PF_A | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCVideo-Info |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-9 |  |  |  |

Table 6.9.1.3.3-9: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportType attribute | "NonEmergency" |  |  |  |
| TriggerID | "PERIODIC" |  |  |  |
| CurrentLocation |  | Information corresponding to the location reporting configuration of table 6.9.1.3.3-3 |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |

Table 6.9.1.3.3-10: SIP MESSAGE from the SS (Step 18, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-info as described in Table 6.9.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-11 |  |  |  |

Table 6.9.1.3.3-11: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | Provision of <ServingEcgi> in the report is requested. |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| GeographicalCoordinate | not present |  |  |  |

Table 6.9.1.3.3-12: SIP MESSAGE from the SS (Step 20, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-info as described in Table 6.9.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-13 |  |  |  |

Table 6.9.1.3.3-13: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "2" | The RequestID that the MCVideo client shall reference in the Report |  |  |

Table 6.9.1.3.3-14: SIP MESSAGE from the UE (Step 22, Table 6.9.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-URI | tsc_MCVideo_PublicServiceId_PF_A | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCVideo-Info |  |  |
| MIME body part |  | Location-info | TS 24.281 [26] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.9.1.3.3-15 |  |  |  |

Table 6.9.1.3.3-15: Location-Info in SIP MESSAGE (Table 6.9.1.3.3-14)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Report | present |  |  |  |
| ReportID | "2" | The same as the ID in the request message. |  |  |
| ReportType | "NonEmergency" |  |  |  |
| CurrentLocation | present | Information corresponding to the location reporting configuration of table 6.9.1.3.3-8 |  |  |
| CurrentServingEcgi | Encrypted <CurrentServingEcgi> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate | not present |  |  |  |