3GPP TSG-RAN5 Meeting #103	R5-242313

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4820 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ID_UAS: Test Model aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UUAA test cases require new AT commands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new AT commands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UUAA test cases can not be fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	Upper Tester ASPs

There is one primitive defined carrying either an MMI or an AT command to be sent to the system adaptor and one common confirmation primitive to be sent by the system adaptor.

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_SYSTEM_REQ |  |  |
| TTCN-3 Type | Record |  |  |
| Cmd |  | TTCN-3 Type | union |
| AT |  | charstring carrying the AT command as defined in TS 27.007 [32], TS 27.005 [31] and TS 27.060 [33] |  |
| MMI |  | - Cmd (charstring) - List of parameters: - Name (charstring) - Value (charstring) |  |
| CnfRequired |  | TTCN-3 Type | Ut_CnfReq_Type |
|  |  | CNF_REQUIRED: SS shall reply with one confirmation (NOTE) NO_CNF_REQUIRED: SS shall swallow any confirmation generated LOCAL_CNF_REQUIRED: SS shall immediately send one confirmation when the command is submitted to the UE i.e. in case of operator interaction when the operator has confirmed the command, but SS shall not wait for the UE responding(NOTE) |  |
| NOTE: In the TTCN, a confirmation shall only be requested in cases when there is no signalling from the UE being triggered by the MMI/AT command |  |  |  |

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_COMMON_CNF |  |  |
| TTCN-3 Type | Record |  |  |
| Result |  | TTCN-3 Type | boolean |
|  |  | true: success false: failure NOTE: The return value false will result in the test immediately finishing with a fatal error. In case of AT commands, the SS shall return false if and only if the UE comes back with "ERROR" as result code according to ITU-T Rec V.250 clause 5.7 [60]. In case of e.g. "+CME ERROR" the SS shall return true and put the UE's response into the ResultString |  |
| ResultString |  | TTCN-3 Type | charstring |
|  |  | response by the UE for commands which request the UE to return a result, optional In case of AT commands, ResultString shall contain the complete line according to the specification of the AT command. Any carriage return and linefeed characters, defined in TS 27.007 [32] clause 3.1, contained in the response shall be included as the textual representations "<CR>" and "<LF>" respectively. |  |

The following MMI commands are defined.

Table 5.1: MMI commands

| Command | Parameters |  |
| --- | --- | --- |
|  | Name | Value |
| "SWITCH_ON" | (none) |  |
| "SWITCH_OFF" | (none) |  |
| "POWER_ON" | (none) |  |
| "POWER_OFF" | (none) |  |
| "INSERT USIM" | "USIM" | <USIM> |
| "REMOVE_USIM" | "USIM" (optional) | <USIM> |
| "CHECK_PLMN" | "PLMN" | <PLMN ID> |
| "CHECK_ETWS_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| “CHECK_ETWS_ALERT” | (none) |  |
| “CHECK_ETWS_NO_ALERT” | (none) |  |
| "CHECK_CMAS_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| “CHECK_CMAS_ALERT” | (none) |  |
| “CHECK_CMAS_NO_ALERT” | (none) |  |
| "HRPD_PDN_CONNECTION” | (none) |  |
| “CHECK_SMS_LENGTH_CONTENTS" | "Length" | <Length> |
|  | "Msg" | <Msg> |
| "CONFIGURE_SMS_ONLY" | (none) |  |
| "DISABLE EPS CAPABILITY" | (none) |  |
| "DETACH_NON_EPS" | (none) |  |
| "CLEAR_STORED_ASSISTANCE_DATA" | (none) |  |
| "CHECK_DTCH_THROUGHCONNECTED" | (none) |  |
| "GERAN_UPLINK_DATA" | (none) |  |
| "SELECT_CSG" | "PLMN" | <PLMN ID> |
|  | "CSG" | <CSG ID> |
| “TRIGGER_USER_RESELECTION” | (none) |  |
| "REQUEST_NON_CALL_RELATED_SS" | (none) |  |
| "MBMS_SERVICE_INTEREST" | "PLMN" | <PLMN ID> |
|  | "Service" | <MBMS Service ID> |
|  | "Interest" | "ON" / "OFF" |
|  | "SAI" | <MBMS SAI> |
| “MBMS_SERVICE_ACTIVE” | "Service" | <MBMS Service ID> |
|  | "SAI" | <MBMS SAI> |
|  | “Active” | "ON" / "OFF" |
| “MBMS_PRIORITY_OVER_UNICAST” | "Priority Over Unicast" | "FALSE" / "TRUE" |
| "CHECK_ETWS_NO_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| "CHECK_CMAS_NO_INDICATION" | “WARNING1” | <WARNING1> |
|  | “WARNING2” | <WARNING2> |
| "CHECK_CALL_DISPLAY" | "ToDisplay" | <NAME>/<NUMBER> |
| "CHECK_CSG" | "CSG" | < CSG ID > |
|  | "Included" | "FALSE" / "TRUE" |
| "CHECK_NITZ_DST" | "DaylightSavingTime:" | DST |
| "CHECK_RESPONSE_DISPLAY" | "ToDisplay" | <STRING> |
| "CHECK_SS_USER_IND" | "Supplementary Service" | <SUPPLEMENTARY SERVICE> |
|  | "ToDisplay" | <STRING> |
| “CHECK_SS_USER_IND_FAILURE” | (none) |  |
| "CONFIGURE_OPERATIONMODE" | "OPERATION MODE" | <OPERATION MODE> |
| “CONFIGURE_ANDSF_RULE” | “ANDSF Rule” | <ANDSF RULE> |
| “WLAN_ANDSF_CONFIGURED” | (none) |  |
| “WLAN_ANDSF_NOT_CONFIGURED” | (none) |  |
| "CMTLR_DUMMY" (see Note) | ”Notification Type” | <NOTIFICATION TYPE> |
| "CCFC_DUMMY" (see Note) | ”Status” | <STATUS> |
| "CNAP_DUMMY" (see Note) | ”Testname” | <TESTNAME> |
| "CTZE_DUMMY" (see Note) | "Year | <YEAR> |
|  | "Month" | <MONTH> |
|  | "Day" | <DAY> |
|  | "Hour" | <HOUR> |
|  | "Minutes" | <MINUTES> |
|  | "Seconds" | <SECONDS> |
|  | “Daylight SavingTime” | <DAYLIGHT SAVING TIME> |
|  | "TimeZone" | <TIMEZONE> |
| "CMWN_DUMMY" (see Note) | (none) |  |
| “D2D_DISCOVERY_CLEAR” | (none) |  |
| “D2D_DISCOVERY_CONFIGURATION” | “ProSeApplicationID” | <charstring> |
|  | "PLMN_List" | <PLMN ID_List> |
|  | T4005 | <T4005> |
| “D2D_DISCOVERY” | “ProSeApplicationID” | <charstring> |
|  | “Announce” | "ON" / "OFF" |
|  | “Monitor” | "ON" / "OFF" |
| “D2D_COMMUNICATION” | “ProSe Layer-2 Group ID” | <charstring> |
|  | “Transmit” | "ON" / "OFF" |
|  | “Receive” | "ON" / "OFF" |
|  | “Max size of UE transmitted Direct Communication data” | <bytes> |
| “D2D_COMMUNICATION_CONFIGURATION” | “ProSe Layer-2 Group ID” | <charstring> |
|  | "PLMN_List" | <PLMN ID_List> |
|  | T4005_Minutes | <T4005> |
|  | “Security Life time timer in minutes” (Default: 0) | <SecurityTimer> |
| "V2X_SIDELINK" | "Data Type" | "IP" / "NON-IP" |
| “INSERT_USIM_OR_PRECONFIGURE” | "USIM" | <USIM> |
| “DELETE_NTN_PLMN_NA” | (none) |  |
| “CONFIGURE_NTN_UE_POSITION” | "SpecReference" | <charstring> |
| Note: These MMI commands are used to indicate to the SS that a UT response is expected by the TTCN in the case when the AT command is not supported. They can be ignored if the UE supports the AT command. The prefix of these commands matches the start of the AT response expected by the TTCN |  |  |

The following AT commands are applied in TTCN.

Table 5.2: AT Commands

| Command | Reference |
| --- | --- |
| ATD | TS 27.007 [32] |
| ATA | TS 27.007 [32] |
| ATH | TS 27.007 [32] |
| AT+CGEQOS | TS 27.007 [32] |
| AT+CGTFT | TS 27.007 [32] |
| AT+CGDSCONT | TS 27.007 [32] |
| AT+CGACT | TS 27.007 [32] |
| AT+CGCMOD | TS 27.007 [32] |
| AT+CGDCONT | TS 27.007 [32] |
| AT+CMGD | TS 27.005 [31] |
| AT+CSMS | TS 27.005 [31] |
| AT+CPMS | TS 27.005 [31] |
| AT+CMGF | TS 27.005 [31] |
| AT+CSCS | TS 27.007 [32] |
| AT+CSCA | TS 27.005 [31] |
| AT+CMGW | TS 27.005 [31] |
| AT+CMSS | TS 27.005 [31] |
| AT+CMMS | TS 27.005 [31] |
| AT+CSMP | TS 27.005 [31] |
| AT+CGEQREQ | TS 27.007 [32] |
| AT+CCLK | TS 27.007 [32] |
| AT+COPS | TS 27.007 [32] |
| AT+CGATT | TS 27.007 [32] |
| AT+CEMODE | TS 27.007 [32] |
| AT+CEN | TS 27.007 [32] |
| AT+CLIP | TS 27.007 [32] |
| AT+CLIR | TS 27.007 [32] |
| AT+COLP | TS 27.007 [32] |
| AT+CCFCU | TS 27.007 [32] |
| AT+CCFC | TS 27.007 [32] |
| AT+CHLD | TS 27.007 [32] |
| AT+CEPPI | TS 27.007 [32] |
| AT+CDU | TS 27.007 [32] |
| AT+CHCCS | TS 27.007 [32] |
| AT+COLR | TS 27.007 [32] |
| AT+CCWA | TS 27.007 [32] |
| AT+CNAP | TS 27.007 [32] |
| AT+CLCK | TS 27.007 [32] |
| AT+CPWD | TS 27.007 [32] |
| AT+CTZR | TS 27.007 [32] |
| AT+CTZU | TS 27.007 [32] |
| AT+CUSD | TS 27.007 [32] |
| AT+CMWI | TS 27.007 [32] |
| AT+CPSMS | TS 27.007 [32] |
| AT+CECALL | TS 27.007 [32] |
| AT+CEDRXS | TS 27.007 [32] |
| AT+CSODCP | TS 27.007 [32] |
| AT+CATM | TS 27.007 [32] |
| AT+CCUTLE | TS 27.007 [32] |
| AT+CUSPCREQ | TS 27.007 [32] |
| AT+CUTCR | TS 27.007 [32] |
| AT+CCBRREQ | TS 27.007 [32] |
| AT+CV2XDTS | TS 27.007 [32] |
| AT+CSPSAIR | TS 27.007 [32] |
| AT+CAPPLEVMR | TS 27.007 [32] |
| AT+CC2APT | TS 27.007 [32] |
| AT+CUUAAPT | TS 27.007 [32] |

AT commands are referred to TS 27.005 [31], TS 27.007 [32] and TS 27.060 [33].

The setting in TTCN of the AT command elements listed in Table 5-3 may require adaptation to the UE dependent interpretation of the element. It shall be handled by the system adaptor.

Table 5-3: AT commands elements

| Element | Reference | TTCN Setting |
| --- | --- | --- |
| <cid> | TS 27.007 [32] | EPS Bearer Identity or PDP Context Identity |