3GPP TSG-RAN5 Meeting #103	R5-242314

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4354 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to 5G V2X test case 12.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table 12.1.7.1.3.3-2 still have some contents that need to be cleaned up. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates in Table 12.1.7.1.3.3-2 removing fields not checked within supportedBandCombinationListSidelinkNR-r16 and removed value n48 of harq-RxProcessSidelink-r16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the values specified will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.7.1	PC5-only operation / Sidelink UE capability transfer via PC5 RRC / One-way and two-way transfer

12.1.7.1.1	Test Purpose (TP)

(1)

with { UE having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE receives a UECapabilityEnquirySidelink message from peer UE }

then { UE sends a UECapabilityInformationSidelink message to peer UE }

}

(2)

with { UE having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE is configured by upper layer to initiate capability transfer procedure }

then { UE sends a UECapabilityEnquirySidelink message to peer UE }

}

(3)

with { UE having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE is configured by upper layer to initiate capability transfer procedure and to provide UE radio access capabilities }

then { UE sends a UECapabilityEnquirySidelink message with ue-CapabilityInformationSidelink to peer UE }

}

12.1.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.9.2.1, 5.8.9.2.2, 5.8.9.2.3 and 5.8.9.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.9.2.1]

This clause describes how the UE compiles and transfers its sidelink UE capability information for unicast to the initiating UE.

Figure 5.8.9.2.1-1: Sidelink UE capability transfer

[TS 38.331, subclause 5.8.9.2.2]

The UE may initiate the sidelink UE capability transfer procedure upon indication from upper layer when it needs (additional) UE radio access capability information.

[TS 38.331, subclause 5.8.9.2.3]

The initiating UE shall set the contents of UECapabilityEnquirySidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink, if needed;

NOTE 1:	It is up to initiating UE to decide whether ue-CapabilityInformationSidelink should be included.

1>	set frequencyBandListFilterSidelink to include frequency bands for which the peer UE is requested to provide supported bands and band combinations;

NOTE 2:	The initiating UE is not allowed to send the UECapabilityEnquirySidelink message without including the field frequencyBandListFilterSidelink.

1>	submit the UECapabilityEnquirySidelink message to lower layers for transmission.

[TS 38.331, subclause 5.8.9.2.4]

The peer UE shall set the contents of UECapabilityInformationSidelink message as follows:

1>	include UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink;

1>	compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilterSidelink, and prioritized in the order of frequencyBandListFilterSidelink (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on).

1>	include into supportedBandCombinationListSidelinkNR as many band combinations as possible from the list of "candidate band combinations", starting from the first entry;

1>	include the received frequencyBandListFilterSidelink in the field appliedFreqBandListFilter of the requested UE capability;

1>	submit the UECapabilityInformationSidelink message to lower layers for transmission.

NOTE:	If the UE cannot include all band combinations due to message size or list size constraints, it is up to UE implementation which band combinations it prioritizes.

12.1.7.1.3	Test description

12.1.7.1.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	Operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS UE uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- UE is equipped with USIM containing default value (as per TS 38.508-1[4] clause 4.8.3.3.3).

- UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.1.7.1.3.2	Test procedure sequence

Table 12.1.7.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The NR-SS-UE sends a UECapabilityEnquirySidelink message from NR-SS-UE on SL-SRB3. | <-- | PC5 RRC: UECapabilityEnquirySidelink | - | - |
| 2 | Check: Does the UE send a UECapabilityInformationSidelink message? | --> | PC5 RRC: UECapabilityInformationSidelink | 1 | P |
| 3 | The UE is configured by upper layers to initiate capability transfer procedure and to include only NR Sidelink operating band which is currently used in this TC for communication over PC5 interface. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| 4 | Check: Does the UE send on SL-SRB3 a UECapabilityEnquirySidelink message? | --> | PC5 RRC: UECapabilityEnquirySidelink | 2 | P |
| 5 | The NR-SS-UE sends a UECapbilityInformationSidelink message. | <-- | PC5 RRC: UECapabilityInformationSidelink | - | - |
| 6 | The UE is configured by upper layers to initiate capability transfer procedure, to include only the NR Sidelink operating band which is currently used in this TC for communication over PC5 interface and to provide UE radio access capabilities. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| 7 | Check: Does the UE send on SL-SRB3 a UECapabilityEnquirySidelink message with ue-CapabilityInformationSidelink-r16 IE? | --> | PC5 RRC: UECapabilityEnquirySidelink | 3 | P |
| 8 | The NR-SS-UE sends a UECapabilityInformationSidelink message. | <-- | PC5 RRC: UECapabilityInformationSidelink | - | - |

12.1.7.1.3.3	Specific message contents

Table 12.1.7.1.3.3-1: UECapabilityEnquirySidelink (step 1, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-6 with condition RX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |
| frequencyBandListFilterSidelink-r16 SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { |  | Includes only the single frequency band and band combination NR Sidelink operating band which is currently used in this TC for communication over the PC5 interface |  |
| FreqBandInformation[1] CHOICE { |  |  |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR of the PC5 operating band |  | px_NR_SidelinkBand |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.1.3.3-2: UECapabilityInformationSidelink (step 2, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-7 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformationSidelink ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier-r16 | Set to the same value as the rrc-TransactionIdentifier-r16 field in UECapabilityEnquirySidelink message in step 1 |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformationSidelink-r16 SEQUENCE { |  |  |  |
| pdcp-ParametersSidelink-r16 SEQUENCE { |  |  |  |
| outOfOrderDeliverySidelink-r16 | Checked |  | pc_outOfOrderDeliverySidelink_r16 |
| } |  |  |  |
| rlc-ParametersSidelink-r16 SEQUENCE { |  |  |  |
| am-WithLongSN-Sidelink-r16 | Checked |  | pc_amWithLongSN_Sidelink_r16 |
| um-WithLongSN-Sidelink-r16 | Checked |  | pc_umWithLongSN_Sidelink_r16 |
| } |  |  |  |
| supportedBandCombinationListSidelinkNR-r16 | Not Checked |  |  |
| supportedBandListSidelink-r16 SEQUENCE (SIZE (1..maxBands)) OF BandSidelinkPC5-r16 { | At least 1 entry | Includes all frequency bands which the UE (= UE Under Test) supports for NR Sidelink acc. to the declared UE capabilities. Values defined in TS 38.523-2 Table A.4.3.1-9 |  |
| BandSidelinkPC5-r16[x] SEQUENCE { |  |  |  |
| freqBandSidelink-r16 | FreqBandIndicatorNR of frequency band which the UE supports for NR Sidelink acc. to the declared UE capabilities |  |  |
| sl-Reception-r16 SEQUENCE { |  |  |  |
| harq-RxProcessSidelink-r16 | Checked |  | pc_harq_RxProcessSidelink_nX (X=16, 24, 32, 64) |
| pscch-RxSidelink-r16 | Not checked |  |  |
| scs-CP-PatternRxSidelink-r16 | Not checked |  |  |
| extendedCP-RxSidelink-r16 | Not checked |  |  |
| } |  |  |  |
| sl-Tx-256QAM-r16 | Not checked |  |  |
| lowSE-64QAM-MCS-TableSidelink-r16 | Not checked |  |  |
| csi-ReportSidelink-r16 SEQUENCE { |  |  |  |
| csi-RS-PortsSidelink-r16 | Not checked |  |  |
| } |  |  |  |
| rankTwoReception-r16 | Not checked |  |  |
| sl-openLoopPC-RSRP-ReportSidelink-r16 | Not checked |  |  |
| sl-Rx-256QAM-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| appliedFreqBandListFilter-r16 SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.1.3.3-3: UECapabilityEnquirySidelink (step 4, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-6 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |
| frequencyBandListFilterSidelink-r16 SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | 1 entry | Includes only the NR Sidelink operating band which is currently used in this TC |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR of the PC5 operating band |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.1.3.3-4: UECapabilityInformationSidelink (steps 5 and 8, Table 12.1.7.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-7 with condition RX

Table 12.1.7.1.3.3-5: UECapabilityEnquirySidelink (step 7, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-6 with condition TX and TWO_WAY_ENQUIRY |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |
| frequencyBandListFilterSidelink-r16 SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | 1 entry | Includes only the NR Sidelink operating band which is currently used in this TC |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR of the PC5 operating band |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-CapabilityInformationSidelink-r16 | Checked, same content as in Table 12.1.7.1.3.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |